# 3GPP TSG-RAN WG4 Meeting #115	R4-2506530

# Malta, 19th – 23rd May, 2025

Title:	Views on general aspects for SBFD

Source:	Qualcomm Incorporated

Agenda item:	7.25.2

Document for:	Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. From RAN4 RF perspective, the objective of the work item is as follows:

The objectives are as follows: - Specify BS RF requirements for SBFD operation at gNB [RAN4]

In RAN4#114bis, a WF was agreed in [2] entailing several aspects regarding SBFD general system design and deployment. In this contribution we share our views on the remaining open items that were considered in [2].

# General aspects

## Declaration and flexibility for SBFD operation

In [2], the following was agreed regarding MR SBFD-capable BS

- For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested. - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing. - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains.

The rationale behind setting a TRP or total conducted power for medium range BS is to account for the implementation dependent solutions at the MR BS adopted to address self-interference and co-channel inter-subband interference that are arising when the BS is operating in an SBFD symbol/ slot.

To recall from the study item phase, TR 38.858 Section 9.3.4 summarizes the conclusion of the feasibility of MR BS as follows:

Based on RAN4 feasibility study on FR1 medium range BS, specifically the analysis on self-interference, co-site inter-sector co-channel inter-subband interference and inter-site inter-subband interference, RAN4 concluded that: - For self-interference analysis, the implementation feasibility of controlling the residual interference to meet the 1dB receiver desensitization target depends on the implementation aspects mentioned in clause 9.3.1. Based on the different assumptions and/or technique adoption for the above-mentioned implementation aspects, based on 5 companies’ technical inputs, companies have come to the following conclusions: - 1dB receiver desensitization target is achievable according to 4 companies. - 1dB receiver desensitization target is not achievable for a base station that strictly meets the RAN4 minimum requirements and/or operates with the maximum BS transmit power according to 2 companies. - Note: Among 5 companies, 1 company shows different feasibility results based on different assumptions.

Since converging views were reached during the study item and to avoid bias towards a specific implementation, RAN4 could agree on the power threshold criteria that was proposed in [2]. Additionally, Table 1 below represents the adopted BS output power for MR BSs during the study item (see Table 9.3.1.1-1 in TR 38.858 for self-interference analysis, note that similar values were considered for co-channel inter-subband interference analysis), where we can observe that the tentatively proposed 33 dBm for MR SBFD-capable BS is not representing actual numbers that were considered during the feasibility study. Thus, to be aligned with envisioned deployments of MR BSs in the field, RAN4 should not adopt a limiting value for X dBm to ensure proper coverage resulting from MR SBFD-capable BS deployment. Given that in TS 38.104, the TRP the MR BS rated carrier TRP output power limit is set to be at most 47 dBm, the tentatively proposed 33 dBm will be too limiting. In this regard, it would be a viable way forward to consider a compromise between maximum TRP values given in TS 38.104 and representative value based on feasibility analysis in TR 38.858.

Table 1 MR BS Output power considered during Rel-18 SBFD SI

| FR1 | Nokia | Ericsson |  |  |  | ZTE | Huawei | Samsung |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS class | Medium Range BS | Medium Range (3GPP minimum requirements) | Medium range (Realistic) | Medium Range (Optimistic RX) | Medium Range (Realistic, lower power) | MR | Medium Range BS | Medium Range BS |
| BS TX Power dBPTX = ① dBm | 36 dBm | 38 dBm | 38 dBm | 38 dBm | 35 dBm | 30 | 38 dBm | 38 dBm |

Proposal 1: RAN4 to consider the MR SBFD-capable power threshold value (X dBm) as a compromise between MR maximum BS rated carrier TRP output power limit given in Table 9.3.1-1, TS 38.104 (i.e., 43 dBm) and representative values adopted in Section 9.3, TR 38.858.

## DL/UL subband configurations support for SBFD

Since in [2] several agreements were made on the DL/UL subband configurations support, it is viable to summarize the latest agreements to ensure clarity of the configurations that were agreed during Rel-19 SBFD work item. In Table 2, we summarize our understanding of the latest agreements for FR1 covering the three different BS classes. Note that for FR2-1, no restrictions are expected for any of the defined BS classes. In this regard, we would like to confirm that Table 1 constitutes the final agreements on the DL/UL subband configuration restrictions for Rel-19 SBFD capable BS.

Table 2 Restrictions on DL/UL/GB for SBFD capable BS

| Restrictions for either DU, UD, or DUD case | Wide area SBFD-capable BS | Medium range SBFD-capable BS | Local area SBFD-capable BS |
| --- | --- | --- | --- |
| Restriction on number of DL subband sizes | Only one DL subband with a specific size is allowed to be declared and tested | Power level threshold criteria whereas above this threshold, FR1 WA restriction applies; Below this threshold, no restriction | No restriction |
| Restriction on number of UL subband sizes | Maximum 3 subband sizes allowed, where each needs to follow FR1 specified channel bandwidth | Power level threshold criteria whereas above this threshold, FR1 WA restriction applies; Below this threshold, no restriction | No restriction |
| Restriction on guardband size | No restriction |  |  |

Proposal 2: RAN4 to confirm Table 2 in this document regarding Rel-19 SBFD capable DL/UL subband configurations.

## Restriction on FR2-1 CHBW support for SBFD DUD case

In [2], the following was agreed regarding FR2-1 CHBW restriction:

- If the SSB symbol is not overlapped with SBFD symbol, - 100MHz for DUD case is allowed for FR2-1. - If the SSB symbol is overlapped with SBFD symbol, - In Rel-19, the BS CHBW for DUD configuration shall be no less than 200MHz for FR2-1.

Recalling that for FR2-1 the sync raster step size is 17.28 MHz while the SSB bandwidth for SCS=240 KHz is 28.8 MHz. In the case that SSBs are allowed to be transmitted in SBFD slots, for DU/UD SBFD configuration one DL subband bandwidth would need to be larger than 46.08 MHz. Thus, with the agreement that the minimum BS channel bandwidth for SBFD operation is 100MHz, no issues are observed. For the DUD configuration, SSBs has to be sent contiguously within only a single DL subband, which has to be larger than 46.08 MHz. From RAN1 perspective, the following was agreed in RAN1#118bis:

RAN1 Working Assumption(#118bis) For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

In our views, the notion of having SSB symbols overlapped with SBFD symbols is up to network implementation, since from RAN1 perspective, SBFD symbols containing the SSB symbols are still considered as SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD-aware UEs. Thus, from RAN4 perspective, no additional restrictions on the channel bandwidth for FR2-1 should be imposed.

Proposal 3: For FR2-1 bands, restriction on channel bandwidth for DUD configuration should be handled by the network, given that RAN1 has agreed that 1) SSB symbols configured with SBFD subbands are SBFD symbols, 2) only DL receptions within DL usable PRBs are allowed for SBFD aware UEs, and 3) SSB block is assumed to be within DL subband.

## Update for HPUE duty cycle evaluation

In [2], the following was discussed:

- RAN4 has discussed the two following points: - For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission; - For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission. - FFS RAN4 specification impact by taking into account the above understanding for SBFD-aware UE.

Recall, as captured in [1], that SBFD can be configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon. Additionally, it is important to note that for the legacy duty cycle evaluation, the calculation is based on the legacy uplink symbols that are transmitted by the UE, not on the symbol assignment only. Thus, in the case of flexible symbol(s) assigned by the network to UL transmissions for a given UE, UE would need to account for such symbol(s) as part of the duty cycle evaluation.

Observation 1: The HPUE duty cycle evaluation is based on UL transmissions carried by the UE and not only on the symbol assignment by the network.

In this regard, comparing this operation to SBFD, the same should apply since UL transmissions carried out by the UE, whether they are based on flexible symbol(s) or SBFD symbol(s), need to be accounted for as part of the duty cycle evaluation framework. In our view, the duty cycle evaluation considers UL transmissions by a UE (SBFD aware or not) independent if such UL transmissions were based on UL symbols, flexible symbols assigned to UL by the network, or UL subband of an SBFD symbol.

Observation 2: Duty cycle evaluation considers UL transmissions by a (SBFD-aware or non SBFD-aware) UE independent if such UL transmissions were based on UL symbols, flexible symbols assigned to UL by the network, or UL subband of an SBFD symbol.

Proposal 4: No additional considerations for SBFD-aware UE is expected to the HPUE duty cycle evaluation given in TS 38.101-1.

Moving to the working procedures, it is important to note that in [1], UE core spec (i.e., TS 38.101-1) is not listed. Accordingly, any modifications to TS 38.101-1 would require a change in the WID first.

Observation 3: Any modifications to TS 38.101-1 would require an update of the Rel-19 SBFD WI to include it in the list of impacted existing TSs.

# Conclusion

In this contribution, we provided our views on general aspects for SBFD operation with the following observations and proposals:

Proposal 1: RAN4 to consider the MR SBFD-capable power threshold value (X dBm) as a compromise between MR maximum BS rated carrier TRP output power limit given in Table 9.3.1-1, TS 38.104 (i.e., 43 dBm) and representative values adopted in Section 9.3, TR 38.858.

Proposal 2: RAN4 to confirm Table 2 in this document regarding Rel-19 SBFD capable DL/UL subband configurations.

Proposal 3: For FR2-1 bands, restriction on channel bandwidth for DUD configuration should be handled by the network, given that RAN1 has agreed that 1) SSB symbols configured with SBFD subbands are SBFD symbols, 2) only DL receptions within DL usable PRBs are allowed for SBFD aware UEs, and 3) SSB block is assumed to be within DL subband.

Observation 1: The HPUE duty cycle evaluation is based on UL transmissions carried by the UE and not only on the symbol assignment by the network.

Observation 2: Duty cycle evaluation considers UL transmissions by a (SBFD-aware or non SBFD-aware) UE independent if such UL transmissions were based on UL symbols, flexible symbols assigned to UL by the network, or UL subband of an SBFD symbol.

Proposal 4: No additional considerations for SBFD-aware UE is expected to the HPUE duty cycle evaluation given in TS 38.101-1.

Observation 3: Any modifications to TS 38.101-1 would require an update of the Rel-19 SBFD WI to include it in the list of impacted existing TSs.

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

- R4-2504751, “Way Forward for [114bis][306] NR_duplex_evo_General”, RAN4#114bis, Samsung.