# 3GPP TSG-RAN WG4 Meeting #115		R4-2506533

# Malta, 19th – 23rd May, 2025

Title:	Views on BS demodulation requirements for MMSE-IRC

Source:	Qualcomm Incorporated

Agenda item:	7.17.4

Document for:	Approval

# 1.	Introduction

During RAN#103 plenary a new study item on BS advanced receiver performance was agreed in [1]. One of the objectives of the WI is as follows:

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In [2], a WF was agreed regarding simulation parameters, scenarios and that are the basis for the demodulation requirements to be defined in this work item. In this contribution, we provide our views on the open items captured in [2].

# 2.1	Test parameters and simulation assumptions

## 2.1.1	Propagation conditions

In [2], the following was agreed:

- TDLA30-10 for HetNet and TDLC300-100 for HomNet

It is important to clarify and finalize the assumptions for the serving and interference propagation channel. In [3], companies have provided results for different propagation channel assumptions for the serving cell, while the interfering cell is fixed to be set to TDLC300-100. Based on our understanding, we would like to confirm that the BS MMSE-IRC Rel-19 requirements will be based on the following assumptions:

  - Propagation condition for serving and interfering cells is TDLA30-10 for HetNet and TDLC300-100 for HomNet

Proposal 1: RAN4 to confirm that the BS MMSE-IRC Rel-19 requirements will be based on TDLA30-10 for HetNet and TDLC300-100 for HomNet for both the serving and interfering cells.

## 2.1.2 	DMRS configuration - Ratio of PUSCH EPRE to DM-RS EPRE

Recall that for legacy Rel-15 PUSCH requirements, we have the below test parameters that have been utilized when BS demodulation requirements are developed, where it is observed that 1) the number of DMRS CDM groups without data is always set to 2 and that 3 dB power boosting is assumed between the DMRS REs and the PUSCH data Res. An offline discussion has started after RAN4#114bis among interested companies within RAN4 to confirm the above.

Our view is that RAN4 may utilize the same assumption and although 3dB mismatch between data and estimated interference might occur when estimating the Covariance matrix Rnn, it is left up to implementation how to address that mismatch, if found impactful on the performance of the MMSE-IRC receiver.

Proposal 2: RAN4 to agree that the ratio of PUSCH EPRE to DM-RS EPRE is -3dB with Number of DM-RS CDM group(s) without data = 2.

Proposal 3: RAN4 to discuss if it will be captured in the spec for the Rel-19 MMSE IRC requirements the assumption of power mismatch between data and estimated interference.

# 2.2	Applicability rule for different test scenarios

In [2], below options were agreed:

- Follow the LTE approach – Only specifies that HomNet requirements are not applicable for Local Area BS. - FFS whether to consider additional applicability rule on how the test is performed: - Option 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped - Option 2: Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes.

In our views, additional applicability rules on how the test is performed are not needed as such additional applicability rules were not considered for LTE. Accordingly, we support option 2 in [2].

Proposal 3: RAN4 to agree not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes.

# Conclusion

In this contribution we provide our views on the remaining issues of BS demodulation aspects for MMSE-IRC receiver. Our observations and proposals can be summarized for that frequency range as follows:

Proposal 1: RAN4 to confirm that the BS MMSE-IRC Rel-19 requirements will be based on TDLA30-10 for HetNet and TDLC300-100 for HomNet for both the serving and interfering cells.

Proposal 2: RAN4 to agree that the ratio of PUSCH EPRE to DM-RS EPRE is -3dB with Number of DM-RS CDM group(s) without data = 2.

Proposal 3: RAN4 to discuss if it will be captured in the spec for the Rel-19 MMSE IRC requirements the assumption of power mismatch between data and estimated interference.

Proposal 4: RAN4 to agree not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes.

# Reference

[1]		RP-240831, “New WID: WI resulting from discussion on Demodulation performance requirements enhancements”, Vivo, RAN#103, Maastricht, Netherlands.

[2] 	R4-2504768, “Way Forward for [114bis][321] NR_demod_Ph5_Part1_General_BS”, RAN4#114bis, China Telecom.

[3]	R4-2504533, “Simulation results summary for BS requirement with MMSE-IRC receiver” RAN4#114bis, Samsung.