3GPP TSG RAN WG4 Meeting #115	R4-2506554

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.31.6

Source:	vivo

Title:	Discussion on RRM performance requirements of NTN for NR Phase 3

Document for:	Discussion

- Introduction

According to the working plan, the discussion on the performance part of NR NTN will begin in this meeting. However, in this document, we will provide our preliminary analysis on the following aspects: performance requirements and RRM test case design for NTN in NR Phase 3.

- Discussion

In the discussion of the RRM core part, the definition of RRM requirements for NTN for NR Phase 3 mainly includes two parts:

1. For (e)Redcap UEs with FR1-NTN

2. For downlink coverage enhancements

First, regarding (e)Redcap UEs with FR1-NTN, the definition of performance requirements primarily focuses on measurement accuracy requirements. The existing RSRP accuracy requirements for FR1 for Redcap can serve as a baseline.

Proposal 1: For the performance requirement in NTN for NR Phase 3, measurement accuracy requirements need to be defined for (e)Redcap UEs with FR1-NTN

As for the definition of test cases, the following aspects already have test cases defined in existing Redcap or NTN scenarios. We believe these can serve as a starting point for discussing the test case list for (e)Redcap UEs with FR1-NTN. For the specific design of these test cases, the existing test cases related to Redcap and NTN test cases can be used as references.

- Cell Re-selection for RedCap UEs with NTN in RRC_IDLE state mobility - Configured Grant based Small Data Transmissions (CG-SDT) for RedCap UEs with NTN - Handover for RedCap UEs with NTN - RRC Connection Mobility Control for RedCap UEs with NTN - UE transmit timing for RedCap UEs with NTN - UE timer accuracy for RedCap UEs with NTN - Timing advance for RedCap UEs with NTN - Radio Link Monitoring for RedCap UEs with NTN - Link Recovery Procedures for RedCap UEs with NTN - Active BWP switch delay for RedCap UEs with NTN - Active TCI state switching delay for RedCap UEs with NTN - UE-specific CBW change for RedCap UEs with NTN - Pathloss reference signal switching delay for RedCap UEs with NTN - NR intra-frequency measurements for RedCap UEs with NTN - NR inter-frequency measurements for RedCap UEs with NTN - L1-RSRP measurements for Reporting for RedCap UEs with NTN

For downlink coverage enhancements, the discussion on the core part primarily focused on analyzing the potential impact of SSB period extension on RRM requirements. Apart from agreeing to define a new requirement for the case of SSB Ês/Iot < -8dB and an unknown NR cell in the following RRC re-establishment requirement, no additional RRM requirements were defined.

Specifically, for the re-establishment requirement in the case of SSB Ês/Iot < -8dB and an unknown NR cell, if the SMTC is 160ms, the corresponding delay of [22] x TSMTC would be quite long (e.g., Tidentify_intra_NR + Tidentify_inter_NR = 7040ms, considering Nfreq = 2). In such cases, the re-establishment timer may have already expired if the timer duration is configured to be relatively short.

We believe that in real-world scenarios, it is not very typical for Ês/Iot to degrade below -8dB and require nearly 10 seconds to search for a cell for re-establishment. Therefore, we are inclined not to define related test cases for this scenario.

## Issue 1-2-B: RRC Re-establishment (SSB Ês/Iot < -8dB and SSB periodicity >20 ms) Agreement (online): - In RRC re-establishment requirements, for the case SSB Ês/Iot < -8dB and unknown NR cell, - Make the following updates - Remove Note 1 in Table 6.2C.1.2.1-1 and Table 6.2C.1.2.1-2 for the applicable scenarios, and the value of TSMTC and TSMTC,i is FFS - Tidentify_intra_NR: MAX (800 ms, [22] x TSMTC) for intra-frequency carrier - Tidentify_inter_NR,i: MAX (800 ms, [22] x TSMTC,i) for inter-frequency carrier

Proposal 2: For NTN for NR Phase 3, RAN4 to define RRM test cases for the following requirements:

- Cell Re-selection for RedCap UEs with NTN in RRC_IDLE state mobility

- Configured Grant based Small Data Transmissions (CG-SDT) for RedCap UEs with NTN

- Handover for RedCap UEs with NTN

- RRC Connection Mobility Control for RedCap UEs with NTN

- UE transmit timing for RedCap UEs with NTN

- UE timer accuracy for RedCap UEs with NTN

- Timing advance for RedCap UEs with NTN

- Radio Link Monitoring for RedCap UEs with NTN

- Link Recovery Procedures for RedCap UEs with NTN

- Active BWP switch delay for RedCap UEs with NTN

- Active TCI state switching delay for RedCap UEs with NTN

- UE-specific CBW change for RedCap UEs with NTN

- Pathloss reference signal switching delay for RedCap UEs with NTN

- NR intra-frequency measurements for RedCap UEs with NTN

- NR inter-frequency measurements for RedCap UEs with NTN

- L1-RSRP measurements for Reporting for RedCap UEs with NTN

- Summary

Proposal 1: For the performance requirement in NTN for NR Phase 3, measurement accuracy requirements need to be defined for (e)Redcap UEs with FR1-NTN

Proposal 2: For NTN for NR Phase 3, RAN4 to define RRM test cases for the following requirements:

- Cell Re-selection for RedCap UEs with NTN in RRC_IDLE state mobility

- Configured Grant based Small Data Transmissions (CG-SDT) for RedCap UEs with NTN

- Handover for RedCap UEs with NTN

- RRC Connection Mobility Control for RedCap UEs with NTN

- UE transmit timing for RedCap UEs with NTN

- UE timer accuracy for RedCap UEs with NTN

- Timing advance for RedCap UEs with NTN

- Radio Link Monitoring for RedCap UEs with NTN

- Link Recovery Procedures for RedCap UEs with NTN

- Active BWP switch delay for RedCap UEs with NTN

- Active TCI state switching delay for RedCap UEs with NTN

- UE-specific CBW change for RedCap UEs with NTN

- Pathloss reference signal switching delay for RedCap UEs with NTN

- NR intra-frequency measurements for RedCap UEs with NTN

- NR inter-frequency measurements for RedCap UEs with NTN

- L1-RSRP measurements for Reporting for RedCap UEs with NTN

- References