3GPP TSG-RAN WG4 Meeting # 115                                R4-2506564

# Malta, MT, 19th – 23rd May, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on General Aspects of AIML Mobility

Agenda Item:	7.23.2

Document for:	Approval

- Introduction

In RAN plenary #105 meeting, a revised SID[1] for Rel-19 was approved to study the AI/ML for mobility in NR. The objectives for RAN4 of this SID are listed as follows.

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation - NOTE 1: RAN1/3 work can be triggered via LS NOTE 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 3: Two-sided model is not included

In this document, we will provide some views on requirements impacts based on RAN2 assumptions of AI/ML for mobility from the RRM perspective.

- Discussion

In previous meeting, we discussed the priorities of different use cases and have reached consensus that RAN4 to start the discussion with RRM measurement prediction use-case [2]. For measurement event prediction use-case and RLF use-case, more progress is needed from RAN2, and RAN4 to only start the discussion of Measurement event prediction use-case and RLF use-case if sufficient progress in RAN2.

RAN4#112bis: Priorities of different use cases Agreement: - RAN4 to start the discussion with RRM measurement prediction use-case from RAN4 #112bis meeting. Agreement: - RAN4 to only start the discussion of Measurement event prediction use-case and RLF use-case if sufficient progress in RAN2. - The earliest time to start the discussion for Measurement event prediction use-case is RAN4#114 meeting (Feb 2025). - The earliest time to start the discussion for RLF use-case is from RAN4#114bis meeting (Apr 2025).

According to the discussion on RAN2, they have reached consensus that RLF prediction would not be studied in R19 due to there is no enough time about RLF. In RAN4#112bis, we agreed that RAN4 to only start the discussion of RLF use-case if sufficient progress in RAN2. Therefore, RAN4 should align with RAN2 and RLF prediction will not be considered in Rel-19.

Agreements on priority - RLF prediction will not be studied in Rel-19 - Further RRM prediction simulations are not expected. For next specification general impact study phase we will consider spatial, temporal, and frenquency domain cell level prediction. FFS if L3 beam level needs to be considered. For now, temporal and frequency domain prediction will be considered with higher priority for detailed study (if needed). - Continue with generalization across cell configurations for RRM prediction - Continue with measurement events

Proposal 1: Align with RAN2 and RLF prediction will not be studied in Rel-19.

In R18 AI/ML for air, there were several typical use cases incorporated, including CSI feedback enhancement, beam management enhancement and positioning enhancement. Since L3 measurement is based on filtering of L1 measurement, the study of AI/ML for air can be leveraged for mobility purpose. Majority of the study of beam management in AI/ML for air can be reused when we extend L1 beam measurement from serving cell to neighbor cell.

It’s noted that both cell level measurement prediction and beam level measurement prediction are within the scope of RRM measurement prediction. We can observe that there are three sub use cases need to be considered for cell level measurement. For case 1, beam level results are predicted first and then cell level results are derived. For case 2 and case 3, cell level results are predicted directly. For beam level measurement prediction, under the discussion in RAN2, L3 filtered beam level prediction cases are lower priority and there are three cases as well.

=> Cell level measurement prediction model For cell level measurement prediction model, at least consider the following cases: Case 1: To predict beam level results, then generate cell level results based on the predicted beam results; Case 2: To directly predict cell level results based on cell level results. Case 3: To directly predict cell level results based on beam level results => Companies can consider to do L3 filtered beam level results for any of this cases. L3 filtered beam level prediction cases are lower priority. Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results. Case 2: To directly predict L3 filtered beam level results based on the L3 beam level measurement results. Case 3: To directly predict L3 filtered beam level results based on the L1 beam level measurement results.

According to the current assumption of RAN2, evaluation scenarios for RRM measurement prediction can be observed from blew table.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain caseA | 2ndgoal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain caseB | 1stgoal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1stgoal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2ndgoal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1stgoal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1stgoal | Intra-cell |

we noticed that intra cell is mentioned in methodology. It needs to be clarified that intra cell prediction in RAN2 includes two scenarios: serving cell prediction for serving cells and neighbor cell prediction for neighbor cells. Therefore, RAN4 should consider both serving cell and neighbor cell measurment prediction for intra-cell.

In addition, based on the current prioritization defined by RAN2 for different scenarios, we need to at least consider secnarios 2, 3, and 4 with the high priority, which involve RRM measurement prediction in temproal domain and frequency domain. For scenario 6 with medium priority, it needs to wait for the progress of RAN2 or AI/ML beam management.

Proposal 2: RAN4 to study the three high-priority scenarios of scenario 2, 3 and 4 first for measurement prediction use-case. And it is proposed to consider all three sub-use cases for cell level measurement prediction model.

- Conclusion

In this contribution, we have the following observations and proposals for the AI/ML monility:

Proposal 1: Align with RAN2 and RLF prediction will not be studied in Rel-19.

Proposal 2: RAN4 to study the three high-priority scenarios of scenario 2, 3 and 4 first for measurement prediction use-case. And it is proposed to consider all three sub-use cases for cell level measurement prediction model.

- Reference

- RP-242393, Revised SID on AIML for mobility in NR , OPPO (rapporteur)

- R4-2420095, WF on RRM requirements for FS_NR_AIML_Mob, Nokia