3GPP TSG-RAN WG4 Meeting #115		 			                        R4-2506585
St. Julian’s, Malta, 19th – 23rd May, 2025
 
Agenda Item: 7.11.5

Source:	SKY Perfect JSAT

Title: Discussion on UE RF requirements for NR NTN Ku band

Document for:	Discussion

- Introduction

In the RAN4#114 WF in R4-2502288[1], it was agreed that the maximum EIRP requirement for the Rel.19 Ku bands VSAT is set at 88.2 dBm. However, there is no consensus on how this should be captured in the UE RF specification, so we provide our view in this contribution.

Additionally, during the previous meeting, it was agreed to update the satellite parameters used for coexistence study[2], and since this updated parameter could affect the tentatively agreed UE minimum peak EIRP value, in this contribution, we provide our views regards to minimum peak EIRP too.

- Analysis and Discussion

Maximum EIRP requirements

In TS 38.101-5[3], there is a NOTE, which is in the table of Maximum output power limits, saying the BW for Ka band when Maximum EIRP was defined. In accordance with how to specify for Ka band, the BW which defined by the number of RBs and SCS combination for the Ku band maximum EIRP would be better to be captured in NOTE as well as Ka band.

Observation 1: In accordance with how to specify for Ka band, the BW which defined by the number of RBs and SCS combination for the Ku band maximum EIRP would be better to be captured in NOTE as well as Ka band.

The currently agreed upon value of 88.2 dBm for the Ku band maximum EIRP is based on the contributions R4-2419558[4] and R4-2501691[5] from SKY Perfect JSAT. The link budget analysis shown in the contributions are below, and the channel bandwidth used when deriving the EIRP of 88.2 dBm is 18.72 MHz.

- R4-2419558 at RAN4#113 Meeting

Table 2.3-4: Example of Link Budget Analysis for GSO Legacy Satellites

(VSAT UE EIRPMAX=88.2dBm)

| Carrier | CQI |  | 1 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Modulation |  | QPSK | QPSK | QPSK | QPSK | 16QAM | 64QAM |
|  | Code Rate x/1024 |  | 78 | 120 | 308 | 602 | 490 | 466 |
|  | Bandwidth | MHz | 18.72 | 18.72 | 18.72 | 18.72 | 18.72 | 18.72 |
|  | Bit Efficiency | bps/Hz | 0.15 | 0.23 | 0.60 | 1.18 | 1.91 | 2.73 |
|  | Throughput | Mbps | 2.85 | 4.39 | 11.26 | 22.01 | 35.83 | 51.11 |
| Uplink | Uplink EIRP | dBW | 58.20 | 58.20 | 58.20 | 58.20 | 58.20 | 58.20 |
|  | Uplink path loss | dB | -207.09 | -207.09 | -207.09 | -207.09 | -207.09 | -207.09 |
|  | Satellite G/T | dB/K | -2.00 | -2.00 | -2.00 | -2.00 | -2.00 | -2.00 |
|  | Bandwidth | dBHz | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 |
|  | Boltzmann Constant | dBW/Hz･K | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 |
|  | Uplink C/N | dB | 4.99 | 4.99 | 4.99 | 4.99 | 4.99 | 4.99 |
| Downlink | Satellite EIRP | dBW | 44.00 | 44.00 | 44.00 | 44.00 | 44.00 | 44.00 |
|  | Output back-off | dB | 8.34 | 8.34 | 8.34 | 8.34 | 8.34 | 8.34 |
|  | Downlink path loss | dB | -205.93 | -205.93 | -205.93 | -205.93 | -205.93 | -205.93 |
|  | Rx Antenna G/T | dB/K | 33.30 | 33.30 | 33.30 | 33.30 | 33.30 | 33.30 |
|  | Bandwidth | dBHz | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 |
|  | Boltzmann Constant | dBW/Hz･K | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 |
|  | Downlink C/N | dB | 18.91 | 18.91 | 18.91 | 18.91 | 18.91 | 18.91 |
| Total | Total C/N | dB | 4.82 | 4.82 | 4.82 | 4.82 | 4.82 | 4.82 |
|  | Required C/N | dB | -9.48 | -6.66 | -1.80 | 2.42 | 6.37 | 10.27 |
|  | C/N Margin | dB | 14.29 | 11.47 | 6.61 | 2.39 | -1.55 | -5.45 |

- R4-2501691 at RAN4#114 Meeting

Table 2.3-1: Link Budget Analysis for GSO Software Defined Satellites (SDS)

(VSAT UE EIRPMAX =88.2dBm) with Consideration of inter-beam interference

| Carrier | CQI |  | 1 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Modulation |  | QPSK | QPSK | QPSK | QPSK | 16QAM | 64QAM |
|  | Code Rate x/1024 |  | 78 | 120 | 308 | 602 | 490 | 466 |
|  | Bandwidth | MHz | 18.72 | 18.72 | 18.72 | 18.72 | 18.72 | 18.72 |
|  | Bit Efficiency | bps/Hz | 0.1523 | 0.23 | 0.60 | 1.18 | 1.91 | 2.73 |
|  | Throughput | Mbps | 2.85 | 4.39 | 11.26 | 22.01 | 35.83 | 51.11 |
| Uplink | Uplink EIRP | dBW | 58.20 | 58.20 | 58.20 | 58.20 | 58.20 | 58.20 |
|  | Uplink path loss | dB | -207.09 | -207.09 | -207.09 | -207.09 | -207.09 | -207.09 |
|  | Satellite G/T | dB/K | 11.00 | 11.00 | 11.00 | 11.00 | 11.00 | 11.00 |
|  | Bandwidth | dBHz | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 |
|  | Boltzmann Constant | dBW/Hz･K | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 |
|  | Uplink C/N | dB | 17.99 | 17.99 | 17.99 | 17.99 | 17.99 | 17.99 |
| Downlink | Satellite EIRP | dBW | 58.00 | 58.00 | 58.00 | 58.00 | 58.00 | 58.00 |
|  | Output back-off | dB | 10.78 | 10.78 | 10.78 | 10.78 | 10.78 | 10.78 |
|  | Downlink path loss | dB | -205.93 | -205.93 | -205.93 | -205.93 | -205.93 | -205.93 |
|  | Rx Antenna G/T | dB/K | 33.30 | 33.30 | 33.30 | 33.30 | 33.30 | 33.30 |
|  | Bandwidth | dBHz | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 |
|  | Boltzmann Constant | dBW/Hz･K | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 |
|  | Downlink C/N | dB | 30.47 | 30.47 | 30.47 | 30.47 | 30.47 | 30.47 |
| Total | Total C/N | dB | 17.75 | 17.75 | 17.75 | 17.75 | 17.75 | 17.75 |
|  | C/I: inter-beam Interference | dB | 10.00 | 10.00 | 10.00 | 10.00 | 10.00 | 10.00 |
|  | C/(N+I) | dB | 9.33 | 9.33 | 9.33 | 9.33 | 9.33 | 9.33 |
|  | Required C/N | dB | -9.48 | -6.66 | -1.80 | 2.42 | 6.37 | 10.27 |
|  | C/N Margin | dB | 18.80 | 15.98 | 11.12 | 6.90 | 2.96 | -0.94 |

This bandwidth is equivalent to 13 RBs allocation with 120 kHz SCS, which is the same as the value already captured in the NOTE of the maximum EIRP for Ka band. It is optimal to specify the VSAT RF requirements for Ku band based on the RF requirements of existing Ka band VSAT, so it makes sense to use the same bandwidth as Ka band. Therefore, we would like to propose the BW for NOTE of the maximum EIRP is 18.72 MHz (i.e. 13 RBs allocation with 120 kHz SCS).

Proposal 1: It is preferable to use 18.72 MHz bandwidth (i.e. 13 RBs allocation with 120 kHz SCS) as the base of the Ku band maximum EIRP because this bandwidth was used in link budget analysis derived the agreed maximum EIRP value and it is appropriate to use same bandwidth as Ka band.

However, since 120 kHz SCS is only adopted for FR2, the combination of the number of RBs and SCS for FR1 needs to be considered. The WID for Ku band states that to specify common FR1 and FR2 radiated RF requirements for NTN VSAT types if possible, and then the EIRP density becomes to vary depending on the numerology if the BW that is the premise of the Ku band maximum EIRP differs between FR1 and FR2, so it is essential to adopt a common BW for FR1 and FR2. Therefore, it is appropriate to use 18.72 MHz as the common BW for FR1 and FR2, and when the number of RBs is derived from 18.72 MHz, it is 52 RBs for 30 kHz SCS and 104 RBs for the 15 kHz SCS.

To summarize above, it is proposed to add the following comments for Ku band in the NOTE of the table of Maximum output power limits.

- Maximum EIRP is defined using 52 RBs allocation with 30 kHz SCS or using 104 RBs allocation with 15 kHz SCS.

- Maximum EIRP is defined using 13 RBs allocation with 120 kHz SCS.

Proposal 2: It is proposed to add the following comments for Ku band in the NOTE of the table of Maximum output power limits.

- Maximum EIRP is defined using 52 RBs allocation with 30 kHz SCS or using 104 RBs allocation with 15 kHz SCS.

- Maximum EIRP is defined using 13 RBs allocation with 120 kHz SCS.

Minimum peak EIRP requirements

As described in WF for Coexistence study R4-2504723[2] in the last meeting, the updated satellite parameters for GEO are agreed as below.

The UE minimum peak EIRP has been derived using satellite parameters of coexistence study. Since the satellite parameters for GEO have been updated to the above consensus values, we reviewed whether the UE minimum peak EIRP is still feasible with the latest satellite parameters.

The link budget analysis using the updated GEO parameter is as below. Our calculations show that the reduction in Satellite RX Gain will result in the Minimum Peak EIRP being higher than the tentatively agreed upon 70dBm, which is for communicating with GEO.

Table 2.2-1 Link budget analysis for minimum peak EIRP

| Satellite Type (Channel Bandwidth) |  | GEO (100MHz) |  | GEO (10MHz) |  |
| --- | --- | --- | --- | --- | --- |
| Elevation angle (VSAT) | deg | 30 | 90 | 30 | 90 |
| Distance (VSAT-SAN) | km | 38612 | 35786 | 38612 | 35786 |
| UL frequency | MHz | 14500 | 14500 | 14500 | 14500 |
| Free space path loss | dB | 207.41 | 206.7 | 207.41 | 206.7 |
| Atmosphere loss | dB | 2 | 2 | 2 | 2 |
| BW | MHz | 100 | 100 | 10 | 10 |
| target SNR | dB | -1 | -1 | -1 | -1 |
| Satellite noise figure | dB | 3 | 3 | 3 | 3 |
| Satellite Rx gain | dBi | 44 | 44 | 44 | 44 |
| Implementation margin | dB | 2 | 2 | 2 | 2 |
| Gain Reduction Beam edge | dB | 3 | 3 | 3 | 3 |
| minimum EIRP | dBm | 78.41 | 77.7 | 68.41 | 67.7 |

However, since we are considering various use cases, including relatively low-throughput scenarios that utilize narrow bandwidths such as the 10 MHz, setting the minimum peak EIRP too high would lead to unnecessarily strict requirements for certain use cases, making it difficult to realize cost-effective terminals.

On the other hand, the calculation results using the 10 MHz bandwidth have confirmed that the minimum peak EIRP of 70dBm is sufficient, so we propose to maintain the minimum peak EIRP at 70dBm for communicating with GEO.

Proposal 3: Maintain the minimum peak EIRP at 70dBm for communicating with GEO, which is the tentatively agreed value.

- Conclusions

Observation 1: In accordance with how to specify for Ka band, the BW which defined by the number of RBs and SCS combination for the Ku band maximum EIRP would be better to be captured in NOTE as well as Ka band.

Proposal 1: It is preferable to use 18.72 MHz bandwidth (i.e. 13 RBs allocation with 120 kHz SCS) as the base of the Ku band maximum EIRP because this bandwidth was used in link budget analysis derived the agreed maximum EIRP value and it is appropriate to use same bandwidth as Ka band.

Proposal 2: It is proposed to add the following comments for Ku band in the NOTE of the table of Maximum output power limits.

- Maximum EIRP is defined using 52 RBs allocation with 30 kHz SCS or using 104 RBs allocation with 15 kHz SCS.

- Maximum EIRP is defined using 13 RBs allocation with 120 kHz SCS.

Proposal 3: Maintain the minimum peak EIRP at 70dBm for communicating with GEO, which is the tentatively agreed value.

- References

- R4-2502288, WF for [114][314] NR_NTN_Ku_Band_UE_SAN_RF; Moderator (CHTTL), RAN4#114

- R4-2504723, Way Forward for [114bis][313] NR_NTN_Ku_Band_Coexistence; Moderator (Intelsat), RAN4#114bis

- 3GPP TS 38.101-5, User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements (Release 18)

- R4-2419558, Ku-band Maximum TRP and EIRP Analysis; SKY Perfect JSAT, RAN4#113

- R4-2501691, Discussion on UE RF requirements for NR NTN Ku band; SKY Perfect JSAT, RAN4#114