# 3GPP TSG-RAN WG4 Meeting #115                             R4-2506598

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.18.3.2

Source: 	CMCC

Title: 			 Discussion on L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

Document for:	Discussion

- Introduction

In RAN #103 meeting, the new WID on RRM enhancements was approved [1]. According to the WID, one of the objectives is FR2-1 SSB based L3 measurement delay reduction for connected mode, the details are duplicated as following.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2

There is discussion on FR2-1 SSB based L3 measurement delay reduction for connected mode in last meeting, and a WF was approved [2]. This contribution provides discussion on L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC.

- Discussion

Issue 1-2-2: Expand solution 1 to FR1 case - Option 1 (Nokia, Apple, CTC): Expand the solution 1 to FR1 band. - Option 1a (Nokia): - Expand the solution 1 to at least FR1 intra-band contiguous CA. - Details: - Network indicates to enable or disable this CSSF enhancement i.e. measuring one serving CC per band for FR1 and FR2 separately. - When network indicates to enable Tentative solution 1 for FR1 bands, UE is required to measure serving and neighbor cells on one serving CC per band when configured with FR1 intra-band contiguous CA. - Option 2 (Huawei): Not to expand solution 1 to FR1 band.

Solution 1 is to measure one serving CC per band. In legacy, for FR2, UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band. With solution 1, UE only need to measure one serving CC per band.

Issue 1-3-1: Tentative solution 1: measure one serving CC per band Agreement: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings, - Network indication of enable/disable this CSSF enhancement is per-UE. - Follow the following principle for serving and neighboring cell measurement - Measure PCC if PCC in band - Measure PSCC if PSCC in band - Measure SCC on which UE is configured to report SSB based measurements, if - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements. Note: Following the WID, the above agreement applies for the measurement without gap.

Solution1 for FR2 is under the consideration that FR2 is noise-limited system rather than interference-limited system, while the situation is different for FR1. In FR1, RSRQ might be different among different serving carriers due to different interference level. In general, it is not preferred to extend solution 1 to FR1. However, as mentioned by companies that it can be considered for certain scenario, e.g. intra-band contiguous CA. Also, whether to enable solution 1 is under network control. With above consideration, we are also fine to extend solution 1 to FR1 intra-band contiguous CA.

Proposal 1: it is proposed to extend solution 1 to FR1 intra-band contiguous CA.

- Conclusion

This contribution provides discussion on L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC. The proposals are:

Proposal 1: it is proposed to extend solution 1 to FR1 intra-band contiguous CA.

- Reference

- RP-240830, New WID: WI resulting from discussion on RRM enhancements

- R4-2504899, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2502592, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2416853, WF for NR_RRM_Ph5_Part1, Apple

- R4-2414026, WF for NR_RRM_Ph5_Part1, Apple

- R4-2410260, Way Forward for [111][228] NR_RRM_Ph5

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5

- R4-2406392, WF for [110bis][230] NR_RRM_Ph5, Apple

- RP-240817, Moderator's summary for discussion on RRM enhancements