# 3GPP TSG-RAN WG4 Meeting #115                              R4-2506609

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.22.3

Source: 	CMCC

Title: 			 Discussion on testing issues for CSI compression two-sided models

Document for:	Discussion

- Introduction

In RAN #106 meeting, a new SID on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2 was approved [1], and the objectives are same as the study aspects that were included in the REL-19 NR_AIML_air WI [2]. The objectives for RAN4 are duplicated as following.

- Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion on testing issues for CSI compression two-sided models.

- Discussion

Option 3 track 2 Endorsed preliminary observations - There is reasonable SGCS convergence when companies train an encoder-decoder pair using the mixed dataset and test using the mixed testset - Around 1% variation in SGCS; see results spreadsheet for more details - Preliminary observation; further results may be added - It is possible for UE vendors to train an encoder based on a frozen test decoder that is functional - When training “own” encoder with the frozen decoder and testing, with all training (frozen decoder and encoder) and testing using the mixed dataset, there is good convergence in average SGCS results from results presented so far - Around [+- 2%] variation in SGCS from the average; see results spreadsheet for more details - Preliminary observation; further results may be added - Up to now, the same encoder structure is considered as used when creating the reference decoder by most companies; the impact of different encoder structure is not yet fully considered Further simulation activity for option 3 track 2 - Definitions - Training Dataset 1 is the dataset used for training of the frozen decoder. - Training Dataset 2 is the dataset used for training of the new encoder based on the frozen decoder. - For the existing results above, Training dataset 1 = training dataset 2 = mixed training dataset. Test dataset = mixed test dataset. - Additional results 1 for RAN4#115: - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset) - For the new encoder: Training dataset 2 = company specific training dataset. Test Dataset = Mixed test dataset. - Additional results 2 for RAN4#115: - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset) - For the new encoder: Training dataset 2 = company specific training dataset. Test dataset = company specific test dataset. - Additional results 3 for RAN4#115: Further results using a lower complexity CNN or transformer encoder - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset) - For the new encoder: Training dataset 2 = mixed. Test Dataset = Mixed test dataset. - Encoder structure: As suggested in [R4-2505156] or company specific selection. Structure and complexity should be reported.

Simulation are performed for additional results 1 and additional results 2. Our simulation results are summarized as following Table 1.

Table 1: simulation results for additional results 1 and additional results 2

| Option 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Joint training |  | New encoder training with (Samsung’s) frozen decoder |  |  |  |
|  |  | Testing on mixed |  | Testing on mixed |  |  | Testing on own |
|  |  | training on mixed | (2nd priority) training on own | training on mixed | Additional results 1 training on own | (2nd priority) training on mixed & own [Note: Not captured in WF] | Additional results 2 training on own |
| Encoder-1 (Agreed encoder, CNN with 47M FLOPs) | CMCC | 0.6885 | 0.6566 | 0.6902 | 0.6847 |  | 0.6692 |

Proposal 1: for option 3 track 2, the additional simulation results are provided in Table 1.

Relation between RAN1 option 3 and RAN1 inter-vendor collaboration direction C For further discussion: - From the online discussion: FFS on the relation between RAN4 test decoder options and RAN1 inter-vendor interoperability directions. The relation between the RAN4 options and RAN1 inter-vendor interoperability directions should be further clarified.

RAN1 agreements related with inter-vendor collaboration are summarized as following.

RAN1 agreements on inter-vendor interoperability Agreements in RAN1#118 meeting Continue the study of following directions - on-device operation and UE side offline engineering - Direction A: Sharing parameters/reference model/dataset that enables UE-side offline engineering (Inter vendor collaboration option 3a/5a/4) - Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1) Conclusion in RAN1#120 meeting - Direction C with standardized model ensures a minimum performance requirement. - The so called fully specified reference model discussed in RAN1 for Direction C is assumed to be equivalent to the to-be-specified model, if specified, for performance requirements in RAN4. - Direction A with standardized model parameter / dataset exchange from NW-side to UE-side may achieves better performance. # Conclusion in RAN1 #120bis meeting - RAN1 concludes that both Direction A and Direction C are feasible. - RAN1 concludes that both sub-option 4-1 and sub-option 3a-1 are feasible. - This includes both 3a-1 with and without target CSI sharing.

According to RAN1, both Direction A nd Direction C are for UE side offline engineering. Direction C with standardized model means fully specified encoder. In RAN4 discussion, for option 3, by definition, a test decoder is standardized. To verify the performance of CSI compression test decoder for calibration of test equipment, RAN4 further agree to define the reference encoder including encoder structure and parameters. For option 4, there are several candidate options. Option 4b is to specify as encoder in the specification which is used to derive a decoder to be implemented by TE vendors. It can be seen that from the perspective of model standardization, RAN1 inter-vendor interoperability Direction C is equivalent to RAN4 option 3 and option 4b, since both are to document encoder in the specifications.

- Option 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors

- Option 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors

- Option 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering

However, whether RAN4 option 3/option 4b and RAN1 direction C are the same need further discussion. We do not think the encoder model specified in RAN4 option 3/option 4b can be used for RAN1 direction C. The key difference is that the intention is different. RAN4 discussion is for test. Even though encoder are standadized, the key point is to make sure the test decoder used by TE is suitable for test. Since RAN4 requirements are minimum, the test encoder/decoder will be simplified. While RAN1 discussion is for inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, which will be used for training in real deployment.

Proposal 2: from the perspective of model standardization, RAN1 inter-vendor interoperability Direction C is equivalent to RAN4 option 3 (with encoder standadized for TE carlibration) and option 4b, since both are to document encoder in the specifications. However, we do not think RAN4 option 3/option 4b and RAN1 direction C are the same. Whether the specified encoder in RAN4 option 3/option 4b can be used for RAN1 direction C need further discussion.

- Conclusion

This contribution provides discussion on testability and interoperability issues for CSI compression. The proposals are:

Proposal 1: for option 3 track 2, the additional simulation results are provided in Table 1.

Table 1:  additional simulation results for option 3 track 2

| Option 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Joint training |  | New encoder training with (Samsung’s) frozen decoder |  |  |  |
|  |  | Testing on mixed |  | Testing on mixed |  |  | Testing on own |
|  |  | training on mixed | (2nd priority) training on own | training on mixed | Additional results 1 training on own | (2nd priority) training on mixed & own [Note: Not captured in WF] | Additional results 2 training on own |
| Encoder-1 (Agreed encoder, CNN with 47M FLOPs) | CMCC | 0.6885 | 0.6566 | 0.6902 | 0.6847 |  | 0.6692 |

Proposal 2: from the perspective of model standardization, RAN1 inter-vendor interoperability Direction C is equivalent to RAN4 option 3 (with encoder standadized for TE carlibration) and option 4b, since both are to document encoder in the specifications. However, we do not think RAN4 option 3/option 4b and RAN1 direction C are the same. Whether the specified encoder in RAN4 option 3/option 4b can be used for RAN1 direction C need further discussion.

- Reference

- RP-243245, New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2505107, WF on AI/ML air interface study, Ericsson

- R4-2502953, WF on AI/ML air interface study, Ericsson

- R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface