3GPP TSG-RAN WG4 Meeting #115	R4-2506632

St Julian’s, Malta, 19th – 23rd May 2025

Source:	Ericsson

Title:	Discussion on Rel-19 NR MIMO Phase 5 demodulation requirements

Agenda item:	7.24.6

Document for:	Discussion

# Introduction

Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

In RAN#105, a revised WID [1] on NR MIMO Phase 5 was approved for Release 19. The WID covers five main objectives, where one of the objectives is to specify 3-antenna-port codebook-based transmissions. The enhancements for uplink performance are copied below from the WID.

In this meeting, we kick-off the demodulation discussions. In this paper, we present our general view on BS demodulation requirements 3-port PUSCH supporting up to 3 layers, along with UE demodulation requirements.

# BS Demodulation requirements

NR currently supports PUSCH configurations with 1, 2, 4, and 8 ports. However, most commercial mobile UEs today are equipped with only 1 or 2 Tx antennas. Looking ahead, as hardware and design technologies continue to evolve, the introduction of UEs with 3 TX antennas is a plausible next step. To support this anticipated development and improve uplink performance, it is beneficial to extend the NR specifications to include support for 3-port PUSCH with up to 3 transmission layers.

2.1 Scenario and channel model

Based on the description in WI [1], it seems still in the similar scope as legacy TN scenario. In that case, same network deployment would be applied.

Observation 1: No new deployment scenario is introduced for 3Tx MIMO transmission.

In general, achieving higher layer transmission requires favourable channel conditions, typically characterized by high SNR. Such conditions are more likely in line-of-sight (LOS) scenarios with relatively low UE mobility. However, from a performance evaluation perspective, it is more insightful to consider worst-case scenarios—such as non-line-of-sight (NLOS) channels and higher Doppler shifts—if such conditions are feasible to test. Since no new scenarios are being introduced, the same channel models used in traditional BS demodulation—such as TDLA30-10, TDLB100-400, and TDLC300-100—can be reused for initial 3Tx simulations. In existing BS demodulation requirements, different channel models are used to evaluate performance across various coverage conditions. A similar approach can be adopted here as well. To reduce complexity, companies may optionally limit the number of test cases, for instance, by specifying requirements for only one or two channel models

Proposal 1: Re-use the legacy channel models, e.g., TDLA30-10/TDLB100-400/TDLC300-100, for initial 3Tx MIMO demodulation simulations.

2.2 Requirement Scope

The TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource selected by the SRI when multiple SRS resources are configured, or if a single SRS resource is configured TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource.

In the current test parameters for normal PUSCH demodulation requirements, TPMI is fixed to 0, meaning only a single-layer or fixed precoding is tested, and no selection among multiple precoding matrices is done. However, this WI focus on non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions. This expands the test scope by requiring multiple TPMI indexes. As discussed in clause 6.1.1.1 of TS 38.214,

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for single-layer transmission.

13100,13010,13001

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for two-layer transmission.

13100100, 13100001, 13 001001

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for three-layer transmission.

13100010001

As per the RAN1 agreements [2] in RAN1#116 meeting,

Observation 2:  For non-coherent uplink precoding by a 3TX UE, new precoders are agreed in RAN1 and need to be considered by RAN4 to define BS demodulation requirements.

Since new precoding matrix for single, two and three-layer transmission need to be considered, the corresponding TPMI index need to be checked for the PUSCH performance requirements.

Proposal 2: Consider new precoding matrix for non-coherent uplink precoding by 3Tx UE to define 3Tx MIMO BS demodulation requirements. Correspondingly, the TPMI index should be assigned for single, two or three-layer transmission.

2.3 Requirement Impact

Enabling 3Tx MIMO primarily affects PUSCH performance as specified in [1]. In the legacy requirements outlined in [3], 2Tx and 4Tx configurations are addressed only within the normal PUSCH demodulation requirements. A similar approach can be adopted for introducing 3Tx demodulation requirements

Proposal 3: Only new normal PUSCH demodulation requirements are considered for 3Tx MIMO.

Some analysis for initial simulations are as follows:

Channel Bandwidth and Subcarrier Spacing (SCS): For the initial simulations, the minimum channel bandwidth for each SCS may be considered. If significant performance differences are observed, it is advisable to also include typical or even the maximum channel bandwidth to ensure adequate test coverage. The following configurations are suggested:

- 15 kHz SCS with 5 MHz and 20 MHz channel bandwidth

- 30 kHz SCS with 10 MHz and 100 MHz channel bandwidth

Modulations: To maintain consistency with 2 and 4Tx requirements, QPSK, 16QAM, and 64QAM can be used to ensure comparable test coverage. Legacy MCS indices can be applied for initial simulations and adjusted later based on simulation outcomes. The following candidate configurations are proposed:

  - TDLA30-10 (low delay spread and low Doppler shift): High-order modulation (up to 64QAM) and higher coding rate (e.g., R = 567/1024) are typical. Therefore, MCS 20 is suggested.

  - TDLB100-400 (medium delay spread and high Doppler shift): A more robust setting using MCS 2 with QPSK modulation is appropriate.

  - TDLC300-100 (high delay spread and medium Doppler shift): MCS 16, using 16QAM modulation, is recommended.

Additionally, higher MCS indices per modulation level may also be considered in initial simulations, especially given the high throughput expectations for the 3Tx scenarios.

Number of Receiver Antennas: For 1Tx and 2Tx configurations, 2, 4, and 8 Rx antennas are typically considered in the requirements. However, since this work item focuses specifically on evaluating 3 uplink layers, the following antenna configurations are proposed for initial simulations: 3Tx 4Rx and 3Tx 8Rx. Since we do not have any other test requirement with 3 and 6 Rx, we can test 3-layer transmission with 4 and 8 Rx antenna.

Waveform: only CP-OFDM is suggested for initial evaluation.

PUSCH mapping type: Both mapping type A and B are considered for 1Tx, 2Tx and 4Tx requirement. From checking the receiver performance perspective, only mapping type A might be enough.

DM-RS configuration: 1+1 DMRS configuration can be taken for initial simulations.

Test metric: 70% normalized throughput.

Proposal 4: It can be feasible to adopt the parameter set in Table 2.3-1 for initial simulations.

- If significant performance differences are observed between channel bandwidths for each SCS, typical bandwidth may be considered in the requirements to ensure comprehensive test coverage.

- Companies are also encouraged to assess the feasibility of using higher MCS values for each modulation level.

Table 2.3-1 Initial simulation parameters for 3Tx MIMO PUSCH demodulation

| nTx | nRx | Mapping type | precoding | DM-RS | Channel model | MCS | SCS [kHz] | CBW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 4 / 8 | A | Enable | 1+1 | TDLB100-400 | 2 | 15 | 5/20 |
|  |  |  |  |  | TDLC300-100 | 16 |  |  |
|  |  |  |  |  | TDLA30-10 | 20 |  |  |
|  |  |  |  |  | TDLB100-400 | 2 | 30 | 10/100 |
|  |  |  |  |  | TDLC300-100 | 16 |  |  |
|  |  |  |  |  | TDLA30-10 | 20 |  |  |

2.4 Specification Impact

Since 3Tx MIMO is a new feature with non-coherent UL codebook, separate test parameter and requirements should be created in section 8.2.16 of TS 38.214. However, it depends on the discussion and agreement among companies.

Observation 3: Since this is a new feature, it may be feasible to add new section to “Sec. 8.2: Performance requirements for PUSCH”.

# UE Demodulation requirements

3.1	Refined Type-I codebook supporting up to 128 CSI-RS ports

RAN1 enhanced Rel-15 Type-I single panel codebook to support more than 32 CSI-RS ports according to the WID [1], and RAN1 are going to introduce many capability options [4].

RAN1 agreed to support 48, 64, and 128 CSI-RS antenna ports for the Rel-19 refined Type-I single panel codebook, as summarized in Table 1. If UE support the Rel-19 refined Type-I codebook, the UE should support at least 64 CSI-RS ports, while it is optional to support 48 and 128 CSI-RS ports.

Table 1	Supported configurations of CSI-RS antenna ports.

| Number of CSI-RS antenna ports | (N1, N2) | (O1, O2) |
| --- | --- | --- |
| 48 | (8, 3) | (4, 4) |
|  | (6, 4) | (4, 4) |
| 64 | (16, 2) | (4, 4) |
|  | (8, 4) | (4, 4) |
| 128 | (16, 4) | (4, 4) |
|  | (8, 8) | (4, 4) |

RAN1 agreed to introduce two Type-I codebook modes to support more than 32 ports, which is called typeI-codebookMode-r19. The supported modes are ‘Mode A’ and ‘Mode B’. Codebook mode A is an extension of Rel-15 type-I codebook, which targets low complexity and low overhead. On the other hand, codebook mode B targets performance improvement to use the larger CSI-RS ports. Both the modes support up to rank 8, and UE can support either or both of modes, according to the UE capability.

When gNB configure CSI reporting, gNB need to configure the corresponding CSI-RS resources for channel measurements (CMR). RAN1 agreed to configure multiple CSI-RS resources (aggregation) to support more than 32 CSI-RS ports, where RAN1 does not enhance the CSI-RS resource configuration, that is, one CSI-RS resource can configure up to 32 CSI-RS ports. This means we need to configure more than one CSI-RS resources to support more than 32 CSI-RS ports. Table 2 shows the relation between the CSI-RS antenna ports, supported number of CSI-RS resource configurations (K), and number of CSI-RS ports per CSI-RS resource (nrofPorts in CSI-RS-ResourceMapping). For example, to configure 128 CSI-RS ports, gNB needs to configure K=4 CSI-RS resources with 32 ports per CSI-RS resource. RAN1 defines two methods to map from CRS-RS resource to the antenna port, which is called portMappingMethod. Two mapping method options are ‘Method 1’ and ‘Method 2’.

If we configure more than one CSI-RS configuration, this consumes radio resources. RAN1 agreed that gNB can send CSI-RS within one slot or within two consecutive slots. It is the basic feature for UE to receive CSI-RS within one slot, and it requires additional capability to receive CSI-RS within two consecutive slots according to the UE feature list (FG 59-2-1-6 [4]).

Table 2	Supported configuration of number of CSI-RS resources.

| Number of CSI-RS antenna ports (Ntot) | Number of CSI-RS resource configurations (K) | Number of CSI-RS ports per CSI-RS resource (N) |
| --- | --- | --- |
| 48 | 2 | 24 |
|  | 3 | 16 |
| 64 | 2 | 32 |
|  | 4 | 16 |
| 128 | 4 | 32 |

RAN4 has already defined PMI reporting requirements with Type-I single panel codebook with 4, 8, 16, and 32 CSI-RS ports. We think RAN4 should define new PMI reporting requirements with Rel-19 refined Type-I codebook to verify the PMI reporting functionalities more than 32 CSI-RS ports.

The latest UE feature list [4] specifies the basic feature is 64 ports with Mode A and B. Considering the number of options and minimizing the total number of test cases, we propose considering four test cases shown in Table 3.

Table 3	Proposed PMI reporting requirements with Rel-19 refined Type-I single panel codebook.

| Test case | CSI-RS ports (N1, N2) | Port mapping method (portMappingMethod) | Codebook mode (typeI-codebookMode-r19) | CMR configuration (K and nrofPorts per CSI-RS resource) | Number of slots for CSI-RS transmission |
| --- | --- | --- | --- | --- | --- |
| Test 1 | 64 (8, 4) | Method 1 | Mode A | K=4 16 CSI-RS ports | One slot |
| Test 2 | 64 (16, 2) | Method 2 | Mode B | K=2 32 CSI-RS ports | One slot |
| Test 3 | 48 (8, 3) | Method 1 | Mode A | K=2 24 CSI-RS ports | One slot |
| Test 4 | 128 (16, 4) | Method 2 | Mode B | K=4 32 CSI-RS ports | Two consecutive slots |

Proposal 5: Define PMI reporting requirements with Rel-19 refined Type-I single panel codebook with 48, 64 and 128 CSI-RS ports.

We can reuse the existing test metric for Type-I codebook PMI reporting tests. For other parameters such as PDSCH FRC and channel model, we also think we can reuse the parameters used for the Rel-15 PMI reporting requirements with 32 CSI-RS ports. However, we may need to revisit PDSCH FRC if the required SNR value is too low to achieve 90% of the peak rate with follow PMI.

Proposal 6: For the PMI reporting requirements with Rel-19 refined Type-I single panel codebook, reuse the test metric for Type-I PMI reporting requirements, that is, throughput ratio between the following PMI and random Type-I PMI, where the same Type-I codebook scheme is used for both follow PMI and random PMI throughput measurements.

Proposal 7: For the PMI reporting requirements with Rel-19 refined Type-I single panel codebook, reuse the test parameters of the existing Rel-15 Type-I single panel PMI reporting requirements with 32 CSI-RS ports as baseline, e.g., TDLA30-5, MCS20 with rank 2.

Proposal 8: RAN4 may consider higher MCS (code rate) for PDSCH FRC, if the required SNR value is too low to achieve 90% of the peak rate with the follow PMI.

In Rel-15, RAN4 defined the CSI reporting requirements targeting 2Rx/4Rx UEs in FR1 FDD/TDD bands. Although RAN4 defined CSI reporting requirements with 1Rx UE (RedCap) in Rel-17 and 8Rx UE in Rel-18, we think it is sufficient to define the PMI requirements for 2Rx/4Rx UEs, considering the minimum performance requirements.

Proposal 9: Define the Rel-19 refined Type-I single panel codebook PMI reporting requirements for 2Rx/4Rx UEs with FR1 FDD SCS=15kHz and FR1 TDD SCS=30kHz.

RAN1 agreed to support two timeline capabilities for PMI calculation:

  - Capability 1: reuses legacy Z2/Z2’ values in TS 38.214 Table 5.4-2, and OCPU = ceil(P/32)

  - Capability 2: Scales Z2/Z2’ by ceil(P/32), but OCPU=1.

This implies the period between CSI-RS resource transmission to CSI reporting becomes roughly double for 48/64 CSI-RS ports and 4 times longer for 128 CSI-RS ports, compared with the existing 32 ports configuration. We should adjust the CSI feedback delay from the time the TE receives the CSI report to the time the TE schedule PDSCH using the reported PMI, according to the configured CSI-RS ports and CSI processing capabilities.

Proposal 10: RAN4 set the CQI/RI/PMI delay according to the number of CSI-RS ports and CSI processing capabilities.

In addition to the Type-I single panel codebook, RAN1 enhanced the Type-I multi-panel codebook to support more than 32 CSI-RS antenna ports. Since RAN4 does not define any PMI reporting requirements with the Type-I multi-panel codebook, we propose not to define PMI reporting requirements with Rel-19 refined Type-I multi-panel codebook supporting more than 32 CSI-RS ports.

Proposal 11: No PMI reporting requirements with Rel-19 refined Type-I multi-panel codebook supporting more than 32 CSI-RS ports.

3.2	Refined Type-II codebook supporting up to 128 CSI-RS ports

RAN1 also enhanced the Type-II codebooks to support more than 32 CSI-RS ports, according to WID. The enhanced Type-II codebooks are summarized as follows [4]:

RAN1 also enhanced the Type-II codebooks to support more than 32 CSI-RS ports, according to WID. The enhanced Type-II codebooks are summarized as follows [2]:

- Refined Rel-16 eType-II codebook to support 48, 64 and 128 CSI-RS ports.

  - Supporting 64 CSI-RS ports is the basic feature, while supporting 48 and 128 CSI-RS ports are added capabilities.

- Refined Rel-17 FeType-II codebook to support 48 and 64 CSI-RS ports.

  - Supporting 64 CSI-RS ports is the basic feature, while supporting 48 CSI-RS ports are added capabilities.

- Refined Rel-18 eType-II Doppler codebook to support 48, 64 and 128 CSI-RS ports.

  - Supporting 64 CSI-RS ports is the basic feature, while supporting 48 and 128 CSI-RS ports are added capabilities.

RAN4 has defined the PMI reporting requirements with Rel-16 eType-II codebook with 16 CSI-RS ports and Rel-18 eType-II Doppler codebook with 16 CSI-RS ports. Considering the minimum performance requirement, we propose to define enahced eType-II with 64 ports by reusing Rel-16 eTypeII PMI reporting requirements.

Proposal 12: Define PMI reporting requirements with Rel-19 refined eType-II codebook with 64 CSI-RS ports, where (N1, N2) = (8, 4).

Proposal 13: For the PMI reporting requirements with Rel-19 refined eType-II codebook, reuse the test metric for eType-II PMI reporting requirements, that is, throughput ratio between the following PMI and random Rel-19 refined Type-I PMI Mode A.

Proposal 14: For the PMI reporting requirements with Rel-19 refined eType-II codebook, reuse the test parameters of the existing Rel-16 eType-II PMI reporting requirements with 16 CSI-RS ports as baseline, e.g., TDLA30-5, MCS20 with rank 2, Codebook Configuration.

We also propose to define Rel-19 refined eType-II codebook PMI reporting requirements for 2Rx/4Rx UEs with FR1 FDD SCS=15kHz and FR1 TDD SCS=30kHz like Type-I PMI reporting requirements.

Proposal 15: Define the Rel-19 refined eType-II codebook PMI reporting requirements for 2Rx/4Rx UEs with FR1 FDD SCS=15kHz and FR1 TDD SCS=30kHz.

3.3 CRI-based CSI reporting up to 128 CSI-RS ports

This is the extension of Rel-15 CRI reporting features to support maximum 128 CSI-RS ports. Since RAN4 does not define the CRI reporting requirements in NR, we don’t think RAN4 needs to specify any requirements for CRI reporting functionality.

Proposal 16: No CRI reporting requirements for Rel-19 MIMO Phase 5 WI.

# Conclusion

4.1	BS Demodulation Requirements

Based on the above discussion following observations are captured:

Observation 1: No new deployment scenario is introduced for 3Tx MIMO transmission.

Observation 2:  For non-coherent uplink precoding by a 3TX UE, new precoders are agreed in RAN1 and need to be considered by RAN4 to define BS demodulation requirements.

Observation 3: Since this is a new feature, it may be feasible to add new section to “Sec. 8.2: Performance requirements for PUSCH”.

Based on the above discussion following proposals are captured:

Proposal 1: Re-use the legacy channel models, e.g., TDLA30-10/TDLB100-400/TDLC300-100, for initial 3Tx MIMO demodulation simulations.

Proposal 2: Consider new precoding matrix for non-coherent uplink precoding by 3Tx UE to define 3Tx MIMO BS demodulation requirements. Correspondingly, the TPMI index should be assigned for single, two or three-layer transmission.

Proposal 3: Only new normal PUSCH demodulation requirements are considered for 3Tx MIMO.

Proposal 4: It can be feasible to adopt the parameter set in Table 2.3-1 for initial simulations.

- If significant performance differences are observed between channel bandwidths for each SCS, typical bandwidth may be considered in the requirements to ensure comprehensive test coverage.

- Companies are also encouraged to assess the feasibility of using higher MCS values for each modulation level.

Table 2.3-1 Initial simulation parameters for 3Tx MIMO PUSCH demodulation

| nTx | nRx | Mapping type | precoding | DM-RS | Channel model | MCS | SCS [kHz] | CBW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 4 / 8 | A | Enable | 1+1 | TDLB100-400 | 2 | 15 | 5/20 |
|  |  |  |  |  | TDLC300-100 | 16 |  |  |
|  |  |  |  |  | TDLA30-10 | 20 |  |  |
|  |  |  |  |  | TDLB100-400 | 2 | 30 | 10/100 |
|  |  |  |  |  | TDLC300-100 | 16 |  |  |
|  |  |  |  |  | TDLA30-10 | 20 |  |  |

4.2	UE Demodulation Requirements

Proposal 5: Define PMI reporting requirements with Rel-19 refined Type-I single panel codebook with 48, 64 and 128 CSI-RS ports.

| Test case | CSI-RS ports (N1, N2) | Port mapping method (portMappingMethod) | Codebook mode (typeI-codebookMode-r19) | CMR configuration (K and nrofPorts per CSI-RS resource) | Number of slots for CSI-RS transmission |
| --- | --- | --- | --- | --- | --- |
| Test 1 | 64 (8, 4) | Method 1 | Mode A | K=4 16 CSI-RS ports | One slot |
| Test 2 | 64 (16, 2) | Method 2 | Mode B | K=2 32 CSI-RS ports | One slot |
| Test 3 | 48 (8, 3) | Method 1 | Mode A | K=2 24 CSI-RS ports | One slot |
| Test 4 | 128 (16, 4) | Method 2 | Mode B | K=4 32 CSI-RS ports | Two consecutive slots |

Proposal 6: For the PMI reporting requirements with Rel-19 refined Type-I single panel codebook, reuse the test metric for Type-I PMI reporting requirements, that is, throughput ratio between the following PMI and random Type-I PMI, where the same Type-I codebook scheme is used for both follow PMI and random PMI throughput measurements.

Proposal 7: For the PMI reporting requirements with Rel-19 refined Type-I single panel codebook, reuse the test parameters of the existing Rel-15 Type-I single panel PMI reporting requirements with 32 CSI-RS ports as baseline, e.g., TDLA30-5, MCS20 with rank 2.

Proposal 8: RAN4 may consider higher MCS (code rate) for PDSCH FRC, if the required SNR value is too low to achieve 90% of the peak rate with the follow PMI.

Proposal 9: Define the Rel-19 refined Type-I single panel codebook PMI reporting requirements for 2Rx/4Rx UEs with FR1 FDD SCS=15kHz and FR1 TDD SCS=30kHz.

Proposal 10: RAN4 set the CQI/RI/PMI delay according to the number of CSI-RS ports and CSI processing capabilities.

Proposal 11: No PMI reporting requirements with Rel-19 refined Type-I multi-panel codebook supporting more than 32 CSI-RS ports.

Proposal 12: Define PMI reporting requirements with Rel-19 refined eType-II codebook with 64 CSI-RS ports, where (N1, N2) = (8, 4).

Proposal 13: For the PMI reporting requirements with Rel-19 refined eType-II codebook, reuse the test metric for eType-II PMI reporting requirements, that is, throughput ratio between the following PMI and random Rel-19 refined Type-I PMI Mode A.

Proposal 14: For the PMI reporting requirements with Rel-19 refined eType-II codebook, reuse the test parameters of the existing Rel-16 eType-II PMI reporting requirements with 16 CSI-RS ports as baseline, e.g., TDLA30-5, MCS20 with rank 2, Codebook Configuration.

Proposal 15: Define the Rel-19 refined eType-II codebook PMI reporting requirements for 2Rx/4Rx UEs with FR1 FDD SCS=15kHz and FR1 TDD SCS=30kHz.

Proposal 16: No CRI reporting requirements for Rel-19 MIMO Phase 5 WI.

# References

- RP-242394, “WID revision: NR MIMO Phase 5”, Samsung, RAN#105, September 2024, Melbourne, Australia.

- R1-2501746, “FL Summary Support for 3TX CB-based Uplink; First Round”, April 2025.

- 3GPP TS 38.104, “NR; Base Station (BS) radio transmission and reception”.

- R1-2502977, “Updated RAN1 UE features list for Rel-19 NR after RAN1 #120bis”, RAN1.