3GPP TSG-RAN WG4 Meeting #115	R4-2506633

St Julian’s, Malta, 19th – 23rd May 2025

Source:	Ericsson

Title:	Discussion on Rel-19 NR NTN Phase 3 SAN demodulation requirements

Agenda item:	7.31.7

Document for:	Discussion

# Introduction

- Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4]

- Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used

  - Specify necessary signalling

  - Update RF requirements accordingly, if needed

- Notes for this objective:

  - The enhancement is not targeting improvements/impacts of MU-MIMO capability

  - The enhancement is not targeted to PUSCH DMRS

  - No enhancement for initial access

  - Enhancements to PRACH are not in scope.

  - This feature may be applicable for UEs operating in terrestrial networks based on a common design

  - Enhancements to PRACH are not in scope.

  - This feature may be applicable for UEs operating in terrestrial networks based on a common design

In RAN#104, a revised WID [1] on NR NTN enhancements was endorsed for Release 19. The objectives regarding SAN demodulation requirements are copied below:

The objectives for UE demodulation are mainly on the support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands.

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1]

- For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4]

- For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1]

  - Enhancements are targeted for the following HD collisions cases (cases 3 and 4):

    - Semi-statically configured DL reception collides with semi-statically configured UL transmission

    - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission

  - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded.

  - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution.

- Notes for this objective:

  - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

In this meeting, we kick-off the discussion by providing the general view on:

-	Uplink capacity/throughput enhancement for FR1-NTN.

-	Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1

NTN bands.

# SAN Demodulation Requirements

This WI focus on optimized capacity performance on uplink through multiplexing techniques, ensuring improved system capacity for large number of UEs under NTN coverage, especially for LEO where rapid access and release of satellite resources are needed. Based on the discussion, agreement summary was made in RAN1#120bis [2]. Some of the agreements are copied as follows:

RAN1#120 agreements

For OCC time synchronization / alignment of multiplexed UEs to maintain orthogonality of the codes used for OCC, OCC group alignment is handled by network scheduling without specification impact.

For RV cycling for OCC with DG-PUSCH, support Option 1

- Option 1: RV cycling is used across OCC groups.

  - Note: when option 1 is applied, RV cycling is applied when the number of repetitions is greater than the OCC length

No optimization in Rel-19 for pairing UEs with OCC2 and UEs with OCC4.

RAN1#119 agreements

- Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs.

- Support Option 1: inter-slot OCC with OCC length 4 to multiplex up to 4 UEs using Hadamard sequences

- RAN1 does not pursue Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs.

- RAN1 does not pursue Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs.

Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4.

Note 2: gNB can ensure the performance of Option 1 by UE grouping with similar CFO (e.g. maximum differential CFO of 50 or 100 Hz or 200 Hz). Without CFO grouping (e.g. maximum differential CFO of 400 Hz), the performance of option 1 is degraded by at least 1 dB in several cases. For CFO grouping, several companies in RAN1 state that CFO grouping is feasible based on network implementation without any new specification impact.

- RAN1 assumes no specification impact for CFO grouping

- RAN1 does not pursue closed-loop frequency adjustment commands.

- RAN1 assumes that RAN4 does not define new UE requirements for CFO.

2.1 General

In the current FR1 NTN PUSCH requirements specified in TS 38.108, Orthogonal Cover Codes (OCC) have not been considered, as the requirements are defined for a 1Tx and 1/2 Rx antenna configuration, which involves single-layer transmission where OCC is not needed. However, as outlined in the WID [1], multiplexing of 2 or 4 UEs with PUSCH repetitions over the same time-frequency resources is now under consideration. To support such multiplexing while maintaining signal orthogonality and minimizing inter-UE interference, it is necessary to define PUSCH demodulation requirements at the Satellite Access Node (SAN) that include OCC. Additionally, the SAN baseband processing must support OCC de-spreading to correctly separate and decode the multiplexed UE transmissions.

Proposal 1: Consider defining SAN PUSCH demodulation requirements for DFT-s-OFDM with orthogonal cover codes.

Further, as per the WID [1], one of the objectives of this WI is support of Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands. However, there is no SAN impact for eRedCap.

Proposal 2: No SAN impact for the support of eRedCap UEs operating in FR1-NTN bands.

Regarding OCC, as per the agreement in RAN1, OCC length 2 with inter-slot OCC to multiplex up to 2 UEs and OCC length 4 to multiplex up to 4 UEs, are considered using Hadamard sequences, i.e.,

For OCC2 (length 2):

- [+1, +1]

- [+1, -1]

For OCC4 (length 4):

- [+1, +1, +1, +1]

- [+1, -1, +1, -1]

- [+1, +1, -1, -1]

- [+1, -1, -1, +1]

Observation 1: Following RAN1 agreements, RAN4 should also consider Hadamard sequences to generate orthogonal cover codes. Intra-symbol pre-DFT OCC need not to considered.

According to the working assumption in RAN1, the agreed scheme for OCC length 2 and 4 for inter-slot OCC is illustrated in Figure 1 and 2, respectively.

Figure 1: Schematic view of inter-slot OCC length 2

Figure 2: Schematic view of inter-slot OCC length 4

2.2 Scope of the Requirement

Regarding the test scope, it would be practical to begin with the existing SAN PUSCH repetition requirements defined in TS 38.108, while incorporating OCC for DFT-s-OFDM PUSCH to enable multiplexing of 2 or 4 UEs when PUSCH repetitions are used.

Observation 2: Current SAN Requirements for PUSCH repetition Type A are with disabled transform precoding. However, as per the WID, transform precoding may be enabled.

Proposal 3: Consider SAN PUSCH repetition type A assumptions, as a starting point to define requirements with OCC.

As discussed in RAN1, inter-OCC group frequency hopping demonstrated up to 3 dB SNR gain at the target BLER under non-line-of-sight (non-LoS) NTN-TDL-A channels. Although frequency hopping for PUSCH repetition was not considered in RAN4, the notable performance improvement highlighted by RAN1 suggests that RAN4 could re-evaluate the potential benefits and consider performance assessments with frequency hopping enabled.

Observation 3: Inter-OCC group frequency hopping may be considered for SAN PUSCH repetition Type A requirements, considering significant gains observed in RAN1.

Further, as per RAN1 discussion, OCC groups are defined, where UEs are grouped based on the carrier-frequency-offset (CFO), starting from the UE with the lowest CFO and continue pairing them in groups of 4 UEs until all UEs are assigned. It was observed that with CFO grouping, the CFO gap within an OCC group can be reduced significantly, considering CFO drift. gNB can ensure the performance of inter-slot OCC by UE grouping with similar CFO (e.g. maximum differential CFO of 50 or 100 Hz or 200 Hz). Also, the starting offset of CFO within the OCC group between different UEs may have a performance impact.

As for current SAN PUSCH repetition type A requirements, the aggregation factor considered is 2, according to TS 38.108. But for the considered OCC-length, RAN4 needs to check, how many repetitions can be feasible. Although, there are no agreements the number of PUSCH repetitions yet, but as observed in RAN1 simulations, OCC works better with higher number of repetitions, typically in the multiples of OCC length [3]. Further, RAN1 discussed on redundancy version (RV) value to have same RV value is used in one OCC group (i.e., OCC length applied to N slots), or if, RV cycling can be additionally used across OCC groups. This can be furtherly checked, when RAN1 reaches a conclusion.

Observation 4: It may be needed to investigate on:

- The impact of CFO drifting on PUSCH performance test.

- The impact of starting offset of CFO within OCC group on PUSCH performance test.

- The number of repetitions feasible for the considered OCC-length and if there is a performance difference seen, for SAN PUSCH repetition type A requirements.

- RV sequence to be used.

Further, RAN1 also considered OCC with transport-block-over-multiple-slots (TBoMS), UCI multiplexing and OCC for PUSCH with 1 PRB. However, since no consensus is obtained in these techniques, RAN4 do not need to analyze them until RAN1 reaches an agreement. Furthermore, companies discussed that phase continuity and power consistency should be maintained during an OCC group. However, RAN4 RF have not been concluded on this issue.

Observation 5: RAN4 SAN demodulation should wait for conclusion on phase and power consistency within OCC group issue in RAN4 RF discussion.

2.3 Simulation Assumptions

As mentioned in proposal 2, we can consider PUSCH repetition type A assumptions for initial simulations focusing on the CP-OFDM and DFT-s-OFDM waveforms, PUSCH aggregation factor based on OCC length, the RV sequence, and frequency hopping.

Proposal 4: Consider simulation assumptions in Table 2.3-1 as a starting point for performance requirements of PUSCH repetition type A with OCC.

Table: 2.3-1 Test parameters for testing PUSCH repetition Type A with OCC

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled/Enabled |
| Channel bandwidth |  | 15kHz SCS: 5MHz 30kHz SCS: 10MHz |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0, 3 [Note 1] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n2, n4 and n8 for OCC-length 2 n4, n8 and n16 for OCC-length 4 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled/Enabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 1: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

# UE Demodulation Requirements

In this section, we discussed the UE demodulation requirements for adding the support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1 NTN bands.

## 3.1	Test scope

For Rel-17 RedCap, following reduced capabilities were considered:

- Reduced maximum UE bandwidth

  - Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20MHz.

  - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100MHz.

- Reduced minimum number of Rx branches

  - For frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1.

  - For frequency bands where a legacy NR UE is required to be equipped with a minimum of 4 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1.

- Maximum number of DL MIMO layers

  - For a RedCap UE with 1 Rx branch, 1 DL MIMO layer is supported.

  - For a RedCap UE with 2 Rx branches, 2 DL MIMO layers are supported.

- Relaxed maximum modulation order

  - Support of 256QAM in DL is optional (instead of mandatory) for an FR1 RedCap UE, that is, the maximum modulation order for RedCap UE is 64QAM.

- Duplex operation

  - HD-FDD type A with the minimum specification impact (Note that FD-FDD and TDD are also supported.)

- CA/DC is not supported, SA only

In Rel-17, RAN4 has defined PDSCH requirements for both HD-FDD and FD-FDD, together with TDD requirements for 1Rx and 2Rx. RAN4 also defined PDCCH, PBCH requirements for RedCap 1Rx UEs.

For Rel-18 eRedCap, RAN4 only defined additional PDSCH requirements since the difference between RedCap and eRedCap is that the peak rate and/or PDSCH BW of eRedCap is limited, and for PDCCH/PBCH, the eRedCap UE should be able to receive with 20MHz CBW, RAN4 can reuse the requirements specified in Rel-17 RedCap.

For Rel-19 NTN enhancement, RedCap and eRedCap UEs need to first pass the corresponding Rel-17, Rel-18 PDSCH/PDCCH/PBCH requirements according to the UE capability, then some additional requirements configured with NTN channel model should be passed as well. Thus, as for the test scope of UE demodulation requirements, we propose to define additional PDSCH requirements with QPKS/16QAM MCS for FDD/HD-FDD SCS=15kHz.

In Rel-18 eRedCap, RAN4 defined two sets of PDSCH demodulation requirements according to capability 1) with reduced baseband bandwidth (FG 48-1) and 2) without reduced baseband bandwidth (FG 48-2). Since FG 48-2 is the capability that the UE receives PDSCH with 10MHz, but the peak rate is limited to 10 Mbps, we think this is close the Rel-17 RedCap. Therefore, we propose to focus on ‘with reduced baseband bandwidth’ for NTN scenario.

Proposal 5: Define PDSCH requirements for NTN+RedCap and NTN+eRedCap, i.e.

|  | NTN+RedCap | NTN+eRedCap |
| --- | --- | --- |
| FR1 HD-FDD, FDD | SCS=15kHz, CBW=10MHz, 1Rx/2Rx, NTN channel model | SCS=15kHz: CBW=10MHz with reduced baseband bandwidth, peak rate ≤ 10,000 bits, 1Rx/2Rx, NTN channel model |

Proposal 6: Rel-17 RedCap FR1 PDCCH requirements are also applicable for Rel-19 NTN eRedCap UE.

Proposal 7: Rel-17 RedCap FR1 PBCH requirements are also applicable for Rel-19 NTN eRedCap UE.

Proposal 8: Do not consider SAN requirements for the support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1 NTN bands.

## 3.2	PDSCH requirements

Since the target of Rel-19 NTN enhancement is to add support of NTN to RedCap and eRedCap UEs, it is very important that those UE to pass the legacy RedCap and eRedCap PDSCH requirements. On top of that, the PDSCH performance under the NTN channel model should also be verified.

For RedCap UEs, we propose the following PDSCH test cases:

| Test num. | Reference channel (Refer to TS 38.101-4) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
|  | R.PDSCH.1-1 FDD R.PDSCH.1-1.1 HD-FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x1, ULA Low | 70 |  |
|  | R.PDSCH.1-1.1 FDD R.PDSCH. 1-1.1 HD-FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 |  |
|  | R.PDSCH.1-2.1 FDD R.PDSCH. 1-1.2 HD-FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x1, ULA Low | 70 |  |
|  | R.PDSCH.1-2.1 FDD R.PDSCH. 1-1.2 HD-FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x2, ULA Low | 70 |  |

For eRedCap UEs, we propose the following PDSCH test cases (with reduced baseband bandwidth):

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
|  | R.PDSCH.1-1.5 FDD R.PDSCH.1-2.2 HD-FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x1, ULA Low | 70 |  |
|  | R.PDSCH.1-1.5 FDD R.PDSCH.1-2.2 HD-FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 |  |
|  | R.PDSCH.1-25.1 FDD R.PDSCH.1-2.3 HD-FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x1, ULA Low | 70 |  |
|  | R.PDSCH.1-25.1 FDD R.PDSCH.1-2.3 HD-FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x2, ULA Low | 70 |  |

Proposal 9: Define PDSCH demodulation requirements with the following test setup:

| Test num. | Reference channel (Refer to TS 38.101-4) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
|  | R.PDSCH.1-1 FDD R.PDSCH.1-1.1 HD-FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x1, ULA Low | 70 |  |
|  | R.PDSCH.1-1.1 FDD R.PDSCH. 1-1.1 HD-FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 |  |
|  | R.PDSCH.1-2.1 FDD R.PDSCH. 1-1.2 HD-FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x1, ULA Low | 70 |  |
|  | R.PDSCH.1-2.1 FDD R.PDSCH. 1-1.2 HD-FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x2, ULA Low | 70 |  |

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
|  | R.PDSCH.1-1.5 FDD R.PDSCH.1-2.2 HD-FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x1, ULA Low | 70 |  |
|  | R.PDSCH.1-1.5 FDD R.PDSCH.1-2.2 HD-FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 |  |
|  | R.PDSCH.1-25.1 FDD R.PDSCH.1-2.3 HD-FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x1, ULA Low | 70 |  |
|  | R.PDSCH.1-25.1 FDD R.PDSCH.1-2.3 HD-FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x2, ULA Low | 70 |  |

In the meantime, we found that it is worth to have discussions on the feasibility of implementing the NGSO channel model for the potential Rel-19 NTN enhancement requirements. RAN4 discuss whether the NGSO channel model is applied for (e)RedCap UE in NTN. We are open to consider it if it finally concludes with no additional margin to be added.

Proposal 10: Open to consider applying the NGSO model for Rel-19 NTN enhancement requirements to (e)RedCap UE PDSCH demodulation requirements.

# UE CSI reporting requirements

RAN4 haven’t defined any CSI reporting requirement for NR NTN before considering that there is large Satellite-UE RTT delay (order of tens of milliseconds delay for both LEO-600 and LEO-1200), It is difficult to guarantee that the reporting CQI is still valid. Even if we assume that CQI reporting values remain valid in certain cases with larger tilt angles and lower orbital altitudes, the overall CQI reporting performance in these NTN scenarios can still be effectively supported by the existing TN-based CQI reporting mechanisms.

Considering the reasoning above, we propose not to add any CSI reporting requirement for NTN + RedCap and NTN+eRedCap in TS 38.101-5.

Proposal 11: Do not define new CSI reporting requirement for (e)RedCap UE in Rel-19 NTN enhancement.

Proposal 12: Existing CSI reporting requirements for RedCap in Rel-17 and eRedCap in Rel-18 should be applied for Rel-19 NTN access capable (e)RedCap UE according to the UE capability.

# Conclusion

  - SAN Demodulation Requirements

Based on the above discussion following observations and proposals are captured:

Proposal 1: Consider defining SAN PUSCH demodulation requirements for DFT-s-OFDM with orthogonal cover codes.

Proposal 2: No SAN impact for the support of eRedCap UEs operating in FR1-NTN bands.

Observation 1: Following RAN1 agreements, RAN4 should also consider Hadamard sequences to generate orthogonal cover codes. Intra-symbol pre-DFT OCC need not to considered.

Observation 2: Current SAN Requirements for PUSCH repetition Type A are with disabled transform precoding. However, as per the WID, transform precoding may be enabled.

Proposal 3: Consider SAN PUSCH repetition type Aassumptions, as a starting point to define requirements with OCC.

Observation 3: Inter-OCC group frequency hopping may be considered for SAN PUSCH repetition Type A requirements, considering significant gains observed in RAN1.

Observation 4: It may be needed to investigate on:

- The impact of CFO drifting on PUSCH performance test.

- The impact of starting offset of CFO within OCC group on PUSCH performance test.

- The number of repetitions feasible for the considered OCC-length and if there is a performance difference seen, for SAN PUSCH repetition type A requirements.

- RV sequence to be used.

Observation 5: RAN4 SAN demodulation should wait for conclusion on phase and power consistency within OCC group issue in RAN4 RF discussion.

Proposal 4: Consider simulation assumptions in Table 2.3-1 as a starting point for performance requirements of PUSCH repetition type A with OCC.

5.2   UE Demodulation Requirements

Based on the above discussion following proposals are captured:

Proposal 5: Define PDSCH requirements for NTN+RedCap and NTN+eRedCap, i.e.

|  | NTN+RedCap | NTN+eRedCap |
| --- | --- | --- |
| FR1 HD-FDD, FDD | SCS=15kHz, CBW=10MHz, 1Rx/2Rx, NTN channel model | SCS=15kHz: CBW=10MHz with reduced baseband bandwidth, peak rate ≤ 10,000 bits, 1Rx/2Rx, NTN channel model |

Proposal 6: Rel-17 RedCap FR1 PDCCH requirements are also applicable for Rel-19 NTN eRedCap UE.

Proposal 7: Rel-17 RedCap FR1 PBCH requirements are also applicable for Rel-19 NTN eRedCap UE.

Proposal 8: Do not consider SAN requirements for the support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1 NTN bands.

Proposal 9: Define PDSCH demodulation requirements with the following test setup:

Proposal 10: Open to consider applying the NGSO model for Rel-19 NTN enhancement requirements to (e)RedCap UE PDSCH demodulation requirements.

Proposal 11: Do not define new CSI reporting requirement for (e)RedCap UE in Rel-19 NTN enhancement.

Proposal 12: Existing CSI reporting requirements for RedCap in Rel-17 and eRedCap in Rel-18 should be applied for Rel-19 NTN access capable (e)RedCap UE according to the UE capability.

# References

RP-243300, Moderator (Thales), New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN#106, Madrid, Spain, December 9-12, 2024.

R1-2502983, “Feature lead summary #1 of AI 9.11.3 on NR-NTN uplink capacity and throughput”, MediaTek, Wuhan, China

R1-2408686. “On uplink capacity/cell throughput enhancement for NR NTN”, Ericsson, RAN1#118bis, Hefei, China.