3GPP TSG-RAN WG4 Meeting #115	   R4-2506666

# St.Julian’s, Malta, 12th – 19th , May , 2025

Agenda Item:	7.25.4.1

Source: 	Ericsson

Title:	Discussion on RRM requirements for UE-to-UE CLI handling (SBFD)

Document for:	Discussion

# Introduction

The agreements on the SBFD CLI handling and SBFD operation during RAN4#114 bis meeting are captured in the approved WF [1].

In this paper we provide our view on the listed open issues on RRM core requirement within related WID [2] objectives:

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

# Discussion

# 2.1 UE to UE CLI handling

### L1-SRS-RSRP measurement

#### Issue 1-2-2: Scheduling restriction - Agreement (online session on Monday with update) - For UL, - Scheduling restrictions always apply, for both FR1 and FR2. - The restricted symbols include symbols for SRS resource and Y symbols before, where Y is re-using same number as R16 requirements. - For the symbols after the SRS resource, not define scheduling restriction on symbols after symbols for SRS resource in TS 38.133, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211. - For DL, - if UE is able to support FDM-ed DL reception and SRS measurement, scheduling restriction does not apply except when SRS resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, otherwise (if UE does not support the capability), scheduling restriction applies. - The restricted symbols include symbols for SRS resource and Y symbols before symbols for SRS resource, where Y is re-using same number as R16 requirements. - FFS whether restricted symbol(s) is needed for the symbol(s) after SRS resource to consider the switching between UL subband and DL subband. Generic assumption for defining RRM requirements, not intend to add it in the core requirement. - For all Method #1, Method #2 and Method #3, - Note: The RF BW is wide enough to cover DL active BWP, including PRBs from both DL subband(s) and UL subband that fall within the DL active BWP.

For scheduling restriction caused by the L1-SRS-RSRP measurement, the measurement is performed by the victim UE within the UL sub-band while the aggressor UE is transmitting the SRS.

At symbol x the UE is performing the SRS measurement within the UL subband, question raises whether UE can direct receive DL data at the symbol x+1.

With the generic assumption that the UE RF BW is wide enough to cover the DL active BWP, this include 2 DL subband+ UL subband in DUD configuration, and 1 DL subband and 1 UL subband. This to our understanding that the UE will have the entire active DL BWP open, receive the SRS or receive DL data at different frequency location. This is quite similar as legacy behavior considering there is no UL subband or DL subband concept, if UE has the entire RF BW open, we don’t see why UE needs to have switching time for switching from UL subband to DL subband.

- Proposal 1: No need to introduce scheduling restriction for DL data at the symbol after SRS resource.

#### Issue 1-2-3: Measurement restriction - Agreements - Measurement requirements do not apply for L1-SRS-RSRP when the measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps. - When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI, FFS the following options - Option 1: Prioritize L1-SRS-RSRP measurement - Option 2: Prioritize L1-CLI-RSSI measurement - Option 3: Leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI

Firstly, we would like to clarify whether network will configure the SRS resources overlapping with the RSSI measurement resource.

For Method #1, the RSSI measurement is being configured within the DL subband, and the SRS will be transmitted within the UL subband, from frequency domain, there is no overlapping issues. For method #3, the RSSI measurement resource is being configured within UL subband which can be partially or fully overlapping with the SRS resource in frequency domain.

Timewise, SRS resources and RSSI measurement can be periodic, semi-persistence or aperiodic, the likelihood that the two types of measurement resources overlapping remains unknown.

We see the benefit to prioritize the SRS-RSRP measurement since it is a targeted measurement towards certain aggressor UE. However, we also see the benefit of have the CLI-RSSI being measured and reported first in order to help network to figure out where the CLI issue is before network configure the victim UE to measure specific aggressor UE.

The purpose of specifying measurement restriction is that there is a consensus which measurement needs to be done first for example when comparing the priority between L1-CLI measurement and typical L1/L3 measurement.

We don’t see this kind of clear priority between the L1-SRS-RSRP and L1-CLI-RSSI measurement, they are two aspects for evaluating the CLI issue. For the simplification of the specification and realistic requirement setting point of view, the network can try to avoid the collision L1-CLI measurement itself.

- Proposal 2: The network configuration will be able to guarantee majority of the measurement resource not overlapping both in frequency and in time domain for L1-SRS-RSRP and L1-CLI RSSI.

#### Issue 1-2-1: Side conditions - Agreements - For SRS Es/Iot, further discuss the following options - Option 1: 1dB - Option 2: > 1dB - For SRS BW, further discuss the following options - Option 1: 24RB - Option 2: 48RB

From network total performance point of view, the side condition not only impacts the accuracy requirement however also impact the requirement coverage. Due to the flexible configuration of the SBFD for DUD and DU, the BW of the UL subband needs to be declared by the base station. Based on RF session agreement and existing RB configuration table from TS 38.104 table 5.3.2-1, we see there is case that the SRS BW cannot be guaranteed for more than 48 RBs for FR1.

- Proposal 3: For L1-SRS-RSRP measurement, SRS BW for setting minimum requirement shall be 24RBs to have better requirement coverage, the SNR side condition can be above 1dB.

### L1-CLI-RSSI measurement

#### Issue 1-3-2: Scheduling restriction - Method #3: - For UL, - Scheduling restrictions always apply, for both FR1 and FR2. - The restricted symbols include symbols for RSSI resource. - Do not define scheduling restriction on symbols after symbols for RSSI resource, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211 clause 4.3.2 - For DL, - Scheduling restriction does not apply except when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, the restricted symbols include symbols for RSSI resource. - Note: the common understanding in RAN4 if the RSSI in UL suband is too strong compare to the wanted signals in DL suband(s), UE may fail to decode the PDCCH or PDSCH. - FFS whether restricted symbol(s) is needed for the symbol(s) after RSSI resource to consider the switching between UL subband and DL subband. - Generic assumption for defining RRM requirements, not intend to add it in the core requirement. - For all Method #1, Method #2 and Method #3, - Note: The RF BW is wide enough to cover DL active BWP, including PRBs from both DL subband(s) and UL subband that fall within the DL active BWP.

For L1-CLI-RSSI method#3, it is rather similar as the L1-SRS-RSRP, when the measurement resources configured at symbol X within UL sub band, whether it is possible to directly receive DL data at symbol X+1 in DL subband.

Based on there is no need for UE expanding or shrinking the BWP at different OFDM symbol, this is a legacy behavior without considering whether it is UL subband or DL subband. UE continue receive either RSSI or DL data within the active DL BWP.

- Proposal 4: No need to introduce scheduling restriction for the symbol after RSSI resources for method#3.

#### Issue 1-3-3: Measurement restriction - Agreements (adhoc session on Tuesday) - Collision between L1-CLI-RSSI measurements with different methods - Capture in RAN4 specs that UE is not expected to perform CLI measurement using Method #1 and Methods#3 in the same OFDM symbol. - Collision between L1-CLI-RSSI measurements and L1/L3 RRM measurements - For Method #1& Method 3: - Measurement requirements do not apply for L1-CLI-RSSI when CLI resources and resources for L1/L3 RRM measurements overlap in time for FR2, FFS for FR1.

Based on the same reference timing for L1-CLI-RSSI and the L1/L3 RRM measurements, we do see there is potential gain to have these two types of measurements performed at the same time. For example, at FR1, there is no RX beam complication, when there is no need to re-tune to another frequency for the measurement gap, we do see the possibility for UE to perform the L1-CLI-RSSI measurement together with other colliding measurement like L1/L3 RRM measurement.

- Proposal 5: For Method #1& Method 3, measurement requirements do not apply for L1-CLI-RSSI when CLI resources are overlapping with measurement gaps at FR1.

# Summary

The following have been proposed in the current contribution:

	Proposal 1: No need to introduce scheduling restriction for DL data at the symbol after SRS resource.

	Proposal 2: The network configuration will be able to guarantee majority of the measurement resource not overlapping both in frequency and in time domain for L1-SRS-RSRP and L1-CLI RSSI.

	Proposal 3: For L1-SRS-RSRP measurement, SRS BW for setting minimum requirement shall be 24RBs to have better requirement coverage, the SNR side condition can be above 1dB.

	Proposal 4: No need to introduce scheduling restriction for the symbol after RSSI resources for method#3.

	Proposal 5: For Method #1& Method 3, measurement requirements do not apply for L1-CLI-RSSI when CLI resources are overlapping with measurement gaps at FR1.

# References

- R4-2504904 “WF on SBFD RRM”, Huawei, April ,2025

- RP-241614 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June ,2024