3GPP TSG-RAN WG4 Meeting #115	R4-2506675

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.8.6

Source:	Samsung

Title:	Discussion on UE demodulation and CSI performance requirements for 7MHz CHBW

Document for:	Discussion

- Introduction

In the RAN plenary RAN#106 meeting the WID [1] for 7MHz CHBW has been approved and further revised in RAN#107 meeting [2]. This is our first meeting on demodulation and CSI reporting performance requirements for 7MHz CHBW WI. The objectives on performance part are as below.

Specify necessary UE/BS performance requirements for NR operation in dedicated FDD FR1 spectrum allocations for irregular channel bandwidths, corresponding to the core requirements: - Specify necessary RRM performance requirements if needed (RAN4) - Specify, if necessary, UE demodulation performance and CSI reporting requirements for the 7 MHz channel bandwidth (RAN4) - Specify, if necessary, BS demodulation performance requirements (RAN4) - Specify necessary BS conformance tests (RAN4)

In this contribution, we provide our analysis and views on the necessity of introducing UE demodulation and CSI reporting requirements for the 7 MHz channel bandwidth.

- UE demodulation performance requirements

Whether to define 7MHz PDSCH/PDCCH/PBCH demodulation performance requirements for single carrier scenario

Currently, the PDSCH/PDCCH/PBCH demodulation performance requirements for single carrier scenario focus on typical channel bandwidth, i.e., 10MHz for FR1 FDD and 40MHz for FR1 TDD.

For the scope of 7MHz channel bandwidth, following agreements achieved on RAN4#114 meeting, in which, 7MHz channel bandwidth is only for FDD bands that also support 5MHz channel bandwidth.

Scope: Agreement: - The 7 MHz channel bandwidth is only applicable to FDD bands that also support 5 MHz channel bandwidth. - In-band NB-IoT can be supported in 7MHz channel BW

In last RAN4#114bis meeting, the draft CR was endorsed [3] which only introduce 7MHz on band n5 and n26. It is clearly that the bands support 7MHz also support 10MHz channel bandwidth. We believe the UE support 7MHz could also support 10MHz channel bandwidth. Hence it is not necessary to introduce 7MHz PDSCH/PDCCH/PBCH demodulation performance requirements for single carrier scenario.

Channel bandwidths for each NR band NR Band; SCS (kHz); UE Channel bandwidth (MHz). 3; 5; 7; 10; 15; 20; 25; 30; 35; 40; 45; 50; 60; 70; 80; 90; 100. n5; 15; 5; 7; 10; 15; 20; 253. 30; 10; 15; 20; 253. 60. n26; 15; 34; 5; 7; 10; 15; 20; 253; 303. 30; 10; 15; 20; 253; 303.

Observation 1: Only band n5 and n26 support 7MHz channel bandwidth, and the band support 7MHz also support 10MHz channel bandwidth.

Proposal 1: It is not necessary to introduce 7MHz PDSCH/PDCCH/PBCH demodulation performance requirements for single carrier scenario.

Whether to define 7MHz PDSCH demodulation performance requirements for CA scenario

For the CA scenario, follow below existing applicability rule, 7MHz bandwidth is reachable even with low probability. Hence it is better to introduce 7MHz PDSCH demodulation performance requirements for CA scenario.

- Existing applicability rules for inter-band CA configuration selection: - Step 1: Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. - Step 2: Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. - Step 3: Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. - Step 4: Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3.

Observation 2: For the CA scenario, follow existing applicability rule, PDSCH demodulation performance 7MHz bandwidth is reachable even with low probability.

Proposal 2: Prefer to introduce FR1 FDD 7MHz PDSCH demodulation performance requirements for CA scenario.

Whether to define SDR requirements for 7MHz CHBW

If PDSCH CA demodulation requirements for 7MHz channel bandwidth will be defined in Rel-19, corresponding configuration for SDR requirements in Table 5.5A-1 and Table 5.5A-4 should be updated accordingly.

Proposal 3: Prefer to update corresponding configuration for SDR requirements in Table 5.5A-1 and Table 5.5A-4 to include 7MHz, if PDSCH CA demodulation requirements for 7MHz channel bandwidth are agreed be defined in Rel-19.

- UE CSI reporting requirements

Whether to define 7MHz CSI reporting requirements for single carrier scenario

Similar logic as 7MHz demodulation requirements for single carrier scenario, it is not necessary to define 7MHz CSI reporting requirements for single carrier scenario.

Proposal 4: It is not necessary to define 7MHz CSI reporting requirements for single carrier scenario.

Whether to define CA CQI reporting requirements for 7MHz CHBW

Similar as PDSCH CA demodulation requirements and SDR requirements for 7MHz channel bandwidth, If PDSCH CA demodulation requirements for 7MHz channel bandwidth will be defined in Rel-19, corresponding configuration for CA CQI requirements in 38.101-4 Table 6.1.2-1 and Table 6.2A.3.1.1-1 should be updated accordingly.

Proposal 5: Prefer to update corresponding configuration for CA CQI requirements in 38.101-4 Table 6.1.2-1 and Table 6.2A.3.1.1-1 to include 7MHz, if PDSCH CA demodulation requirements for 7MHz channel bandwidth are agreed to be defined in Rel-19.

- Conclusion

In this contribution, we provide analysis and views on UE demodulation performance and CSI requirements for 7MHz CHBW WI.

The proposals and observations could be summarized as:

Observation 1: Only band n5 and n26 support 7MHz channel bandwidth, and the band support 7MHz also support 10MHz channel bandwidth.

Proposal 1: It is not necessary to introduce 7MHz PDSCH/PDCCH/PBCH demodulation performance requirements for single carrier scenario.

Observation 2: For the CA scenario, follow existing applicability rule, PDSCH demodulation performance 7MHz bandwidth is reachable even with low probability.

Proposal 2: Prefer to introduce FR1 FDD 7MHz PDSCH demodulation performance requirements for CA scenario.

Proposal 3: Prefer to update corresponding configuration for SDR requirements in Table 5.5A-1 and Table 5.5A-4 to include 7MHz, if PDSCH CA demodulation requirements for 7MHz channel bandwidth are agreed be defined in Rel-19.

Proposal 4: It is not necessary to define 7MHz CSI reporting requirements for single carrier scenario.

Proposal 5: Prefer to update corresponding configuration for CA CQI requirements in 38.101-4 Table 6.1.2-1 and Table 6.2A.3.1.1-1 to include 7MHz, if PDSCH CA demodulation requirements for 7MHz channel bandwidth are agreed to be defined in Rel-19.

- Reference

[1] RP-243323, New WID on 7 MHz Channel Bandwidth for n26 and n5, RAN#106 meeting, China Telecom, T-Mobile USA

[2] RP-250373, Revised WID on NR FR1 7 MHz Channel Bandwidth, RAN#107 meeting, T-Mobile USA, China Telecom

[3] R4-2505136, Introduction of 7 MHz channel bandwidth, RAN4#114bis meeting, Nokia