3GPP TSG-RAN WG4 Meeting #115	R4-2506683

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.27.5

Source:	Samsung

Title:	Discussion on UE demodulation and CSI performance requirements for Rel-19 NES

Document for:	Discussion

- Introduction

The WID [1] for “New WID: Enhancements of network energy savings for NR” has been approved in the RAN#102 meeting and updated in the RAN#107 meeting [2], in which the performance part starts from RAN4#115 in May 2025 and targeting to complete by RAN#111 in March 2026. The objectives for core part are as below:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

Accordingly, the objective for performance part is

- Specify corresponding performance requirements and test cases for the network energy saving techniques [RAN4].

In this contribution, we share our views and analysis on the general test scope for demodulation and CSI requirements of Rel-19 NES.

- Discussion

The objectives for Rel-19 NES could be summarized as

Objective 1: on-demand SSB SCell operation

Objective 2: on-demand SIB1 for UEs in idle/inactive mode

Objective 3: adaptation of common signal/channel transmissions

  - PDSCH requirements

Whether to define new PDSCH/PDCCH/PBCH demodulation performance requirements for Rel-19 NES

From RAN1 discussion progress and the WID description [2], we know that objective 1 focus on the on-demand SSB based SCell activation/deactivation signaling, and SCell time/frequency synchronization, L1/L3 measurements and SCell activation procedure, which has no impact on PDSCH/PDCCH/PBCH demodulation performance. The objective 2 is targeting on idle/inactive mode, which clearly do no has impact on PDSCH/PDCCH/PBCH demodulation performance. The objective 3 is about SSB/PRACH/Paging adaptation, the time/frequency domain resources for these common signal/channel transmissions may change, however, the demodulation performance requirements generally based on some typical configurations without time/frequency domain resources variation. Therefore, there is no need to define new PDSCH/PDCCH/PBCH demodulation performance requirements for Rel-19 NES.

Proposal 1: Do not define new PDSCH/PDCCH/PBCH demodulation requirements for Rel-19 NES.

  - CSI requirements

Whether to define CSI reporting requirements for Rel-19 NES

Since there are no new codebook type and CQI report mode introduced for NES topic in Rel-19, and also no new PMI and CQI signal processing needed for NES topic, thus we don’t prefer to introduce CSI reporting requirements for Rel-19 NES.

Proposal 2: Do not define new CSI reporting requirements for Rel-19 NES.

- Conclusion

In this contribution, we provide analysis and views on the general test scope for demodulation and CSI requirements of Rel-19 NES. The proposals could be summarized as:

Proposal 1: Do not define new PDSCH/PDCCH/PBCH demodulation requirements for Rel-19 NES.

Proposal 2: Do not define new CSI reporting requirements for Rel-19 NES.

- Reference

[1] RP-234065, New WID: Enhancements of network energy savings for NR, Ericsson, RAN#102 meeting

[2] RP-250343, Revised WID on enhancements of network energy savings for NR, Ericsson, RAN#107 meeting