3GPP TSG-RAN WG4 Meeting #115	R4-2506728

Malta, MT, 19th – 23rd May, 2025

Title: 	Discussion on RRM requirements for supporting intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s)

Source: 	Huawei, HiSilicon

Agenda item:	7.4.3

Document for:	Discussion

Introduction

Supporting of intra-band non-collocated EN-DC/NR-CA deployment for Type 2 UE has been completed in Rel-18. The requirements for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and FR1 intra-band non-contiguous NR-CA for Type 2 UE were specified.

The new R19 WI is to further support intra-band non-collocated EN-DC/NR-CA deployment scenarios for Type 4 FWA UE. The objectives of the WID are duplicated as below [1].

- 4.1 Objective of SI or Core part WI or Testing part WI The core part of the work item includes: - Specify the core requirements for type 4a/4b capable FWA UE that supports non-co-located scenarios for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and/or FR1 intra-band non-contiguous NR-CA for up to 4-layer DL MIMO per CC, where MRTD>CP is assumed. - RF power imbalance requirements of up to 25dB - RRM requirements including MTTD and MRTD requirements and other identified requirement such as scheduling restriction/availability, SCell operation related delay and interruption. - Work is limited to EN-DC/NR-CA for bands 42, n77/n78 - Discuss and if needed, specify the UE behavior when the UE supports type 4a/b but operates in co-located scenarios depending on whether network signaling is provided. R18 handling of UE behavior can be used as a reference.

In RRM session, in RAN4#112bis meeting, the impacted RRM core requirements were identified in [R4-2416850], and in RAN4#113 meeting, CR split among companies was approved in [R4-2420090]. In RAN4#114bis meeting, the draft CRs from companies were endorsed in RAN4#114bis meeting and it was agreed that further update the UE capability and BS signalling based on RF/RAN2 conclusion.

In RF session, regarding UE capability and network signalling for Type 4 UE, one LS was approved to send to RAN2. The contribution is to discuss the principle how to capture these R18 UE capability and BS signalling to RRM requirements.

Discussion

Background of UE capability and new BS signalling for Type 4

In LS R4-2505161, UE capability for Type 4a/4b has reached completed conclusion.

UE capability Type - For intra-band non-contiguous NR-CA, Type 4b is considered, Type 4a is not considered - For inter-band non-contiguous EN-DC with overlapping or partially overlapping bands, both Type 4a and Type 4b are considered UE requirements: - Reuse the power imbalance of Type 2 - Reuse the MRTD and MTTD requirements of Type 2 UE capability: - Define one capability for NR CA and one capability for EN-DC - For EN-DC, Type 4a and Type 4b capability can be differentiated by legacy capability, such as MIMO-CapabilityDL-r10 on E-UTRA carrier - If UE reports capability for Type 4a/4b, Type 2 capability shall be deemed as supported by default, and UE still needs to report Type 2 capability if UE supports it in order to keep backward compatibility (i.e., for legacy BS)

In summary, one UE capability is introduced for NR CA and one capability is introduced for EN-DC. The Type 4 capable UE by default supports Type 2, and the UE shall report Type 2 capability as well.

Based on the approved LS R4-2505161 and agreements in previous meeting, the BS signalling is summarized as below.

BS signalling - No new NW signaling needed from Type 4 to single carrier fallback. - When maxMIMO-layers > 4, Type 1(collocated) requirements apply. No new BS signaling is needed - When maxMIMO-layers=2, UE follow Rel-18 specification. No new BS signaling is needed - If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present, UE is Type 1. - If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent, UE is Type 2. - When maxMIMO-layers=4, new BS signalling(s) is needed for NR-CA and EN-DC, and the details are up to RAN2 NOTE: maxMIMO-layers intends for the legacy BS signaling

For maxMIMO-layers=4 case, RAN4 didn’t define specific switching behaviour (such as when IE is present switch from type 4 to type 1 or the inverse, when IE is present switch from type 1 to type 4) and leave RAN2 to decide.

- 2.2 RRM requirements

Before discussing RRM requirements for R19 non-collocated, it’s worth pointing out that R18 BS signalling nonCollocatedTypeNR-CA/ nonCollocatedTypeMRDC is only present for a UE configured with maxMIMO-layers ≤2. For the case UE configured with maxMIMO-layers =4, it is also collocated, however no any related description in RAN2 and RAN4 specification (see below RAN2 and RAN4 context).

RAN2 spec nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15].. nonCollocatedTypeMRDC This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements.. RAN4 RRM spec (take BFD requirements as an example) - 8.5.5 Requirements for SSB based candidate beam detection #### 8.5.5.1 Introduction The requirements in this clause apply for each SSB resource in the set q1 configured for a serving cell, provided that the SSBs configured for candidate beam detection are actually transmitted within UE active DL BWP during the entire evaluation period specified in clause 8.5.5.2 unless the UE supports bwpOperationMeasWithoutInterrupt-r18, provided that the SSB is within the configured UE-specific CBW. The requirements in this clause apply when UE is required to perform candidate beam detection on no more than 1 serving cell per band unless otherwise specified. For UE supporting intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided for the configured FR1 intra-band non-contiguous CA, the requirements in this clause apply when UE is required to perform candidate beam detection on no more than 2 serving cells per band if these 2 serving cells are in non-contiguous carriers, and no more than 1 serving cell per band otherwise..

In R19, the same principle as in R18 can be followed, that is, no description on UE behaviour when maxMIMO-layer＞4 (although it is regarded as collocated).

Proposal 1: When defining requirements for R19 non-collocated WI, the same principle as in R18 is followed, that is, there is no description on UE behaviour when maxMIMO-layer＞4 (it is collocated).

As per the latest RF agreements, when maxMIMO-layers=2, UE follow Rel-18 specification. No new BS signaling is needed, that is:

  - If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present, UE is Type 1.

  - If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent, UE is Type 2.

The case is clear and the corresponding requirements have been already captured in R18 requirements.

Observation 1: The requirements for the case when maxMIMO-layers=2 has already been captured in R18 non-collocated RRM requirements.

As per the latest RF agreements, how to design R19 BS signalling for the case maxMIMO-layers=4 doesn’t reach consensus in RAN4. Whether the BS signalling uses the default UE behaviour way or the way of “absent, present with value 0, present with value 1” is left to RAN2. Anyway, we believe the final BS signalling can clearly outline each status (collocated/non-collocated) when maxMIMO-layers=4.

When defining requirements for R19 non-collocated, the same principle as R18 shall be followed, that’s, RRM requirements focus on UE behaviour when maxMIMO-layers=4. As RAN2 IE is not decided yet, companies can use general description of UE capabilities and BS signalling in RRM requirements:

UE supporting [interBandMRDC-WithOverlapDL-Band-r19]

UE supporting [intraBandNR-CA-non-collocated-r19]

[R19 type 4 capable UE is in non-collocated condition]

[R19 type 4 capable UE is in collocated condition]

The above description will be further updated according to RAN2 agreements.

Proposal 2: When defining requirements for R19 non-collocated WI, the same principle as R18 shall be followed, that’s, RRM requirements are defined when maxMIMO-layers =4.

Proposal 3: For drafting RRM CRs, companies can use general description of UE capabilities and BS signalling:

-UE supporting [interBandMRDC-WithOverlapDL-Band-r19]

-UE supporting [intraBandNR-CA-non-collocated-r19]

- [R19 type 4 capable UE is in non-collocated condition]

- [R19 type 4 capable UE is in collocated condition]

The above description will be further updated according to RAN2 agreements.

Conclusion

The contribution provides some analysis for supporting intra-band non-collocated EN-DC/NR-CA deployment scenarios from RRM requirements perspective.

Proposal 1: When defining requirements for R19 non-collocated WI, the same principle as in R18 is followed, that is, there is no description on UE behaviour when maxMIMO-layer＞4 (it is collocated).

Observation 1: The corresponding requirements for the case when maxMIMO-layers=2 has already been captured in R18 non-collocated RRM requirements.

Proposal 2: When defining requirements for R19 non-collocated WI, the same principle as R18 shall be followed, that’s, RRM requirements are defined when maxMIMO-layers =4.

Proposal 3: For drafting RRM CRs, companies can use general description of UE capabilities and BS signalling:

-UE supporting [interBandMRDC-WithOverlapDL-Band-r19]

-UE supporting [intraBandNR-CA-non-collocated-r19]

- [R19 type 4 capable UE is in non-collocated condition]

- [R19 type 4 capable UE is in collocated condition]

The above description will be further updated according to RAN2 agreements.

Reference

- RP-240838, New WID on Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s)