3GPP TSG-RAN WG4 Meeting # 115	R4-2506730

Malta, MT, 19th – 23rd May, 2025

Title: 	Discussion on Event triggered L1 measurement reporting

Source: 	Huawei, HiSilicon

Agenda item:	7.29.3.1

Document for:	Discussion

Introduction

The revised R19 mobility WI was agreed in [1]. The objectives are duplicated as below.

- 4.1 Objective of SI or Core part WI or Testing part WI - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

This contribution provides analysis on Event triggered L1 measurement reporting.

Discussion

Some left open issues in [2] are to be discussed in the followings.

- Issue 2-1: applicability of L1-RSRP reporting delay Candidate options: - Option 1: (vivo) - RAN4 starts the discussion on L1-RSRP reporting delay with the follow assumptions: - No indicate TCI change during the evaluation period. - No configuration change w.r.t measurement RS during the evaluation period - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period - Option 2 (Apple) - Seems no need to capture assumptions listed under option-1 into core requirements.

We understand the assumptions in option 1 come from the below RAN2 agreement on TTT. TTT timer is not restarted even if current beam changes. So option 1 is to ensure no other procedure (e.g., TCI change or RS configuration change) would change on serving beam during L1 RSRP reporting evaluation.  However we don’t think such assumption shall be captured in core requirements, as these would strongly restrict network configuration and implementation. Recalling the legacy L3 RSRP measurement test, the serving cell is kept unchanged during the event reporting procedure. The similar way can be followed for L1-RSRP measurement.

- TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT. - TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

Proposal 1: The proposed L1-RSRP reporting assumptions are not captured in core requirements. During performance phase, RAN4 can consider to ensure serving beam kept unchanged (e.g., no indicate TCI change, and no configuration change) for L1 event trigger reporting test cases.

The issue on event triggered reporting delay for L1 reporting is still open although we had a general agreement as below.

- Issue 2-2: Event triggered reporting delay for L1 reporting Agreement: - Reuse the framework from L3 event triggered reporting. Discuss the details further.

In legacy requirements for L3 measurement, it is specified that the event triggered measurement reporting delay shall be less than the L3-RSRP measurement period (Tidentify intra with index or T identify intra without index).

- 9.2.4.3 Event Triggered Reporting Reported RSRP, RSRQ, and RS-SINR measurements contained in event triggered measurement reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2). The UE shall not send any event triggered measurement reports as long as no reporting criteria is fulfilled. The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on. The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify intra with index or T identify intra without index defined in clause 9.2.5.1 or clause 9.2.6.2. When L3 filtering is used an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap. A cell is detectable only if at least one SSBs measured from the Cell being configured remains detectable during the time period Tidentify_intra_without_index or Tidentify_intra_with_index as defined in clause 9.2.5.1 or clause 9.2.6.2. If a cell which has been detectable at least for the time period Tidentify intra without index or Tidentify intra with index defined in clause 9.2.5.1 or clause 9.2.6.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again with the same spatial reception parameter and triggers an event, the event triggered measurement reporting delay shall be less than TSSB_measurement_period_intra provided the timing to that cell has not changed more than 3200/2µ Tc while the measurement gap has not been available and L3 filtering has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap.

In RAN4#114 meeting, companies discussed event triggered reporting delay for L1 reporting in LTM. One controversial issue is that for Event LTM3 (Beam of candidate cell becomes amount of offset better than beam of serving cell), when the RS periodicity for candidate cell and serving cell are different, how to determine the evaluation periodicity for event triggered reporting. In last meeting, one high level proposal was agreed as below, however it needs further discussion on which cell the below “L1-RSRP measurement period” refers to.

- Issue 2-2: Event triggered reporting delay for L1 reporting Agreement: - Reuse the framework from L3 event triggered reporting. Discuss the details further.

When serving beam and the beam from candidate cell have different periodicities, e.g., the periodicity of serving beam is 80 ms, and that of the candidate beam is 20 ms, if the evaluation periodicity is determined based on the periodicity of the candidate beam (i.e., 20 ms), then UE shall evaluate the configured event every 20 ms. It means that UE is required to evaluate the event LTM3 4 times within 80ms and at each evaluation, the latest measurement result of candidate cell would compare with “old” measurement result of serving cell. We think one reasonable approach is that the event instance evaluation shall be based on new results from both current and new beam. It means that the evaluation period is supposed to be the maximum RS periodicity of serving cell and candidate cell.

We know there is a related LS in R19 MIMO5 topic, where the current beams and new beams come from the same cell. If the same RS periodicity is assumed, no interaction between two different cells is needed in MIMO topic. However in LTM, things are different. Restriction on same RS periodicity requires some information exchange between serving cell and candidate cells. So it seams not reasonable to assume the RS periodicity between two cells are the same.

It also shall be noted, for event triggered MR, as no periodicity for reporting (i.e., Treport) is configured, the L1-RSRP measurement period for neighbor cell and serving cell (e.g., in Table 9.14.5.1) shall assume Treport=0.

For Event-triggered Periodic Reporting, the first report shall meet the requirements as event triggered MR.

Table 9.14.5.1-1: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_intra in FR1 for UE incapable of multiCellL1-measRTD-greaterThan-CP-r18

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |
| NOTE 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: K = 1.5. NOTE 3: The requirements apply when the max RTD between the serving cell and any candidate cell on which UE is required to perform L1-RSRP measurement on the same frequency layer is not larger than CP length of the cell on the frequency layer. |  |

Proposal 2:

- Except for periodic event-triggered MR, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period of the cells corresponding to the event.

    - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0.

- For periodic event-triggered MR: For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

    - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval.

- Issue 2-7: Impact on other LTM requirements Agreement: - For FR1: - UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, - Case 1: leaving condition of the event is configured and corresponding condition is not met, - the related cell switch delay and TCI state activation requirements are applicable - Case 2: leaving condition of the event is NOT configured, FFS - For FR2: FFS

As per FR2 SSB based L1-RSRP measurement principle in R18 mobility, UE would prioritize L1-RSRP measurement on the TCI state which is in active TCI list. Whether UE would measurement on the TCI which is not in the active TCI list is left to UE implementation and no requirements are defined.

- PL1_sharing is defined as - When number of neighboring cells configured with SSB based L1-RSRP measurement is 1 - PL1_sharing = 2, if any symbol of the SSBs from serving cell and neighbor cell are overlapping or adjacent (in time domain) - PL1_sharing = 1, otherwise - When number of neighboring cells configured with SSB based L1-RSRP measurement is more than 1 - When none of TCI state(s) of the intra-frequency neighbor cells or inter-frequency neighbor cells without gap are in the active TCI state list - PL1_sharing = 3*NNeighbor_Cell, where NNeighbor_Cell is the number of neighbor cells to measure on intra-frequency and inter-frequency without gap - Otherwise - PL1_sharing = 3*NNeighbor_Cell_in_list, where NNeighbor_Cell_in_list is the number of neigbour cells including intra-frequency neighbor cell(s) and inter-frequency neighbor cell(s) without gap whose TCI state(s) are in the active TCI state list. No requirements are defined for any other cell(s) whose TCI state(s) is not in the active TCI state list.

So in last meeting companies raised a case that a beam which is not in active TCI list triggered an event-triggered L1 report, afterwards if NW activated some other TCI states, UE may stop measuring the beam. So when UE receives the cell switch command or TCI state activation command, the reported beam may become unknown. Therefore the optimization for FR1 is not applicable to FR2.

For FR1, we don’t think the case that only event entering condition is configured is valid. As per RAN2 configuration, both event entering and leaving conditions use the same hysteresis value, and the entering condition and leaving condition are explicitly defined with equation (see below context in RAN2).

where

Observation1: It is invalid that only event entering condition is configured and leaving condition is not configured.

Proposal 3:

- For FR1:

  - UE has sent an event-triggered L1 report due to the entering condition is met and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, and leaving condition of the event is not met

    - the related cell switch delay and TCI state activation requirements are applicable

- The above optimization is not applicable to FR2

Conclusion

The contribution provides analysis on Event triggered L1 measurement reporting. The following proposals are proposed:

Proposal 1: The proposed L1-RSRP reporting assumptions are not captured in core requirements. During performance phase, RAN4 can consider to ensure serving beam kept unchanged (e.g., no indicate TCI change, and no configuration change) for L1 event trigger reporting test cases.

Proposal 2:

- Except for periodic event-triggered MR, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period of the cells corresponding to the event.

    - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0.

- For periodic event-triggered MR: For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event.

    - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval.

Observation1: It is invalid that only event entering condition is configured and leaving condition is not configured.

Proposal 3:

- For FR1:

  - UE has sent an event-triggered L1 report due to the entering condition is met and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, and leaving condition of the event is not met

    - the related cell switch delay and TCI state activation requirements are applicable

- The above optimization is not applicable to FR2

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R4-2504965, WF on RRM requirements for NR_Mob_Ph4_Part1), Apple