3GPP TSG-RAN WG4 Meeting #115	R4-2506777

Malta, MT, 19 – 23 May, 2025

Title: 	Discussion on RRM requirements for less than 5MHz Ph2

Source: 	Huawei, HiSilicon

Agenda item:	7.3.3

Document for:	Discussion

Introduction

RRM core requirements for less than 5MHz Ph2 are closed in RAN4#114, and CR is agreed in [1]. One issue that was not discussed before is the support of high speed requirements on SCell with less than 5MHz.

In this paper, we will provide our views on the support of high speed requirements on SCell.

Discussion

In R18, high speed RRM measurement is supported based on the following objective in the R18 WID.

- Specify requirements for UE speeds up to 500km/h for Band n100.

As a result, high speed RRM measurement requirements are defined for less than 5MHz, e.g. in Table 9.2.5.1-23.

Table 9.2.5.1-23: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1)

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120 ms, ceil(7 x Kp) x SMTC period) |
| DRX cycle≤ 320 ms | max(120ms, ceil(M2 x 7 x Kp) x max(SMTC period,DRX cycle)) |
| DRX cycle>320 ms | ceil(7 x Kp) x DRX cycle |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. |  |

RAN4 should discuss whether high speed RRM measurement is supported when a cell with less than 5MHz is an SCell of a UE. In our view, this should be supported because the high speed use case (railway) is for band n100, and if a cell with less than 5MHz is deployed to serve the high speed use case, high speed RRM measurement is meaningful no matter if the cell is used as PCell or SCell for a high speed UE. This is the reason why high speed RRM measurement has been defined for SCell in R17 for normal BW. On UE side, supporting high speed RRM measurement means a high speed UE can benefit from CA.

As to the exact requirements, the relevant requirements for SSB index reading for normal BW are in Table 9.2.5.1-3 for activated SCell and Table 9.2.5.1-10 for deactivated SCell. The enhancement in high speed RRM measurement requirements is the scaling factor M2 for DRX cycle≤ 320 ms. It is straightforward to apply the same enhancement for less than 5MHz based on the same UE capability.

Table 9.2.5.1-3: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120 ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(120 ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil(3 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

Table 9.2.5.1-10: Time period for time index detection, deactivated SCell (FR1)，when highSpeedMeasCA-Scell-r17 is configured

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(3 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(3 x Kp) x max(measCycleSCell, M2 Note 1xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(3 x Kp)x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

In RAN4#114-bis, some companies raised concern that RAN4 has not evaluated the performance combining the HST enhancements to SSB index reading performance with 12 PRB SSB. In our understanding, the HST enhancement, which reduces scaling factor M2 for DRX cycle≤ 320 ms from 1.5 to 1 for SMTC periodicity <= 40 ms, is based on more frequent UE measurement in HST scenario, and RAN4 did not conduct simulation in R17 when introducing the enhancement for normal BW, so we do not see clear issue to apply the same enhancement for less than 5MHz.

Proposal: Define high speed RRM measurement requirements for SCell with less than 5MHz. The same requirements for normal BW are re-used.

Conclusions

In this paper we provided our views on the support of high speed requirements on SCell.

Proposal: Define high speed RRM measurement requirements for SCell with less than 5MHz. The same requirements for normal BW are re-used.

Reference

- R4-2502608, Big CR for less than 5MHz RRM requirements in Rel-19, Intel Corporation