3GPP TSG-RAN WG4 Meeting #115	R4-2506780

Malta, MT, 19 – 23 May, 2025

Title: 	Discussion on performance requirements for 7MHz

Source: 	Huawei, HiSilicon

Agenda item:	7.8.5

Document for:	Discussion

Introduction

In RAN#107, a revised WID [1] for support of 7MHz CBW was approved. It has the following objective for the performance part.

Specify necessary UE/BS performance requirements for NR operation in dedicated FDD FR1 spectrum allocations for irregular channel bandwidths, corresponding to the core requirements: - Specify necessary RRM performance requirements if needed (RAN4) - Specify, if necessary, UE demodulation performance and CSI reporting requirements for the 7 MHz channel bandwidth (RAN4) - Specify, if necessary, BS demodulation performance requirements (RAN4) - Specify necessary BS conformance tests (RAN4)

In this paper, we will provide our views on RRM performance requirements for 7MHz CBW.

Discussion

The RRM performance requirements mainly include measurement accuracy requirements and test cases.

Measurement accuracy requirements

For the measurement accuracy requirements, we do not see any impact from support of 7MHz CBW. The SSB can be fully accommodated within the CBW, so the existing accuracy requirements should apply. CSI-RS with 48 PRB (side condition of the requirements) cannot be fully accommodated within the CBW, so the existing accuracy requirements cannot apply, but it is the same case for 5MHz. 5MHz has been supported since R15, and there has been no issue identified related to lack of applicable CSI-RS accuracy requirements so far, so we do not see clear motivation to define new CSI-RS accuracy requirements for 7MHz.

Test cases

RRM test cases are defined for selected CBW, and 10MHz is selected for 15kHz for most test cases. It is clear that if RAN4 cannot define RRM test cases for all the CBWs supported in spec. As most of the RRM requirements are agnostic to CBW, the principle has been that if a requirement is verified with selected CBW, UE is assumed to be able to meet the requirement with other CBW.

3MHz is a special case. Due to punctured SSB and CORESET BW, RAN4 defined new core requirements for 3MHz. In performance part, RAN4 only defined test cases for requirements that are impacted by 3MHz. This is because there is no UE that only supports 3MHz, so requirements that are not impacted by 3MHz are verified by existing test cases with regular CBW like 10MHz which is also supported by the UE.

For 7MHz, there is no new core requirements defined, and we understand there is also no UE that supports only 7MHz. Following the existing principle, we do not see clear need to define new test case or update existing test case for it.

Proposal: Confirm no impact to RRM performance requirements for supporting 7MHz CBW.

Conclusions

In this paper we provided our views on RRM performance requirements for 7MHz CBW.

Proposal: Confirm no impact to RRM performance requirements for supporting 7MHz CBW.

Reference

- RP-250373, Revised WID on NR FR1 7 MHz Channel Bandwidth, T-Mobile USA, China Telecom