3GPP TSG RAN WG4 Meeting #115		 R4-2506835

Malta, MT, 19th – 23th May, 2025

Agenda Item:	7.5.2

Source: 		vivo

Title: 	TP for TR38.755 on fall-back considerations

Document for:	Approval

1. Introduction

In RAN4#114 the TR [1] has been approved. In this paper we provide our view for the fall-back considerations about CA configuration and architecture switching.

2. TP

Proposed TP for TR38.755

<Start of proposed TP>

8.2	Study on semi-statically switch hardware resources procedure and fall back considerations

When either the in-gap interference or the self-interference is too high, the receiver is hard to maintain the shared receiving state, i.e., one RX RF chain, and the “fallback” behaviour needs to be studied. In order to make the application of this function more future-oriented, some higher CA combinations are also considered.

In total, two aspects should be involved when discussing fallback behaviour: “CA fallback” and “architecture switching”.

“CA fallback” means releasing at least one SCell in one band of the band combination, while “architecture switching” means UE architecture is changed from “FC” architecture (i.e., One Rx RF Chain) to legacy CA architecture (i.e., fully separated Rx Chain or partially shared Rx Chain) at least for one band of a band combination.[2]

If we go further and categorize “CA fallback”, the following scenarios are involved:

- Legacy CA combination falls back to lower order CA

- Legacy CA combination belongs to inter-band CA and intra-band CA with more than 2CCs, etc.

- Legacy CA combination falls back to “non-CA” mode

- Legacy CA combination belongs to intra-band CA combination with 2CCs.

If we go further and categorize “architecture switching”, the following scenarios are involved:

- Architecture switches from “FC” mode to fully-separate mode

- Before sharing, the architecture should be fully-separate mode

- Architecture switches from “FC” mode to partially-shared mode

- Before sharing, the architecture should be partially-shared mode

Based on the analysis above, the fallback behaviour should be totally divided into the following three scenarios:

- Scenario 1: CA fallback without architecture switching

- Scenario 2: CA fallback with architecture switching

- Scenario 3: architecture switching without CA fallback

For Scenario 1:

When the UE receives two CCs in “FC” mode but experiences significant performance degradation on one CC due to excessive interference, the NW could just release the degraded CC. The local oscillator frequency can either remain unchanged or revert to the center frequency of the remaining CC. If ensuring the PCC's reception performance is mandatory, the SCC can be directly released, with the LO shifted to the PCC's center frequency and the filter bandwidth narrowed—effectively reverting to a single-carrier transmission architecture.

This fallback mechanism is similar to legacy CA combination fallback procedure, so the UE should inherently support this capability by default without requiring additional feature.

For Scenario 2:

Since this scenario involves a switch in the receiver architecture, additional chain support is required to transition CC1+CC2 from a shared to an independent reception state. This capability is not inherently supported by default and depends on UE implementation. A new UE capability indication would be needed. [3]

For Scenario 3:

Similar to Scenario 2, maintaining the same CA combination while switching CC1+CC2 from a shared back to an independent reception state requires either: Reducing the MIMO layers for part of the CCs, or using additional idle standby links to facilitate the transition. Thus, this fallback still depends on UE implementation and requires additional capability support.

Regarding the decision-making authority for switching-to-sharing and fall-back behaviours, whether there is any change to the CA combination type, or MIMO layer should be judged firstly, if so then it could only be configured by the NW. If not then it could also be triggered by the UE itself.

#### 8.2.1. Case1: Partially shared switch to fully shared & fallback behaviour

According to session 5.2, the partially shared architectures contain both type2: Internal Rx path split, and type3: External Rx path split. The two partially shared architectures are expressed in the same form as below, i.e., the antenna is shared between CC1/2/3, and the other devices before down-conversion such as LNA, BPF are omitted here.

When partially shared architecture switches to fully shared architecture, for band nX CC1 and CC2 would be received by one RX RF chain together instead of each CC taking up one chain. Therefore, there would be one chain being released after each antenna.

According to the utilization of the released chain, there are two sub-scenarios: the released chain is not used (Fig 8.2.1-1), or the released chain is used to aggregate more CCs in the same band (Fig 8.2.1-3). As the new aggregated CCs belong to the same band and there is no new demand in the UE's hardware capability, it could be predicted that there is no new capability needed except for supporting the “FC” architecture.

- After sharing, the released chain is not used.

- Partially separate→ “FC”

Fig 8.2.1-1. Partially shared switch to “FC” mode

As can be seen in Fig 8.2.1-1, before sharing the partially shared architecture composes of one set of RX chain (one main Rx + one diversity Rx). After sharing half of the chain is released and unused which may have the benefit of power saving.

- Fallback behaviour:

Fig 8.2.1-2. Fallback to lower order CA combination (scenario1), or CA combination unchanged (scenario3)

In Fig 8.2.1-2, when fall back behaviour is triggered, there are two options:

Falling back to scenario1, which means only the combination type becomes lower without changing architecture.

Falling back to scenario3, which means CA combination and layer unchanged, while it needs architecture switching, i.e., CC1 and CC2 should be split into 2 chains, and each has one LO supported.

- After sharing, the released chain is used to aggregate more CCs in the same band.

- Partially separate→ “FC” & Fallback behaviour

The following analysis takes the 4RX-chain architecture as an example, and the analysis of the 2RX-chain architecture is similar. 4RX UE contains one main receiver branch and three diversity receiver branches, which could be seen in Fig 8.2.1-3.

Fig 8.2.1-3. 4RX UE, switch to “FC” mode

Fig 8.2.1-4. 4RX UE, fall-back to a lower order CA combination

In Fig 8.2.1-4, for 4Rx UE the fallback behaviour would lead to a lower order CA combination with architecture changed (scenario1) or unchanged (scenario2).

#### 8.2.2. Case2: fully separate switch to fully shared & fallback behaviour

According to the discussion before, there is another popular implementation architecture before sharing: fully-separate mode, which means each CC has exclusive access to one single RX RF chain from antenna to ADC.[4]

The following analysis primarily focuses on the utilization of idle links after switching to “FC” mode and its differences from the partially shared architecture. The fallback behaviour is similar and therefore omitted here.

Fig 8.2.2-1. After switching to “FC” mode, more CC from another band could be aggregated (case A), or MIMO layer could be higher

When fully-separate architecture changes to fully-shared architecture, some RX chains would also be released and the band combination or MIMO layer could be increased. Since the switching scheme of case A may involve changes in frequency and/or bandwidth of some devices as the part of the architecture would switch to receiving the CC3 from another band, it should be supported by additional capabilities.

<End of proposed TP>

Reference:

- R4-2505158 draft TR 38.755_v020_r1-cl

- R4-2503366, On fallback behavior and signalling aspect for Fragmented carriers, Samsung, #114bis.

- R4-2504283, On signalling aspect of fragmented carriers, HW, #114bis.

- R4-2411310, Views on UE RF architecture and NW deployment assumption for fragmented carriers, Samsung, TELUS, Bell mobility, #112.