3GPP TSG-RAN4 Meeting #115	R4-2506837

Malta, MT, 19th – 23th May, 2025

Agenda item:			7.1.2.2.1

Source:	vivo

Title:	Discussion on UE RF power enhancement for NR single carrier

Document for:	Discussion

- Introduction

In RAN#114bis meeting, the WF[1] has been agreed, and the remaining issues are as follows:

### Sub-topic 1-1: Applicable requirements Issue 1-1-1: RB based or CBW based extension - WF - Symmetrical/ Asymmetric for maximum extension at each side are determined by 0.5*NRB - If a limited number of additional fixed values are introduced, the extension is determined by X*NRB at each side, where X < 0.5 - The extended RBs are aligned with common RB grid Issue 1-1-2: Fixed or flexible extension - Agreement in main session: - Fixed value (0.5*CBW and/or 0.5*NRB) for Symmetrical/ Asymmetric by default for UE supporting the MPR reduction feature - In addition, consider a limited number of additional fixed values with capability Issue 1-1-3: Derivation of new inner RB region in extended CBW - WF - FFS in next meeting Issue 1-1-4: Guard band for extended CBW - Options - Option 1: No need to define multiple UE guard bands for initial CBW and extended CBW, UE should have one guard band per each side. All frequency segments within UE extended CBW and outside UE maximum transmission bandwidth (number of RBs in 38.101-1) act as UE total guard bands (i.e., UE initial guard band + additional guard band according to UE extended CBW NW indication), where this UE cannot be allocated and OOBE edges are the outer edges of UE total guard band. - Option 2: NRB based extension and associated guard band: - maximum bandwidth extension: The extended RB BW NRBext inherits the guard-bands (GBext) from the next corresponding existing higher CBW that uses the next higher NRB versus NRBext. - flexible NRB-based extension: Similarly to the ceil(NRB/2) extension, the guard band mapping for the fractional NRB extension can inherit the guard band from the next corresponding existing higher CBW that uses the next higher NRB versus NRBext - Option 3: The GBs of extended UE CBW should only be retained on the side that is far away from the original CBW. The GBs of extended UE CBW reuse the size of original GB of UE CBW. - Option 4: The GBs of the extended CBW shall be the remaining portion obtained by subtracting the RB numbers from the specified channel bandwidth when the extended CBW is selected from Table 5.3.2-1 in 3GPP TS 38.101-1. - Others - WF - FFS in next meeting ### Sub-topic 1-2: Applicable requirements Issue 1-2-1: A-MPR handling - Proposals - Option 1: Only apply the power domain enhancement scheme to channels which are not subject to A-MPR. - Option 2: - If A-MPR is specified for a given RB allocation X, then the UE performs max(MPRenh(X), A-MPR(X)) - If A-MPR is not specified for a given RB allocation, then the UE performs max(MPR(X), A-MPR(X)) - Option 3: In cases where additional emission should be met, the UE performs the max(MPR, A-MPR) function based on the specified A-MPR and the initial UE CBW MPR and thus set its backoff ignoring any MPR reduction. - Option 4: max(MPRenh(X), A-MPR(X)), where A-MPR(X) is determined based on a given RB allocation, and RB region with respect to initial CBW, WF and modulation etc, and MPRenh(X) is determined based on RB region/etc. within the extended UE CBW. - Others - WF - Only apply the power domain enhancement scheme to channels which are not subject to A-MPR in Rel-19. Issue 1-2-2: Whether the extended IBE is applied in the GB of original UE CBW Issue1 Issue 2; Option1. Allocate the extended RBs from the GB of the original UE; Option2. Allocate the extended RBs from the edge of the UE CBW. Option1. Individual evaluation of the extended RBs & Horizontal line; Case1; Case2. Option2. Combine evaluation of the extended RBs with the non-allocated RBs of the UE CBW & Stepped line; Case3; Case4. Note: It is assumed that extended RBs are RB aligned with the UE and BS RBs for the listed cases.. - WF: - Case 1 and case 3 could be further down selected - GB of UE CBW may or may not be scheduled to other UEs - Extended IBE can be applied in the GB of UE CBW - Relaxation can be considered for the GB, based on calculation from the existing IBE formula from the last RB of original UE CBW Issue 1-2-3: Derivation of the extended IBE - Options - Option 1: Fixed IBE used for extended CBW at each side of UE CBW, the value is derived from the outmost allocated RB of UE CBW (illustrated in the figure below) - Option 2: Combine the evaluation of the extended RBs with the non-allocated RBs of the UE CBW: Stepped line from the first non-allocated RB to the last extended RB, NRB’ = NRB + extended RBs. - Option 3: Others - WF - Discuss together with issue 1-2-2, and down select the options in next meeting. Issue 1-2-4: Relaxation of the extended IBE - Proposals - Proposal 1: For the IBE derivation for the extended CBW, use the general part of legacy IBE requirements and fix the EVM parameter to the one for BPSK. (Xiaomi) - Proposal 2: (vivo) - If the extended RBs are allocated from the GB of the original UE (case1/3), then the IBE in the GB should be relaxed to ensure the MPR of outer allocation reduces to the inner level: - Way 1: Skip the IBE evaluation for the extended RBs allocated in the GB. - Way2: Relax EVM in the IBE formula to that for BPSK for extended RBs. IBE_general = max(PRB - 30 dB, ) - WF - For case 1 in issue 1-2-2, use BPSK EVM in the IBE formula for all extended RBs - For case 3 in issue 1-2-2, FFS the relaxation ### Sub-topic 1-3: Signaling Issue 1-3-1: NW signaling - WF - FFS in next meeting Issue 1-3-2: UE capability - Agreement in main session: - An optional UE capability to indicate whether it supports the CBW extension based MPR reduction feature - To discuss the following aspects - Whether to have separate UE capabilities for scenario 1 and scenario 2 - Whether to indicate multiple values of fixed extension capability

- Discussion

## 2.1 Extension scheme

### 2.1.1 Extension ratio

Based on the WF in #114bis meeting, 0.5 is agreed as one extension ratio per side, and a limited number of additional fixed values could also be considered with the limitation of less than 0.5. Given the scenario where UEs may be positioned close to the edge of the BS coverage area, a set of moderately varied extension ratios is required. However, employing an excessive number of ratios would introduce unnecessary signaling overhead, especially since these ratios are typically constrained to values below 0.5 and exhibit only marginal differences between adjacent settings. In hence, we propose to add only one other extension case as ceil(1/4 * NRB) so that at most 2 bits code per side is sufficient to indicate the extension information, for example: 00 for non-extension, 01 and 10 for extension ratio = 1/4, 11 for extension ratio = 1/2.

Proposal 1: Except for 1/2, only considering 1/4 as the extension ratio.

Proposal 2: At most 2bit binary code is enough to indicate the three extension cases per side, e.g., no extension is signaled with 00, ceil(1/4 * NRB) is signaled with 01/10, ceil(1/2 * NRB) is signaled with 11.

### 2.1.2 Guard band for extended CBW

First of all, through the indication information it should be clear to the UE that the extended RB per side is ceil(x*NRB), e.g., x=1/4, 1/2. And the evaluation range inside the extension part is also clear as IBE is based on per RB granularity. At the same time, the evaluation of OOBE requirements should be based on a clear BW boundary, i.e., the extended UE CBW, which also matches the WF[2] in the 114 meeting.

Observation 1: Both the extension CBW and the extension RB numbers should be clear to the UE to identify the applying range of OOBE and IBE requirements.

According to the agreement in #114bis, the GB of the original UE would be occupied by the extended RBs and only the GB of the UE CBW after extension would be maintained per side. Therefore, how to define the BW of the GB is related to the mapping relationship of extended RB and extended CBW. And actually, inside the GB no requirements are needed so the specific BW of the GB is not such important as the specific value of extended RB and extended CBW.

To be easier, we could use the extension ratio not only on the RB number but also on the BW size, and the GB bandwidth of extended part is obtained by subtracting the bandwidth occupied by the extended RB from (extended bandwidth + GB of original UE). For example, when we consider the extension ratio as 0.5 on the left side, it represents that the extension RB number on the left side is ceil(0.5*NRB), and the extension CBW on the left side is also 0.5* original CBW. And the GB on this side would be naturally derived as (GB of original UE + 0.5* original CBW- ceil(0.5*NRB)*12*SCS). Similarly, if asymmetric extension is indicated, e.g., 1/4 on the left side and 1/2 on the right side, then the GB_L = (GB of original UE + 0.25* original CBW- ceil(0.25*NRB)*12*SCS), GB_R = (GB of original UE + 0.5* original CBW- ceil(0.5*NRB)*12*SCS).

And the table below shows some examples of the mapping relationship between the GB and the original CBW&NRB.

| SCS [kHz] | CBW [MHz] | NRB | GB of original UE [kHz] | ExR (extension ratio of left or right side) | Extended CBW (=ExR *CBW) [MHz] | Extended RB (= ceil (ExR *NRB)) | GB_EX (= GB of original UE + Extended CBW - Extended RB *12*SCS) [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15k | 5 | 25 | 242.5 | 1/4 | 1.25 | 7 | 232.5 |
|  |  |  |  | 1/2 | 2.5 | 13 | 402.5 |
|  | 20 | 106 | 452.5 | 1/4 | 5 | 27 | 592.5 |
|  |  |  |  | 1/2 | 10 | 53 | 912.5 |
|  | 50 | 270 | 692.5 | 1/4 | 12.5 | 68 | 952.5 |
|  |  |  |  | 1/2 | 25 | 135 | 1392.5 |
| 30k | 5 | 11 | 505 | 1/4 | 1.25 | 3 | 675 |
|  |  |  |  | 1/2 | 2.5 | 6 | 845 |
|  | 50 | 133 | 1045 | 1/4 | 12.5 | 34 | 1305 |
|  |  |  |  | 1/2 | 25 | 67 | 1925 |
|  | 100 | 273 | 845 | 1/4 | 25 | 69 | 1005 |
|  |  |  |  | 1/2 | 50 | 137 | 1525 |

Proposal 3: The GB bandwidth is obtained by subtracting the bandwidth occupied by the extended RB from the sum of extended bandwidth and GB of original UE, ExR is the extension ratio:

- GB_L = (GB of original UE + ExR_L* original CBW) - ceil(ExR_L*NRB)*12*SCS,

- GB_R = (GB of original UE + ExR_R* original CBW) - ceil(ExR_R*NRB)*12*SCS.

## 2.2 Derivation of the extended IBE

There are two remaining schemes about the IBE evaluation of the extension part:

case 1. Individual evaluation of the extended RBs: Horizontal line with the value equal to the outermost non-allocated RB of UE CBW.

case 2. Combine the evaluation of the extended RBs with the non-allocated RBs of the UE CBW: Stepped line from the first non-allocated RB to the last extended RB, NRB’ = NRB + extended RBs.

Apparently, option 1 has no effect on the IBE inside the original UE CBW. However, for option 2, as NRB’ changes with the extended RB, it may also change the IBE inside UE, as the equation is as follows:

IBE_general = max(PRB - 30 dB, )

It is well known that, generally the first several non-allocated RBs would follow the IBE requirement of the equation（1）, and the stepped line is visible in these non-allocated RBs:

20*log10EVM -3-5*∆RB-1LCRB        (1)

This means that for the non-allocated RBs closer to the allocated ones, they are not generally affected by equation (2) below. However, for more distant non-allocated RBs, the value of (1) gradually decreases with ΔRB increasing, causing the step-like curve to decline. As a result, the IBE may eventually need to follow the value given by (2).

-25-10lg(NRB/LCRB)                (2)

One of the prerequisites for this to happen is that (1) < (2), i.e.,

-25-10lgNRBLCRB> 20*log10EVM -3-5*∆RB-1LCRB     (3)

To model the case where (1) takes extreme small values (The stepped line declines to a relative low level and gives (2) the chance to dominant), we could place all the allocated RBs on one side of the UE, at which point the value of ∆RB at the outermost non-allocated RBs takes the value of ∆RB≈NRB-LCRB, and thus inequality (3) changes to:

-25-10lgNRBLCRB> 20*log10EVM -3-5*NRB-LCRB-1LCRB   (4)

By derivation, if 1/LCRB is ignored, then for QPSK (EVM = 0.175), the inequality is satisfied when the following conditions are met: NRB/LCRB⪆2.5, which may exist for outer allocations where the RB allocation is close to one side of the UE. Therefore, the change in NRB may affect the IBE within the UE CBW.

Observation 2: There are two options on how to evaluate IBE in the extension RBs:

- Case 1: Individual evaluation of the extended RBs: Horizontal line with the value equal to the outermost non-allocated RB of UE CBW.

- Case 2: Combine the evaluation of the extended RBs with the non-allocated RBs of the UE CBW: Stepped line from the first non-allocated RB to the last extended RB, NRB' = NRB + extended RBs.

Observation 3: If case2 is selected, i.e. NRB' = NRB + extended RBs for combined evaluation of extended RBs and non-allocated RBs within the UE CBW, the IBE within the UE CBW may be tightened.

| Case1. Individual evaluation of the extended RBs & Horizontal line | Case1 |
| --- | --- |
| Case2. Combine evaluation of the extended RBs with the non-allocated RBs of the UE CBW & Stepped line | Case2 |

In total, we propose to down-select case 1 with relaxation for extension part: use BPSK EVM in the IBE formula for extended RBs.

Proposal 4: It is recommended to choose case 1 with relaxation for extension part: use BPSK EVM in the IBE formula for extended RBs.

## 2.3. New inner region

One essential issue is that, the extended RBs are used to help re-define a new inner region based on the legacy allocated RB recourses. Therefore, the derivation of the new inner RB region within the extended CBW should not alter the reference system for RB allocation, but only modify the boundary of the inner region.

Observation 4: The derivation of the new inner RB region within the extended CBW should not alter the reference system for RB allocation, but only modify the boundary of the inner region.

In the current spec, the definition of inner region is captured as follows:

Based on the corresponding concepts and definition format, considering the diverse extension ratios and also the asymmetric extension scenarios, the definition of the new inner region is categorized as the following four cases:

- Only the higher side of original UE is extended by a*NRB.

- Only the lower side of original UE is extended by a*NRB.

- Symmetrical extension on both sides of original UE by a*NRB.

- Asymmetric extension on both sides of original UE, a*NRB on the higher side and b*NRB on the lower side.

As can be seen in the following figure, the big blue triangle on the left represents the new RB region of the extended UE (maximum LCRB = NRB’), the light blue part is the inner region of the extended UE. The smaller yellow triangle on the right represents the RB region of the original UE (LCRB = NRB), and the orange part is the inner region of original UE.

Fig 0. Illustration of RB region of extended UE and original UE

Therefore, it is easy to infer that the new inner region of the original UE after extension is the intersection area of the inner region of the extended UE CBW with the region of the original UE CBW, which is surrounded by the red dotted line as shown in the following figures. And the critical issue is how to define the intersection area through precise equations.

Case 1. Only the higher side is extended by a*NRB, NRB’=NRB+a*NRB:

For new inner region’s RBStart,Low, the equation does not change as it still belongs to the boundary of the inner area of the original UE.

However, for RBStart,High there are two distinct lines: One represents the RBStart,High boundary of inner region of the extended UE, another represents the RBStart,High boundary of the whole RB region of original UE, and the two lines intersect at the following point:

NRB’-1/2LCRB-LCRB = NRB-LCRB

Thus, the demarcation point of LCRB is: 2α*NRB, and the new inner region of original UE after extension is:

Fig 1-1. Only the higher side is extended by a*NRB

RBStart,Low  = max(1, floor(LCRB /2)),

If LCRB<=2α*NRB,

RBStart,High = NRB-LCRB,

Else,

RBStart,High = NRB’ – RBStart,Low – LCRB，

The RB allocation is an Inner RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart  ≤  RBStart,High

LCRB  ≤  ceil(1/2 *NRB’)

For example, if only the higher side is extended by 1/2 *NRB, then the condition of LCRB<=2α*NRB=NRB would always holds, therefore,

RBStart,Low  = max(1, floor(LCRB /2)),

RBStart,High = NRB-LCRB,

The RB allocation is an Inner RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart  ≤  RBStart,High

LCRB  ≤  ceil(1/2 *NRB’)

Actually, if more precise, it could be derived from ‘RBStart,Low ≤ RBStart ≤ RBStart,High’ that, LCRB ≤ ceil(2/3 *NRB).

Fig 1-2. Only the higher side is extended by 0.5*NRB

Case 2. Only the lower side of original UE is extended by a*NRB, NRB’=NRB+a*NRB:

For new inner region’s RBStart,High, the equation does not change as it still belongs to the boundary of the inner area of the original UE.

However, for RBStart,Low there are two distinct lines: One represents the RBStart,Low boundary of inner region of the extended UE, another represents the RBStart,Low boundary of the whole RB region of original UE, and the two lines intersect at the following point:

1/2LCRB-α*NRB=0

Thus, the demarcation point of LCRB is: 2α*NRB, and the new inner region of original UE after extension is:

Fig 2-1. Only the lower side of original UE is extended by a*NRB

RBStart,High = NRB– floor(LCRB /2)– LCRB,

If LCRB<=2α*NRB,

RBStart,Low  = 0

Else,

RBStart,Low  = max(1, floor((LCRB /2)-α*NRB)) = floor((LCRB /2)-α*NRB)

The RB allocation is an Inner RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart  ≤  RBStart,High

LCRB  ≤  ceil(1/2 *NRB’)

For example, if only the lower side is extended by 1/2 *NRB, then the condition of LCRB<=2α*NRB=NRB would always holds, so,

RBStart,Low  = 0,

RBStart,High = NRB– floor(LCRB /2)– LCRB,

The RB allocation is an Inner RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart  ≤  RBStart,High

LCRB  ≤  ceil(1/2 *NRB’)

Actually, if more precise, it could be derived from ‘RBStart,Low ≤ RBStart ≤ RBStart,High’ that, LCRB ≤ ceil(2/3 *NRB).

Fig 2-2. Only the lower side of original UE is extended by 0.5*NRB

Case 3. Symmetrical extension on both sides of original UE by a*NRB, NRB’=NRB+2a*NRB:

Similarly, this case is kind of merger of case1&2, therefore:

If LCRB<=2a*NRB,

RBStart,Low  = 0，RBStart,High = NRB-LCRB

Else,

RBStart,Low  = floor((LCRB /2)-a*NRB)，RBStart,High = NRB + a*NRB – floor(LCRB /2)– LCRB

The RB allocation is an Inner RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart  ≤  RBStart,High

LCRB  ≤  ceil(1/2 *NRB’)

Fig 3-1.

For example, if both the lower & higher sides are extended by 1/2 *NRB, then the condition of LCRB<=2α*NRB=NRB would always holds, therefore,

RBStart,Low  = 0，RBStart,High = NRB-LCRB

The RB allocation is an Inner RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart  ≤  RBStart,High

LCRB  ≤  ceil(1/2 *NRB’) = NRB

Apparently, the whole RB region of original UE becomes inner region.

Fig 3-2.

Case 4. Asymmetric extension on two sides of original UE, a*NRB on the higher side and b*NRB on the lower side.

This is the most complicated case and should be divided into some sub-cases based on the relationship between a and b. And the relationship between LCRB and

- When a > b：

Fig 4-1.

If LCRB<2b*NRB,

RBStart,Low  = 0，RBStart,High = NRB-LCRB

If 2β*NRB<LCRB<2α*NRB,

RBStart,Low  = 0，RBStart,High = (NRB + b*NRB) – floor(LCRB /2)– LCRB

If LCRB>2α*NRB,

RBStart,Low  = floor((LCRB /2)-a*NRB), RBStart,High = (NRB + b*NRB) – floor(LCRB /2)– LCRB

- When a < b：

Fig 4-2.

If LCRB<=2a*NRB,

RBStart,Low  = 0，RBStart,High = NRB-LCRB

If 2a*NRB<LCRB<2b*NRB,

RBStart,Low  = max(0,floor((LCRB /2)-a*NRB)), RBStart,High = NRB-LCRB

If LCRB>2b*NRB,

RBStart,Low  = floor((LCRB /2)-a*NRB), RBStart,High = (NRB + b*NRB) – floor(LCRB /2)– LCRB

The RB allocation is an Inner RB allocation if the following conditions are met:

RBStart,Low ≤ RBStart ≤ RBStart,High

LCRB ≤ ceil(1/2 *NRB’)

In total, the formulas in the previous parts can actually be consolidated into Case 4. Specifically:

- For higher-side-only extension, set a = 0.

- For lower-side-only extension, set b = 0.

- For symmetric extension on both sides, set a = b.

By applying these parameter adjustments to the formula in Case 4, we can derive the new inner region formulas for Cases 1–3.

Therefore, the unified formula for the inner region after bandwidth extension is as follows:

Proposal 5: When the extension information is a*NRB on the higher side and b*NRB on the lower side, a or b could be 0 to represent one-side extension case, and the new inner region could be derived as follows:

- When a > b：

- If LCRB<=2b*NRB,

RBStart,Low  = 0，RBStart,High = NRB-LCRB

- If 2b*NRB<LCRB<=2a*NRB,

RBStart,Low  = 0，RBStart,High = (NRB + b*NRB) – floor(LCRB /2)– LCRB

- If LCRB>2a*NRB,

RBStart,Low  = floor((LCRB /2)-a*NRB), RBStart,High = (NRB + b*NRB) – floor(LCRB /2)– LCRB

- When a < b：

- If LCRB<=2a*NRB,

RBStart,Low  = 0，RBStart,High = NRB-LCRB

- If 2a*NRB<LCRB<=2b*NRB,

RBStart,High = NRB-LCRB

- If a=0, RBStart,Low = max(1, floor(LCRB /2))

- If a>0, RBStart,Low = max(0, floor((LCRB /2)-α*NRB))

- If LCRB>2b*NRB,

RBStart,Low  = floor((LCRB /2)-α*NRB), RBStart,High = (NRB + b*NRB) – max(1, floor(LCRB /2))– LCRB

- When a = b：

- If LCRB<=2a*NRB,

RBStart,Low  = 0，RBStart,High = NRB-LCRB

- If LCRB>2α*NRB,

RBStart,Low  = floor((LCRB /2)-a*NRB)，RBStart,High = NRB + a*NRB – floor(LCRB /2)– LCRB

The RB allocation is an Inner RB allocation if the following conditions are met:

RBStart,Low ≤ RBStart ≤ RBStart,High

LCRB ≤ ceil(1/2 *NRB’)

## 2.4 UE capability & signalling

Based on the agreement, there are two separate UE capabilities needed to be defined:

Capability 1: An optional UE capability indicating support for MPR reduction feature based on 0.5*CBW extension per side.

Capability 2: An optional UE capability indicating support for MPR reduction feature based on x*CBW extension per side, x contains several values smaller than 0.5.

And if one UE support Capability 1, it could support the extension of 0.5*CBW per side. Similarly, if one UE support Capability 2, it could support the extension of x*CBW per side, e.g., x=0.25.

Proposal 6: There are two separate UE capabilities needed to be defined:

- Capability 1: An optional UE capability indicating support for MPR reduction feature based on 0.5*CBW extension per side.

- Capability 2: An optional UE capability indicating support for MPR reduction feature based on x*CBW extension per side, x contains several values smaller than 0.5.

The new mechanism of the UE to reduce MPR should be discussed:

First of all, the UE reports Capability 1/2, and the NW would judge the deployment environment. When the gap on one side of the UE CBW is wide enough, the NW should send the signaling ExtendLowerSideRel19, or ExtendHigherSideRel19, or ExtendLowerSideRel19 + ExtendHigherSideRel19 to the UE to indicate the extension on the left or right side. ExtendLowerSideRel19 and ExtendHigherSideRel19 both contain 2bit binary code: 00 represents non-extension, 01 and 10 represents the extended CBW & extended NRB = 1/4 *NRB, 11 represents the extended CBW & extended NRB = 1/2 *NRB.

Then according to the extension information, the UE would refer to the new inner region in Proposal 5, and the corresponding MPR would be used.

Proposal 7: Based on the UE reporting the Capability 1/2, the following signaling mechanism could be considered:

- It is up to the NW to judge the gap width on both sides of the UE.

- When the gap on one side of the UE CBW is wide enough, the NW should send the signaling ExtendLowerSideRel19, or ExtendHigherSideRel19, or ExtendLowerSideRel19 + ExtendHigherSideRel19 to the UE to indicate the extension ratio on the left or right side.

- ExtendLowerSideRel19 and ExtendHigherSideRel19 both contain 2bit binary code: 00 represents non-extension, 01 and 10 represents the extended CBW = 1/4 * CBW & extended NRB = 1/4 *NRB, 11 represents the extended CBW = 1/2 * CBW & extended NRB = 1/2 *NRB.

- According to the extension information, the UE would refer to the new inner region in Proposal 5, and the corresponding MPR would be used.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46.NR_ENDC_RF_Ph4 | 46-1 | MPR reduction of based on 0.5*CBW extension per side | Support for MPR reduction feature of single UL carrier based on the 0.5*CBW extension per side, containing both symmetric and asymmetric extension. | N/A | Yes | N/A | UE can not support MPR reduction for single UL carrier with 0.5* CBW extension | Per band | No | YES (only FR1) | N/A |  | Optional with capability signalling |
| 46.NR_ENDC_RF_Ph4 | 46-2 | MPR reduction based on x*CBW extension per side. | Support for MPR reduction feature of single UL carrier based on x*CBW extension per side, containing both symmetric and asymmetric extension. | 46-1 | Yes | N/A | UE can not support MPR reduction for single UL carrier with [x]* CBW extension | Per band | No | YES (only FR1) | N/A | Candidate value of X: [0.25] | Optional with capability signalling |
| 46.NR_ENDC_RF_Ph4 | 46-3 | MPR reduction of intra-band C CA for FR2 based on configured UL BWchannel_CA | Support for MPR reduction of intra-band CA for FR2 based on configured UL BWchannel_CA | N/A | Yes | N/A | UE cannot support MPR reduction of intra-band c CA for FR2 based on configured UL BWchannel_CA | Per UE | Yes | YES (only FR2) | N/A |  | Optional with capability signalling |
| 46.NR_ENDC_RF_Ph4 | 46-4 | MPR reduction of intra-band C CA for FR2 based on activated CABW | Support for MPR reduction of intra-band CA for FR2 based on activated CABW | N/A | Yes | N/A | UE cannot support MPR reduction of intra-band CA for FR2 based on activated CABW | Per UE | Yes | YES (only FR2) | N/A |  | Optional with capability signalling |
| 46.NR_ENDC_RF_Ph4 | 46-5 | MPR reduction of FR1 intra-band C CA based on activated carrier | Support for MPR reduction of intra-band C CA based on single CC when only single CC is activated. | N/A | Yes | N/A | UE cannot support MPR reduction of FR1 intra-band C CA based on activated carrier | Per UE | No | YES (only FR1) | N/A |  | Optional with capability signalling |

- Conclusion

Proposal 1: Except for 1/2, only considering 1/4 as the extension ratio.

Proposal 2: At most 2bit binary code is enough to indicate the three extension cases per side, e.g., no extension is signaled with 00, ceil(1/4 * NRB) is signaled with 01/10, ceil(1/2 * NRB) is signaled with 11.

Observation 1: Both the extension CBW and the extension RB numbers should be clear to the UE to identify the applying range of OOBE and IBE requirements.

Proposal 3: The GB bandwidth is obtained by subtracting the bandwidth occupied by the extended RB from the sum of extended bandwidth and GB of original UE, ExR is the extension ratio:

- GB_L = (GB of original UE + ExR_L* original CBW) - ceil(ExR_L*NRB)*12*SCS,

- GB_R = (GB of original UE + ExR_R* original CBW) - ceil(ExR_R*NRB)*12*SCS.

Observation 2: There are two options on how to evaluate IBE in the extension RBs:

- Case 1: Individual evaluation of the extended RBs: Horizontal line with the value equal to the outermost non-allocated RB of UE CBW.

- Case 2: Combine the evaluation of the extended RBs with the non-allocated RBs of the UE CBW: Stepped line from the first non-allocated RB to the last extended RB, NRB' = NRB + extended RBs.

Observation 3: If case2 is selected, i.e. NRB' = NRB + extended RBs for combined evaluation of extended RBs and non-allocated RBs within the UE CBW, the IBE within the UE CBW may be tightened.

Proposal 4: It is recommended to choose case 1 with relaxation for extension part: use BPSK EVM in the IBE formula for extended RBs.

| Case1. Individual evaluation of the extended RBs & Horizontal line | Case1 |
| --- | --- |
| Case2. Combine evaluation of the extended RBs with the non-allocated RBs of the UE CBW & Stepped line | Case2 |

Observation 4: The derivation of the new inner RB region within the extended CBW should not alter the reference system for RB allocation, but only modify the boundary of the inner region.

Proposal 5: When the extension information is a*NRB on the higher side and b*NRB on the lower side, a or b could be 0 to represent one-side extension case, and the new inner region could be derived as follows:

- When a > b：

- If LCRB<=2b*NRB,

RBStart,Low  = 0，RBStart,High = NRB-LCRB

- If 2b*NRB<LCRB<=2a*NRB,

RBStart,Low  = 0，RBStart,High = (NRB + b*NRB) – floor(LCRB /2)– LCRB

- If LCRB>2a*NRB,

RBStart,Low  = floor((LCRB /2)-a*NRB), RBStart,High = (NRB + b*NRB) – floor(LCRB /2)– LCRB

- When a < b：

- If LCRB<=2a*NRB,

RBStart,Low  = 0，RBStart,High = NRB-LCRB

- If 2a*NRB<LCRB<=2b*NRB,

RBStart,High = NRB-LCRB

- If a=0, RBStart,Low = max(1, floor(LCRB /2))

- If a>0, RBStart,Low = max(0, floor((LCRB /2)-α*NRB))

- If LCRB>2b*NRB,

RBStart,Low  = floor((LCRB /2)-α*NRB), RBStart,High = (NRB + b*NRB) – max(1, floor(LCRB /2))– LCRB

- When a = b：

- If LCRB<=2a*NRB,

RBStart,Low  = 0，RBStart,High = NRB-LCRB

- If LCRB>2α*NRB,

RBStart,Low  = floor((LCRB /2)-a*NRB)，RBStart,High = NRB + a*NRB – floor(LCRB /2)– LCRB

The RB allocation is an Inner RB allocation if the following conditions are met:

RBStart,Low ≤ RBStart ≤ RBStart,High

LCRB ≤ ceil(1/2 *NRB’)

Proposal 6: There are two separate UE capabilities needed to be defined:

- Capability 1: An optional UE capability indicating support for MPR reduction feature based on 0.5*CBW extension per side.

- Capability 2: An optional UE capability indicating support for MPR reduction feature based on x*CBW extension per side, x contains several values smaller than 0.5.

Proposal 7: Based on the UE reporting the Capability 1/2, the following signaling mechanism could be considered:

- It is up to the NW to judge the gap width on both sides of the UE.

- When the gap on one side of the UE CBW is wide enough, the NW should send the signaling ExtendLowerSideRel19, or ExtendHigherSideRel19, or ExtendLowerSideRel19 + ExtendHigherSideRel19 to the UE to indicate the extension on the left or right side.

- ExtendLowerSideRel19 and ExtendHigherSideRel19 both contain 2bit binary code: 00 represents non-extension, 01 and 10 represents the extended CBW = 1/4 * CBW & extended NRB = 1/4 *NRB, 11 represents the extended CBW = 1/2 * CBW & extended NRB = 1/2 *NRB.

- According to the extension information, the UE would refer to the new inner region in Proposal 5, and the corresponding MPR would be used.

- References

- R4-2505099, WF on power domain enhancement, Huawei, HiSilicon, RAN4 #114bis.

- R4-2503013, WF on power domain enhancement, Huawei, HiSilicon, RAN4 #114.