3GPP TSG-RAN WG4 Meeting # 115	R4-2506859

Malta, MT, 19th – 23rd May, 2025

Source: 	vivo

Title: 	Iot NTN HPUE Rx Requirements

Agenda Item:	7.10.3.3.2

Document for:	Approval

Introduction

In last meeting, progress was made for PC2 case, and agreements were made for PC2 in [2].

Issue 3-1-1: PC2 1Tx RSD (full duplex eMTC based)

- Proposals

  - Reference sensitivity values

    - 256:

      - 0 dB (MTK, Samsung, Ericsson)

      - 1.1dB (Qualcomm)

    - 255:

      - 0.8dB (MTK)

      - 1.2dB (Qualcomm)

  - No differentiation of handheld and non-handheld for RSD

- WF (Ad-hoc agreement):

  - Reference sensitivity values

    - 256: [0.5] dB

    - 255: 1 dB

Now the only remaining issue is PC1 case. It is also noted that the whole RSD part is only applicable to eMTC based with full duplex case for Iot-NTN, and not applicable to half-duplex cases, and the scenarios did not change in last meeting.

In this paper, further discussion was made.

HPUE Impact for Iot-NTN RSD

General

As already agreed, for all the half duplex mode cases, no RSD is needed. Only for the case of eMTC based Iot NTN with full duplex, RSD is needed to be discussed.

Table 1: Status for requirements consideration and feasibility (eMTC based Iot NTN with full duplex)

|  | Power Class | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- | --- |
| eMTC based with full duplex | PC1 | 1Tx | Non-handheld |
|  | PC2 | 1Tx | Both |

For this particular case, actual the guidelines should be the same to NR-NTN, since similar reference components are expected. However, there are two main difference: Only 1Tx and 1Rx

Observation 1(Re-submission): For the RSD analysis of eMTC based Iot NTN with full duplex, the key differences from NR-NTN is only 1Tx and 1Rx be considered. Other conditions (reference components conditions, duplex gap) are similar.

PC1

The current agreed WF is as following:

Issue 3-1-2: PC1 RSD (Non-handheld only, full duplex eMTC based)

- Proposals

  - Option 1: 0dB for 254/255/256 (oppo, Ericsson)

  - Option 2: 0dB for at least band 256 (Samsung)

  - Option 3: non-zero value as following: (MediaTek)

| NTN Band | 1.4 MHz (dB) |
| --- | --- |
| 256 | [3.7] |
| 255 | [4.7] |
| 254 | [3.7] |

- Recommended WF

  - TBD. Further explanation from proponents of option 3 needed

As discussed in NR-NTN, currently PC1 is only for 1Tx and non-handheld case in LTE and NR TN, and typically share PC3 Rx requirements without any RSD. For PC1 1Tx, if following de-sense analysis in 37.880, the current NTN band 254/255/256 are easier compared to legacy PC1 bands, and 0dB RSD can be deduced.

For PC1 Iot-NTN, currently only non-handheld requirements would be defined for eMTC based full-duplex, as the duplexer for 1Tx PC1 UE is currently deemed as not that feasible. In this sense, more clear reference components is needed if non-zero value is proposed.

Observation 2: If based on legacy cavity duplexer which is used for BS, the RSD can be 0dB.

It is admitted that the example components such as cavity filter used in the 37.880 is very large form factor. Since it is in the order of centimetres and the thermal effects is also not that good, it is quite challenging even for some FWA devices. In other hand, the thermal effects is also challenging and for cavity filter.

In another direction, the current technology for mobile phone duplexers is also evolving. E.g. BAW and FBAR is improving the power level, and the margin shortage compared to what PC1 needs is shrinking. With the price of cost, IL, and area, there are still room for future improvement based on current technology. So it might be also reasonable to keep some more possibilities to use the technology originated from smart phone duplexers, by introducing some relaxations for PC1.

Observation 3: Introduce some relaxation for future RSD for non-handheld may also be reasonable.

Proposal 3 (New): More possibilities of reference components and related relaxations might be considered for PC1. However, if no more and agreements on this can be reached, 0dB can still be used since this is in line with legacy cases.

Conclusion

In this paper, the RSD requirements have been discussed for NR NTN HPUE.

General

Observation 1(Re-submission): For the RSD analysis of eMTC based Iot NTN with full duplex, the key differences from NR-NTN is only 1Tx and 1Rx be considered. Other conditions (reference components conditions, duplex gap) are similar.

PC1

Observation 2: If based on legacy cavity duplexer which is used for BS, the RSD can be 0dB.

Observation 3: Introduce some relaxation for future RSD for non-handheld may also be reasonable.

Proposal 3: More possibilities of reference components and related relaxations might be considered. However, if no more and agreements on this can be reached, 0dB can still be used since this is in line with legacy cases.

- References

[1] R4-2504682, Topic summary for [114bis][124] NR_IoT_NTN_HPUE_part2, Moderator(Vivo), RAN4#114bis

[2] R4-2505114, WF on HPUE UE requirement for IoT-NTN, vivo, RAN4#114bis

[3] R4-2503843, Iot NTN HPUE Rx Requirements, vivo, RAN4#114bis

[4] R4-2504385, Discussion on IoT-NTN HPUE RX RSD, MediaTek, RAN4#114bis