# 3GPP TSG-RAN WG4 Meeting #115	R4-2506870

Malta, MT, 19th – 23rd May, 2025

Source: 	vivo

Title: 	Discussion on simultaneous operation between GNSS and UL transmission in NR NTN

Agenda Item:	7.31.2

Document for:	Approval

Introduction

An LS [1] was sent to RAN2 on simultaneous operation between GNSS and NR NTN, asking to introduce improvements to existing IDC solutions to address this in-device coexistence issue. A received LS [2] was received from RAN2, requiring more information on requirements, e.g. the periodicity and duration of IDC free time to extend IDC TDM solution. In the last meeting, the draft reply LS content was discussed and there was no consensus on duration for IDC free time and duty cycle for autonomous denial solution. In this contribution, we provide our views on the periodicity and duration of IDC free time and duty cycle for autonomous denial solution.

Discussion

The content of the response LS [2] from RAN2 was captured as follows:

1. Overall Description: RAN2 would like to thank RAN4 for the original LS in R4-2419898. RAN2 has considered the simultaneous operation between GNSS and UL transmission in NR NTN and concluded not to work on the enhancements to the existing IDC solutions in Release 18. RAN2 might consider the extension to IDC TDM solution or autonomous denial solution in Release 19, and would like to receive more information on requirements, e.g. on the periodicity and duration of IDC free time to help the UE read the GNSS in NTN. 2. Actions: To RAN4: ACTION: RAN2 respectfully asks RAN4 to take the aforementioned RAN2 decisions into account and provide the corresponding requirements, e.g. the periodicity and duration of IDC free time to help the UE read the GNSS.

IDC TDM solution is a mechanism defined in 3GPP standards to address in device interference between multiple radio technologies. In this discussion, it mainly focuses on the simultaneous operation between GNSS and UL transmission in NR NTN. In the LS, RAN2 asks more information on the periodicity and duration of time for UE read GNSS in NTN.

For GNSS pattern, it serves different purposes, e.g. power efficiency, interference mitigation, resource optimization, enhancing positioning performance. For some typical scenarios, they can be different as shown in the following table:

| Application Scenario | Period | Duration | Duty Cycle |
| --- | --- | --- | --- |
| High-Accuracy (e.g., Navigation) | 1 second | Continuous operation (no sleep) | 100% |
| Power-Sensitive Applications (e.g., IoT Devices) | 60 seconds | 5 seconds | ~8.3% |
| Balanced Power and Accuracy Scenarios (e.g., Vehicle Tracking) | 10 seconds | 2 seconds | 20% |

Besides above GNSS typical patterns, there were some values defined in 3GPP. GNSS-PositionFixDuration-r18 is a feature introduced in 3GPP LTE Rel-18 to dynamically configure the duration of GNSS positioning fixes. It was introduced in Rel-18 to:

- Support diverse application scenarios.

- Optimize device power consumption.

- Improve network resource utilization.

- Enhance positioning technology.

The duration of GNSS positioning fixes based on the configured value of GNSS-PositionFixDuration-r18 can be 1s~31s. short positioning duration is suitable for fast positioning, medium for general positioning needs and long for scenarios requiring high accuracy or continuous tracking.

#### GNSS-PositionFixDuration (From 36.331) The IE GNSS-PositionFixDuration indicates the time duration required for the UE to acquire a GNSS position. Value s1 corresponds to 1 second, s2 corresponds to 2 seconds and so on. GNSS-PositionFixDuration information element -- ASN1START GNSS-PositionFixDuration-r18 ::= ENUMERATED{ s1, s2, s3, s4, s5, s6, s7, s13, s19, s25, s31} -- ASN1STOP

However, we think these values cannot be directly used for the duration for IDC free time. Some large values for GNSS positioning duration should be left out. In the last meeting, we discussed LS reply on extension of IDC solution and no conclusion was made. There were two remaining issue we couldn’t reach consensus:

- IDC free duration 2s or 10s

- 50% of duty cycle for autonomous denial solution

For IDC free duration, companies had different views on the values for 2s or 10s. Some company think if UE needs more than 2s IDC free time, it better goes to idle mode. However, some companies would like to keep 10s for GNSS measurement gap since it can leave more flexibility of UE implementation.

From our perspective, we prefer to take 2s as baseline for IDC free time. Since GNSS duration of 2s is typical for Balanced Power and Accuracy Scenarios. 10s is too long and it is intended for some high accuracy application. It has too much impact on the whole network performance. Also, with 2s GNSS duration, current IE for the duration of IDC free time can still work.

Proposal 1: It is suggested to take 2 seconds as duration of IDC free time for simultaneous operation between GNSS and UL transmission in NR NTN.

For autonomous denial solution, avoiding transmitting 50% of duty cycle is too much. For the typical GNSS pattern for balanced power and accuracy scenarios, duration is 2 second with 10 seconds period, which implies 20% duty cycle. We should take this as a baseline.

Proposal 2: It is suggested to take 20% duty cycle for autonomous denial solution for simultaneous operation between GNSS and UL transmission in NR NTN.

Conclusion

This contribution discusses GNSS pattern information to RAN2 for further enhancements for IDC TDD solution, allowing simultaneous operation between GNSS and NR NTN. The following proposals are made:

Proposal 1: It is suggested to take 2 seconds as duration of IDC free time for simultaneous operation between GNSS and UL transmission in NR NTN.

Proposal 2: It is suggested to take 20% duty cycle for autonomous denial solution for simultaneous operation between GNSS and UL transmission in NR NTN.

- References

[1] R4-2419898, LS on simultaneous operation between GNSS and NR NTN, RAN4#113.

[2] R2-2501568, Response LS on simultaneous operation between GNSS and NR NTN, RAN2#129.