# 3GPP TSG-RAN WG4 Meeting #115	R4-2506871

Malta, MT, 19th – 23rd May, 2025

Source: 	vivo

Title: 	Discussion on the reply LS on requirements for phase continuity and power consistency for OCC with PUSCH in NR NTN Ph3

Agenda Item:	7.31.3.2

Document for:	Approval

Introduction

An incoming LS [1] was received from RAN1 asking RAN4 whether the same phase continuity requirements as for DMRS bundling can be applied for OCC length 2 and/or OCC length 4. In this contribution, we discussed this issue and provided our reply for this LS.

Discussion

The content of incoming LS [1] was captured as follows:

1. Overall description RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH. - FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling. RAN1 agreed the following: 2. Actions: RAN1 respectfully ask RAN4 whether the same phase continuity requirements as for DMRS bundling can be applied for OCC length 2 and/or OCC length 4 under the same conditions as for DMRS bundling, or if new requirements are needed.

In order to keep the orthogonality of different OCC codes from different UE, it is necessary to have requirements on power consistency and phase consistency. However, we think no new evaluation is needed since we can reuse the requirements for DMRS bundling. Since both DMRS bundling and OCC groups fundamentally depend on phase stability and shared hardware (UE RF), synchronization (CFO/TA), and channel estimation constraints:

- Enforcing DMRS phase continuity implicitly safeguards OCC orthogonality.

- UE RF design (e.g., LO phase noise) must holistically satisfy both requirements.

This reuse reduces standard/implementation complexity and enhances system efficiency.

To enable DMRS bundling for PUSCH and PUCCH in Rel-17 coverage enhancement scenarios, maintaining phase continuity and power consistency (within defined tolerances) is essential.

- Power Consistency: The existing aggregate power tolerance requirement (per TS 38.101-1) remains applicable; no additional requirements are introduced.

- Phase Continuity: New requirement is defined to constrain phase variations (e.g., ≤25° between adjacent slots and ≤30° between the first slot and slot p), ensuring coherent DMRS integration across bundled transmissions.

For maximum allowable phase difference for one OCC group with PUSCH, we think the requirements for DMRS bundling in Table 6.4.3.5-1 of TS 38.101-1 can still apply and no new evaluation is needed. RAN1 can take the following table as baseline.

Table 6.4.2.5-1: Maximum allowable phase difference for DMRS bundling

| UL channel | Modulation order | Phase difference between any slot p-1 and slot p (NOTE 2) | Phase difference between slot 0 and any slot p (NOTE 3) |
| --- | --- | --- | --- |
| PUSCH | Pi/2 BPSK, QPSK | [25] degrees | [30] degrees |
| PUCCH | Pi/2 BPSK, BPSK, QPSK |  |  |
| NOTE 1: The UE capability of the length of maximum duration refers to the maximum time duration during which UE is able to meet the phase continuity requirements, assuming no phase consistency violating events defined in TS 38.214 in between. NOTE 2: This requirement applies for FDD and TDD bands, for supported DMRS bundling configurations ≤ 8 slots. NOTE 3: This requirement applies only for FDD bands, for supported DMRS bundling configurations of 16 slots. |  |  |  |

For the conditions when the above requirements are applicable can refer to Section 6.4.2.2 of TS 38.101-5:

The above requirements are applicable when all the following conditions are met within the measurement time window: - RB allocation in terms of length and frequency position does not change, and intra-slot and inter-slot frequency hopping is not activated. - Modulation order does not change. - No network commanded TA takes effect. - The TPMI precoder does not change. - There is no change in UE transmission power level, and no change in the level of P-MPR applied by the UE. - UE is not scheduled with uplink transmission of other physical channel/signal in-between the PUSCH or PUCCH transmissions. - Doppler conditions are set to zero and delay conditions are set to constant.

Proposal 1: The same phase continuity requirements for DMRS bundling in TS38.101-5 can be applied for the duration of one OCC group with PUSCH in NR NTN.

Conclusion

This contribution discusses phase continuity requirements for the duration of one OCC group with PUSCH in NR NTN. And we see no issue of reusing the maximum allowable phase difference for DMRS bundling. The following proposals are made:

Proposal 1: The same phase continuity requirements for DMRS bundling in TS38.101-5 can be applied for the duration of one OCC group with PUSCH in NR NTN.

- References

[1] R4-2505305_R1-2503071, LS on requirements for the phase continuity and power consistency for OCC with PUSCH in NR NTN Ph3, RAN1, MediaTek.