# 3GPP TSG-RAN WG4 Meeting #115	R4-2506875

Malta, MT, 19th – 23rd May, 2025

Source: 	vivo

Title: 	Discussion on general aspects for gNB SBFD

Agenda Item:	7.25.2

Document for:	Approval

Introduction

In the last meeting, there was a proposal on HPUE duty cycle evaluation for SBFD-aware UE. The WF for this issue was captured in [1]. In this contribution, we provide our views on this issue.

Discussion

Some discussion points were captured in the WF [1] on HPUE SAR:

##### Issue 1-7-1: Update for HPUE duty cycle evaluation Way forward: - RAN4 has discussed the two following points: - For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission; - For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission. - FFS RAN4 specification impact by taking into account the above understanding for SBFD-aware UE.

For HPUE duty cycle, we prefer not to change the specification for UE RF.

We went through a lot of tdocs discussing HPUE SAR issue dating back to Rel-15. The wording ‘uplink symbols transmitted’, not ‘uplink symbols’ standalone, means uplink time [2] or uplink transmission time [3] that can be scheduled to ensure SAR. RAN4 sent the LS [3] to RAN2 to design the UE capability for maxUplinkDutyCycle-PC2-FR1, the exact wording that is ‘evaluation period that can be scheduled for uplink transmission’.

| maxUplinkDutyCycle-PC2-FR1 Indicates the maximum percentage of symbols during a certain evaluation period that can be scheduled for uplink transmission to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies. This field is applicable for FR1 power class 2 UE and also applicable for FR1 power class 1.5 UE as specified in clause 6.2.1 of TS 38.101-1 [2]. If the field and maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 are both absent, 50% shall be applied as the upper limit of the UL duty cycle for power class 2. Value n60 corresponds to 60%, value n70 corresponds to 70% and so on. This capability is not applicable to IAB-MT. | Band | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |

We can find the interpretation for ‘uplink symbols transmitted’ in many approved tdocs, LS, or TS. The meaning of ‘uplink symbols transmitted’ is ‘uplink time’ or ‘uplink transmission time’ or ‘uplink transmission’. It is still true even for SBFD symbols. As long as UE transmits in SBFD symbols, it can be counted as ‘uplink symbols transmitted’. There is no ambiguity about this wording.

The issue ‘Flexible’ symbols for SAR evaluation was also discussed in HPUE duty cycle SAR [4]. We think this is quite the same with SBFD symbols. As long as UE transmits in the evaluation period, it can be counted as ‘uplink symbols transmitted’. The wording ‘uplink symbols transmitted’ tries to screen the various time domain configuration.

We think current wording is OK even for SBFD symbols, there is no need to change this part.

Observation 1: The term ‘uplink symbols transmitted’ as a whole, not ‘uplink symbols’ standalone, means ‘uplink time’, ‘uplink transmission time’ or ‘uplink transmission’ during the evaluation period for HPUE SAR.

Proposal 1: No change for HPUE duty cycle SAR part in RAN4 specification.

Conclusion

This contribution discusses HPUE duty cycle. The following observation and proposal are made:

Observation 1: The term ‘uplink symbols transmitted’ as a whole, not ‘uplink symbols’ standalone, means ‘uplink time’, ‘uplink transmission time’ or ‘uplink transmission’ during the evaluation period for HPUE SAR.

Proposal 1: No change for HPUE duty cycle SAR part in RAN4 specification.

- References

[1] R4-2504751, Way Forward for [114bis][306] NR_duplex_evo_General, Samsung, RAN4#114bis.

[2] R4-1805776, WF on FR1 HPUE behaviour, CMCC, RAN4#86bis.

[3] R4-1805786, LS on the UE capability of maxUplinkDutyCycle for NR FR1 power class 2 UE, CMCC, RAN4#86bis.

[4] R4-1801125, WF on NR TDD UL/DL configurations and HPUE behaviour, Vivo, Qualcomm, Softbank, Sprint, RAN4#AH-1801