# 3GPP TSG-RAN WG4 Meeting #115	R4-2506898

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	4.5.1

Source: 	Ericsson

Title:	Clarification on Rel-16 mandatory gap patterns in Rel-16

Document for:	Discussion

# Clarification

In Athens meeting, we proposed to clarify the UE behaviour when the UE reports the mandatory gap pattern capability supportedGapPattern-NRonly-r16 in NR SA on top of Rel-15 supportedGapPatterns. In our understanding, Rel-16 mandatory gap pattern is the gap pattern introduced for NR SA without E-UTRA measurement objects. When UE reports to support such gap pattern, it means UE supports NR only measurement but not support LTE&NR measurement. On the contrary, if UE reports to support a gap pattern in Rel-15 supportedGapPatterns, it means UE supports both NR and LTE measurement wo restriction. However, it will be a mismatch based on today’s capability design when UE reports to support a pattern in Rel-15 supportedGapPatterns but reports not support the pattern in Rel-16 supportedGapPattern-NRonly-r16. From NW side, it’s difficult to understand the UE reporting capability with such mismatch.

|  |  | Support for GP2-GP11 |  |  |
| --- | --- | --- | --- | --- |
| supportedGapPatterns | supportedGapPatterns-NRonly-r16 | LTE measurements only | NR FR1 + LTE measurements | NR FR1 measurements only |
| 0 | 0 | No | No | No |
| 0 | 1 | No | No | Yes |
| 1 | 0 | Yes | Yes | Yes |
| 1 | 1 | Yes | Yes | Yes |

In last meeting, we proposed to clarify that UE is not allowed to report mismatch between Rel-16 supportedGapPatterns-NRonly-r16 and Rel-15 supportedGapPatterns. During the discussion, some companies proposed to ignore the UE reporting in Rel-16 once UE reports to support the gap pattern in Rel-15. Firstly, we want to understand whether such error reporting can happen from UE side. If yes, we propose to send LS to RAN2 to clarify UE behaviour to ignore the bit in supportedGapPatterns-NRonly-r16 if UE reports ‘1’ to support the same pattern in Rel-15 supportedGapPatterns.

RAN4 #113 Further discuss the following options. - Option 1: clarify that UE is not allowed to report ‘0’ for one pattern in supportedGapPatterns-NRonly-r16 if UE reports ‘1’ to support the same pattern in Rel-15 supportedGapPatterns. - Option 2: clarify that if UE reports 1 for a specific gap pattern in R15 capability, NW can ignore R16 capability for the same gap pattern.

Proposal 1: UE vendors firstly to clarify whether the gap pattern mismatch can happen in real field.

Proposal 2: RAN4 to send LS to RAN2 to clarify the UE behaviour on gap pattern reporting if both Rel-16  supportedGapPatterns-NRonly-r16 and Rel-15 supportedGapPatterns are reported and if mismatch in reporting can occur.

Proposal 3: RAN4 to send LS to RAN2 to clarify that UE is not allowed to report ‘0’ for one pattern in supportedGapPatterns-NRonly-r16 if UE reports ‘1’ to support the same pattern in Rel-15 supportedGapPatterns if UE currently didn’t implement such mismatch reporting.

- Summary

In this contribution, we discussed the mandatory gap capability mismatch. Based on the discussions, we have made following proposals and observations:

Proposal 1: UE vendors firstly to clarify whether the gap pattern mismatch will happen in real field.

Proposal 2: RAN4 to send LS to RAN2 to clarify the UE behaviour on gap pattern reporting in both Rel-16  supportedGapPatterns-NRonly-r16 and Rel-15 supportedGapPatterns if UE has already implemented such mismatch reporting.

Proposal 3: RAN4 to send LS to RAN2 to clarify that UE is not allowed to report ‘0’ for one pattern in supportedGapPatterns-NRonly-r16 if UE reports ‘1’ to support the same pattern in Rel-15 supportedGapPatterns if UE currently didn’t implement such mismatch reporting.