3GPP TSG-RAN WG4 Meeting #115		R4-2506915
Malta, MT, 19th – 23rd May, 2025

Title:	R19 MSD for PC1.5 with HPUE on both FDD and TDD

Source:	OPPO

Agenda item:	6.8.5

Document for:	Approval

- Introduction

In Rel-19, the CA/DC HPUE basket WI [1] includes PC1.5 with PC2 FDD + PC2/1.5 TDD band combinations which needs the MSD analysis before introducing these band combinations.

In last RAN4 meeting, paper [2] pointed out that the PC1.5 for some CA/DC band combinations have already been defined in the spec but with the power capability in each band as PC3 FDD + PC1.5 TDD, and the MSD was defined based on 23dBm FDD + 27.8dBm TDD due to the power capability limitation in each band. Now, with the PC2 FDD + PC2/1.5 TDD band combination introduced, the MSD needed will be different.

This paper discusses how to treat this issue.

- MSD for FDD PC2+TDD PC2/1.5

As discussed in the introduction part, currently some PC1.5 CA/DC band combinations have been introduced in Rel-18 when 3Tx feature was introduced. At that time, there is no FDD HPUE in the spec, therefore, the PC1.5 CA/DC was based on power capability of PC3+PC1.5 in each band as shown in figure 1 below. Correspondingly, the MSD was calculated based on 23dBm in PC3 band and 24.8 *2 in the PC1.5 band as shown in figure 2 below.

Figure 1 Power capability of each band in CA PC1.5

Figure 2 Power configuration of CA PC1.5 for UE with PC3 FDD + PC1.5 TDD capability

Observation 1:   Some PC1.5 CA/DC band combinations have been introduced in Rel-18 when 3Tx feature was introduced. At that time, there is no FDD HPUE in the spec, therefore, the PC1.5 CA/DC was based on power capability of PC3+PC1.5 in each band. Correspondingly, the MSD was calculated based on 23dBm in PC3 band and 24.8 *2 in the PC1.5 band.

As pointed out in [2], the MSD for 23dBm FDD +27.8dBm TDD is smaller than 26dBm FDD +26dBm TDD. Below are two approaches to handle this situation:

- Approach 1: Define separate requirements in the spec with information of power capability in each band.

Table 1 is an example of how this approach looks like.

The benefit of this approach is that it aligns with the power capability information of each band in the MOP table as shown in Figure 1 and is precise in the MSD definition and also testing configurations.

It should be noticed that in this table the Tx number information doesn’t show, and MSD is only based on the Tx power of each band.

|  | MSD for PC3 in FDD and PC1.5 in TDD | MSD for PC2 in FDD and PC2/1.5 in TDD |
| --- | --- | --- |
| CA_n71-n77 | 16dB NOTE: UE Tx power for FDD band shall be set at min{23dBm, Pcmax,L,f,c} and Tx power for TDD band shall be set at min{27.8dBm, Pcmax,L,f,c}. | 26dB NOTE: UE Tx power for FDD band shall be set at min{26dBm, Pcmax,L,f,c} and Tx power for TDD band shall be set at min{26dBm, Pcmax,L,f,c}. |

- Approach 2: Define same requirements based on the worst case.

This approach uses the worst MSD for each band comb i.e.PC2+PC2 power configuration. And similar as approach 1, it doesn’t consider the Tx number information in the MSD definition.

However, one point needs to be carefully noticed is that for UE with PC3+PC1.5 power capability, it cannot support the power configuration of 26dBm+26dBm in MSD tests. The power configuration needs to be clear and avoid UE cannot reach the targeted power.

If go with this approach, one problem might be the work handling considering some times one company request PC3 FDD + PC1.5 TDD, but the other company request PC2 FDD + PC2 TDD and these two band combinations are fall into different TR. Cross checking will cause additional efforts.

|  | MSD for (PC2 FDD + PC2/1.5 TDD), and (PC3 FDD + PC1.5 TDD) |
| --- | --- |
| CA_n71-n77 | 26dB NOTE: - If FDD band supports PC2, the UE Tx power for FDD band shall be set at min{26dBm, Pcmax,L,f,c} and Tx power for TDD band shall be set at min{26dBm, Pcmax,L,f,c}, - otherwise, the UE Tx power for FDD band shall be set at min{23dBm, Pcmax,L,f,c} and Tx power for TDD band shall be set at min{27.8dBm, Pcmax,L,f,c}. |

Observation 2:   If define same requirements for both (PC2 FDD+PC2/1.5 TDD), and (PC3 FDD+PC1.5 TDD), the configured power in MSD requirement needs to be differentiate, otherwise, there is possibility that UE only support PC3 in FDD cannot reach the targeted power level of 26dBm.

Comparing the above two approaches, we slightly prefer to use approach 1 which is more accurate in MSD definition and clean in the work management by reducing the cross checking on the other band combination progress.

Proposal 1:         Consider following two approaches of MSD definition in the PC1.5 CA/DC with HPUE on both FDD and TDD bands:

- Approach 1: Define separate requirements in the spec with information of power capability in each band.

|  | MSD for PC3 in FDD and PC1.5 in TDD | MSD for PC2 in FDD and PC2/1.5 in TDD |
| --- | --- | --- |
| CA_n71-n77 | 16dB NOTE: UE Tx power for FDD band shall be set at min{23dBm, Pcmax,L,f,c} and Tx power for TDD band shall be set at min{27.8dBm, Pcmax,L,f,c}. | 26dB NOTE: UE Tx power for FDD band shall be set at min{26dBm, Pcmax,L,f,c} and Tx power for TDD band shall be set at min{26dBm, Pcmax,L,f,c}. |

- Approach 2: Define same requirements based on the worst case.

|  | MSD for (PC2 FDD + PC2/1.5 TDD), and (PC3 FDD + PC1.5 TDD) |
| --- | --- |
| CA_n71-n77 | 26dB NOTE: - If FDD band supports PC2, the UE Tx power for FDD band shall be set at min{26dBm, Pcmax,L,f,c} and Tx power for TDD band shall be set at min{26dBm, Pcmax,L,f,c}, - otherwise, the UE Tx power for FDD band shall be set at min{23dBm, Pcmax,L,f,c} and Tx power for TDD band shall be set at min{27.8dBm, Pcmax,L,f,c}. |

Proposal 2:         Adopt option 1 in Rel-19 MSD analysis, i.e., 26dBm+26dBm power configuration is used in IMD MSD analysis for PC1.5 CA/DC with HPUE in both FDD and TDD bands.

- Conclusion

This paper analyzed the MSD for PC1.5 CA/DC with HPUE on both FDD and TDD bands, got below proposals.

Observation 1:   Some PC1.5 CA/DC band combinations have been introduced in Rel-18 when 3Tx feature was introduced. At that time, there is no FDD HPUE in the spec, therefore, the PC1.5 CA/DC was based on power capability of PC3+PC1.5 in each band. Correspondingly, the MSD was calculated based on 23dBm in PC3 band and 24.8 *2 in the PC1.5 band.

Observation 2:   If define same requirements for both (PC2 FDD+PC2/1.5 TDD), and (PC3 FDD+PC1.5 TDD), the configured power in MSD requirement needs to be differentiate, otherwise, there is possibility that UE only support PC3 in FDD cannot reach the targeted power level of 26dBm.

Proposal 1:         Consider following two approaches of MSD definition in the PC1.5 CA/DC with HPUE on both FDD and TDD bands:

- Approach 1: Define separate requirements in the spec with information of power capability in each band.

|  | MSD for PC3 in FDD and PC1.5 in TDD | MSD for PC2 in FDD and PC2/1.5 in TDD |
| --- | --- | --- |
| CA_n71-n77 | 16dB NOTE: UE Tx power for FDD band shall be set at min{23dBm, Pcmax,L,f,c} and Tx power for TDD band shall be set at min{27.8dBm, Pcmax,L,f,c}. | 26dB NOTE: UE Tx power for FDD band shall be set at min{26dBm, Pcmax,L,f,c} and Tx power for TDD band shall be set at min{26dBm, Pcmax,L,f,c}. |

- Approach 2: Define same requirements based on the worst case.

|  | MSD for (PC2 FDD + PC2/1.5 TDD), and (PC3 FDD + PC1.5 TDD) |
| --- | --- |
| CA_n71-n77 | 26dB NOTE: - If FDD band supports PC2, the UE Tx power for FDD band shall be set at min{26dBm, Pcmax,L,f,c} and Tx power for TDD band shall be set at min{26dBm, Pcmax,L,f,c}, - otherwise, the UE Tx power for FDD band shall be set at min{23dBm, Pcmax,L,f,c} and Tx power for TDD band shall be set at min{27.8dBm, Pcmax,L,f,c}. |

Proposal 2:         Adopt option 1 in Rel-19 MSD analysis, i.e., 26dBm+26dBm power configuration is used in IMD MSD analysis for PC1.5 CA/DC with HPUE in both FDD and TDD bands.

- Reference

[1] RP-250806 Revised WID, China Telecom, RAN#107 2025-03

[2] R4-2504614 MSD for 3Tx UL CA combinations with 2Tx FDD + 1Tx TDD, T-Mobile USA, #114bis 2025-04