3GPP TSG-RAN WG4 Meeting #115		R4-2506920
Malta, MT, 19th – 23rd May, 2025

Title:	R19 on NTN SAR

Source:	OPPO

Agenda item:	7.10.3.3.1

Document for:	Approval

- Introduction

NTN SAR has been discussed for a long time in this WI, however, with no much progress. The group needs to focus on the SAR solutions that can avoid UE power back off caused by SAR considering Tx power is already quite limited in NTN system. Any large power back off may cause Radio Link Failure.

This paper discusses the NTN SAR related issues per WF [1] as below with duty cycle-based SAR solutions, and also discussed the topic raised in last meeting, i.e. RF component capability in RF transmit time.

Issue 2-1-2: Whether and how to introduce duty cycle related method. - WF - Further discuss the proposed schemes related to duty cycle related SAR scheme. - Other schemes or no duty cycle related scheme are not precluded. Other (For information): - Interesting companies can provide inputs on UE RF components capabilities for FDD and HD-FDD especially for handheld HPUE, including NR-NTN and Iot-NTN in next meeting.

- Discussion

  - Tx time restriction

Duty cycle approach has been widely used in the TN system to solve the SAR issue in TDD bands, TDD+TDD band combinations and TDD+FDD band combinations at least from specification perspective. In principle, this can also be applied to NTN HPUE.

The main problem in current TN duty cycle solution is no differentiation of UE operating status in the field, i.e. whether UE need to meet SAR or not in certain period. For example, if UE is put on the table and far away from human body, in this case the SAR should not be considered and UE can transmit with 100% UL duty cycle. If follow current TN duty cycle scheme, then UE will always be restricted to 50% UL duty cycle. This makes UE be degraded in the network.

Therefore, in [2] some enhancements have been provided, i.e. the duty cycle counting needs to take UE operating status into account.

Proposal 1:         Duty cycle-based SAR solution should not limit UE Tx time when SAR is not required in certain UE operating status.

  - Whether mandate UE power back off

In TN system, especially FR1 HPUE, power back off is mandated when scheduled duty cycle exceeds the reported duty cycle capability. However, as discussed in section 2.1, without differentiating UE operating status, mandating UE to do power back off might limits UE UL performance epically when SAR is not required. This makes the solution cannot be implemented in the real network.

To make the duty cycle be more meaningful and be implemented in the end, while keep the changes to current duty cycle scheme as small as possible, it might be better to still mandate UE power back off as in TN, but differentiate UE working status in terms of whether SAR is required in a time duration.

For example, UE indicates whether the following time duration needs to be counted in UL duty cycle statistics to facilitate NW scheduling and also SAR tests. The time duration can be 1s or even longer considering SAR tests is in minutes level.

Proposal 2:         To make the duty cycle scheme be useful in the NW, while keep the changes to current duty cycle scheme as small as possible, it might be better to still mandate UE power back off as in TN, but differentiate UE working status in terms of whether SAR is required in a time duration.

        - For example, UE indicates whether the following time duration (1s or even longer) needs to be counted in UL duty cycle statistics to facilitate NW scheduling and also SAR tests.

  - RF component Tx time restriction

It was raised in last meeting that some RF components like duplexers might have some Tx time restriction due to thermal issues and ask companies to check.

Based on the discussion, it seems this restriction is caused by following

- Currently, the FDD HPUE in 38.101-1 restricts UE Tx time to 50% (default maxUplinkdutycycle value) if UE doesn’t indicate specific capability value to NW. And power back off is required when the scheduled duty cycle exceeds UE maxUplinkdutycycle capability due to SAR.

- Duplexers are designed according to this 50% duty cycle restriction. And if scheduled duty cycle exceeds 50%, then in some cases UE will face thermal/heating issues, and power back off may happen.

And from the impression of offline discussions, it seems the proposal is not mandating UE to do power back off when the scheduled duty cycle exceeds 50%.

Here, it seems mixed duty cycle restrictions caused by SAR and thermal together. The maxUplinkdutycycle defined in 38.101-1 is optional capability, and UE can report values between 50% and 100%. If UE has problems no matter due to SAR or thermal, it can choose to report the proper value that it can meet. This seems already can solve the concern of mandating UE to do power back off when scheduled duty cycle > 50%?

Or, it considers some other factors like environmental temperature and UE may do power back off when the environmental temperature is high, and may not do power back off when the environmental temperature is low? If this is the case, then some dynamic indication to NW may be needed?

Another point not quite clear is that FDD HPUEs have been introduced since Rel-17 with 1Tx or 2Tx. It hasn’t been raised such thermal caused Tx time restriction though it is clear that higher Tx power will cause higher temperature especially in small size components. Thermal caused issue might be different from component to component, does it can be solved by implementation?

Observation 1:   It is unclear how this RF component Tx time restriction happens in terms of thermal, and may need to clarify below questions:

        - The maxUplinkdutycycle defined in 38.101-1 is optional capability, and UE can report values between 50% and 100%. If UE has problems no matter due to SAR or thermal, it can choose to report the proper value that it can meet. This seems already can solve the concern of mandating UE to do power back off when scheduled duty cycle > 50%, e.g. UE can report the largest value that it doesn’t need to do power back off?

        - Does the thermal issue raised considers some other factors like environmental temperature and UE may do power back off when the environmental temperature is high, and may not do power back off when the environmental temperature is low? If this is the case, then some dynamic indication to NW may be needed?

        - Thermal caused issue might be different from component to component, does it can be solved by implementation?

- Conclusion

In this paper we have analyzed the NTN SAR regulations and found there are some differences from TN SAR. Then analyzed the reason why current TN SAR solutions haven’t been implemented by any UE, and regarding these problems some enhancements are proposed.

Proposal 1:         Duty cycle-based SAR solution should not limit UE Tx time when SAR is not required in certain UE operating status.

Proposal 2:         To make the duty cycle scheme be useful in the NW, while keep the changes to current duty cycle scheme as small as possible, it might be better to still mandate UE power back off as in TN, but differentiate UE working status in terms of whether SAR is required in a time duration.

        - For example, UE indicates whether the following time duration (1s or even longer) needs to be counted in UL duty cycle statistics to facilitate NW scheduling and also SAR tests.

Observation 1:   It is unclear how this RF component Tx time restriction happens in terms of thermal, and may need to clarify below questions:

        - The maxUplinkdutycycle defined in 38.101-1 is optional capability, and UE can report values between 50% and 100%. If UE has problems no matter due to SAR or thermal, it can choose to report the proper value that it can meet. This seems already can solve the concern of mandating UE to do power back off when scheduled duty cycle > 50%, e.g. UE can report the largest value that it doesn’t need to do power back off?

        - Does the thermal issue raised considers some other factors like environmental temperature and UE may do power back off when the environmental temperature is high, and may not do power back off when the environmental temperature is low? If this is the case, then some dynamic indication to NW may be needed?

        - Thermal caused issue might be different from component to component, does it can be solved by implementation?

- Reference

[1] R4-2505114 WF on HPUE RF requirements for IoT-NTN, vivo, #144bis 2025-04

[2] R4-2504172 R19 on NTN SAR, OPPO, #144bis 2025-04