3GPP TSG-RAN WG4 Meeting #115	R4-2506929

# Malta, MT, 19th – 23rd May, 2025

Source: 	KDDI

Title: 	(NR_newRAT-Core) WF on NR A-MPR reevaluation scenarios on n1 for Japan

Agenda Item:	4.2.1

Document for:	Discussion

- 1. Introduction

In the previous meeting, RAN4 triggered NR A-MPR reevaluation scenarios and to specify new NS_05/NS_05U for n1 PC3 and PC2 on n1 for Japan [1]. In last meeting, WF [2] was agreed. We propose way forwards for NS_05U and NS_05 A-MPR in band n1 for PC2 and PC3.

- 2. Discussion

  - PC2 A-MPR region for NS_05U and NS_05

A-MPR region definition

In previous meetings, four companies proposed PC2 A-MPR region for NS_05U and NS_05 [3][4][5][6] and RAN4 also approved WF on PC3 A-MPR region. Based on them, in advance, we propose WF as starting points to discuss PC2 A-MPR region and reach common consensus ASAP in this meeting.

    - PC2 NS_05U

We propose WF as a starting point to discuss PC2 A-MPR for NS_05U region as follows.

Proposal 1: Consider Table 1 as PC2 A-MPR region for NS_05U.

Table 1: A-MPR regions for NS_05U (Power Class 2)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A1 |  | Region A2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | RBstart |
| 5 |  | N/A | N/A | N/A | N/A |
| 10 | < 1930.7 | ≥ 1.35*RBstart + 4.32MHz/12/scs (24RB) | ≥ 0 | ≤ 2.16MHz/12/scs (12RB) | ≤ 1.8MHz/12/scs (10RB) |
| 15 | 1927.5 ≤ fc <1932.5 | ≥ RBstart*1.2 + 3.96MHz/12/scs(22RB ) | ≥ 0 | ≤ RBstart*1.2 + 3.96MHz/12/scs(22RB) ≤ 4.5MHz/12/scs (25RB) | ≤ 3.6MHz/12/scs (20RB) |
| 15 | 1932.5 ≤ fc < 1938.2 | ≥ 8.28MHz/12/ scs (46RB) | ≤ 1.26MHz/12/scs (7RB) | ≤ 2.16MHz/12/scs (12RB) | ≤ 2.16MHz/12/scs (12RB) |
| 20 | < 1945.7 | ≥ 1.44*Rbstart + 4.16MHz/scs/12 (23RB) | ≥ 0 | ≤ 6.48MHz/12/scs; (36RB) ≤ RBstart*1.5 + 4.14MHz/12/scs (23RB) | ≤ 4.68MHz/12/scs (26RB) |
| NOTE: When NS_05U is signalled, Region A3 is applicable and adopts its definitions from NS_100 requirement. |  |  |  |  |  |

    - PC2 NS_05

We propose WF as a starting point to discuss PC2 A-MPR region for NS_05 as follows.

Proposal 2: Consider Table 2 as PC2 A-MPR region for NS_05.

Table 2: A-MPR regions for NS_05 (Power Class 2)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A1 |  | Region A2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | RBstart |
| 5 |  | N/A | N/A | N/A | N/A |
| 10 | < 1930.7 | ≥ 1.35*RBstart + 4.32MHz/12/scs (24RB) | ≥ 0 | ≤ 2.16MHz/12/scs (12RB) | ≤ 1.8MHz/12/scs (10RB) |
| 15 | 1927.5 ≤ fc <1932.5 | ≥ RBstart*1.2 + 3.96MHz/12/scs(22RB ) | ≥ 0 | ≤ RBstart*1.2 + 3.96MHz/12/scs(22RB) ≤ 4.5MHz/12/scs (25RB) | ≤ 3.6MHz/12/scs (20RB) |
| 15 | 1932.5 ≤ fc < 1938.2 | ≥ 8.28MHz/12/ scs (46RB) | ≤ 1.26MHz/12/scs (7RB) | ≤ 2.16MHz/12/scs (12RB) | ≤ 2.16MHz/12/scs (12RB) |
| 20 | < 1945.7 | ≥ 1.0*12*SCS*RBstart + 4.14MHz/12/scs (23RB) | ≥ 0 | ≤ 6.48MHz/12/scs; (36RB) ≤ RBstart*1.5 + 4.14MHz/12/scs (23RB) | ≤ 4.68MHz/12/scs (26RB) |
| NOTE: When NS_05U is signalled, Region A3 is applicable and adopts its definitions from NS_100 requirement. |  |  |  |  |  |

A-MPR Value for PC2 2Tx

In the last meeting, one company observed that 2Tx A-MPR is 0.5~1dB larger than 1Tx AMPR in the same configurations for NS_05U and NS_05 [5][6]. And then, they proposed to accommodate 2Tx PA configurations and simplify the tables, define one set AMPR for n1 PC2 NS_05 covering both 1Tx and 2Tx. The A-MPR needed for 2Tx in all configurations need FFS.

From our business point of view, we can accept above proposal by add 0.5~1dB to 1Tx A-MPR values automatically so that RAN4 is able to conclude n1 PHS protection update related works ASAP.

Proposal 3: Accommodate 2Tx PA configurations and simplify the tables, define one set A-MPR for PC2 NS_05U and NS_05 covering both 1Tx and 2Tx.

Proposal 4: Take an approach automatically adding 0.5~1dB to 1Tx A-MPR values for 2Tx in all configurations for PC2 NS_05U and NS_05.

A-MPR Value for PC2 NS_05U and NS_05

If RAN4 agree with proposal 3 and 4, based on companies’ proposals[3][4][5][6], we propose WF in Annex section of this contribution as starting points to discuss A-MPR value for NS_05U and NS_05. On the other hand, companies will propose/update their A-MPR values in this meeting, therefore RAN4 can further discuss details offline in this meeting.

Observation 1: Based on companies’ proposals and our WF in Annex section of this contribution, RAN4 can further discuss A-MPR value for NS_05U and NS_05 offline in this meeting.

  - A-MPR Region Split

WF on PHS protection update (R4-2505222, RAN4#114bis)

In the last meeting, regarding A-MPR region definition, RAN4 agreed with WF [2] as follows.

Sub-topic 2-1: PUCCH transmissions for meeting limits without A-MPR < Issue 2-1-1: PUCCH transmissions for meeting limits without A-MPR > Agreement: - Encourage companies to evaluate the feasibility of the following: - Split the agreed Region A2 into two new regions

Proposal 5: Discuss to split the agreed Region A2 into two new regions.

- 3. Conclusion

Proposal 1: Consider Table 1 as PC2 A-MPR region for NS_05U.

Table 1: A-MPR regions for NS_05U (Power Class 2)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A1 |  | Region A2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | RBstart |
| 5 |  | N/A | N/A | N/A | N/A |
| 10 | < 1930.7 | ≥ 1.35*RBstart + 4.32MHz/12/scs (24RB) | ≥ 0 | ≤ 2.16MHz/12/scs (12RB) | ≤ 1.8MHz/12/scs (10RB) |
| 15 | 1927.5 ≤ fc <1932.5 | ≥ RBstart*1.2 + 3.96MHz/12/scs(22RB ) | ≥ 0 | ≤ RBstart*1.2 + 3.96MHz/12/scs(22RB) ≤ 4.5MHz/12/scs (25RB) | ≤ 3.6MHz/12/scs (20RB) |
| 15 | 1932.5 ≤ fc < 1938.2 | ≥ 8.28MHz/12/ scs (46RB) | ≤ 1.26MHz/12/scs (7RB) | ≤ 2.16MHz/12/scs (12RB) | ≤ 2.16MHz/12/scs (12RB) |
| 20 | < 1945.7 | ≥ 1.44*Rbstart + 4.16MHz/scs/12 (23RB) | ≥ 0 | ≤ 6.48MHz/12/scs; (36RB) ≤ RBstart*1.5 + 4.14MHz/12/scs (23RB) | ≤ 4.68MHz/12/scs (26RB) |
| NOTE: When NS_05U is signalled, Region A3 is applicable and adopts its definitions from NS_100 requirement. |  |  |  |  |  |

Proposal 2: Consider Table 2 as PC2 A-MPR region for NS_05.

Table 2: A-MPR regions for NS_05 (Power Class 2)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A1 |  | Region A2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | RBstart |
| 5 |  | N/A | N/A | N/A | N/A |
| 10 | < 1930.7 | ≥ 1.35*RBstart + 4.32MHz/12/scs (24RB) | ≥ 0 | ≤ 2.16MHz/12/scs (12RB) | ≤ 1.8MHz/12/scs (10RB) |
| 15 | 1927.5 ≤ fc <1932.5 | ≥ RBstart*1.2 + 3.96MHz/12/scs(22RB ) | ≥ 0 | ≤ RBstart*1.2 + 3.96MHz/12/scs(22RB) ≤ 4.5MHz/12/scs (25RB) | ≤ 3.6MHz/12/scs (20RB) |
| 15 | 1932.5 ≤ fc < 1938.2 | ≥ 8.28MHz/12/ scs (46RB) | ≤ 1.26MHz/12/scs (7RB) | ≤ 2.16MHz/12/scs (12RB) | ≤ 2.16MHz/12/scs (12RB) |
| 20 | < 1945.7 | ≥ 1.0*12*SCS*RBstart + 4.14MHz/12/scs (23RB) | ≥ 0 | ≤ 6.48MHz/12/scs; (36RB) ≤ RBstart*1.5 + 4.14MHz/12/scs (23RB) | ≤ 4.68MHz/12/scs (26RB) |
| NOTE: When NS_05U is signalled, Region A3 is applicable and adopts its definitions from NS_100 requirement. |  |  |  |  |  |

Proposal 3: Accommodate 2Tx PA configurations and simplify the tables, define one set A-MPR for PC2 NS_05U and NS_05 covering both 1Tx and 2Tx.

Proposal 4: Take an approach automatically adding 0.5~1dB to 1Tx A-MPR values for 2Tx in all configurations for PC2 NS_05U and NS_05.

Observation 1: Based on companies’ proposals and our WF in Annex section of this contribution, RAN4 can further discuss A-MPR value for NS_05U and NS_05 offline in this meeting.

Proposal 5: Discuss to split the agreed Region A2 into two new regions.

- 4. Reference

[1] R4-2418439 (NR_newRAT-Core) NR A-MPR reevaluation scenarios on n1 for Japan, KDDI, RAN4#113

[2] R4-2505222 WF on PHS protection update, KDDI, RAN4#114bis

[3] R4-2500421 n1 NS_05 A-MPR proposals, Nokia, RAN4#114bis

[4] R4-2503562 n1 NS_05 and NS_05U PC2 A-MPR proposal, Qualcomm, RAN4#114bis

[5] R4-2504166 R16 n1 NS_05 PC2 AMPR, OPPO, RAN4#114bis

[6] R4-2504167 R16 n1 NS_05U PC2 and PC3 AMPR, OPPO, RAN4#114bis

- 5. Annex