3GPP TSG-RAN WG4 Meeting #115	R4-2506972

# Malta, MT, 19th – 23rd May, 2025

Title:	Discussion on partial sphere metric

Source:	Samsung

Agenda item:	7.14.3.2

Document for:	Discussion

1.	Introduction

In RAN4#114bis meeting, the usage scenario and performance metric for NTN handheld UE was agreed as following [1, R4-2504763]:

Issue 2-1-2: performance metric for NR-NTN and IoT-NTN only support scenario 2 Agreements: o For IoT-NTN and NR-NTN, typical usage scenarios are: § Scenario 1: both head+hand mode and hand only mode. § Scenario 2: hand only mode targeted for specific UE orientation usage § Scenario 3: hand only mode targeted for arbitrary UE orientation usage o UE shall declare support for only one scenario per band. The declaration details of each scenario should be further discussed. o For IoT-NTN and NR-NTN handheld UEs declared Scenario 2, the performance metrics are partial sphere metric. FFS details o For IoT-NTN and NR-NTN handheld UEs declared Scenario 3, the performance metrics are full sphere TRP and TRS

Partial sphere metric was agreed as the performance metric for NTN handheld UE declaring scenario 2. As a grand new OTA metric compared with legacy full sphere TRP TRS, it is necessary to discuss the fundamental principles first of all.

In this contribution, we propose several fundamental issues for partial sphere metric and share our views on the basic principles before discussing the metric details.

2. 	Discussion

2.1 DUT orientation for initial test condition

The coordinate system is the basis of discussing OTA performance metric and test method. The DUT orientation for initial test condition is based on the standardized coordinate system.

The reference coordinate system and DUT default alignment to coordinate system for NR FR1 TRP TRS can be found in Annex A of TR 38.870:

Annex A: UE coordinate system A.1 Reference coordinate system This annex defines the measurement coordinate system for NR FR1 TRP and TRS measurements. The reference coordinate system, as defined in [9] is provided in Figure A.1-1 below while A.1-2 shows the DUT in the default alignment for Free Space. Figure A.1-1: Reference coordinate system Figure A.1-2: DUT default alignment to coordinate system The following aspects are necessary: - A basic understanding of the top and bottom of the device is needed in order to define unambiguous DUT positioning requirements for the test, e.g., in the drawings used in this annex, the earpiece is on the top center of the device (front), the three buttons are on the bottom of the device (front) and the camera is on the top of the device (back). - An understanding of the origin and alignment of the coordinate system inside the test system, i.e. the directions in which the x, y, z axes point inside the test chamber, is needed in order to define unambiguous DUT orientation and measurement angles.

The full sphere TRP TRS metric for single AoA testing is not sensitive to DUT orientation for initial test condition, so it could be enough to consider only the default DUT orientation as indicated in Figure A.1-2 of TR38.870. However, when the performance metric would be hemisphere metric or partial sphere metric, the DUT orientation for initial test condition matters, different DUT orientation for initial test condition may lead to different test result especially for constant step size measurement grid.

Observation 1:	for partial sphere metric, different DUT orientation for initial test condition may lead to different test result especially for constant step size measurement grid.

Moreover, if the DUT orientation for initial test condition can be any arbitrary orientation, the existing DUT positioning system may not support full freedom initial DUT orientation unless an additional positioner is to be introduced. This is a big change to existing test system.

Observation 2:	existing DUT positioning system may not support full freedom initial DUT orientation unless an additional positioner is to be introduced which is a big change to existing test system.

Our preference is to test partial sphere metric OTA with existing positioning system of the SISO OTA chamber. With existing test system, the most achievable freedom is to define more alternative DUT orientations for initial test condition. In the testability study for 2AoA test method, more DUT orientations are introduced in Table B.3-1, Table B.3-2 and Table B.3-3 of TR38.871. For convenience, these Tables are copied in Annex of this paper. These 12 DUT orientations can be a reference for the NTN partial sphere metric and test.

Based on above observations and discussion, it is proposed to consider following principles

Proposal 1:	partial sphere metric should be compatible with existing DUT positioning system, i.e., DUT orientation for initial test condition should not rely on the introduction of a new positioner with more freedom.

Proposal 2:	DUT orientation for initial test condition shall be standardized. If more than one allowed, refer to the 12 DUT orientations in Table B.3-1, Table B.3-2 and Table B.3-3 of TR38.871 as starting point.

2.2 upper hemisphere declaration

According to the endorsed CR [2, R4-2504728], hemisphere metric is one special example of partial sphere metric, i.e., UHRP and UHIS.

5.3.1.2 Upper Hemisphere Radiated Power The UHRP metric is defined as total radiated power in three-dimensional upper hemisphere. FFS details

How to define the upper hemisphere in the UE coordination system is not clear yet. There could be following possibilities:

- Hemisphere cut by equator (e.g., the hemisphere with z axis as centroid)

- Hemisphere cut by longitude (e.g., the hemisphere with x axis or y axis or other vector in xy plane as centroid)

RAN4 should discuss the hemisphere metric and declaration by eliminating above ambiguity.

Proposal 3:	RAN4 should discuss the hemisphere metric and declaration by eliminating the ambiguity at least from below alternatives

- Alt1: Hemisphere cut by equator (e.g., the hemisphere with z axis as centroid)

- Alt2: Hemisphere cut by longitude (e.g., the hemisphere with x axis or y axis or other vector in xy plane as centroid)

2.3 partial sphere declaration

According to the endorsed CR [2, R4-2504728], partial hemisphere metric is introduced and details to be further studied.

5.3.1.3 Partial sphere Radiated Power (PsRP) The PsRP metric is defined as the total power radiated in the Theta and Phi range from x to y degrees. FFS details

How to define the partial sphere in the UE coordination system is not clear yet. There could be following possibilities:

- A dome area cut by latitude (phi range 0~360deg, theta range 0~x)

- A quasi-rectangular area cut by two longitude (phi range x1~x2) and two latitude (theta range y1~y2)

RAN4 should discuss the partial sphere metric and declaration by eliminating above ambiguity.

Proposal 4:	RAN4 should discuss the partial sphere metric and declaration by eliminating the ambiguity at least from below alternatives

- Alt1: A dome area cut by latitude (phi range 0~360deg, theta range 0~x)

- Alt2: A quasi-rectangular area cut by two longitude (phi range x1~x2) and two latitude (theta range y1~y2)

3. 	Conclusion

Observation 1:	for partial sphere metric, different DUT orientation for initial test condition may lead to different test result especially for constant step size measurement grid.

Observation 2:	existing DUT positioning system may not support full freedom initial DUT orientation unless an additional positioner is to be introduced which is a big change to existing test system.

Proposal 1:	partial sphere metric should be compatible with existing DUT positioning system, i.e., DUT orientation for initial test condition should not rely on the introduction of a new positioner with more freedom.

Proposal 2:	DUT orientation for initial test condition shall be standardized. If more than one allowed, refer to the 12 DUT orientations in Table B.3-1, Table B.3-2 and Table B.3-3 of TR38.871 as starting point.

Proposal 3:	RAN4 should discuss the hemisphere metric and declaration by eliminating the ambiguity at least from below alternatives

- Alt1: Hemisphere cut by equator (e.g., the hemisphere with z axis as centroid)

- Alt2: Hemisphere cut by longitude (e.g., the hemisphere with x axis or y axis or other vector in xy plane as centroid)

Proposal 4:	RAN4 should discuss the partial sphere metric and declaration by eliminating the ambiguity at least from below alternatives

- Alt1: A dome area cut by latitude (phi range 0~360deg, theta range 0~x)

- Alt2: A quasi-rectangular area cut by two longitude (phi range x1~x2) and two latitude (theta range y1~y2)

4. 	References

- R4-2504763    “WF for [114bis][328] TRP_TRS_MIMO_OTA_Ph3”, vivo

- R4-2504728    “Draft CR to 38.870 on NTN OTA performance metric and test method”, vivo, Samsung, Apple

- TR 38.870

- TR 38.871

5. 	Annex

B.3	Test conditions and angle definitions (of TR38.871)

This sub-annex provides the test conditions and angle definitions for twelve permitted device orientations for the initial test condition. With the convenience of defining the permitted alignments/orientations, they are divided into three groups as shown from Table B.3-1 to Table B.3-3. The DUT orientation to be tested is chosen from Table B.3-1, Table B.3-2 and Table B.3-3 by UE vendor declaration.

Table B.3-1: Test conditions and angle definitions for Alignment Option 1

| Test condition | DUT orientation | Diagram |
| --- | --- | --- |
| Orientation 1 (Option 1) | α = 0º; β = 0º; γ = 0º |  |
| Orientation 1 (Option 2) | α = 0º; β = 0º; γ = 180º |  |
| Orientation 2 (Option 1) | α = 180º; β = 0º; γ = 0º |  |
| Orientation 2 (Option 2) | α = 0º; β = 180º; γ = 0º |  |
| NOTE 1: The combination of rotations is captured by matrix M=Rz()•Ry()•Rx() |  |  |

Table B.3-2: Test conditions and angle definitions for Alignment Option 2

| Test condition | DUT orientation | Diagram |
| --- | --- | --- |
| Orientation 1 (Option 1) | α = 0º; β = -90º; γ = 0º |  |
| Orientation 1 (Option 2) | α = 0º; β = -90º; γ = 180º |  |
| Orientation 2 (Option 1) | α = 180º; β = 90º; γ = 0º |  |
| Orientation 2 (Option 2) | α = 0º; β = 90º; γ = 0º |  |
| NOTE 1: The combination of rotations is captured by matrix M=Rz()•Ry()•Rx() |  |  |

Table B.3-3: Test conditions and angle definitions for Alignment Option 3

| Test condition | DUT orientation | Diagram |
| --- | --- | --- |
| Orientation 1 (Option 1) | α = 90º; β = 0º; γ = 0º |  |
| Orientation 1 (Option 2) | α = 90º; β = 0º; γ = 180º |  |
| Orientation 2 (Option 1) | α = -90º; β = 0º; γ = 0º |  |
| Orientation 2 (Option 2) | α = 90º; β = 180º; γ = 0º |  |
| NOTE 1: The combination of rotations is captured by matrix M=Rz()•Ry()•Rx() |  |  |