3GPP TSG-RAN WG4 Meeting #115	R4-2506973

# Malta, MT, 19th – 23rd May, 2025

Title:	On performance metric for FR1 dynamic MIMO OTA

Source:	Samsung

Agenda item:	7.14.3.3

Document for:	Discussion

1.	Introduction

About the performance metric of dynamic MIMO OTA, it is agreed that different device orientations and rotations shall be combined into a single CDF curve. The open issues include the required percentile value and how to handle Uma and Umi channel models in the final requirement. [1]

Issue 1-2: Dynamic MIMO OTA test and UE performance metric Agreement: - All measured throughput values for different device orientations and rotations shall be combined to derive a single CDF curve for the dynamic MIMO OTA test. FFS whether the performance metric should be based on three (10%/50%/90%) percentiles at the CDF. - Use the following as a starting point: - include both UMa and UMi channels via post measurement data processing. FFS how to do the post data processing.

In this contribution, we share our views on above highlighted item – the required percentile value in final requirement derivation.

2. 	Discussion

In last meeting, there were mainly two proposals for discussion, one is to prioritize the 50%-tile CDF of CTMT [2, R4-2503652], the other is to consider all three percentile values (10%, 50%, 90%) with an average [3, R4-2503990]. The average shown in [3] is as below:

Consider taking the average of {CDF10%TMTUMi, CDF50%TMTUMi, CDF90%TMTUMi, CDF10%TMTUMa, CDF50%TMTUMa, CDF90%(TMTUMa)} as the UE performance metric for dynamic MIMO OTA to reflect an overall performance of a UE.

The group is open with average approach but concerns about the weighting in the average. An arithmetic average without weighting among three percentile values (10%, 50%, 90%) leads to more bias in 90%-tile throughput results and the contribution from 10%-tile throughput results would be underestimated.

One way to overcome the bias issue is to adopt different weighting factors for the three different percentile values (10%, 50%, 90%). The throughput result at 50%-tile can be equally weighted, but the throughput result at 10%-tile should be increasingly weighted and the throughput result at 90%-tile should be decreasingly weighted.

The test result at 10%-tile means that 90% out of all data pool is no smaller than the throughput at 10%-tile;

The test result at 90%-tile means that 10% out of all data pool is no smaller than the throughput at 10%-tile.

That is to say, the test result at 10%-tile stands for 90% of total measurements while the test result at 90%-tile stands for 10% of total measurements, so the weighting ratio between the test result at 10%-tile and the test result at 90%-tile can be designed as 9:1.

With above principle, the weighting factor ratio among 10%-tile, 50%-tile, 90%-tile can be 9:5:1. However, it is not certain the group could achieve consensus about the weighting factors for the average approach, in our understanding, it is still a simple approach to prioritize 50%-tile only.

Base on above discussions, it is proposed for RAN4 to discuss the dynamic MIMO OTA performance metrics between the two options:

- Option 1: take 50%-ile CDF of CTMT

- Option 2: take a weighted average among 10%-tile, 50%-tile, 90%-tile CDF of CTMT, and consider the weighting factor ratio among 10%-tile, 50%-tile, 90%-tile as 9:5:1

Proposal 1:	it is proposed for RAN4 to discuss the dynamic MIMO OTA performance metrics between the two options:

-	Option 1: take 50%-ile CDF of CTMT

-	Option 2: take a weighted average among 10%-tile, 50%-tile, 90%-tile CDF of CTMT, and consider the weighting factor ratio among 10%-tile, 50%-tile, 90%-tile as 9:5:1

3. 	Conclusion

Proposal 1:	it is proposed for RAN4 to discuss the dynamic MIMO OTA performance metrics between the two options:

-	Option 1: take 50%-ile CDF of CTMT

-	Option 2: take a weighted average among 10%-tile, 50%-tile, 90%-tile CDF of CTMT, and consider the weighting factor ratio among 10%-tile, 50%-tile, 90%-tile as 9:5:1

4. 	References

- R4-2502296    “WF for [114][328] MIMO_OTA_Ph3”, CAICT

- R4-2503652    “On performance metric for FR1 dynamic MIMO”, Samsung

- R4-2503990    “On Rel-19 MIMO OTA”, CAICT