3GPP TSG RAN WG4 Meeting #115		 R4-2506986

St Julian’s, Malta  19th – 23th May, 2025

Agenda Item:	7.5.2

Source: 	MediaTek Inc.

Title: 	TP for TR38.755: Summary for NR FR1 DL fragment carriers study

Document for:	Approval

1. Introduction

The SI[1] for Rel-19 NR FR1 DL fragmented carriers has been studied and discussed from RAN4#112 to RAN4#115. Method for reducing number of UE Rx chains, architectures has been discussed. The power spectral density difference between carriers of co-located adjacent channel operators also have been studied. Further more, the potential adjusted RF requirements and signalling indication and UE behaviour were also been studied. The contribution provide summary of these aspects.

2. TP for TR38.755

<Start of TP>

9 Summary for NR FR1 DL fragment carriers study

The non-contiguous intra-band carrier aggregation (NC IB CA) have been introduced in the RAN4 specs for years. The legacy UE receiver is implemented with partially-shared architecture which requires separate Rx paths and Rx LOs for receiving each carriers that limit the CA capabilities as the maximum supported number of component carriers. In this study, RAN4 figured out one Rx RF chain (fully shared receiver) architecture to reduce the number of Rx chains for receiving intra-band DL NCCA so that UE can support receiving more DL carriers with remaining Rx chains.

From the study on power spectral density difference between carriers of co-located adjacent channel operators, the in-gap interference to wanted signal ratio (ISR ratio) can distribute from 20dB to less than 0dB due to different gNB antenna pattern assumptions from system level simulation. The simulation further shows if the network shifts i.e., non-collocated adjacent channel operators, the delta RSRP can distribute from tens of dBs to negative tens of dBs which implies the in-gap interference rejection capability need to be as good as possible.

The adjusted RF requirements for example bands have also been evaluated by companies. There would be REFSENS degradation due to one Rx RF chain mode on both PCC and SCC in the FDD bands due to self-Tx leakage impact. The network vendor express strong concerns to the degradation on the Pcell when one Rx RF chain mode is configured.

In summary, if degradation on Pcell is strictly forbidden, no PCC degradation worse than REFSENS level in one Rx RF chain mode, then the operation scenarios can be considered for intra-band non-contiguous CA:

- FDD band DL fragmented carriers

  - CBW of both DL fragmented carriers + Wgap are small enough

  - UE locates near gNB that UL Tx power level don’t need to be large

  - For higher order CA combination, both intra-band DL fragmented carriers are SCC (UL is in another band)

  - The frequency allocation of PCC and SCC to avoid worst case configuration on DL performance i.e., PCC/SCC swapping

- TDD band DL fragmented carriers

The signalling indication has also been discussed and agreed on the following two aspects[22]. The down select or final decision can be made during work item phase:

- Alternative 1: use the legacy band combination reporting scheme + Rx sharing capability indication

- Alternative 2: use the new notation or new bandwidth class for CA configurations with single Rx RF Chain

As for UE behaviour regarding CA fallback and architecture switching were also discussed. The baseline fallback when UE is configured one Rx RF chain mode is fallback to single CC. As for other fallback configurations, option2 and option 3 as illustrated in [Fig XXXXX-1], that may happen with architecture switching together, there’s UE capability dependency that may need additional UE capability indication such as ”CA fallback combination with architecture switching”.

The criteria of enabling one Rx RF chain mode have also been discussed. One approach is rely on existing channel quality reporting and procedures without new indication from UE. The [new measurement property] may still be needed when applying legacy measurement approach. The other alternative is UE can do measurement to determine and indicate to the network whether it can support non-contiguous CCs with one Rx RF chain. The flow of UE state change is also captured in the technical report for reference.

With the study outcome, RAN4 conclude it is feasible for NR UE to enable one Rx RF chain mode to receive two DL non-contiguous carriers.

<End of TP>