3GPP TSG RAN WG4 Meeting #115		R4- 2507005

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.14.3.1

Source:	Vodafone, Telecom Italia S.p.A., Deutsche Telekom

Title:	Discussion on OTA requirements for XR Headworn Devices

WI/SI:	TRP_TRS_MIMO_OTA_Ph3

Release:	Rel-19

Document for:	Approval

# 1	Introduction

During RAN4#114-bis meeting, XR TRP/TRS test procedures and alternative test solutions were discussed [1]. This contribution introduces a modified approach for XR OTA requirements, aiming to ensure good performance for XR headworn devices, simplify antenna design, and reduce testing time.

# 2	Discussion

For mid-bands, four RX antennas are typically required to capture the full spatial nuances of the antenna’s radiation pattern. However, for XR headworn devices, the minimum requirement in these bands has been reduced to two RX antennas. This reduction has several implications, leading to the following observations:

- Observation 1: With fewer RX antennas, the likelihood for creation of deep nulls in downlink radiation pattern increases. The more antennas are spatially distributed, the smoother the radiation patter thereby reducing the likelihood of deep nulls.

- Observation 2: With a reduced number of antennas, there is a potential for coverage gap. Even if the averaged TRP & TRS values appears acceptable, localized regions of poor performance may still exist. In a fixed-use scenario against user’s body like that of XR headworn, these gaps can significantly degrade the user experience, especially in areas where consistent connectivity is essential.

- Observation 3:  UE manufacturers must now optimize antenna designs and calibration procedures, considering that fewer RX antennas might lead to insufficient performance, such as low throughput and coverage.

General XR TRP/TRS test procedure

The current TRP/TRS minimum requirement approach has been sufficient for many devices, but it presents a significant shortcoming for XR headworn devices, which are worn with a consistent orientation against the user's body. In such scenarios, even if the overall TRP/TRS meets the required standard, a few low-power measurements (deep nulls) in certain directions may lead to inconsistent coverage. For XR headworn devices, where performance uniformity is crucial for maintaining seamless connectivity in an immersive experience, such nulls can lead to user dissatisfaction and degraded performance and coverage.

These issues underscore the need for a more granular analysis approach that does not allow the averaging process to obscure significant performance deficiencies. The fixed orientation of XR headworn makes them particularly vulnerable to the effects of deep nulls, and with the reduced RX antennas, it is imperative that evaluation and minimum requirement methods evolve accordingly. At the same time, the fixed orientation can be exploited to optimize antenna designs to prioritize certain directions and deprioritize others.

Proposal 1: Introduce high priority and low priority directions for XR headworn TRP/TRS minimum requirements, each with its own minimum TRP/TRS values. Relaxed minimum requirements are expected for low priority directions, while higher minimum requirements are expected for high priority directions. The determination of the range for both directions can be discussed.

Proposal 2: Introduce a new metric for XR headworn devices as an additional minimum requirement for high priority directions. This metric could be implemented without altering the current measurement procedure or requiring additional equipment, ensuring seamless integration into existing testing frameworks.

Proposal 3: The following are the key elements for introducing a new metric for XR headworn:

- Measurement Procedures: The standard method for TRP measurement could remain unchanged. Effective Isotropic Radiated Power (EIRP) and Effective Isotropic Sensitivity (EIS), are measured and saved for each angle/position.

- Introducing a Post-Measurement Threshold Analysis: Once the data is collected, each individual measurement point is compared against threshold values of EIRP and EIS. Determining the exact threshold values will be FFS. Every measurement point must meet or exceed the agreed-upon threshold (above min EIRP and EIS threshold values).

Alternative test solutions

One of the challenges for XR headworn devices during OTA testing is their lower battery life compared to handheld devices. Increasing the grid size for TRP and TRS testing can help reduce the measurement time.

Proposal 4: Increase the measurement grid size for XR low priority directions.

# 3	Conclusions

In the following, the observations and proposals of the contribution are summarized:

Observation 1: With fewer RX antennas, the likelihood for creation of deep nulls in downlink radiation pattern increases. The more antennas are spatially distributed, the smoother the radiation patter thereby reducing the likelihood of deep nulls.

Observation 2: With a reduced number of antennas, there is a potential for coverage gap. Even if the averaged TRP & TRS values appears acceptable, localized regions of poor performance may still exist. In a fixed-use scenario against user’s body like that of XR headworn, these gaps can significantly degrade the user experience, especially in areas where consistent connectivity is essential.

Observation 3:  UE manufacturers must now optimize antenna designs and calibration procedures, considering that fewer RX antennas might lead to insufficient performance, such as low throughput and coverage.

Proposal 1: Introduce high priority and low priority directions for XR headworn TRP/TRS minimum requirements, each with its own minimum TRP/TRS values. Relaxed minimum requirements are expected for low priority directions, while higher minimum requirements are expected for high priority directions. The determination of the range for both directions can be discussed.

Proposal 2: Introduce a new metric for XR headworn devices as an additional minimum requirement for high priority directions. This metric could be implemented without altering the current measurement procedure or requiring additional equipment, ensuring seamless integration into existing testing frameworks.

Proposal 3: The following are the key elements for introducing a new metric for XR headworn:

- Measurement Procedures: The standard method for TRP measurement could remain unchanged. Effective Isotropic Radiated Power (EIRP) and Effective Isotropic Sensitivity (EIS), are measured and saved for each angle/position.

- Introducing a Post-Measurement Threshold Analysis: Once the data is collected, each individual measurement point is compared against threshold values of EIRP and EIS. Determining the exact threshold values will be FFS. Every measurement point must meet or exceed the agreed-upon threshold (above min EIRP and EIS threshold values).

Proposal 4: Increase the measurement grid size for XR low priority directions.

# 4	References

- R4-2504763, “WF for [114bis][328] TRP_TRS_MIMO_OTA_Ph3”, vivo, April 2025.