3GPP TSG-RAN WG4 Meeting #115                                                          R4-2507006

Malta, MT, 19th - 23rd May, 2025

Source: 	NTT DOCOMO, INC.

Title:	Discussion on modifiedMPR-behaviour for n1

Agenda item:	5.3.1

Document for:	Approval

- Introduction

Due to changes in the requirements for protecting PHS from n1, reevaluating A-MPR values for n1 is currently discussed. RAN4 agreed that it can use modifiedMPR-behaviour and reuse NS_05 and NS_05U instead of introducing new NS to introduce new smaller A-MPR for protecting DECT. Companies may have different understandings of how to describe modifiedMPR-behaviour in the specifications of each release. Therefore, this paper shares our views to clarify these.

- Background

Based on regulatory changes in Japan, it is necessary to protect DECT instead of PHS for n1 newly. The requirements for protection are relaxed, so RAN4 is currently discussing reevaluating A-MPR values for n1. Regarding NS value to be used to indicate the new A-MPR requirements, the agreement at RAN4#113 is captured in WF [1] as follows.

< Issue 1-1-3: NS value to be used to indicate the new A-MPR requirements > Way Forward: - Introduce new A-MPR in Rel-19 spec and reuse NS_05/NS_05U instead of introducing new NS. UE can support it as optional before Rel-19.

NS_05 and NS_05U have been introduced since Rel-15 for protecting PHS from n1. Based on the above agreement, RAN4 can use modifiedMPR-behaviour and reuse NS_05 and NS_05U instead of introducing new NS to introduce new smaller A-MPR for protecting DECT. However, at the last meeting, it was not decided how to align the modifiedMPR-Behavior among releases [2].

< Issue 2-5-1: modifiedMPR-Behaviour > Way forward: - Continue further discussion based on more inputs in next meeting.

Companies may have different understandings. Therefore, clarifying these points will help reuse existing NS and introduce the new smaller A-MPR for n1.

- Discussion

As organized in our contribution [3], when referencing the TS38101-1 specifications for each release, it is noted that in the modifiedMPR Table defining the field modifiedMPR-behaviour, "shall" is used when it is mandatory to meet the new MPR requirement, while "may" is used when it is optional. As an example, the following is excerpt from the modifiedMPR table in TS38.101-1 v18.8.0.

Table 3-1: Excerpt from modifiedMPR table in TS38101-1 v18.8.0 (Table L.1-1: Definitions of the bits in the field modifiedMPR-Behaviour)

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n71 | 0 (leftmost bit) | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)71AA UE EN-DC |
| n34 | 0 (leftmost bit) | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, PC 1.5 MPR as defined in Table 6.2D.2-2 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |

Observation 1: In the modifiedMPR Table, "shall" is used when it is mandatory to satisfy the new MPR, and "may" is used when it is optional.

In the draft CRs submitted at RAN4#114bis [4][5], "shall" is used for Rel-19 and "may" is used for releases prior to Rel-18. According to the above Observation 1, this indicates that it is mandatory to use a smaller new A-MPR value from Rel-19 onwards, while allowing the UE to optionally support the new MPR requirements for releases prior to Rel-18. This appears to be consistent with previous agreements made in RAN4.

Table 3-2: modifiedMPR Table for release prior to Rel-18 [4]

Table 3-3: modifiedMPR Table for Rel-19 [5]

Moreover, from the perspective of network implementation, this approach is preferable. The network can easily understand that the UE meets the new MPR requirements for Rel-19 when the UE sets the modifiedMPR-behaviour bit to 1, or the old MPR requirements when it is set to 0. If it were not mandatory to set the modifiedMPR-behaviour bit to 1 for Rel-19, the network implementation would become more complex. In this case, when the modifiedMPR-behaviour bit is set to 0, the network would need to check which release the UE supports (e.g., accessStratumRelease) to determine whether the old or new MPR requirements are met. To avoid complicating the network implementation, it is preferable to use "shall" for Rel-19.

Observation 2: By making it mandatory for UEs supporting Rel-19 to set the modifiedMPR-behaviour bit to 1, the network eliminates the need to verify which release the UE supports, thus allowing for easier understanding of which MPR requirements the UE meets.

Proposal 1: To avoid complications for the network, the modifiedMPR Table for n1 uses "shall" to mandate setting the bit to 1 for Rel-19, and "may" to allow for it to be optional for releases prior to Rel-18.

Nevertheless, I understand the motivation behind the need to simplify the current modifiedMPR Table, which is quite convoluted. In fact, as has already been pointed out, the n41 case features multiple instances of "shall" and "may", resulting in ambiguous interpretations. This confusion is considered avoidable thorough discussions during the introduction phase. RAN4 can amend the existing contradictory statements within the modifiedMPR Table and ensure that the similar contradictions do not arise in the future.

Observation 3: RAN4 can avoid complexity by correcting several contradictions in the current regulations and making an effort to thoroughly discuss the validity of the modifiedMPR Table during its future introduction.

- Conclusion

Here we summarize our proposals:

Observation 1: In the modifiedMPR Table, "shall" is used when it is mandatory to satisfy the new MPR, and "may" is used when it is optional.

Observation 2: By making it mandatory for UEs supporting Rel-19 to set the modifiedMPR-behaviour bit to 1, the network eliminates the need to verify which release the UE supports, thus allowing for easier understanding of which MPR requirements the UE meets.

Proposal 1: To avoid complications for the network, the modifiedMPR Table for n1 uses "shall" to mandate setting the bit to 1 for Rel-19, and "may" to allow for it to be optional for releases prior to Rel-18.

Observation 3: RAN4 can avoid complexity by correcting several contradictions in the current regulations and making an effort to thoroughly discuss the validity of the modifiedMPR Table during its future introduction.

References

- R4-2420481, “WF on PHS protection update”, KDDI, OPPO, RAN4#113

- R4-2505222, “WF on PHS protection update”, KDDI, OPPO, RAN4#114bis

- R4-2501736, “Clarification of modifiedMPR-behaviour for n1, NTT DOCOMO, INC., RAN4#114

- R4-2504570, “Draft CR to 38.101-1: Modification of additional requirements indicated by NS_05/NS_05U”, Ericsson, Samsung, RAN4#114bis

- R4-2504571, “Draft CR to 38.101-1: Modification of additional requirements indicated by NS_05/NS_05U”, Ericsson, Samsung, RAN4#114bis