3GPP TSG RAN WG4 Meeting #115	R4-2507019

Saint Julian's, MT, 19 May 2025 – 23 May 2025

Agenda Item:	5.19.1

Source: 	Ericsson

Title:	Maintenance of L1/L2 based inter-cell mobility

Document for:	Discussion

# Introduction

RAN4 has agreed to support LTM without L1-RSRP support or without L1-RSRP reports. In this contribution, we address some of the key remaining open issues related to supporting LTM implementation.

# Discussion

Collision between SSB and Valid RO for LTM

During the last meeting, the prioritization between SSB and RACH occasions was discussed. The following agreements were made.

Agreement on reply LS

- RACH shall be prioritized over SSB, if the collision is outside MG.

- If collision happens within MG, it is up to UE implementation, i.e., UE may transmit or drop RACH when the RACH occasion colliding with the MG occasion.

< Agreement >

- Further discuss the definition of RACH occasion colliding with the MG occasion.

One of the open issues in the above agreement is the definition of RO collision with MG. When the UE is requested to perform RACH on candidate cells, RF retuning is required both before and after RACH, depending on the scenario. This retuning can take up to Y = 2 ms, as reported by the UE via pdcch-RACH-SwitchingTimeList-r18.

If the NW configures PRACH preamble format 1, which spans 3 slots, and the UE reports Y = 2 ms, the total RO duration can extend to 7 slots (2 slots for RF retuning before, 3 slots for the preamble, and 2 slots for RF retuning after). As shown in Figure 1, this extended RO may result in MG colliding with two consecutive ROs (assuming RACH periodicity = 10 ms, which maybe a typical RACH configuration periodicity), which may be undesirable.

Figure 1: Illustration of RO and MG collision

- For the definition of MG colliding with RO occasion, consider RACH preamble colliding with any symbol of MG. Whether both RF retuning before the RACH preamble and RF retuning after the RACH preamble transmission need to be considered can be FFS, to address the case of MG colliding with two consecutive ROs.

Clarification PDCCH order to RACH delay

As shown in the copied 3GPP text below, 38.213 specifies the delay for determining PDCCH order to RACH by referencing certain components in 38.133. One such component is ∆RF/BB preparation.

TS 38.133 specifies that the component ∆RF/BB preparation=TBWPSwitchDelay if PRACH is outside UL BWP of PCell, but withing UL BWP of any active SCell. TBWPSwitchDelay refer to Table 8.6.2-1 in 38.133 for the BWP switch delays for different SCS and mentions the smaller SCS should be used to determine the switch time.

However, in case of LTM, the switch could be to transmit PRACH format 0/1 where SCS is 1.25kHz or 5 kHz. Since the BWP switch delay do not refer to 1.25KHz or 5KHz, it is not clear about which delay is applicable. The same issue is addressed in RAN1 by using the SCS configuration of µ=0, and we suggest RAN4 to clarify the same.

- If PRACH is outside UL BWP of PCell, but withing UL BWP of any active SCell, for PRACH transmission using 1.25kHz or 5 kHz SCS, UE determines BWP switch delay assuming µ=0.

Clarification of UE capabilities 39-3-5 and 39-3-6:

Following agreement was reached in last meeting.

  - The first component of RAN4 FG 39-3-5 maxSSB-PerFreqLayerL1-Meas-r18 counts both LTM candidate cell(s) and serving cells for SSB L1-RSRP measurement, no matter SSB L1-RSRP measurement on serving cell is configured in LTM-CSI-ResourceConfig-r18 or not.

  - For RAN4 FG 39-3-5, include the candidate values of 12, 16, 20, 24 to existing candidate values.

    - FFS: include the candidate values of 32, 40, 48, 64.

  - FFS Update the 39-3-6 FG candidate values description from Component 2 of 2-24 to component 1 of 2-24.

| Index | FG | Components | Candidate values | Our understanding |
| --- | --- | --- | --- | --- |
| 39-3-5 | Number of SSB resources for L1-RSRP measurement per frequency layer | 1. The max number of SSB resources UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps 2. The max number of SSB resources UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps | Candidate value of Component 1: {1,2,3,4,5,6,7,8, 12, 16, 20, 24} Candidate value of Component 2: {1,2,3,4,5,6,7,8, 12, 16, 20, 24} | This FG is per frequency layer, and our understanding of this FG is that it is only applicable to LTM candidates. The main reason for this is, RAN4 did not define L1-RSRP per frequency layer till Rel-17 as the L1-RSRP is only supported for serving cell. One can argue that all the serving cells are intra-frequency cells and should be included. However, as we can see, the candidate values are lower than UE capability of serving cell L1-RSRP measurement (beamManagementSSB-CSI-RS). We also observed that there is an error in the candidate values, and it seems to have written only from FR1 perspective and may not work for FR2. Hence, we see a need for updating the candidate values and send LS to RAN2. |
| 39-3-6 | Number of total SSB resources to be measured | The max number of total SSB resources of serving cells and neighbouring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. | Candidate values: {2,4,8,12,16,32,64} Note: the value should be not smaller than UE capability of beamManagementSSB-CSI-RS (Component 2 of 2-24) | Since the UE may use existing capability of beamManagement to accommodate LTM, we agreed that UE should not report smaller than beamManagementSSB-CSI-RS. However there seems to be an error in the note. We refer to Component 2 of FG 2-24 of the MIMO, but the component 2 refer to maxNumberCSI-RS-Resource. |

- FG 39-3-5 is only applicable to LTM candidates and the candidate values should be updated to cover the FR2 candidate cells as well. Include the candidate values of 32, 40, 48, 64 to existing candidate values.

- Update the 39-3-6 FG candidate values description from Component 2 of 2-24 to component 1 of 2-24.

# Summary and Conclusion

In this contribution we have analysed RRM requirements for L1/L2 based inter-cell mobility and made following proposals.

- For the definition of MG colliding with RO occasion, consider RACH preamble colliding with any symbol of MG. Whether both RF retuning before the RACH preamble and RF retuning after the RACH preamble transmission need to be considered can be FFS, to address the case of MG colliding with two consecutive ROs.

- If PRACH is outside UL BWP of PCell, but withing UL BWP of any active SCell, for PRACH transmission using 1.25kHz or 5 kHz SCS, UE determines BWP switch delay assuming µ=0.

- FG 39-3-5 is only applicable to LTM candidates and the candidate values should be updated to cover the FR2 candidate cells as well. Include the candidate values of 32, 40, 48, 64 to existing candidate values.

- Update the 39-3-6 FG candidate values description from Component 2 of 2-24 to component 1 of 2-24.

# References

- R4-2410302 WF on mobility enhancement’s part 1, Mediatek

- RP-221799 “WID on Mobility enhancements”, MediaTek