3GPP TSG-RAN WG4 Meeting # 115	R4-2507023

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.22.3

Source: 	National Taiwan University

Title: 	Evaluation Criterion for AI/ML Interoperability from Standardization of Testing and Verification Perspective

Document for:	Approval

Introduction

We present our view on partially specified decoder and reference decoder derivation procedure in this contribution.

Discussion

Discussion for Option 4 Inter-Operability

The following was agreed in the WF for option 4 [1]

- Provide results for - Your company’s “own” decoder and “own encoder” - Other companies uploaded “own” decoders and your company’s “own” decoder (this is a test for interoperability between encoders and test decoders)

We have the following agreements in TR 38.843 for interoperability and test repeatability:

- Test repeatability should be ensured (variation among TE vendor implementations should be bound)

- Other vendors should also be able to develop such a decoder and which can deliver similar performance

Based on the agreement in the TR, we need to check the implementation variation across different decoders from option 4a and 4b from test repeatability perspective. We propose the following

Proposal 1: For all the “own” test decoders, verify the test repeatability and interoperability by the following procedure

- Select one (arbitrary) encoder

- Connect the encoder to “own” decoder 1, run the test based on the agree test dataset and record the SGCS

- Repeat step 2 for all the “own” decoders, record all the SGCS

- The variation of the SGCSs from all the “own” decoders should fall within a threshold agreed in RAN4 to claim that the reference encoder provided by Samsung or the dataset generated from the enc/dec pair from Samsung fulfil the test repeatability and interoperability agreed in the TR

If we have concern about single encoder sample bias, we can repeat step 1 to 4 for multiple encoders

In addition, none of these options are complete enough for TE vendors to derive the test decoders that meet the test repeatability requirements. For example, in 4a, there are different ways to train the test decoders and the achieved loss functions from the specified dataset can vary a lot, and we still don’t have any test repeatability guarantee across different test decoders. Similarly, the joint training with reference encoder and yield different test decoders without guaranteed test repeatability. Therefore, additional requirements/specifications are needed for dataset based and reference encoder-based approach. We provide the suggestions for further specifications for each option in the following:

Dataset based approach 4a: we can specify a loss/similarity function of the test decoder output and the specified dataset for a given input as follows

z∈Zfgref(z),gtest(z)<δ,

where Z is the set of decoder inputs (latent messages) in the specified dataset, and f is the chosen loss/similarity function to evaluate the difference between the decoder output from the dataset corresponding to the latent message z, gref(z), and the test decoder output from z, gtest(z).

The drawback of the approach is that both the test decoder (and implicitly the DUT encoder) are heavily restricted by the dataset due to the specified constraint, and we don’t expect that the DUT encoder can work in practice unless in an environment that is very closely resembled with the specified dataset. In fact, the purpose of the test is, as agreed, “verify whether the minimum performance of AI/ML functionality/feature can be achieved”. But we don’t expect such dataset shared between the UE vendors and infra vendors is available for all the application scenarios, and therefore the performance achieved here is definitely not the minimum performance we can expect in the field. Alternatively, if we consider the approach proposed in the previous meeting (and included also in section 2.2), we can derive a more flexible requirement that can enable both TE vendors and UE vendors to develop decoders and encoders that can be trained by different datasets than the specified one. Such a flexibility can get us closer to the agreed test goal, since we allow the encoders and decoders to be trained by other dataset as long as the specified requirement in the form of latent message/decoder output ordering is fulfilled. To be more specified, the alternative requirement is:

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where Ytest is the test decoder output space.

Similarly, we can add the requirement for option 4b as

x∈Xfgref(z(x)),gtest(z(x))<δ,

where z(x) is the reference encoder output (latent message) of encoder input x, and X is the sampled encoder input during test decoder verification procedures. Therefore, we also need to specified the generation procedures of X.

Alternatively, based on the proposal in section 2.2, we can specify the requirements of the test decoder as

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where Ytest is the test decoder output space and Z is the reference encoder output space.

In summary, we have the following proposal:

Proposal 2: Additional steps for option 4a and 4b feasibility study:

- Define the decoder verification criterion from one of the following options (for 4a and 4b, respectively):

(Option 4a: specify dataset)

- 4a-1: RAN4 derive the threshold δ via simulation study (setup following option 3 discussion), and check whether it’s feasible to derive a test decoder satisfying

z∈Zfgref(z),gtest(z)<δ,

where Z is the set of decoder inputs (latent messages) in the specified dataset (from step 2), and f is the chosen loss/similarity function gref(z) is the test decoder output and gtest(z) is from the dataset.

- 4a-2: RAN4 studies whether it is feasible to derive a test decoder satisfying

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where the notations are the same as 4a-1, and Ytest is the test decoder output space.

(Option 4b: specify reference encoder)

- 4b-1: RAN4 derive the threshold δ and the encoder input X generation procedure via simulation study (setup following option 3 discussion), and check whether it’s feasible to derive a test decoder satisfying

x∈Xfgref(z(x)),gtest(z(x))<δ,

where z(x) is the reference encoder output (latent message) of encoder input x, and X is the sampled encoder input in the test decoder verification procedures. Note that RAN4 needs to specified the generation procedures of X and specifying

- 4b-2: RAN4 studies whether it is feasible to derive a test decoder satisfying

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where Ytest is the test decoder output space and Z is the reference encoder output space.

- RAN4 check if the decoders derived from the above procedure can satisfy test repeatability requirement, i.e., the loss function delta between two different decoders when connecting to the same encoder, is within a RAN4 agreed margin. If yes, we can conclude that option 4 (at least one sub-option) is feasible.

Observation 1: Option 4a-1 and 4b-1 require both TE vendors and UE vendors to train the test decoder and DUT encoder with the dataset or reference encoder, while option 4a-2 and 4b-2 allow TE vendors and UE vendors to train the test decoder and DUT encoder with arbitrary dataset or encoders/decoders as long as the specified ordering requirements are satisfied.

Test Decoder Selection Criterion

We summarize the potential criterions for test decoder selection below:

- Achievable performance metrics: SGCS or NMSE

- Complexity: including flops or model storage size, can consider to set an upper bound

- Robustness (based on performance metrics): whether the decoder can achieve satisfactory performance when connecting to encoder with different structures with the decoder, but trained with the decoder’s input and output dataset

Given the agreements on decoder structure, complexity is mostly bounded, and therefore we may not need to consider it. Most of discussions so far were focusing on the achievable performance as criterion. However, in the agreed procedure, after selected a decoder, the next step is:

Each company brings results for training of “own encoder” with selected decoder(s)

  - performance alignment to be checked/discussed

  - using “own” data or data shared by other companies

in which we need to check the performance alignment across the encoders trained by a common dataset (as an option), similar to the robustness mentioned above. In order to achieve performance alignment, it is preferred to select a decoder that achieves similar performance when connecting to different encoders with different structures, e.g., the structure options discussed previously for the test decoder, but trained with the same dataset produced by the decoder, i.e., the decoder input and output dataset. Otherwise, even if a decoder can achieve a better performance when connected to the jointly trained encoder, that decoder might fail to achieve performance alignment in the agreed next step, and RAN4 has to reselect the decoder. Based on the above analysis, we have the following proposal:

Proposal 3: Consider the test decoder selection criterion based on the agreed step, i.e., check whether performance alignment across contributing companies’ “own encoder”, which may have different structures, can be achieved. 
Therefore, companies can bring the test decoder proposal together with performance metrics achieved when connecting to different encoders with different structures, e.g., the structure options for test decoder discussion, but trained with the decoder input and output dataset from the proposed decoder.

Conclusion

Proposal 1: For all the “own” test decoders, verify the test repeatability and interoperability by the following procedure

- Select one (arbitrary) encoder

- Connect the encoder to “own” decoder 1, run the test based on the agree test dataset and record the SGCS

- Repeat step 2 for all the “own” decoders, record all the SGCS

- The variation of the SGCSs from all the “own” decoders should fall within a threshold agreed in RAN4 to claim that the reference encoder provided by Samsung or the dataset generated from the enc/dec pair from Samsung fulfil the test repeatability and interoperability agreed in the TR

If we have concern about single encoder sample bias, we can repeat step 1 to 4 for multiple encoders

Proposal 2: Additional steps for option 4a and 4b feasibility study:

        - Define the decoder verification criterion from one of the following options (for 4a and 4b, respectively):

(Option 4a: specify dataset)

- 4a-1: RAN4 derive the threshold δ via simulation study (setup following option 3 discussion), and check whether it’s feasible to derive a test decoder satisfying

z∈Zfgref(z),gtest(z)<δ,

where Z is the set of decoder inputs (latent messages) in the specified dataset (from step 2), and f is the chosen loss/similarity function gref(z) is the test decoder output and gtest(z) is from the dataset.

- 4a-2: RAN4 studies whether it is feasible to derive a test decoder satisfying

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where the notations are the same as 4a-1, and Ytest is the test decoder output space.

(Option 4b: specify reference encoder)

- 4b-1: RAN4 derive the threshold δ and the encoder input X generation procedure via simulation study (setup following option 3 discussion), and check whether it’s feasible to derive a test decoder satisfying

x∈Xfgref(z(x)),gtest(z(x))<δ,

where z(x) is the reference encoder output (latent message) of encoder input x, and X is the sampled encoder input in the test decoder verification procedures. Note that RAN4 needs to specified the generation procedures of X and specifying

- 4b-2: RAN4 studies whether it is feasible to derive a test decoder satisfying

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where Ytest is the test decoder output space and Z is the reference encoder output space.

        - RAN4 check if the decoders derived from the above procedure can satisfy test repeatability requirement, i.e., the loss function delta between two different decoders when connecting to the same encoder, is within a RAN4 agreed margin. If yes, we can conclude that option 4 (at least one sub-option) is feasible.

Proposal 3: Consider the test decoder selection criterion based on the agreed step, i.e., check whether performance alignment across contributing companies’ “own encoder”, which may have different structures, can be achieved. 
Therefore, companies can bring the test decoder proposal together with performance metrics achieved when connecting to different encoders with different structures, e.g., the structure options for test decoder discussion, but trained with the decoder input and output dataset from the proposed decoder.

Reference

[1] R4-2505107