3GPP TSG-RAN WG4 Meeting #115	R4-2507025

# Malta, MT, 19 - 23 May, 2025

Title: 	Discussion on NTN testing for NGSO demodulation performance requirements

Source: 	Huawei,HiSilicon

Agenda item:	7.10.5.3

Document for:	Discussion

Background

During RAN4#114bis meeting, Way Forward [1] for NTN_testing_NGSO_channel_model_and_demod was approved. In this contribution, we share our view on NTN testing for NGSO demodulation performance requirements.

Discussion

Impact on RAN4 demodulation requirements with TE-emulated time vary doppler shift and propagation delay models

- Proposals: - Option 1: - Keep the same NR NTN PDSCH requirements (SNR) even if the time-varying Doppler shift and propagation delay model is applied. - Keep the same IoT NTN UE demodulation requirements (SNR) even if the time-varying Doppler shift and propagation delay model is applied. - Option 2: Considering worst case for frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply. Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models - Option 3: - RAN4 shall start to agree parameters for using the NGSO channel model in order to understand whether the margin produced is acceptable to define requirements with. - RAN4 shall use the new NGSO channel model for all performance requirements, in it’s current state. - Option 4: After concluding discussions on the time-varying Doppler shift and propagation delay, initiate simulation campaigns to analyse the impact on demodulation requirements.

Here we provider our simulation results by considering 20% of the worst case as stated in Option 2 for frequency error and timing error as Following Table 2.1-1.

Table 2.1-1 simulation results considering 20% of the worst case for frequency error and timing error

| Case Number | SCS (kHz) / CBW (MHz) | Propagation condition | MCS | Antenna configuration | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Ideal | +Error |
| 1 | 15 / 10 | NTN-TDLA100-200 | 4 | 1x2 | -2.80 | -2.65 |
| 2 | 15 / 10 | NTN-TDLC5-200 | 13 | 1x2 | 4.86 | 5.24 |

Based on the simulation results, about 0.4dB performance difference for MCS13 can be observed, it is reasonable to add certain additional margin, such as 0.5dB, on top of the current NTN performance requirements for NTN NGSO testing by considering the timing varying doppler shift and propagation delay modelling.

Add 0.5 dB margin for NR NTN PDSCH requirements.

Test cases for NR NTN with applying time varying Doppler Shift and propagation delay model

- agreement - Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 38.101-5 Clause 8.2.1.2.2.1 Test 1-1 and Test 1-2. - FFS on test 1-3 and 1-4 - Note: test 1-3 is related with UE capability to support 32 HARQ process, and test 1-4 is related with UE capability with HARQ disable

From our understanding, UE performance under time-varying Doppler shift and propagation delay model can be ensured by only defining limited and basic cases, there is no need to cover all cases. This is also aligned with the WID scope. We prefer not to consider tests 1-3 and 1-4 for NR NTN with applying time varying Doppler Shift and propagation delay model.

Not consider tests 1-3 and 1-4 for NR NTN with applying time varying Doppler Shift and propagation delay model.

Test case for NB-IoT NTN with applying the time-varying Doppler shift and propagation delay model

- agreement - Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 36.102 Clause 8.3.1.1 Test 1 - FFS If the LEO-600 model is not enough to cover the test time of Test 1, RAN4 should consider using the LEO-1200 model

Based on TS 36.521-4, the test time for the test 1 is about 243.36s. LEO-600 model is enough to cover the minimum test time of Test 1, no need to consider LEO-1200 model that is also out of the work scope.

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | Min Gap between consecutive NPDCCH taking into account RTT (ms) | Actual Gap between consecutive NPDCCH taking into account RTT (ms) | MNSF (Min No Sub frames) HD-FDD (Note2) |
| --- | --- | --- | --- | --- | --- |
| 8.3.1.1.1 Test 1 | R.NB.1 FDD (200kHz, QPSK,1/2) (1x1) NTN TDLC5-200 | 845 (Note 1) | 241 | 288 | 243360 |

Confirm that LEO-600 model is enough to cover the minimum test time of Test 1.

Test case for eMTC NTN with applying the time-varying Doppler shift and propagation delay model

- agreement - FFS to consider Cat-M1 demodulation requirement with applying time varying Doppler shift and propagation delay model - If considering Cat-M1 demodulation requirement with applying time varying Doppler shift and propagation delay model - Cover test 1 and test 2 according to TS 36.102 Clause 8.2.1.1 - FFS to cover test 3

Firstly, from specification completion point of view, minimum and basic test cases for eMTC NTN are needed. Then, UE performance under time-varying Doppler shift and propagation delay model can be ensured by minimum and basic test cases, there is no need to cover all cases. We prefer to only consider test 1 and test 2 for Cat-M1 demodulation requirement with applying time varying Doppler shift and propagation delay model.

Only consider test 1 and test 2 for Cat-M1 demodulation requirement with applying time varying Doppler shift and propagation delay model.

Proposals

In this contribution, we share our view on NTN testing for NGSO for demodulation performance requirements. Our observations and proposals are:

- Add 0.5 dB margin for NR NTN PDSCH requirements.

Not consider tests 1-3 and 1-4 for NR NTN with applying time varying Doppler Shift and propagation delay model.

Confirm that LEO-600 model is enough to cover the minimum test time of Test 1.

Only consider test 1 and test 2 for Cat-M1 demodulation requirement with applying time varying Doppler shift and propagation delay model.

Reference

R4-2504700, Way Forward for [114bis][323] NTN_testing_NGSO_channel_model_and_demod, RAN4#114bis, Samsung