3GPP TSG RAN WG4 Meeting #115	R4-2507056

Saint Julian's, MT, 19 May 2025 – 23 May 2025

Agenda Item:	7.7.4

Source: 	Ericsson

Title:	RRM requirements for low band carrier aggregation via switching

Document for:	Discussion

# Introduction

New WID [1] on Low band carrier aggregation via switching is agreed in RAN#106. In this contribution, we analyze the potential impact to RAN4 RRM requirements based on issues identified in last meeting WF, R4-2502588.

# Discussion

## General method to specify the requirements

In the previous meeting, RAN4 began examining the impact of RRM requirements for SDL SCell and FDD PCell. We believe the existing RRM requirements can be used as baseline. We propose that the RRM impact can be managed using two methods.

Method 1: Discussing a method to extend the delay requirement if SSB/SMTC/MG/CSI-RS/DRX ON periodicity are missed at UE due to switching to another cell. Once we agree on this delay extension principle, it can be applied to all related procedures on PCell and SCell. This approach was similarly utilized in Release 16 for defining requirements on a carrier subject to CCA.

Method 2: for some of the requirements, without extending or relaxing the particular requirement, we can specify an applicability condition under which the requirements are valid for low-band CA via switching.

We think RAN4 does not need to choose between method 1 and 2. Some of the requirement’s impact can be addressed with method 1 and some other requirements impact can be addressed with method 2.

- RAN4 to further investigate the RRM requirements impact with at least one of the following methods.

- Method 1: Agree on the principle of extending the delay based on missed reception occasion of necessary DL signals/channels and apply this principle to all the aforementioned procedures.

- Method 2: Specify an applicability condition under which the requirements are valid for low-band CA via switching.

The rest of the discussion is organized as follows:

- RRM requirements impact for SDL SCell

- RRM requirements impact for FDD PCell

- Timing requirements

- LB CA switching delay and potential interruptions

## SSB assumptions for R19 LB CA via switching

From the WF [1] we have:

  - [moderator]: Decide between option 1 and option 2. The SSB-less operation here does not mean SSB is not transmitted on SDL SCell, and that means UE may use the same operation as for SSB-less case, e.g., skip L3 measurement on SSB-less SCell.

    - Option 1 (OPPO, LGE, MTK, Nokia, Xiaomi, QC): RAN4 to use SSB based operation as baseline for Rel-19 low band CA via switching.

    - Option 2 (Apple): RAN4 to use SSB-less based operation as baseline for Rel-19 low band CA via switching.

Already in RAN4#114 we stated [3]:

“In the RAN plenary, two cases were discussed: SCell with SSB and SCell without SSB. We would like to point out that UE supports SSB-less SCell with optional UE capability. Since this WI is proposed by the operators and concerns existing on-field issues, RAN4 should strive to support as much UE capability as possible. Therefore, as a baseline, SCell with SSB should be supported for a UE supporting low band carrier via switching. This means that if we consider SCell without SSB, there should not be two optional UE capabilities (i.e., low band CA via switching with SSB and low band CA via switching without SSB) for supporting low band CA via switching. We are open to considering SCell without SSB after the baseline solution is supported”.

We then went on to propose that for a UE supporting low band CA via switching, SCell with SSB operation should be mandatorily supported and that low band CA via switching, SCell without SSB operation can be considered after the SCell with SSB operation is supported.

- RAN4 to use SSB based operation as baseline for Rel-19 low band CA via switching.

- SCell without SSB operation can be considered after the SCell with SSB operation is supported.

## MRTD assumptions for R19 LB CA via switching

From the WF [1] we have:

  - FFS:

    - Option 1 (OPPO, vivo, QC, ZTE, Nokia, Xiaomi, CATT, Samsung, HW, MTK, LGE): The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us.

    - Option 3 (Ericsson): MRTD = CP for in LB-LB CA with switching.

In the previous section we make an argument that SCell without SSB operation can be considered after the SCell with SSB operation is supported.

We note in [4], that in 3GPP rel-18 NES WI the following was added to TS 38.133, section 8.3.2. This means that 3GPP RAN4 has already standardized SSB less cases as part of NES rel-18 WI.

… For a UE supporting scellWithoutSSB-InterBandCA-r18 if the SCell being activated belongs to FR1 and if the UE is not provided with SSB ....... and if there is one collocated active reference serving cell on different FR1 band, when the following conditions are fulfilled, - The RTD between the target SCell and the collocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, …

Observation 1: 	3GPP RAN4 has already standardized SSB less cases as part of NES rel-18 WI, with MRTD = CP.

We can re-use the system development in rel-18 NES since the use case in LB-LB CA with switching in rel-19 when there is no SSB on SDL-carrier is similar to the case we already standardized in RRM TS 38.133 for rel-18 NES inter-band CA without SSB on SCell.

- If the SSB less SCell is considered to be in scope, use NES rel-18 WI as baseline for specifying LB-LB CA between collocated FDD-carrier and SSB-less SDL-carrier via switching in rel-19.

- Since we propose NES rel-18 WI as baseline for specifying LB-LB CA between collocated FDD-carrier and SSB-less SDL-carrier via switching in rel-19, we inherit MRTD = CP from NES rel-18, but option 1 MRTD = 3 µs works as well.

## RRM requirements impact for SDL SCell

### When the SCell is deactivated

In the last meeting, the following agreement was reached.

- For the SCell which is deactivated,

  - The switching pattern for SDL SCell is not applied.

  - If the deactivated SDL SCell measurement requirement is needed, FFS whether UE follows legacy deactivated SCell measurement requirements to perform deactivated SDL SCell measurement or follow a new measurement requirements for deactivated SDL SCell measurement.

Whether the deactivated SCell measurement is needed:

In the last meeting, a proposal was discussed to potentially skip L3 measurements for deactivated SCells. This proposal is based on the fact that PCell and SCell are collocated, share the same TAG, and operate in low-frequency bands.

To assess the feasibility of this proposal, we did a short simulation to analyse the L3-RSRP difference between the PCell and SCell. We make following observations from these simulations:

- Pedestrian UEs: The L3-RSRP difference between PCell and SCell is generally < 6 dB for low-mobility UEs.

- Limitations: Results are specific to pedestrian UEs and do not account for mobile UEs.

- Variability: ~10% of UEs showed an L3-RSRP difference > 6 dB, with some differences reaching up to 18 dB.

Based on the simulation results observations, we propose the following:

Conditional Skipping: RAN4 to specify conditions under which L3-RSRP measurement for deactivated SCells can be skipped, focusing on scenarios with pedestrian UEs in collocated PCell/SCell configurations in low-frequency bands.

- RAN4 to consider skipping deactivated SCell L3-RSRP for the low mobility UEs in collocated PCell/SCell configurations in low-frequency bands.

For other cases we suggest following approach:

In the existing specification as well, as pointed by other companies in last meeting, the interruption requirements are specified for deactivated SCell measurements when the measCycleSCell is 640ms or larger. Some part of the interruption requirement of the existing spec is produced below for reference.

Interruptions on PCell or activated SCell(s) due to measurements when an SCell is deactivated are allowed with up to 0.5 % probability of missed ACK/NACK when the configured measCycleSCell [2] is 640 ms or longer. - If the PCell or activated SCell(s) is not in the same band as the deactivated SCell, the UE is only allowed to cause interruptions on PCell or activated SCell(s) immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in table 8.2.2.2.2-1.

Unlike legacy interruption requirement, for low band CA via switching, we believe that interruption requirements should apply to all values of measCycleSCell. It's important to note that the reasons for interruption differ between legacy deactivated SCell measurements and deactivated SDL SCell measurements. In legacy, the UE does not need to remain on the SCC or keep the RF chain active if the measCycleSCell is 640 ms or longer. However, in the low band CA via switching scenario, the UE must switch to the deactivated SCell to perform measurements, requiring active transitions regardless of the measurement cycle duration.

- RAN4 to define the interruption requirement to allow SDL SCell measurement on deactivated SCell without switch pattern being applicable. For deactivated SDL SCell measurement, interruptions on PCell or activated SCell(s) are allowed with up to 0.5% probability of missed ACK/NACK regardless of the configured measCycleSCell.

### When the SCell is being activated

In the last meeting, the following agreement was reached.

- For the SCell which is in activation procedure,

  - The switching pattern for SDL SCell is not applied.

        - RAN4 defines SCell activation requirement assuming UE utilizes the RS occasions to perform SDL SCell activation

  - the above RS occasions are FFS

The RS occasions depend on the type of SCell activation procedure considered. So far RAN4 defined many cases of SCell activation and for simplicity these can be referred as normal SCell activation and fast SCell activation procedure.

For normal SCell activation procedure:

The UE primarily needs to access the SCell to perform SCell activation-related tasks, which may include coarse Automatic Gain Control (AGC), fine AGC, cell search, L1-RSRP measurement, and fine timing adjustment. Given the multiple cases considered in the existing specifications and as per the last meeting agreement of UE is allowed to interrupt the pattern to access the SCell for activation, we believe that following the existing SCell activation procedure simplifies the workload.

Fast SCell activation procedure:

RS Occasions: RS used for SCell activation are TRS and A-TRS.

Pattern Interruption: During TRS and A-TRS occasions, the UE is permitted to access the SCell by interrupting the existing switching pattern as per last meeting agreement.

SCell activation delay: Follow the existing SCell activation delay and the side conditions.

- For both normal and fast SCell activation procedures, reuse the existing requirements by allowing the UE to switch to the SCell during SSB, TRS, A-TRS, and CSI-RS occasions. In other words, the SCell activation delay requirement remains unaffected, as the switching pattern is not enforced during the SCell activation process.

UE measurement behaviour during the SCell activation:

The next point to consider is whether to conduct the AGC measurement during the SSB occasion or the SMTC occasion. SMTC occasion-based activation was introduced to save UE power. However, since low-band carrier aggregation through switching functions in half-duplex mode between the FDD cell and the SDL cell, the number of SSB/SMTC occasions may be reduced compared to standard CA operation. Therefore, we prefer SSB-based measurement during activation over SMTC-based measurement for faster SCell activation and efficient load balancing. We would like to point that the similar enhancements (e.g., shortMeasInterval-r18) are introduced as part of the Rel-18 SCell enhancements.

- RAN4 to agree on applying the enhancements introduced in Rel-18 for SCell activation—such as shortMeasInterval-r18—to LB CA as well.

### When the SCell is activated

When the SCell is activated, the UE must carry out several RRM procedures. We will examine how the switch pattern affects these procedures based on the following agreed WF in the last meeting.

For R19 LB CA via switching, RAN4 to not define SDL activated SCell L3 measurement requirement if SSB-less based SCell operation is used. For R19 LB CA via switching, FFS: - Whether and how RAN4 to define SDL activated SCell L3 measurement requirement if SSB based SCell operation is used. - RAN4 to define a new BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for R19 LB CA - the existing BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirement can be used as baseline for R19 LB CA

#### L3 measurements

Our analysis for skipping the L3 measurements using simulation results holds true for activated SCell measurements as well. As we discussed in deactivated SCell measurement requirements, we suggest the following proposal.

- RAN4 to consider skipping activated SCell L3-RSRP for the low mobility UEs in collocated PCell/SCell configurations in low-frequency bands.

For other cases we suggest following the method 1 specified in section 2.1 of this paper.

While there are other type of measurement requirements that are impacted, we consider intra-frequency without gap as an example to discuss the principle for RRM requirement relaxation or extending the delay The measurement period for L3 measurements is as mentioned below.

Table 9.2.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1)

| DRX cycle | TSSB_measurement_period_intra |
| --- | --- |
| No DRX | max (200 ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max (200 ms, ceil(1.5x 5 x Kp) x max(SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil(5 x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

According to the measurement period outlined in the above table, the measurement period relies on the parameters SMTC, Kp, and CSSFintra. Due to the switch pattern and the half-duplex nature of the LB CA via switching, these parameters may be affected. We look at the defintion of these parameters for analyzing the RRM requirements impact.

Kp: This factor determines the number of SMTC occasions not overlapping with any MG relative to the total available SMTC occasions, including those overlapped by gaps. Existing defintion of the Kp is shown below.

Kp is the scaling factor for an SSB frequency layer to be measured without GAP. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows: - For a window W of duration max(SMTC period, xRP_max), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion: - Ntotal is the total number of SMTC occasions within the window, including those overlapped with GAP and MUSIM gap occasions within the window, and - Navailable is the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions within the window W, after accounting for measurement GAP and MUSIM gap collisions by applying the collision rules for GAP and MUSIM gap in clauses 9.1.8.3, 9.1.10.4, 9.1.10.5, 9.1.12.3, and 9.1.13.3, respectively. Kp = 1 when Navailable = 0. - xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap. Requirements in this clause do not apply when Navailable = 0 due to fully overlapping between SMTC occasions and MUSIM gap occasions within the window W.

Because of the switch pattern design, introducing a new scaling factor, Kp_LB, is necessary. Kp_LB will account for SMTC occasions on the relevant cell that neither overlap with gaps nor are missed due to switch patterns. This scaling factor can be defined to capture the SMTC occasions available for L3 measurement and same is highlighted in green, as illustrated in Figure 2. The precise formula for Kp_LB is contingent on RAN1 design and requires further analysis (FFS).

Figure 1: L3 measurement ocassions for LB CA

- RAN4 to define a new Kp_LB to address the impact of switch patterns on the measurement period.

SMTC Period: Certain SMTC occasions may be unavailable for L3 measurement due to switching between cells, and it can result in a prolonged measurement delay. We think the non availability of the SMTC occasion for the L3 measurement can be captured in the Kp_LB design. Due to this, we think, SMTC period defintion need not change.

CSSFintra: When all the Measurement Gap (MG) and SMTC occasions overlap, the inter-frequency and intra-frequency carriers must share the SMTC occasion. Again, since this is influenced by the RAN1 pattern design, we propose to defer the final determination of the CSSFintra factor, marking it as FFS.

- RAN4 to further discuss CSSFintra for the LB CA.

#### L1 measurement requirements:

We look at the legacy L1-RSRP requirements for the impact analysis for LB CA L1 measurement requirements and later we check if we can apply same principles for other L1 measurements as well.

Table 9.5.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB for FR1 Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P)*TSSB). DRX cycle ≤ 320ms; max(TReport, ceil(1.5*M*P)*max(TDRX,TSSB)). DRX cycle > 320ms; ceil(M*P)*TDRX. Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting.. - M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise For FR1, - P=1/(1-T_SSB/MGRP), when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and - P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB. TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting.

From the table above, we observe that there is no need to change the value of M, as the number of samples remains consistent with legacy requirements for L1-RSRP measurements. However, switching between FDD and SDL may result in some SSBs being unavailable for L1-RSRP measurement. In legacy, when SSBs overlap with MG, a sharing factor P is used to address the non-availability of SSBs due to overlap of SSB with MG, thus specifying the remaining SSB occasions for L1-RSRP. In LB CA with switching, SSBs might be unavailable for L1-RSRP not only due to switching between FDD and SDL but also because of overlap with MG (similar to legacy). Both factors must be considered when defining SSB occasions for L1-RSRP measurement or a sharing factor for L1-RSRP. A new sharing factor, PLB, can be introduced to account for the non-availability of SSBs due to the switch pattern and overlap with MG. PLB can be defined to capture the SSB occasions available for L1-RSRP measurement and same occasions are highlighted in green, as shown in figure 3. Since PLB addresses SSB availability, we propose reusing the existing TSSB definition, TDRX and TReport.

Figure 2: L1-RSRP measurement occasions for LB CA

We observe that RAN4 followed similar approach for defining the BFD, CBD and L1-SINR requirements. We copy the relevant requirements below for reference.

BFD requirements: Table 8.5.2.2-1: Evaluation period TEvaluate_BFD_SSB for FR1 Configuration; TEvaluate_BFD_SSB (ms). no DRX; Max(50, Ceil(5 ´ P) ´ TSSB). DRX cycle ≤ 320ms; Max(50, Ceil(7.5 ´ P) ´ Max(TDRX,TSSB)). DRX cycle > 320ms; Ceil(5 ´ P) ´ TDRX. Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length.. For FR1, - P=11-TSSBMGRP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB. - P = 1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB. CBD requirements: Table 8.5.5.2-1: Evaluation period TEvaluate_CBD_SSB for FR1 Configuration; TEvaluate_CBD_SSB (ms). non-DRX, DRX cycle ≤ 320ms; Max(25, Ceil(3 ´ P) ´ TSSB). DRX cycle > 320ms; Ceil(3 ´ P) ´ TDRX. Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length.. For FR1, - P=11-TSSBMGRP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB, - P = 1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB. L1-SINR requirements: Table 9.8.4.2-1: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR for FR1 Configuration; TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms). non-DRX; max(TReport, ceil(M*P)*TSSB). DRX cycle ≤ 320 ms; max(TReport, ceil(1.5*M*P)*max(TDRX,TSSB)). DRX cycle > 320 ms; ceil(M*P)*TDRX. NOTE 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR channel measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity..

- For L1 measurements, RAN4 to define new sharing factor, PLB to account for the non-availability of SSBs due to the switch pattern and SSB overlap with MG.

#### TCI state switch delay

We summarize different TCI state switch delay supported in the existing spec. We did not consider unified TCI state switch for the analysis, but we expect similar conclusions should be applicable for unified TCI state switch as well.

|  | Known TCI state | Unknown TCI state | Comment |
| --- | --- | --- | --- |
| MAC CE based | n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc) / NR slot length | n+ THARQ +3Nslotsubframe,µ + TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc) / NR slot length | - Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state |
| DCI based | timeDurationForQCL |  |  |
| RRC based | n+ (TRRC_processing +TOk*(Tfirst-SSB + TSSB-proc)) / NR slot length | n+ (TRRC_processing +TL1-RSRP +TOuk*(Tfirst-SSB + TSSB-proc)) / NR slot length | Tfirst-SSB is time to first SSB transmission after L1-RSRP measurement when TCI state switching involves QCL-TypeD; - Tfirst-SSB is time to first SSB transmission after RRC processing time at the UE for other QCL types; - The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state |
| Active TCI state list update | n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. | NA | As defined in MAC CE based switch |

Among the delay components outlined in the table above regarding legacy requirements, we think THARQ is not impacted due to LB CA via switching as it is configured by the NW using the k1 value and the NW configuration already considers the switch pattern. Among the other components, we believe the parameter that is affected by the switch pattern is Tfirst-SSB. This is because Tfirst-SSB involves the UE accessing SSB signals for measurement, and the time taken to access these signals depends on the switch pattern. We consider other components to be related to UE processing delays, which we believe remain unaffected. Therefore, we propose redefining Tfirst-SSB for LB CA via switching, suggesting it should be defined as the first SSB available on the respective cell.

Due to UE operation on the SDL, and the potential HARQ delay on the FDD carrier, UE may receive the SSB on the SDL carrier before sending the HARQ on the FDD carrier. In this case, we think the delay should be max (HARQ, SSB reception time). RAN4 can further work on the delay equation.

- RAN4 to change the defintion of Tfirst-SSB for the TCI state switch delay and it is to be defined as the first SSB available on the respective cell after the switch command.

- RAN4 to consider changes to TCI state switch requirement equation to consider the SSB being received on SDL before the HARQ being sent on FDD.

## RRM requirements impact for FDD PCell

We think the RRM requirements analysis of L3 measurement, L1 measurements should hold true for PCell as well.

- The principle of scaling factor agreed for SDL SCell, to be applicable for PCell as well.

Other than the requirements discussed for the SDL SCell, we think couple of procedures such as RLM and CBD on the PCell needs special attention.

RLM-RS can be SSB and/or CSI-RS. In legacy, RLM was supported only on SpCell. Since the WI says Strive to minimize the RAN1 impact, we do not foresee RAN1 discussing/introducing the RLM on SCell.

Further UE is expected to perform RLM measurement on the RLM-RS at least once in 2 radio frame and send OOS indication to the higher layer if the OOS is detected.

Based on the switch pattern, the availability of RLM-RS on the PCell may be impacted. If the RLM-RS is impacted due to the switch pattern, RLM evaluation period requirements need further discussion. Since the SDL SCell do not have UL support, HARQ feedback is carried on the PCell. If the radio link of PCell fails, even if the SCell DL is good, UE communication with SCell is as good as lost. Hence, we suggest RAN4 to investigate the RLM and faster link recovery on the PCell. For example, if any of the following condition is met, UE could prioritize performing RLM and link recovery on the slots where the RLM-RS or SSB is transmitted by ignoring the pattern on those slots.

- N1 number of out of sync are triggered on PCell; or

- T310 timer is started.

Similarly, if the BFD is detected on the PCell, UE could prioritize performing CBD on the slots where the CBD-RS is transmitted by ignoring the pattern on those slots (i.e., even if the slot containing CBD-RS belonging to SCell, UE prioritize the CBD-RS reception)

- RAN4 to further discuss the switch pattern applicability when the RLM or BFD is triggered on the PCell and study potential solution for faster link recovery.

## LB CA switching delay and potential interruptions

When RAN4 defines interruption requirements, we typically specify the location and duration of the interruption. Before discussing the necessity of these requirements, we first review RAN1 agreements on the semi-static switch pattern.

In RAN1#120bis, the following agreements were made:

Agreements regarding the semi-static switching pattern: Agreement - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2 - The pattern starts from the beginning of SFN 0 of PCell - Restriction of maximum X switch(es) within Y slot(s), - FFS: detailed value and relation of X and Y. - FFS: other restriction Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the time unit (resolution) of the semi-static switching pattern configuration is slot (1ms for 15kHz SCS). Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms

Agreements regarding the frequency location (i.e., the carrier on which the interruption occurs) of the switching gap: Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, down select one of the following solutions - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier Alt 3: Switching gap is always assumed at the SDL carrier

Agreements regarding the time location of the switching gap (i.e., position within a slot). Agreement For Rel-19 low NR band carrier aggregation via switching, the semi-static switching pattern and corresponding switching gap are based on the downlink timing (DL reception time at UE side) of Pcell. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed) - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA FFS: whether it has specification impact

From the RAN1 agreements, we see that RAN1 has already begun discussing the switching gap, including its location and duration. According to the RAN1 agreement, the network ensures the switching gap covers at least the switching period as stated in RAN4 LS (R1-2501702) and UL TA (if needed). This in inlin with the interruption/scheduling requirement we specify in RAN4.

For the SDL to FDD switch direction, it has been agreed that the switching gap ends at the end of the SDL slot, as illustrated in Figure 3. For the FDD to SDL switch, the frequency location of the switching gap is still to be determined. We anticipate that RAN1 will discuss this open issue in the May meeting. If RAN1 can agree on the FDD to SDL switching gap and its components, RAN4 will be well-positioned to decide if additional interruption requirements are needed beyond the RAN1-defined switching gap. Therefore, we believe RAN4 can wait for one more meeting to discuss the necessity of defining interruption requirements.

- RAN4 to wait for the RAN1 progress to make decision on defining the interruption requirement

Figure 3: Illustration of switching locations in both frequency and time for both FDD to SDL switch and SDL to FDD switch including the effect of timing advance (TA).

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- RAN4 to further investigate the RRM requirements impact with at least one of the following methods.

- Method 1: Agree on the principle of extending the delay based on missed reception occasion of necessary DL signals/channels and apply this principle to all the aforementioned procedures.

- Method 2: Specify an applicability condition under which the requirements are valid for low-band CA via switching.

- RAN4 to use SSB based operation as baseline for Rel-19 low band CA via switching.

- SCell without SSB operation can be considered after the SCell with SSB operation is supported.

- If the SSB less SCell is considered to be in scope, use NES rel-18 WI as baseline for specifying LB-LB CA between collocated FDD-carrier and SSB-less SDL-carrier via switching in rel-19.

- Since we propose NES rel-18 WI as baseline for specifying LB-LB CA between collocated FDD-carrier and SSB-less SDL-carrier via switching in rel-19, we inherit MRTD = CP from NES rel-18, but option 1 MRTD = 3 µs works as well.

- RAN4 to consider skipping deactivated SCell L3-RSRP for the low mobility UEs in collocated PCell/SCell configurations in low-frequency bands.

- RAN4 to define the interruption requirement to allow SDL SCell measurement on deactivated SCell without switch pattern being applicable. For deactivated SDL SCell measurement, interruptions on PCell or activated SCell(s) are allowed with up to 0.5% probability of missed ACK/NACK regardless of the configured measCycleSCell.

- For both normal and fast SCell activation procedures, reuse the existing requirements by allowing the UE to switch to the SCell during SSB, TRS, A-TRS, and CSI-RS occasions. In other words, the SCell activation delay requirement remains unaffected, as the switching pattern is not enforced during the SCell activation process.

- RAN4 to agree on applying the enhancements introduced in Rel-18 for SCell activation—such as shortMeasInterval-r18—to LB CA as well.

- RAN4 to consider skipping activated SCell L3-RSRP for the low mobility UEs in collocated PCell/SCell configurations in low-frequency bands.

- RAN4 to define a new Kp_LB to address the impact of switch patterns on the measurement period.

- RAN4 to further discuss CSSFintra for the LB CA.

- For L1 measurements, RAN4 to define new sharing factor, PLB to account for the non-availability of SSBs due to the switch pattern and SSB overlap with MG.

- RAN4 to change the defintion of Tfirst-SSB for the TCI state switch delay and it is to be defined as the first SSB available on the respective cell after the switch command.

- RAN4 to consider changes to TCI state switch requirement equation to consider the SSB being received on SDL before the HARQ being sent on FDD.

- The principle of scaling factor agreed for SDL SCell, to be applicable for PCell as well.

- RAN4 to further discuss the switch pattern applicability when the RLM or BFD is triggered on the PCell and study potential solution for faster link recovery.

- RAN4 to wait for the RAN1 progress to make decision on defining the interruption requirement.

# References

[1]		RP-243317, New WID on low band CA via switching

[2]		RP-241537

[3]		R4-2502588, WF on RRM requirements for NR_LBCA_Sw, Moderator (Apple)