3GPP TSG-RAN WG4#115	R4-2507069

Malta, MT, May 19th – 23rd, 2025

Agenda item:	5.3.3

Source: 	Qualcomm France

Title: 	Handling of Optional bandwidths across releases

Document for:	Approval

# Introduction

Handling of few optional bandwidths across releases is discussed. Please note that WF [1] contained also MSD work related to this, which we shall address in separate Tdoc.

- Background

WF was agreed in RAN4-114bis [1]:

WF#1 on handling optional CBW across Releases

Agreement 1:

- Change across Releases from “optional support” to “mandatory support” is evaluated on a case-by-case basis. Interested companies are invited to share their views on the following options for the following CBW:

- 3MHz CBW:

  - option 1: remains optional across Releases for all bands except for band n106 where mandatory support is kept across Releases,

  - option 2: changes to mandatory support in Release 19, but this CBW is not accounted for MSD analyses.

- 5MHz CBW for band n41/n90:

  - option 1: remains optional across Releases

  - option 2: changes to mandatory support in Release 19

  - option 3: remove this CBW from band n41/n90.

- 35MHz CBW for band n71

  - Option 1: UL remains optional across releases

  - Option 2: changes UL to mandatory support in Release 19

Agreement 2:

It is agreed that 100MHz CBW for band n46, n96 and n102 becomes mandatory support in Release 19.

WF#2 on MSD test point maintenance for BCS4/5 due to new smallest/greatest CBW

Agreement:

- Impact on 1UL MSD test points: agreements from [2] and captured in RAN4 PRD apply.

- Impact on 2UL MSD test points: At the exception of 3MHz, and [7MHz] CBW, an MSD test point is needed when a new smallest CBW is enabled by BCS4/5 even if CBW is optional support in current Release.

WF#3 on MSD evaluation

Agreement: interested companies are invited to evaluate the following MSDs [1]

CA_n5A-n71A:

- Consider changing 1st test point to 2nd optional test point based on [2],

- Evaluate the following band n5 MSD requirements,

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n5 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | [FFS] | >ACLR2 |

CA_n71A-n85A:

PC3: Consider voiding footnotes 6 and 7 in Table 7.3A.6-1.

PC2: Consider voiding footnotes 4 and 5 in Table 7.3A.6-1a-1	.

CA_n77A-n102A, CA_n78A-n102A:  Evaluate the following PC3 cross-band isolation MSD requirements.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n102 | n77 | 5975 | 100 | 30 | 270 (RBstart=0) | 4195 | 10 | [FFS] | >ACLR2 |
| n102 | n78 | 5975 | 100 | 30 | 270 (RBstart=0) | 3795 | 10 | [FFS] | >ACLR2 |

CA_n3A-n74A, CA_n3A-n75A:

- Consider the following change of 1st test point to 2nd optional test point based on [2].

- Evaluate the following band n74 and band n75 MSD requirements,

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n74 | 1730 | 40 | 15 | 50 (RBstart=0) | 1515.5 | 5 | [FFS] | >ACLR2 |
| n3 | n75 | 1730 | 40 | 15 | 50 (RBstart=0) | 1514.5 | 5 | [FFS] | >ACLR2 |

- Discussion

Agreement 1:

3MHz CBW:

  - option 1: remains optional across Releases for all bands except for band n106 where mandatory support is kept across Releases,

  - option 2: changes to mandatory support in Release 19, but this CBW is not accounted for MSD analyses.

Our preference is option 1, i.e. to keep 3MHz optional across Releases for all bands except for band n106 where mandatory support is kept.

Proposal 1: Keep 3MHz optional across Releases for all bands except for band n106

5MHz CBW for band n41/n90:

  - option 1: remains optional across Releases

  - option 2: changes to mandatory support in Release 19

  - option 3: remove this CBW from band n41/n90.

In our understanding, 5MHz for n41/n90 is not deployed anywhere e.g. due to 30kHz SCS not supporting 5MHz BW’s. Our preference is to keep 5MHz for n41/n90 as optional across Releases. We would not object in case this CBW from n41/n90 were removed either.

Proposal 2: Keep or remove 5MHz CBW from band n41/n90

35MHz CBW for band n71

  - Option 1: UL remains optional across releases

  - Option 2: changes UL to mandatory support in Release 19

We don’t have a very clear view on this one. If there is real demand for this by operators’ then making it mandatory should definitely considered. If there is no clear demand for this, then making it mandatory may not be the best option as optimizing the performance especially for UL BW which comes along with degraded REFSENS due to TX impairments as this BW has TX impairments (degraded REFSENS) might end up being useless.

Our preference is to keep this as optional for now, but we are open to discuss with interested parties on this.

Proposal 3: Keep 35MHz n71 UL optional across releases unless otherwise agreed in discussions with interested parties

- Conclusion

Considerations on handling of optional bandwidths across releases were provided with the following proposals.

Proposal 1: Keep 3MHz optional across Releases for all bands except for band n106

Proposal 2: Keep or remove 5MHz CBW from band n41/n90

Proposal 3: Keep 35MHz n71 UL optional across releases unless otherwise agreed in discussions with interested parties

- Reference

[1] R4-2505214, “WF on handling optional channel bandwidth across releases”, Skyworks