3GPP TSG-RAN WG4#115	R4-2507073

Valletta, Malta, May 19th – 23rd, 2025

Agenda item:	7.1.2.3.4

Source: 	Qualcomm France

Title: 	SRS IL imbalance indication

Document for:	Approval

# Introduction

SRS IL imbalance indication considered in this contribution.

- WF from RAN4#114bis

### Sub-topic 4-1: General considerations for SRS IL imbalance issue

Issue 4-1-1: Whether to solve SRS IL imbalance issue in Rel-19

Way forward:

- No further discussion on the issue in future meetings and focus on identification of possible SRS IL imbalance issue solutions.

  - NOTE: This does not mean that any solution for SRS IL imbalance issue is specified in Rel-19

Issue 4-1-2: SRS IL solution applicability to 2Rx/4Rx/6Rx/8Rx

Agreement

- FFS on the potential SRS IL solution applicability to 2Rx/4Rx/8Rx UEs in addition to 6Rx UEs

Issue 4-1-3: SRS IL imbalance solution as optional feature

Agreement

- If any new requirements or procedures for SRS IL handling are defined in Rel-19, they will be defined as optional UE features

### Sub-topic 4-2: SRS IL imbalance issue solutions

Way forward

- RAN4 to further discuss the candidate SRS IL imbalance solution framework based on agreements for issues 4-2-1 and 4-2-2.

- Companies are encouraged to provide views on additional aspects including

  - SRS output power testing

  - Enhancements to PCmax equations to improve SRS IL compensation

Issue 4-2-1: SRS IL imbalance UE self-compensation

Agreement:

- RAN4 assumptions on UE behavior for Rel-19 SRS IL handling:

  - For SRS transmissions Rel-19 UE is expected to perform self-compensation of SRS IL up to its configured maximum output power

    - FFS if any additional clarifications on behavior are needed

    - No RAN1/4 specification impacts (i.e. this behavior is aligned with existing specifications)

Issue 4-2-2: SRS IL imbalance UE assistance and reporting

Agreement:

- The following two possible general solutions may be considered for further consideration

  - Solution 1: Introduce UE capability indication on whether UE performs SRS IL self-compensation

    - FFS on the detailed solution including clarification on self-compensation and whether static or dynamic indication will be needed

  - Solution 2: Extend Type 3 SRS PHR reporting to carriers with PUSCH subject to RAN1 confirmation on feasibility

    - Further check whether solution 2 is in the scope or not

Considerations of any solutions should be based on system level throughput evaluations, and implementation complexity

- Discussion

Issue 4-2-1: SRS IL imbalance UE self-compensation

UE behaviour and hence UE self-compensation is RAN1 specification matter (TS38.213), not RAN4 matter. There is no need to make any additions/changes into RAN4 specification because of this. If changes are proposed, RAN4 would need to ask RAN1 view on those.

Proposal 1: No clarifications/changes to RAN4 specification related to UE self-compensation.

Issue 4-2-2: SRS IL imbalance UE assistance and reporting

Solution 1:

Adding a UE capability to indicate whether UE performs self-compensation or not was proposed by some companies. It would be very weird to add such capability indication without asking RAN1 whether TS38.213 mandates UE self-compensation or not; If it does, then there is no point adding the bit as UE’s not indicating that would be incompliant as per RAN1. If it does not, then RAN4 assumption in 4-2-1 was wrong and all things should be re-considered.

Proposal 2: Do not add UE capability to indicate support of UE SRS IL self-compensation.

Instead of adding any UE capability on UE self-compensation, RAN4 should discuss how current ΔTRxSRS should be tested. The fact that as of today, there is no verification of ΔTRxSRS. As indicated earlier, we would be willing to consider adding UL RMC’s so that ΔTRxSRS can be verified by RAN5 as having ΔTRxSRS requirements in RAN4 but no means to verify those does not make a lot of sense. Based on the discussion in previous meeting, there is also quite a bit reluctance to do even that as some companies provided such views. Having said that we are ok to consider adding these RMC’s but if they are added than they are applicable to all UE’s i.e. they are not behind any UE capability. The discussion on adding UL RMC’s should consider the following aspects.

- Whether UL RMC’s to verify SRS power at least at max power level are added

- If SRS power were tested below max power, should RAN4 and RAN1 view on UE compensation before doing that?

- From which release, if any should UL RMC’s be added to verify SRS power

Proposal 3: Discuss whether UL RMC’s to verify SRS power at least at max power level can be added

Solution 2:

With regards to this one, our view from the previous meetings remains unchanged. We don’t see that the proposed solutions would improve overall situation, even though they would cause a lot of extra complexity.

Any reporting per SRS resource would only work when SRS resource consists of one SRS port. For SRS resource consisting of 2 ports, if would not help because the IL can be (and often is) different. Hence reporting one shared delta value would be inaccurate e.g. in case when power available on SRS port#3 is 21dB, and power available in SRS port#4 is 17dB.

We re-iterate our comments from previous meetings that is when the UE compensates SRS IL when possible by power reserves, any extra indication would not help as the said delta would be zero as long as UE is able to compensate. There have been no simulation results to show system-level gains for the cases where UE compensates when it has power to do so and does not compensate when there is no power remaining.

It is obvious that the system-level benefits, if any, would be very small especially for UE’s compensating the SRS IL. In far-cell scenario where compensation may not be possible, SRS may not be that beneficial anyhow independent of indication or no indication. In any kind of indication there would be quite a bit of additional complexity, and no guarantee that NW would use any of the additional information.

The proposed new MAC CE would include lot’s of reported numbers which would involve significant amount of extra complexity with uncertain benefits and no guarantee that NW would use any of the additional information given infra vendors have very diverse views on this.

Observation 1: Proposed SRS IL indication scheme would not work for all cases.

Observation 2: Proposed SRS IL indication scheme would introduce quite a bit of extra complexity.

Observation 3: There have been no simulation results to show system level gains for the cases where UE compensates when it has power to do so and does not compensate when there is no power remaining.

Proposal 4: Do not pursue option 2.

- Conclusion

SRS IL indication was discussed with the following observations and proposals.

Proposal 1: No clarifications/changes to RAN4 specification related to UE self-compensation.

Proposal 2: Do not add UE capability to indicate support of UE SRS IL self-compensation.

Observation 1: Proposed SRS IL indication schemes would not work for all cases.

Observation 2: Proposed SRS IL indication schemes would introduce quite a bit of extra complexity.

Observation 3: There have been no simulation results to show system level gains for the cases where UE compensates when it has power to do so and does not compensate when there is no power remaining.

Proposal 3: Discuss whether UL RMC’s to verify SRS power at least at max power level can be added

Proposal 4: Do not pursue option 2.

- Reference

[1] R4-2505101, “WF on 6Rx RF requirements and SRS IL imbalance”, AT&T