3GPP TSG-RAN WG4 Meeting # 115	R4-2507082

# St Julian’s, Malta, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.863 | CR | CRNum | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 38.863 on SAN RF requirements for less than 5MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TR 38.863 is included as ‘Impacted existing TS/TR’ in the approved WID in RP-242901 for less than 5MHz in bands n255 and n256. Corresponding materials on UE RF requirements has been endorsed in the Rel-19 draft CR R4-2504710, but materials on SAN RF requirements are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add materials on SAN RF requirements for less than 5MHz according to the approved WFs in R4-2416564 and R4-2419793. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SAN RF requirements for less than 5MHz remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause under 7.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The index ‘x’ should be replaced with the next available index under 7.3.4 when implementing this draft CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

#### 7.3.4.x	SAN RF requirements for 3MHz channel bandwidth

##### 7.3.4.x.1	Conducted Tx/Rx requirements

The agreed changes to SAN RF conducted Tx/Rx requirements for 3MHz channel bandwidth are provided in Table 7.3.4.x.1-1.

Table 7.3.4.x.1-1: Conducted Tx/Rx requirements for 3MHz channel bandwidth

| NR SAN RF conducted Tx/Rx requirement in TS 38.108 [26] | Proposal for 3MHz channel bandwidth in FR1-NTN bands |
| --- | --- |
| 6.2 Satellite Access Node output power | No specification impact. |
| 6.3.2 RE power control dynamic range | No specification impact. |
| 6.3.3 Total power dynamic range | Minimum requirement calculated according to transmission bandwidth configuration in Table 6.3.3.2-1. |
| 6.4 Transmit ON/OFF power | No specification impact. |
| 6.5 Transmitted signal quality | No specification impact. |
| 6.6.2 Occupied bandwidth | No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Requirement limits as other channel bandwidth, but assume different adjacent channel carriers in Table 6.6.3.2-1 and Table 6.6.3.2-2. |
| 6.6.4 Out-of-band emissions | No specification impact. |
| 6.6.5 Transmitter spurious emissions | No specification impact |
| 6.7 Transmitter intermodulation | No specification impact. |
| 7.2 Reference sensitivity level | Add minimum requirements for 3 MHz in Table 7.2.2-1 and Table 7.2.2-2. Scaling from 5 MHz can be used. Use FRC G-FR1-A1-7, same as in TN. |
| 7.3 Dynamic range | Add minimum requirements for 3 MHz in Table 7.3.2-1. Scaling from 5 MHz can be used. Reuse TN FRC G-FR1-A2-15 (to be introduced to A.2). |
| 7.4.1 Adjacent Channel Selectivity (ACS) | Define ACS requirement similar to TN for 3MHz in Table 7.4.1.2-1, and assume same interferer frequency offset values as TN 3 MHz in Table 7.4.1.2-2. |
| 7.4.2 In-band blocking | No specification impact. |
| 7.5 Out-of-band blocking | No specification impact. |
| 7.6 Receiver spurious emissions | No specification impact. |
| 7.7 Receiver intermodulation | No specification impact. |
| 7.8 In-channel selectivity | Add minimum requirements for 3 MHz in Table 7.8.2-1 and Table 7.8.2-2. Reuse TN FRC G-FR1-A1-20 (to be introduced to A.1). |
| A.1 Fixed Reference Channels for RF Rx requirements (QPSK, R=1/3) | FRC for 3MHz in-channel selectivity need to be added in Table A.1-1. Reuse TN FRC G-FR1-A1-20. Rename the FRC to indicate “NTN” can be discussed as Rel-18 maintenance. |
| A.2 Fixed Reference Channels for dynamic range (16QAM, R=2/3) | FRC for 3MHz needs to be added in Table A.2-1. Reuse TN FRC G-FR1-A2-15. Rename the FRC to indicate “NTN” can be discussed as Rel-18 maintenance. |

##### 7.3.4.x.2	Radiated Tx/Rx requirements

The agreed changes to SAN RF radiated Tx/Rx requirements for 3MHz channel bandwidth are provided in Table 7.3. 4.x.2-1.

Table 7.3.4.x.2-1: Radiated Tx/Rx requirements for 3MHz channel bandwidth

| NR SAN RF conformance requirement in TS 38.181 [25] | Expected updates for 3MHz channel bandwidth in FR1-NTN bands |
| --- | --- |
| 4.1.2 Acceptable uncertainty of Test System | Add maximum test system uncertainty for 3 MHz in Table 4.1.2.2-1 and Table 4.1.2.2-2. |
| 4.7.2 Test signal used to build Test Configurations | Add a note in Table 4.7.2-1 to use 3 MHz channel bandwidth if supported. |
| 6.2 Satellite Access Node output power | No specification impact. |
| 6.3.2 RE power control dynamic range | No specification impact. |
| 6.3.3 Total power dynamic range | Add minimum requirements for 3 MHz in Table 6.3.3.5-1. |
| 6.4 Transmit ON/OFF power | No specification impact. |
| 6.5 Transmitted signal quality | Add minimum requirements for 3 MHz in Table 6.5.3.5-2. |
| 6.6.2 Occupied bandwidth | Add minimum requirements for 3 MHz in Table 6.6.2.4.2-1. |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Requirement limits as other channel bandwidth, but assume different adjacent channel carriers in Table 6.6.3.5.2-1 and Table 6.6.3.5.2-2. |
| 6.6.4 Out-of-band emissions | No specification impact. |
| 6.6.5 Transmitter spurious emissions | No specification impact |
| 6.7 Transmitter intermodulation | No specification impact. |
| 7.2 Reference sensitivity level | Add minimum requirements for 3 MHz in Table 7.2.5-1 and Table 7.2.5-2. Scaling from 5 MHz can be used. Use FRC G-FR1-A1-7, same as in TN. |
| 7.3 Dynamic range | Add minimum requirements for 3 MHz in Table 7.3.5-1. Scaling from 5 MHz can be used. Reuse TN FRC G-FR1-A2-15 (to be introduced to A.2). |
| 7.4.1 Adjacent Channel Selectivity (ACS) | Define ACS requirement similar to TN for 3MHz in Table 7.4.1.5-1, and assume same interferer frequency offset values as TN 3 MHz in Table 7.4.1.5-2. |
| 7.4.2 In-band blocking | No specification impact. |
| 7.5 Out-of-band blocking | No specification impact. |
| 7.6 Receiver spurious emissions | No specification impact. |
| 7.7 Receiver intermodulation | No specification impact. |
| 7.8 In-channel selectivity | Add minimum requirements for 3 MHz in Table 7.8.5-1 and Table 7.8.5-2. Reuse TN FRC G-FR1-A1-20 (to be introduced to A.1). |
| 9.2 Radiated transmit power | No specification impact. |
| 9.3 OTA SAN Output power dynamics | No specification impact. |
| 9.4 OTA output power dynamic range | Similar update as conductive requirements to add 3MHz channel bandwidth. |
| 9.5 OTA transmit ON/OFF Power | No specification impact. |
| 9.6 OTA transmitted signal quality | Similar update as conductive requirements to add 3MHz channel bandwidth. |
| 9.7 OTA unwanted emissions | Similar update as conductive requirements to add 3MHz channel bandwidth. |
| 9.8 OTA Transmitter intermodulation | No specification impact. |
| 10.2 OTA sensitivity | Similar update as conductive requirements to add 3MHz channel bandwidth. |
| 10.3 OTA reference sensitivity level | Similar update as conductive requirements to add 3MHz channel bandwidth. |
| 10.4 OTA dynamic range | Similar update as conductive requirements to add 3MHz channel bandwidth. |
| 10.5 OTA In-band selectivity and blocking | Similar update as conductive requirements to add 3MHz channel bandwidth. |
| 10.6 OTA Out-of-band blocking | No specification impact. |
| 10.7 OTA receiver spurious emissions | No specification impact. |
| 10.8 OTA receiver intermodulation | No specification impact. |
| 10.9 OTA In-channel selectivity | Similar update as conductive requirements to add 3MHz channel bandwidth. |
| A.1 Fixed Reference Channels for RF Rx requirements (QPSK, R=1/3) | FRC for 3MHz in-channel selectivity need to be added in Table A.1-1. Reuse TN FRC G-FR1-A1-20. Rename the FRC to indicate “NTN” can be discussed as Rel-18 maintenance. |
| A.2 Fixed Reference Channels for dynamic range (16QAM, R=2/3) | FRC for 3MHz needs to be added in Table A.2-1. Reuse TN FRC G-FR1-A2-15. Rename the FRC to indicate “NTN” can be discussed as Rel-18 maintenance. |

<End of change>