3GPP TSG-RAN4 Meeting #115	R4-2507090

# Malta, MT, 19th – 23rd May, 2025

Source:	LG Electronics

Title: 	On Ku-band VSAT reference sensitivity and maximum input level

Agenda Item:	7.11.5

Document for:	Discussion and Approval

# Introduction

RAN4 has been discussing requirements for NTN Ku-band VSAT RE-requirements. Way forwards [1] and [2] were agreed in RAN WG4 Meeting #114bis and agreements in these documents and company inputs captured into [3] are used as inputs in this analysis.

This document discusses the VSAT reference sensitivity and maximum received input level requirement and proposes a draft CR to capture the requirement into TS 38.101-5 [4].

# Discussion

## VSAT RX antenna gain and Noise Figure

Reference [2] agrees the VSAT antenna gain and noise figure assumtions for the three different VSAT types, for which the co-existence simulations will be performed. These VSAT types and their antenna gain assumtions are listed in Table 3 of the [2] and antenna gain values derived from assumtions and receiver Noise figure are shown in Table 1 below. The antenna array gain with 35degrees elevation, which corresponds to 110degress field og view for the satellites is derived from antenna gain simulations, but can also be estimated with well-known formula of GainMAX-20log10(cos()), where  is the electronic steering angle.

The peak antenna gains below are assuming that antenna is optimized for certain frequency, but often antennas are used multiple frequencies that are only close to optimum, like TX and RX bands within the Ku-band (DL ~ 11GHz and TX ~ 14GHz). In such cases one must decide, frequency where the antenna performance is optmized, which then means that the other frequenmcies will experience slightly compromized antenna performance. When it comes to UEs or VSAT it is usually mandatory to optimize the antenna(s) for TX as cost (thermal, size, weight, cost) of TX design trade-offs are far higher than RX trade-offs. Based on simulations for the Ku-band the antenna gain trade-off for RX, when array is optmized for TX is about 1.5-2dB. Here 2 dB is assumed.

Table 1 VSAT RX antenna and receiver characteristics

| VSAT Antenna Characteristics |  |  |  |
| --- | --- | --- | --- |
| Antenna pattern | Phase array | Phase array | Parabolic Antenna |
| Array size | 45x45 | 78x78 | n.a. |
| Peak antenna gain 90deg elevation - TXRX trade-off | 36.6 – 2 = 34.6 dB | 41.3 - 2 = 39.2 dB | 34.3 dB |
| Peak antenna gain 35deg elevation | 31.7 -2 = 29.7 dB | 36.5 – 2 = 34.5 dB | 34.3 dB |
| Noise figure | 6 dB | 6 dB | 2.5 dB |

Figure 1. Antenna gains with different electronic steering angles in Vertical coverage range

Figure 2. Simulated gain and 3dB beam width for the phased array antennas

## SAN antenna gain output power density

Reference [2] Table 2 shows the SAN antenna gain and EIRP density. These are also shown in Table 2 below. As the satellite output power is indicated with EIRP density it means that wider bandwidth signals are transmitter with higher ourput power.

However, it’s necessary to take into account that target VSAT is not always at the center of the satellite beam, where the satellite reaches the maximum EIRP density, but VSAT may be located at the edge of the beam where the EIRP density is ~3dB lower. in Table 2 shows also the 3dB beamwidth and the beam diameter on the earth’s surface, although for reference sensitivity and maximum received input power analysis it’s enough to assume that satellite output power may be at the maximum or 3dB below the maximum.

Table 2 Satellite Parameters

| Satellite orbit |  | GEO | LEO-600 |
| --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |
| Equivalent satellite antenna aperture | Ku-band (i.e. 11 GHz for DL) | 4 m | 0.91 m |
| Satellite EIRP density |  | 43 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  | 43 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.400 deg | 1.768 deg |
| Satellite beam diameter |  | 250 km | 19 km |

## Down-link link budget

The downlink signal SNR at the VSAT receiver antenna input (EIS) can be calculated by subtracting the path loss from the signal power leaving the satellite antenna and comparing this power with equivalent noise power at the same point.

For path loss the key input parameter is the distance between the satellite and the operating frequency. Figure 3 shows how the distance between the VSAT and satellite is calculated with different elevation angle and distance and corresponding path loss for LEO600, LEO1200 and GEO scenario are shown in Table 3 and Table 4.

Figure 3. Geometries for calculating the distance between the VSAT and satellite (LEO use case as example)

Table 3. Distance between the VSAT and satellite in different scenarios

| Distance | LEO 600 | LEO 1200 | GSO |  |
| --- | --- | --- | --- | --- |
| h (zebith distance) | 600 | 1200 | 35786 | km |
| x (horizon distance) | 2831 | 4093 | 41681 | km |
| y (beta elevation distance) | 967 | 1831 | 38181 | km |
| beta (elevation) | 35 | 35 | 35 | deg |

Table 4. Path loss in different scenarios

| Path loss | LEO 600 | LEO 1200 | GSO |  |
| --- | --- | --- | --- | --- |
| Zenith | 170.1 | 176.1 | 205.6 | dB |
| Horizon | 183.6 | 186.8 | 206.9 | dB |
| Bera elevation | 174.3 | 179.8 | 206.2 | dB |
| beta (elevation) | 35 | 35 | 35 | deg |
| Fc | 12750 | 12750 | 12750 | MHz |

## Reference measurement channels

Reference sensitivity and EIS reference sensitivity are always defined using certain input signal, which is referred as reference measurement channel (RMC). For reference sensitivity requirements the RMC is usually the QPSK modulated channel with 1/3 code rate, which gives 95% throughput with about -1dB SNR. [4] 3GPP TS 38.101-5 currently defines following RMCs for down-link in A.3.2 Reference measurement channels for PDSCH performance requirements.

Observation 1: Currently PDSCH QPSK RMC is defined only for 10MHz channel with 15kHz SCS, for 50, 100 and 200MHz with 60kHz SCS and for 50, 100, 200 and 400MHz with 120kHz SCS. There are no RMC defined for 30kHz SCS.

Proposal 1: It’s proposed to use the existing RMCs for defining the EIS reference sensitivity requirements for Ku-band VSAT supporting FR1 and/or FR2 numerologies. New QPSK reference channels with different CBW and SCS can be introduced at later point in time or during this WI but are not mandatory to complete the Ku-band WI.

Table 5. Reference measurement channels for PDSCH performance requirements

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 50 | 100 | 200 | 50 | 100 | 200 | 400 |
| Subcarrier spacing | kHz | 15 | 60 | 60 | 60 | 120 | 120 | 120 | 120 |
| Number of allocated resource blocks | PRBs | 52 | 66 | 132 | 264 | 32 | 66 | 132 | 264 |
| MCS index |  | 4 13 | 4 13 19 | 4 13 19 | 4 13 19 | 4 13 | 4 13 | 4 13 | 4 13 |
| Modulation |  | QPSK 16QAM | QPSK 16QAM 64QAM | QPSK 16QAM 64QAM | QPSK 16QAM 64QAM | QPSK 16QAM 64QAM | QPSK 16QAM 64QAM | QPSK 16QAM 64QAM | QPSK 16QAM 64QAM |
| Target Coding Rate |  | 0.30 0.48 | 1/3 0.48 1/2 | 1/3 0.48 1/2 | 1/3 0.48 1/2 | 1/3 0.48 1/2 | 1/3 0.48 1/2 | 1/3 0.48 1/2 | 1/3 0.48 1/2 |

## EIS reference sensitivity and maximum input level

The EIS reference sensitivity and maximum input level calculations are shown in the Table 6. Signal level at the VSAT antenna input for all three VSAT types and LEO600 and GSO satellite orbits is analyzed in three different cases as listed below:

Case A: VSAT at the center of the satellite beam, VSAT antenna elevation 90deg (Highest SNR)

Case B: VSAT at the center of the satellite beam, VSAT antenna elevation 35deg

Case C: VSAT at the edge of the satellite beam, VSAT antenna elevation 35deg (Lowest SNR)

For each case the lowest row of the table shows the margin against the targeted -1dB SNR of the used Reference Measurement Channel (RMC). The results are summarized Table 7, Table 8 and Table 9.

Table 7 shows the minimum signal power needed to meet 1dB SNR. This is only dependent on the VSAT noise figure and antenna gain taking into account the elevation angle in each case.

Table 8 shows the signal power at the VSAT antenna input. This is only dependent on the satellite parameters, scenario (LEO, GSO) and geometry of the each case.

Table 9 compares these two and shows the margin to meet -1dB SNR in each case.

Detailed analysis as shown in the listed tables is only shown for FR2 numerology and 50MHz CBW as values for the FR1 numerology and 10MHz CBW can be defined by scaling the above mentioned ones.

Table 6. EIS reference sensitivity and maximum input level details for 60kHz SCS and 50MHz channel

| VSAT noise at antenna input (radiated) |  |  | Array |  |  |  | Array |  |  |  | Array |  |  |  | Array |  |  |  | Parabolic |  |  |  | Parabolic |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VSAT antenna type |  |  | 45x45 | 45x45 | 45x45 |  | 45x45 | 45x45 | 45x45 |  | 78x78 | 78x78 | 78x78 |  | 78x78 | 78x78 | 78x78 |  | 60cm | 60cm | 60cm |  | 60cm | 60cm | 60cm |  |
| Channel BW |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |  | 50 | 50 | 50 |  | 50 | 50 | 50 |  | 50 | 50 | 50 |  | 50 | 50 | 50 | MHz |
| No |  |  | -173.8 | -173.8 | -173.8 |  | -173.8 | -173.8 | -173.8 |  | -173.8 | -173.8 | -173.8 |  | -173.8 | -173.8 | -173.8 |  | -173.8 | -173.8 | -173.8 |  | -173.8 | -173.8 | -173.8 | dBm/sqrt(Hz) |
| BW (66*12*0.06= 47.520MHz) |  |  | 76.8 | 76.8 | 76.8 |  | 76.8 | 76.8 | 76.8 |  | 76.8 | 76.8 | 76.8 |  | 76.8 | 76.8 | 76.8 |  | 76.8 | 76.8 | 76.8 |  | 76.8 | 76.8 | 76.8 | dB |
| Noise Figure |  |  | 6 | 6 | 6 |  | 6 | 6 | 6 |  | 6 | 6 | 6 |  | 6 | 6 | 6 |  | 2.5 | 2.5 | 2.5 |  | 2.5 | 2.5 | 2.5 | dB |
| Nt |  |  | -91.1 | -91.1 | -91.1 |  | -91.1 | -91.1 | -91.1 |  | -91.1 | -91.1 | -91.1 |  | -91.1 | -91.1 | -91.1 |  | -94.6 | -94.6 | -94.6 |  | -94.6 | -94.6 | -94.6 | dBm |
| VSAT antenna elevation |  |  | 90 | 35 | 35 |  | 90 | 35 | 35 |  | 90 | 35 | 35 |  | 90 | 35 | 35 |  | 90 | 35 | 35 |  | 90 | 35 | 35 | deg |
| VSAT antenna gain |  |  | 34.6 | 29.7 | 29.7 |  | 34.6 | 29.7 | 29.7 |  | 39.3 | 34.5 | 34.5 |  | 39.3 | 34.5 | 34.5 |  | 34.3 | 34.3 | 34.3 |  | 34.3 | 34.3 | 34.3 | dB |
| Equivalent Noise at antenna input ~50MHz |  |  | -125.6 | -120.8 | -120.8 |  | -125.6 | -120.8 | -120.8 |  | -130.4 | -125.6 | -125.6 |  | -130.4 | -125.6 | -125.6 |  | -128.9 | -128.9 | -128.9 |  | -128.9 | -128.9 | -128.9 | dBm |
| Link budget |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Satellite |  |  | LEO | LEO | LEO |  | GSO | GSO | GSO |  | LEO | LEO | LEO |  | GSO | GSO | GSO |  | LEO | LEO | LEO |  | GSO | GSO | GSO |  |
| VSAT location within DL beam |  |  | Center | Center | Edge |  | Center | Center | Edge |  | Center | Center | Edge |  | Center | Center | Edge |  | Center | Center | Edge |  | Center | Center | Edge |  |
| Satellite Antenna gain |  |  | 38.5 | 38.5 | 35.5 |  | 43 | 43 | 40 |  | 38.5 | 38.5 | 35.5 |  | 43 | 43 | 43 |  | 38.5 | 38.5 | 35.5 |  | 43 | 43 | 40 | dB |
| 3dB Beam Width |  |  | 1.768 | 1.768 | 1.768 |  | 0.4 | 0.4 | 0.4 |  | 1.768 | 1.768 | 1.768 |  | 0.4 | 0.4 | 0.4 |  | 1.768 | 1.768 | 1.768 |  | 0.4 | 0.4 | 0.4 | deg |
| Beam diameter |  |  | 19 | 19 | 19 |  | 250 | 250 | 250 |  | 19 | 19 | 19 |  | 250 | 250 | 250 |  | 19 | 19 | 19 |  | 250 | 250 | 250 | km |
| Satellite EIRP density in dBW |  |  | 7 | 7 | 4 |  | 43 | 43 | 40 |  | 7 | 7 | 4 |  | 43 | 43 | 40 |  | 7 | 7 | 4 |  | 43 | 43 | 40 | dBW/MHz |
| Satellite EIRP density in dBm |  |  | 37 | 37 | 34 |  | 73 | 73 | 70 |  | 37 | 37 | 34 |  | 73 | 73 | 70 |  | 37 | 37 | 34 |  | 73 | 73 | 70 | dBm/MHz |
| 1MHz > 50MHz |  |  | 16.8 | 16.8 | 16.8 |  | 16.8 | 16.8 | 16.8 |  | 16.8 | 16.8 | 16.8 |  | 16.8 | 16.8 | 16.8 |  | 16.8 | 16.8 | 16.8 |  | 16.8 | 16.8 | 16.8 | dB |
| Satellite EIRP 50/10MHz |  |  | 53.8 | 53.8 | 50.8 |  | 89.8 | 89.8 | 86.8 |  | 53.8 | 53.8 | 50.8 |  | 89.8 | 89.8 | 86.8 |  | 53.8 | 53.8 | 50.8 |  | 89.8 | 89.8 | 86.8 | dBm |
| Distance |  |  | 600 | 967 | 967 |  | 35786 | 38181 | 38181 |  | 600 | 967 | 967 |  | 35786 | 38181 | 38181 |  | 600 | 967 | 967 |  | 35786 | 38181 | 38181 | km |
| Frequency |  |  | 12750 | 12750 | 12750 |  | 12750 | 12750 | 12750 |  | 12750 | 12750 | 12750 |  | 12750 | 12750 | 12750 |  | 12750 | 12750 | 12750 |  | 12750 | 12750 | 12750 | MHz |
| Path loss |  |  | 170.1 | 174.3 | 174.3 |  | 205.6 | 206.2 | 206.2 |  | 170.1 | 174.3 | 174.3 |  | 205.6 | 206.2 | 206.2 |  | 170.1 | 174.3 | 174.3 |  | 205.6 | 206.2 | 206.2 | dB |
| Atmosphere loss |  |  | 2 | 2 | 2 |  | 2 | 2 | 2 |  | 2 | 2 | 2 |  | 2 | 2 | 2 |  | 2 | 2 | 2 |  | 2 | 2 | 2 | dB |
| Total propagation loss |  |  | 172.1 | 176.3 | 176.3 |  | 207.6 | 208.2 | 208.2 |  | 172.1 | 176.3 | 176.3 |  | 207.6 | 208.2 | 208.2 |  | 172.1 | 176.3 | 176.3 |  | 207.6 | 208.2 | 208.2 | dB |
| Signal power at antenna input ~50MHz |  |  | -118.3 | -122.5 | -125.5 |  | -117.9 | -118.4 | -121.4 |  | -118.3 | -122.5 | -125.5 |  | -117.9 | -118.4 | -121.4 |  | -118.3 | -122.5 | -125.5 |  | -117.9 | -118.4 | -121.4 | dBm |
| Received signal SNR |  |  | 7.3 | -1.7 | -4.7 |  | 7.8 | 2.4 | -0.6 |  | 12.1 | 3.1 | 0.1 |  | 12.5 | 7.2 | 4.2 |  | 10.5 | 6.4 | 3.4 |  | 11.0 | 10.4 | 7.4 | dB |
| Target SNR |  |  | -1 | -1 | -1 |  | -1 | -1 | -1 |  | -1 | -1 | -1 |  | -1 | -1 | -1 |  | -1 | -1 | -1 |  | -1 | -1 | -1 |  |
| Margin |  |  | 8.3 | -0.7 | -3.7 |  | 8.8 | 3.4 | 0.4 |  | 13.1 | 4.1 | 1.1 |  | 13.5 | 8.2 | 5.2 |  | 11.5 | 7.4 | 4.4 |  | 12.0 | 11.4 | 8.4 | dB |

Table 7. Summary for EIS Reference sensitivity to meet -1dB SNR in selected cases A, B and C

|  |  |  |  |  |  |  | =ROUND(EISREFSENS) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EISREFSENS_50MHz (dBm) |  |  | Case A | Case B | Case C |  | Case A | Case B | Case C |
| VSAT location within DL beam |  | VSAT | Center | Center | Edge |  | Center | Center | Edge |
| VSAT antenna elevation |  | NF | 90 | 35 | 35 |  | 90 | 35 | 35 |
| Array 45x45 | LEO | 6 | -126.6 | -121.8 | -121.8 |  | -127 | -122 | -122 |
| Array 45x45 | GEO | 6 | -126.6 | -121.8 | -121.8 |  | -127 | -122 | -122 |
| Array 78x78 | LEO | 6 | -131.4 | -126.6 | -126.6 |  | -131 | -127 | -127 |
| Array 78x78 | GEO | 6 | -131.4 | -126.6 | -126.6 |  | -131 | -127 | -127 |
| Parabolic 60cm | LEO | 2.5 | -129.9 | -129.9 | -129.9 |  | -130 | -130 | -130 |
| Parabolic 60cm | GEO | 2.5 | -129.9 | -129.9 | -129.9 |  | -130 | -130 | -130 |

Observation 2: Based on the used assumptions for VSAT antenna parameters and noise figure the minimum EIS Reference Sensitivity for 50MHz RMC  is -121.8dBm, which can be used as is (Reference sensitivity is often defined with granularity of 0.1dB) or rounded to closest dB value of -122 or -121dBm. For Ka-band the 50MHz EIS Reference Sensitivity requirement is -122dBm. It’s to be noted that actual vendor declared EISREFSENS_50M is an integer value as already defined in 38.101-5 sub clause 10.3.2.

Observation 3: As the satellite output power scales with the bandwidth of the transmitted signal the requirements for other channel bandwidths (including the 15kHz SCS 10MHz Reference Measurement Channel) can be derived using a scaling factor that is calculate using the transmission bandwidths.

Observation 4: Transmission bandwidth for the 50MHz 120kHz SCS RMC is 66·12·0.06= 47.52MHz. Transmission bandwidth for the 10MHz 15kHz SCS RMC is 52·12·0.015= 9.36MHz and hence the 10MHz EIS Reference Sensitivity requirement should be 10·log10(47.52/9.36)= -7.06dB lower than the 50MHz requirement.

Observation 5: Based on the used assumptions for VSAT antenna parameters and noise figure and above mentioned scaling the minimum EIS Reference Sensitivity for 10MHz reference channel is EIS Reference Sensitivity for 50MHz RMC -7.06= -128.8dBm.

Observation 6: The proposed scaling approach is not a new, but already used in the existing 38.101-5 Table Table 10.3.2-1: OTA reference sensitivity requirement for NTN VSAT, where the reference bandwidth is indicated as follows: NOTE 1: The “factor” represents the normalized factor to scale EIS for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz. Due to the already defined scaling there is no need to define separate Reference Sensitivity Requirement for different numerology and CBW.

Table 8. Summary for EIS signal level in selected cases A, B and C

| Received 50MHz signal power at the antenna input (dBm) |  |  | Case A | Case B | Case C |
| --- | --- | --- | --- | --- | --- |
| VSAT location within DL beam |  | VSAT | Center | Center | Edge |
| VSAT antenna elevation |  | NF | 90 | 35 | 35 |
| Array 45x45 | LEO | 6 | -118.3 | -122.5 | -125.5 |
| Array 45x45 | GEO | 6 | -117.9 | -118.4 | -121.4 |
| Array 78x78 | LEO | 6 | -118.3 | -122.5 | -125.5 |
| Array 78x78 | GEO | 6 | -117.9 | -118.4 | -121.4 |
| Parabolic 60cm | LEO | 2.5 | -118.3 | -122.5 | -125.5 |
| Parabolic 60cm | GEO | 2.5 | -117.9 | -118.4 | -121.4 |

Observation 7: Based on the used assumptions for satellite parameters and geometries the maximum received EIS power for the 50MHz signal is -117.9dBm.

Observation 8: As the satellite output power scales with the frequency as already noted in Observation 3 the value for 50MHz CBW of -117.9dBm needs to be scaled to take into account the maximum supported CBW. For FR1 numerology with maximum CBW of 100MHz and for FR2 numerology with maximum CBW of 400MHz 3 and 9dB needs to be added correspondingly.

Observation 9: For FR2 numerology bands with maximum CBW of 400MHz the maximum input level is -117.9 +9= -108.9dBm. For Ka-band with similar CBW options 50, 100, 200 and 400MHz the Maximum input level requirement is -109.6dBm.

Observation 10: For FR1 numerology bands with maximum CBW of 100MHz the maximum input level is -117.9 +3= -114.9dBm.

Table 9. Margin to -1dB SNR in selected cases A, B and C

| Margin to -1dB SNR (dB) |  |  | Case A | Case B | Case C |
| --- | --- | --- | --- | --- | --- |
| VSAT location within DL beam |  | VSAT | Center | Center | Edge |
| VSAT antenna elevation |  | NF | 90 | 35 | 35 |
| Array 45x45 | LEO | 6 | 8.3 | -0.7 | -3.7 |
| Array 45x45 | GEO | 6 | 8.8 | 3.4 | 0.4 |
| Array 78x78 | LEO | 6 | 13.1 | 4.1 | 1.1 |
| Array 78x78 | GEO | 6 | 13.5 | 8.2 | 5.2 |
| Parabolic 60cm | LEO | 2.5 | 11.5 | 7.4 | 4.4 |
| Parabolic 60cm | GEO | 2.5 | 12.0 | 11.4 | 8.4 |

Observation 11: There is a negative margin in some of the analyzed cases. This this does not mean that system does not work in those cases, but it indicates that under such conditions the system needs to be operated differently, either by using signal with stronger coding or by increasing the output power density at the satellite end of the link. Alternatively it is also possible to require VSAT to have lower NF or higher antenna gain, which would enable introduction of more stringent Reference Sensitivity Requirements.

Table 10 shows the received signal SNR in the analyzed cases.

Table 10. Received signal SNR in selected cases A, B and C

| SNR (dB) |  |  | Case A | Case B | Case C |
| --- | --- | --- | --- | --- | --- |
| VSAT location within DL beam |  | VSAT | Center | Center | Edge |
| VSAT antenna elevation |  | NF | 90 | 35 | 35 |
| Array 45x45 | LEO | 6 | 7.3 | -1.7 | -4.7 |
| Array 45x45 | GEO | 6 | 7.8 | 2.4 | -0.6 |
| Array 78x78 | LEO | 6 | 12.1 | 3.1 | 0.1 |
| Array 78x78 | GEO | 6 | 12.5 | 7.2 | 4.2 |
| Parabolic 60cm | LEO | 2.5 | 10.5 | 6.4 | 3.4 |
| Parabolic 60cm | GEO | 2.5 | 11.0 | 10.4 | 7.4 |

Observation 12: The maximum SNR in the analyzed cases is 12.5dB. This implies that under existing assumptions support for modulation order higher than 16QAM is not meaningful. However, similarly as in observation 11 it is possible to operate the system differently, which may enable use of 64QAM modulation as is also done for Ka-band. Maximum input power is defined for QPSK, 16QAM and 64QAM modulated RMC, but VSAT is naturally only required to meet the RF requirements for the modulations that it can support.

## How to capture the requirements into the specification

Observation 13: Existing [4] TS 38.101-5 Clause 10 requirements for Radiated receiver characteristics is already structured in the way that it neatly introduces the requirements for multiple bands, different VSAT types , Fixed and Mobile, with mechanical and electronic steering antenna. However, further modifications are needed to introduce the requirements for multiple bands that supporting different numerologies.

Following is proposed:

Proposal 2: For OTA Reference Sensitivity see 2.6.1 OTA Reference Sensitivity and/or Annex A: Draft CR.

Proposal 3: For proposal 2 it’s further proposed to discuss the need for OCGN Pattern for Reference sensitivity case where the SNR for the input signal should be low.

Proposal 4: For OTA Maximum Input Level see 2.6.2 OTA Maximum input level and/or Annex A: Draft CR.

Proposal 5: It’s observed that R/Code Rate for 16QAM and 64QAM Reference Measurement Channels defined in Annex 3.2 is 0.48, while R= 1/2 is defined in tables 10.4.2-1 and 10.4.3-1. Although the delta is small discussion on alignment between the definition of the RMC and RF requirement is proposed.

Observation 14: Table formats for both 10.4.2 Minimum requirement for Mobiles VSAT and 10.4.3 Minimum requirement for Fixed VSAT have been modified although there are no new requirements introduced in this proposal for the Fixed VSAT on bands n512, n511 and n510. This modification is done to align the table formats, to improve the readability and future compatibility of the specification.

## OTA Reference Sensitivity

< START OF CHANGE #1 >

10.3	OTA reference sensitivity level

10.3.1	General

The OTA REFSENS requirement is a directional requirement and is intended to ensure the minimum OTA reference sensitivity level at the centre of the quiet zone in the RX beam peak direction. The OTA reference sensitivity power level EISREFSENS is the minimum mean power received over the air at the RIB, at which the throughput shall meet or exceed the requirements for a specified reference measurement channel.

10.3.2	Minimum requirement

The throughput shall be ≥ 95 % of the maximum throughput of the QPSK reference measurement channels as specified in Annexes A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1) with peak reference sensitivity specified in Table 10.3.2-1. EISREFSENS_50M declared by the vendor is an integer value in the range specified in Table 10.3.2-2 for different types of NTN VSAT. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle).

The EIS of Rx beam peak direction should be verified within the declared minimum elevation angle supported for receiving. The steered beam peak directions can be achieved by mechanical steering and/or electronic steering according to VSAT Type. Where the supported minimum elevation angle shall be declared by manufacturer and within the range of 3°≤minimum elevation angle≤75°, and it can be expressed as (90-θ) if the coordinate systems in Figure 10.3.2-1 below is taken as an example.

Figure 10.3.2-1 Example measurement grid for EIS with the declared supported minimum elevation angle

Table 10.3.2-1: OTA reference sensitivity requirement for NTN VSAT

| NTN VSAT channel bandwidth (MHz) | UL/DL RB allocation | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- |
| 5, 10, 15, 20, 25, [30], 35, 50, 100, 200, 400 | Full RB allocation NRB as specified in sub-clause 5.3.2 | EISREFSENS_50MHz + 10log10(NRB x SCS x 12 / factor) (NOTE 1) |
| NOTE 1: The “factor” represents the normalized factor to scale EIS for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz. |  |  |

Table 10.3.2-2: The range of EISREFSENS_50MHz declared by vendor per NTN VSAT

| Operating band | NTN VSAT class | NTN VSAT type | SCS (kHz) | The range of EISREFSENS_50MHz (dBm) |
| --- | --- | --- | --- | --- |
| n512, n511 | Fixed VSAT | 1, 2 | 60, 120 | ≤ -122 |
|  |  | 3 | 60, 120 | ≤ -115.6 |
| n512, n511, n510 | Mobile VSAT | 4, 5 | 60, 120 | ≤ -122 |
| n248, n247 | Mobile VSAT | 4, 5 | 15, 30 | [≤ -122] |
| n509, n508 | Mobile VSAT | 4, 5 | 120 | [≤ -122] |

< END OF CHANGE #1 >

## OTA Maximum input level

< START OF CHANGE #2 >

10.4	Maximum input level

10.4.1	General

The maximum input level is defined as the maximum mean power, for which the throughput shall meet or exceed the minimum requirements for the specified reference measurement channel.

The maximum input level is defined as a directional requirement. The requirement is verified in beam locked mode in the direction where peak gain is achieved.

10.4.2	Minimum requirement for Mobile VSAT

For mobile VSAT, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A with one sided dynamic OCNG Pattern for the DL-signal as described in Annex A.5.1.1 with parameters specified in Table 10.4.2-1. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 10.4.2-1: Mobile VSAT maximum input level

| Rx Parameter | Band | VSAT type | SCS (kHz) | Units | Channel bandwidth (MHz) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5, 10, 15, 20, 25, 30, 35, 50, 100 | 50, 100, 200, 400 |
| Power in transmission bandwidth configuration | n512 n511 n510 | 4, 5 | 60, 120 | dBm | Not applicable | -109.6 (NOTE 2, 3, 4) |
|  | n248 n247 | 4, 5 | 15, 30 | dBm | [-114.9] (NOTE 5) | Not applicable |
|  | n509 n508 | 4, 5 | 120 | dBm | Not applicable | [-108.9] (NOTE 2, 3, 4) |
| NOTE 1: The transmitter shall be set to [4 dB] below the PUMAX,f,c as defined in sub-clause 9.2.3, with uplink configuration specified in Table 10.3.2-1. NOTE 2: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: QPSK with R=1/3 NOTE 3: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: 16QAM with R=1/2 NOTE 4: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: 64QAM with R=1/2 NOTE 5: Reference measurement channel is specified in Annex A.3.2.1.1: QPSK with R= 0.3 and 16 QAM with R= 0.48 |  |  |  |  |  |  |

10.4.3	Minimum requirement for Fixed VSAT

For fixed VSAT, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A (with one sided dynamic OCNG Pattern for the DL-signal as described in Annex A.5.1.1) with parameters specified in Table 10.4.3.-1. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 10.4.3-1: Fixed VSAT maximum input level

| Rx Parameter | Band | VSAT type | SCS (kHz) | Units | Channel bandwidth (MHz) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5, 10, 15, 20, 25, 30, 35, 50, 100 | 50, 100, 200, 400 |
| Power in transmission bandwidth configuration | n512 n511 n510 | 4, 5 | 60, 120 | dBm | Not applicable | -109.6 (NOTE 2, 3, 4) |
| NOTE 1: The transmitter shall be set to [4 dB] below the PUMAX,f,c as defined in sub-clause 9.2.3, with uplink configuration specified in Table 10.3.2-1. NOTE 2: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: QPSK with R=1/3 NOTE 3: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: 16QAM with R=1/2 NOTE 4: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: 64QAM with R=1/2 |  |  |  |  |  |  |

< END OF CHANGE #2 >

# Conclusions

This document discusses the VSAT reference sensitivity and maximum received input power requirement and proposes a draft CR to capture the requirement into TS 38.101-5 [4].

Observation 1: Currently PDSCH QPSK RMC is defined only for 10MHz channel with 15kHz SCS, for 50, 100 and 200MHz with 60kHz SCS and for 50, 100, 200 and 400MHz with 120kHz SCS. There are no RMC defined for 30kHz SCS.

Proposal 1: It’s proposed to use the existing RMCs for defining the EIS reference sensitivity requirements for Ku-band VSAT supporting FR1 and/or FR2 numerologies. New QPSK reference channels with different CBW and SCS can be introduced at later point in time or during this WI but are not mandatory to complete the Ku-band WI.

Observation 2: Based on the used assumptions for VSAT antenna parameters and noise figure the minimum EIS Reference Sensitivity for 50MHz RMC  is -121.8dBm, which can be used as is (Reference sensitivity is often defined with granularity of 0.1dB) or rounded to closest dB value of -122 or -121dBm. For Ka-band the 50MHz EIS Reference Sensitivity requirement is -122dBm. It’s to be noted that actual vendor declared EISREFSENS_50M is an integer value as already defined in 38.101-5 sub clause 10.3.2.

Observation 3: As the satellite output power scales with the bandwidth of the transmitted signal the requirements for other channel bandwidths (including the 15kHz SCS 10MHz Reference Measurement Channel) can be derived using a scaling factor that is calculate using the transmission bandwidths.

Observation 4: Transmission bandwidth for the 50MHz 120kHz SCS RMC is 66·12·0.06= 47.52MHz. Transmission bandwidth for the 10MHz 15kHz SCS RMC is 52·12·0.015= 9.36MHz and hence the 10MHz EIS Reference Sensitivity requirement should be 10·log10(47.52/9.36)= -7.06dB lower than the 50MHz requirement.

Observation 5: Based on the used assumptions for VSAT antenna parameters and noise figure and above mentioned scaling the minimum EIS Reference Sensitivity for 10MHz reference channel is EIS Reference Sensitivity for 50MHz RMC -7.06= -128.8dBm.

Observation 6: The proposed scaling approach is not a new, but already used in the existing 38.101-5 Table Table 10.3.2-1: OTA reference sensitivity requirement for NTN VSAT, where the reference bandwidth is indicated as follows: NOTE 1: The “factor” represents the normalized factor to scale EIS for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz. Due to the already defined scaling there is no need to define separate Reference Sensitivity Requirement for different numerology and CBW.

Observation 7: Based on the used assumptions for satellite parameters and geometries the maximum received EIS power for the 50MHz signal is -117.9dBm.

Observation 8: As the satellite output power scales with the frequency as already noted in Observation 3 the value for 50MHz CBW of -117.9dBm needs to be scaled to take into account the maximum supported CBW. For FR1 numerology with maximum CBW of 100MHz and for FR2 numerology with maximum CBW of 400MHz 3 and 9dB needs to be added correspondingly.

Observation 9: For FR2 numerology bands with maximum CBW of 400MHz the maximum input level is -117.9 +9= -108.9dBm. For Ka-band with similar CBW options 50, 100, 200 and 400MHz the Maximum input level requirement is -109.6dBm.

Observation 10: For FR1 numerology bands with maximum CBW of 100MHz the maximum input level is -117.9 +3= -114.9dBm.

Observation 11: There is a negative margin in some of the analyzed cases. This this does not mean that system does not work in those cases, but it indicates that under such conditions the system needs to be operated differently, either by using signal with stronger coding or by increasing the output power density at the satellite end of the link. Alternatively it is also possible to require VSAT to have lower NF or higher antenna gain, which would enable introduction of more stringent Reference Sensitivity Requirements.

Observation 12: The maximum SNR in the analyzed cases is 12.5dB. This implies that under existing assumptions support for modulation order higher than 16QAM is not meaningful. However, similarly as in observation 11 it is possible to operate the system differently, which may enable use of 64QAM modulation as is also done for Ka-band. Maximum input power is defined for QPSK, 16QAM and 64QAM modulated RMC, but VSAT is naturally only required to meet the RF requirements for the modulations that it can support.

Observation 13: Existing [4] TS 38.101-5 Clause 10 requirements for Radiated receiver characteristics is already structured in the way that it neatly introduces the requirements for multiple bands, different VSAT types , Fixed and Mobile, with mechanical and electronic steering antenna. However, further modifications are needed to introduce the requirements for multiple bands that supporting different numerologies.

Following is proposed:

Proposal 2: For OTA Reference Sensitivity see 2.6.1 OTA Reference Sensitivity and/or Annex A: Draft CR.

Proposal 3: For proposal 2 it’s further proposed to discuss the need for OCGN Pattern for Reference sensitivity case where the SNR for the input signal should be low.

Proposal 4: For OTA Maximum Input Level see 2.6.2 OTA Maximum input level and/or Annex A: Draft CR.

Proposal 5: It’s observed that R/Code Rate for 16QAM and 64QAM Reference Measurement Channels defined in Annex 3.2 is 0.48, while R= 1/2 is defined in tables 10.4.2-1 and 10.4.3-1. Although the delta is small discussion on alignment between the definition of the RMC and RF requirement is proposed.

Observation 14: Table formats for both 10.4.2 Minimum requirement for Mobiles VSAT and 10.4.3 Minimum requirement for Fixed VSAT have been modified although there are no new requirements introduced in this proposal for the Fixed VSAT on bands n512, n511 and n510. This modification is done to align the table formats, to improve the readability and future compatibility of the specification.

# References

[1] R4-2504720, Way Forward for [114bis][314] NR_NTN_Ku_Band_UE_SAN_RF, CHTTL, TSG-RAN WG4 Meeting #114bis, April 2025

[2] R4-2504723, Way Forward for [114bis][313] NR_NTN_Ku_Band_Coexistence, Intelsat, TSG-RAN WG4 Meeting #114bis, April 2025

[3] R4-2502315, Co-existence simulation assumptions for NR NTN Ku band (Running document), Ericsson, TSG-RAN WG4 Meeting #114, Feb 2025

[4] 3GPP TS 38.101-5, User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements (Release 18)

# Annex A: Draft CR

3GPP TSG-RAN4 Meeting #115	R4-2507099

Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | Draft | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for Ku-band VSAT OTA Reference Sensitivity and Maximum Input Level |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ku_bands-Core |  |  |  |  |  | Date: |  |  | 2025-05-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add OTA Reference Sensitivity and Maximum Input Level requirements for Ku-band VSAT |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For OTA Reference Sensitivity Add requirements for bands n248 & n247 and n509 & n508 into tables 10.3.2-1 and 10.3.2-2 - For Maximum Input Level Add requirements for bands n248 & n247 and n509 & n508 into table 10.4.2-1. Modify the table format of table 10.4.3-1 to align with modifications made to table 10.4.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | OTA Reference Sensitivity and Maximum Input Level requirements are missing for Ku-band VSAT |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.2, 10.4.2, 10.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

# 10	Radiated receiver characteristics

## 10.1	General

Unless otherwise stated, the receiver characteristics are specified over the air (OTA) at the RIB for Ka bands fixed and mobile VSAT. The power level for all DL wanted signals and interference is defined assuming a 0 dBi reference antenna located at the center of the quiet zone.

## 10.2	Polarization characteristics

The minimum requirements on the receiver characteristics apply under either LHCP (Left Hand Circular Polarization) or RHCP (Right Hand Circular Polarization) or Linear Polarization.

## 10.3	OTA reference sensitivity level

### 10.3.1	General

The OTA REFSENS requirement is a directional requirement and is intended to ensure the minimum OTA reference sensitivity level at the centre of the quiet zone in the RX beam peak direction. The OTA reference sensitivity power level EISREFSENS is the minimum mean power received over the air at the RIB, at which the throughput shall meet or exceed the requirements for a specified reference measurement channel.

### 10.3.2	Minimum requirement

The throughput shall be ≥ 95 % of the maximum throughput of the QPSK reference measurement channels as specified in Annexes A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1) with peak reference sensitivity specified in Table 10.3.2-1. EISREFSENS_50M declared by the vendor is an integer value in the range specified in Table 10.3.2-2 for different types of NTN VSAT. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle).

The EIS of Rx beam peak direction should be verified within the declared minimum elevation angle supported for receiving. The steered beam peak directions can be achieved by mechanical steering and/or electronic steering according to VSAT Type. Where the supported minimum elevation angle shall be declared by manufacturer and within the range of 3°≤minimum elevation angle≤75°, and it can be expressed as (90-θ) if the coordinate systems in Figure 10.3.2-1 below is taken as an example.

Figure 10.3.2-1 Example measurement grid for EIS with the declared supported minimum elevation angle

Table 10.3.2-1: OTA reference sensitivity requirement for NTN VSAT

| NTN VSAT channel bandwidth (MHz) | UL/DL RB allocation | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- |
| 5, 10, 15, 20, 25, [30], 35, 50, 100, 200, 400 | Full RB allocation NRB as specified in sub-clause 5.3.2 | EISREFSENS_50MHz + 10log10(NRB x SCS x 12 / factor) (NOTE 1) |
| NOTE 1: The “factor” represents the normalized factor to scale EIS for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz. |  |  |

Table 10.3.2-2: The range of EISREFSENS_50MHz declared by vendor per NTN VSAT

| Operating band | NTN VSAT class | NTN VSAT type | SCS (kHz) | The range of EISREFSENS_50MHz (dBm) |
| --- | --- | --- | --- | --- |
| n512, n511 | Fixed VSAT | 1, 2 | 60, 120 | ≤ -122 |
|  |  | 3 | 60, 120 | ≤ -115.6 |
| n512, n511, n510 | Mobile VSAT | 4, 5 | 60, 120 | ≤ -122 |
| n248, n247 | Mobile VSAT | 4, 5 | 15, 30 | [≤ -122] |
| n509, n508 | Mobile VSAT | 4, 5 | 120 | [≤ -122] |

## 10.4	Maximum input level

### 10.4.1	General

The maximum input level is defined as the maximum mean power, for which the throughput shall meet or exceed the minimum requirements for the specified reference measurement channel.

The maximum input level is defined as a directional requirement. The requirement is verified in beam locked mode in the direction where peak gain is achieved.

### 10.4.2	Minimum requirement for Mobile VSAT

For mobile VSAT, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A with one sided dynamic OCNG Pattern for the DL-signal as described in Annex A.5.1.1 with parameters specified in Table 10.4.2-1. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 10.4.2-1: Mobile VSAT maximum input level

| Rx Parameter | Band | VSAT type | SCS (kHz) | Units | Channel bandwidth (MHz) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5, 10, 15, 20, 25, 30, 35, 50, 100 | 50, 100, 200, 400 |
| Power in transmission bandwidth configuration | n512 n511 n510 | 4, 5 | 60, 120 | dBm | Not applicable | -109.6 (NOTE 2, 3, 4) |
|  | n248 n247 | 4, 5 | 15, 30 | dBm | [-114.9] (NOTE 5) | Not applicable |
|  | n509 n508 | 4, 5 | 120 | dBm | Not applicable | [-108.9] (NOTE 2, 3, 4) |
| NOTE 1: The transmitter shall be set to [4 dB] below the PUMAX,f,c as defined in sub-clause 9.2.3, with uplink configuration specified in Table 10.3.2-1. NOTE 2: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: QPSK with R=1/3 NOTE 3: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: 16QAM with R=1/2 NOTE 4: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: 64QAM with R=1/2 NOTE 5: Reference measurement channel is specified in Annex A.3.2.1.1: QPSK with R= 0.3 and 16 QAM with R= 0.48 |  |  |  |  |  |  |

### 10.4.3	Minimum requirement for Fixed VSAT

For fixed VSAT, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A with one sided dynamic OCNG Pattern for the DL-signal as described in Annex A.5.1.1 with parameters specified in Table 10.4.3.-1. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 10.4.3-1: Fixed VSAT maximum input level

| Rx Parameter | Band | VSAT type | SCS (kHz) | Units | Channel bandwidth (MHz) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5, 10, 15, 20, 25, 30, 35, 50, 100 | 50, 100, 200, 400 |
| Power in transmission bandwidth configuration | n512 n511 n510 | 4, 5 | 60, 120 | dBm | Not applicable | -109.6 (NOTE 2, 3, 4) |
| NOTE 1: The transmitter shall be set to [4 dB] below the PUMAX,f,c as defined in sub-clause 9.2.3, with uplink configuration specified in Table 10.3.2-1. NOTE 2: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: QPSK with R=1/3 NOTE 3: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: 16QAM with R=1/2 NOTE 4: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: 64QAM with R=1/2 |  |  |  |  |  |  |

< END OF CHANGE #1 >