3GPP TSG-RAN WG4 Meeting #115                                          R4-2507100

# Malta, 19th – 23rd of May 2025

Title: 	Updates on SAN RF requirements for NTN NB-IoT TDD

Agenda Item:	7.33.4 – SAN RF requirements

Release:	Rel-19

Source: 	THALES, Iridium Satellite LLC

Document for:	Discussion

# WID information

A new WID Introduction of IoT-NTN TDD mode (RP-242415, New WID on introduction of IoT-NTN TDD mode, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024) was approved in RAN#105. That WID has been revised in RAN#106, as the study phase had been completed. Then, a work plan for Revised WID on Introduction of IoT-NTN TDD mode (RP-243293, Revised WID on introduction of IoT-NTN TDD mode, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024) is proposed for RAN 1, 2, and 4.

## Core part WI

The work item aims to specify enhancements for NB-IoT NTN to enable NTN operation with a NB-IoT TDD mode leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures for the NB-IoT operation in the targeted unpaired MSS allocated band (TN deployment is not expected in this band). The feature is not intended to be applicable to existing 3GPP bands.

The work objectives assume the following:

- LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763)

- Target the 1616-1626.5 MHz MSS allocated band

- Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT)

- Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with N=9 for the target MSS allocated band.

This work item includes the following objectives:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including:

  - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2]

      - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period. RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band.

  - Other necessary impacts on higher layers [RAN2]

  - RRM and RF core requirements [RAN4]

- Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4].

  - Specify band numbering

  - Specify SAN and UE RF characteristics

  - Specify DL and UL channelization.

  - Specify channel bandwidth as 200 kHz

  - Note1: No NTN-NTN coexistence study needed.

  - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands

Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements)

## Performance part WI

Specify the following:

- RRM performance requirements, as needed [RAN4]

- Necessary SAN and UE demodulation requirements [RAN4]

- SAN conformance testing requirements [RAN4]

## Current Work Plan

Current WP from R4-2502291 (from RAN4#114):

| RAN#107 (Mar. 2025) | RAN1#121 | 0.5 | - Conclude on the solutions for the TDD support in NB IoT NTN - Further discuss (if needed) related RRC parameters and send LS to RAN2 - Finalize RAN1 CRs |
| --- | --- | --- | --- |
|  | RAN2#130 | 0.5 | - Complete the definition and RRC configuration of signaling of the periodic pattern, and associated UE procedures (if any) - Conclude support of TDD mode in NB IoT NTN - Discuss stage-2 design of TDD support in NB IoT NTN - Discuss RAN2 CRs |
|  | RAN4#115 | 0.5+0.25 | - Continue discussion on the UE AMPR, UE frequency error requirement, - Continue discussion on repetition number to be considered (associated with D/U=8 for 8 ms Rx/Tx time and N=9 for 90 ms periodicity) for both UE and SAN, - Continue discussion (at least) FRC for REFSENS, in-channel selectivity, dynamic range, and related parameters such as number of tones, Tx/Rx time, related SNR target for specific FRC configuration, - Derive related values for the at least for previous mentioned requirements: REFSENS, in-channel selectivity, dynamic range, etc., - Start discussion on UE and SAN CRs for RF part and SAN conformance testing, - Start CR split to prepare Draft CRs for the Core requirements |

Previous SAN & UE agreements from RAN4#114 were described in WF R4-2502292 (“Way Forward for [114][318] IoT_NTN_TDD_UE_SAN_RF”, THALES) and from RAN4#114-bis in WF R4-2504719 (“Way Forward for [114bis][316] IoT_NTN_TDD_UE_SAN_RF”).

# SAN Requirements for IoT_NTN_TDD

In this contribution we would like to enhance several SAN decisions previously taken at RAN4#114 & RAN4#114-bis.

In RAN4#114-bis it was agreed (among other proposals):

Sub-topic 3-3: REFSENS in TS 36.108

Issue 3-3-1: 15 kHz tone vs 3.75 kHz tone

Agreement:

- Option 1: Keep single tone 15 kHz (with different variants) and remove 3.75 kHz single-tone configuration for SAN reference sensitivity for NTN TDD NB-IoT in TS 36.108.

- Option 2: 3.75 kHz can be kept if companies are bringing contributions to support this configuration with a Tx time below 8 ms.

At RAN4#115 we propose:

Proposal 1: RAN4 to agree with keeping single tone 15 kHz (with different variants) and remove 3.75 kHz single-tone configuration for SAN reference sensitivity for NTN TDD NB-IoT in TS 36.108 (Option 1/REFSENS in TS 36.108).

In RAN4#114-bis it was agreed (among other proposals):

Issue 3-3-2: New A14-1 variants (FRC REFSENS)

Agreement:

- Option 1: Consider at least one of the new A14-1 configurations below for NTN TDD NB-IoT:

Table A.14-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A14-1 for FDD | A14-2 for FDD | A14-1 new Option1 for TDD | A14-1 new Option2 for TDD | A14-1 new Option3 for TDD |
| --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 kHz | 15 kHz | 15 kHz |
| Number of tone | 1 | 1 | 1 | 1 | 1 |
| Diversity | No | No | No | No | No |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK | [π/4] QPSK | [π/4] QPSK |
| Frequency offset | 0 | 0 | 0 | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 | 4 ms | 4 ms | 4 ms |
| Number of NPUSCH repetition | 1 | 1 | 1 | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 | 0 / 0 | 3 / 3 | 2 / 1 |
| Payload size (bits) | 32 | 32 | 16 bits | 40 bits | 24 bits |
| Allocated resource unit | 2 | 2 | 1 | 1 | 1 |
| Code rate (target) | 1/3 | 1/3 | ⅓ | ⅓ | ⅓ |
| Code rate (effective) | 0.29 | 0.29 | 0.42 | 0.33 | 0.25 |
| Transport block CRC (bits) | 24 | 24 | 24 bits | 24 bits | 24 bits |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 96 | 192 | 192 |
| Total symbols per resource unit | 96 | 96 | 96 | 96 | 96 |
| Tx time (ms) | 16 | 64 | 8 ms | 8 ms | 8 ms |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |

- Option 2: 3.75 kHz can be kept for NTN TDD NB-IoT if companies are bringing contributions to support this configuration with a Tx time below 8 ms.

At RAN4#115 we propose:

Proposal 2: RAN4 to consider (at least) new FRC configuration A14-1 Option 1 below for NTN TDD NB-IoT:

Table A.14-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A14-1 for FDD | A14-2 for FDD | A14-1 new Option1 for TDD | A14-1 new Option2 for TDD | A14-1 new Option3 for TDD |
| --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 kHz | 15 kHz | 15 kHz |
| Number of tone | 1 | 1 | 1 | 1 | 1 |
| Diversity | No | No | No | No | No |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK | [π/4] QPSK | [π/4] QPSK |
| Frequency offset | 0 | 0 | 0 | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 | 4 ms | 4 ms | 4 ms |
| Number of NPUSCH repetition | 1 | 1 | 1 | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 | 0 / 0 | 3 / 3 | 2 / 1 |
| Payload size (bits) | 32 | 32 | 16 bits | 40 bits | 24 bits |
| Allocated resource unit | 2 | 2 | 1 | 1 | 1 |
| Code rate (target) | 1/3 | 1/3 | ⅓ | ⅓ | ⅓ |
| Code rate (effective) | 0.29 | 0.29 | 0.42 | 0.33 | 0.25 |
| Transport block CRC (bits) | 24 | 24 | 24 bits | 24 bits | 24 bits |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 96 | 192 | 192 |
| Total symbols per resource unit | 96 | 96 | 96 | 96 | 96 |
| Tx time (ms) | 16 | 64 | 8 ms | 8 ms | 8 ms |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |

In RAN4#114-bis it was agreed (among other proposals):

Issue 3-3-3: SNR targets for new A14-1 variants (FRC REFSENS)

Agreement:

- Option 1: Consider the SNR target values corresponding to the new NTN TDD NB-IoT A14-1 configurations:

Table A.14-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A14-1 for FDD | A14-2 for FDD | A14-1 new Option1 for TDD | A14-1 new Option2 for TDD | A14-1 new Option3 for TDD |
| --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 kHz | 15 kHz | 15 kHz |
| Number of tone | 1 | 1 | 1 | 1 | 1 |
| Diversity | No | No | No | No | No |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK | [π/4]QPSK | [π/4]QPSK |
| Frequency offset | 0 | 0 | 0 | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 | 4 ms | 4 ms | 4 ms |
| Number of NPUSCH repetition | 1 | 1 | 1 | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 | 0 / 0 | 3 / 3 | 2 / 1 |
| Payload size (bits) | 32 | 32 | 16 bits | 40 bits | 24 bits |
| Allocated resource unit | 2 | 2 | 1 | 1 | 1 |
| Code rate (target) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 | 0.42 | 0.33 | 0.25 |
| Transport block CRC (bits) | 24 | 24 | 24 bits | 24 bits | 24 bits |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 96 | 192 | 192 |
| Total symbols per resource unit | 96 | 96 | 96 | 96 | 96 |
| Tx time (ms) | 16 | 64 | 8 ms | 8 ms | 8 ms |
| SNR target value (dB) | -2.1 dB as per TR 36.802 | as per TR 36.802 | -1,17 dB | 0,54 dB | -0,61 dB |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |

- Option 2: 3.75 kHz can be kept for NTN TDD NB-IoT if companies are bringing SNR values to support this configuration with a Tx time below 8 ms.

At RAN4#115 we propose:

Proposal 3: RAN4 to consider (at least) new SNR target value corresponding to the new configuration NTN TDD NB-IoT A14-1 Option 1 below for NTN TDD NB-IoT:

Table A.14-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A14-1 for FDD | A14-2 for FDD | A14-1 new Option1 for TDD | A14-1 new Option2 for TDD | A14-1 new Option3 for TDD |
| --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 kHz | 15 kHz | 15 kHz |
| Number of tone | 1 | 1 | 1 | 1 | 1 |
| Diversity | No | No | No | No | No |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK | [π/4]QPSK | [π/4]QPSK |
| Frequency offset | 0 | 0 | 0 | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 | 4 ms | 4 ms | 4 ms |
| Number of NPUSCH repetition | 1 | 1 | 1 | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 | 0 / 0 | 3 / 3 | 2 / 1 |
| Payload size (bits) | 32 | 32 | 16 bits | 40 bits | 24 bits |
| Allocated resource unit | 2 | 2 | 1 | 1 | 1 |
| Code rate (target) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 | 0.42 | 0.33 | 0.25 |
| Transport block CRC (bits) | 24 | 24 | 24 bits | 24 bits | 24 bits |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 96 | 192 | 192 |
| Total symbols per resource unit | 96 | 96 | 96 | 96 | 96 |
| Tx time (ms) | 16 | 64 | 8 ms | 8 ms | 8 ms |
| SNR target value (dB) | -2.1 dB as per TR 36.802 | as per TR 36.802 | -1,17 dB | 0,54 dB | -0,61 dB |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |

In RAN4#114-bis it was agreed (among other proposals):

Issue 3-3-4: Equations and parameter values to compute REFSENS

Agreement: As starting point, the following equations and parameter values to compute REFSENS for TDD NTN NB-IoT SAN can be considered:

REFSENS [dBm] = -174 dBm/Hz noise density +10*LOG10(15*1000 Hz) bandwidth + SAN NF + IM + SNR_target,

where:

- SAN NF (Noise Figure):

  - NF LEO = 4.3 dB

  - NF GEO = 7.4 dB

- IM (Implementation Margin) = 2 dB

- (new TDD only) SNR_target values:

  - A14-1 new Option1 for TDD: -1.17dB;

  - A14-1 new Option2 for TDD: 0.54 dB;

  - A14-1 new Option3 for TDD: -0.61 dB,

At RAN4#115 we propose:

Proposal 4: The following equations and parameter values to compute REFSENS for TDD NTN NB-IoT SAN can be considered:

REFSENS [dBm] = -174 dBm/Hz noise density +10*LOG10(15*1000 Hz) bandwidth + SAN NF + IM + SNR_target,

where:

- SAN NF (Noise Figure):

  - NF LEO = 4.3 dB

  - NF GEO = 7.4 dB

- IM (Implementation Margin) = 2 dB

- (new TDD only) SNR_target values:

  - A14-1 new Option1 for TDD: -1.17dB;

  - A14-1 new Option2 for TDD: 0.54 dB;

  - A14-1 new Option3 for TDD: -0.61 dB,

In RAN4#114-bis it was agreed (among other proposals):

Issue 3-3-5: Resulted REFSENS values (different options)

Agreement: In accordance with new A14-1 configurations, RAN4 to select between following (one or more) REFSENS values for TDD NB-IoT NTN band 249 SAN specification:

- Option 1:

  - GEO: -124.0 dBm;

  - LEO: -127.1 dBm;

- Option 2:

  - GEO: -122.3 dBm;

  - LEO: -125.4 dBm;

- Option 3:

  - GEO: -123.4 dBm;

  - LEO: -126.5 dBm;

- Other options not precluded.

| Reference channel | A14-1 for FDD | A14-2 for FDD | A14-1 new Option1 for TDD | A14-1 new Option2 for TDD | A14-1 new Option3 for TDD |
| --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 kHz | 15 kHz | 15 kHz |
| Number of tone | 1 | 1 | 1 | 1 | 1 |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK | QPSK | QPSK |
| Tx time (ms) | 16 | 64 | 8 ms | 8 ms | 8 ms |
| SNR target value (dB) | -2.1 dB as per TR 36.802 | .. as per TR 36.802 | -1,17 dB | 0,54 dB | -0,61 dB |
| REFSENS (dBm) | GEO: -124.9 LEO: -128.0 (current) |  | GEO: -124.0 LEO: -127.1 (new proposed) | GEO: -122.3 LEO: -125.4 (new proposed) | GEO: -123.4 LEO: -126.5 (new proposed) |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |

At RAN4#115 we propose:

Proposal 5: In accordance with new A14-1 configurations, RAN4 to select between following REFSENS values for TDD NB-IoT NTN band 249 SAN specification:

- Option 1:

  - GEO: -124.0 dBm;

  - LEO: -127.1 dBm;

- Option 2:

  - GEO: -122.3 dBm;

  - LEO: -125.4 dBm;

- Option 3:

  - GEO: -123.4 dBm;

  - LEO: -126.5 dBm;

- Other options not precluded.

| Reference channel | A14-1 for FDD | A14-2 for FDD | A14-1 new Option1 for TDD | A14-1 new Option2 for TDD | A14-1 new Option3 for TDD |
| --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 kHz | 15 kHz | 15 kHz |
| Number of tone | 1 | 1 | 1 | 1 | 1 |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK | QPSK | QPSK |
| Tx time (ms) | 16 | 64 | 8 ms | 8 ms | 8 ms |
| SNR target value (dB) | -2.1 dB as per TR 36.802 | .. as per TR 36.802 | -1,17 dB | 0,54 dB | -0,61 dB |
| REFSENS (dBm) | GEO: -124.9 LEO: -128.0 (current) |  | GEO: -124.0 LEO: -127.1 (new proposed) | GEO: -122.3 LEO: -125.4 (new proposed) | GEO: -123.4 LEO: -126.5 (new proposed) |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |

In RAN4#114-bis it was agreed (among other proposals):

Issue 3-3-6: REFSENS (with updates from previous meeting RAN4#114)

Agreement:

- Option 1: For SAN TS 36.108, adapt FRC configuration with new TDD frame structure (maximum 8 ms transmission time), for example:

  - For conducted specifications:

For SAN supporting NTN NB-IoT operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.1 with parameters specified in Table 7.2.2-3 and 7.2.2-4 for SAN type 1-H in all FDD operating bands and specified in Table 7.2.2-5 and 7.2.2-6 for SAN type 1-H in all TDD operating bands.

Table 7.2.2-3: Reference sensitivity levels of SAN supporting NB-IoT operation in FDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -124.9 |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -130.9 |

Table 7.2.2-4: Reference sensitivity levels of SAN supporting NB-IoT operation in FDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -128.0 |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -134.0 |

Table 7.2.2-5: Reference sensitivity levels of SAN supporting NB-IoT operation in TDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 new Option | TBD |
| 200 | 3.75 | TBD in Annex A.14 | TBD |

Table 7.2.2-6: Reference sensitivity levels of SAN supporting NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 new Option | TBD |
| 200 | 3.75 | TBD in Annex A.14 | TBD |

  - For OTA specifications:

For SAN supporting standalone NB-IoT operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameter specified in Table 10.3.2-3 and Table 10.3.2-4 for FDD bands and Table 10.3.2-5 and Table 10.3.2-6 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

Table 10.3.2-3: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in FDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -124.9 - ΔOTAREFSENS |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -130.9 - ΔOTAREFSENS |

Table10.3.2-4: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in FDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -128.0 - ΔOTAREFSENS |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -134.0 - ΔOTAREFSENS |

Table 10.3.2-5: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in TDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 new Option | TBD- ΔOTAREFSENS |
| 200 | 3.75 | TBD in Annex A.14 | TBD - ΔOTAREFSENS |

Table10.3.2-6: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 new Option | TBD - ΔOTAREFSENS |
| 200 | 3.75 | TBD in Annex A.14 | TBD - ΔOTAREFSENS |

- Option 2: 3.75 kHz can be kept for NTN TDD NB-IoT REFSENS if companies are bringing contributions to support this configuration with a Tx time below 8 ms.

At RAN4#115 we propose:

Proposal 6: For SAN TS 36.108, adapt FRC configuration with new TDD frame structure (maximum 8 ms transmission time), for example:

  - For conducted specifications:

For SAN supporting NTN NB-IoT operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.1 with parameters specified in Table 7.2.2-3 and 7.2.2-4 for SAN type 1-H in all FDD operating bands and specified in Table 7.2.2-5 and 7.2.2-6 for SAN type 1-H in all TDD operating bands.

Table 7.2.2-3: Reference sensitivity levels of SAN supporting NB-IoT operation in FDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -124.9 |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -130.9 |

Table 7.2.2-4: Reference sensitivity levels of SAN supporting NB-IoT operation in FDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -128.0 |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -134.0 |

Table 7.2.2-5: Reference sensitivity levels of SAN supporting NB-IoT operation in TDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 new Option 1 in Annex A.14 | -124.0 |
| 200 | 3.75 | TBD in Annex A.14 | TBD |

Table 7.2.2-6: Reference sensitivity levels of SAN supporting NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 new Option 1 in Annex A.14 | -127.1 |
| 200 | 3.75 | TBD in Annex A.14 | TBD |

  - For OTA specifications:

For SAN supporting standalone NB-IoT operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameter specified in Table 10.3.2-3 and Table 10.3.2-4 for FDD bands and Table 10.3.2-5 and Table 10.3.2-6 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

Table 10.3.2-3: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in FDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -124.9 - ΔOTAREFSENS |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -130.9 - ΔOTAREFSENS |

Table10.3.2-4: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in FDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -128.0 - ΔOTAREFSENS |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -134.0 - ΔOTAREFSENS |

Table 10.3.2-5: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in TDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 new Option 1 in Annex A.14 | -124.0 - ΔOTAREFSENS |
| 200 | 3.75 | TBD in Annex A.14 | TBD - ΔOTAREFSENS |

Table10.3.2-6: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 new Option 1 in Annex A.14 | -127.1 - ΔOTAREFSENS |
| 200 | 3.75 | TBD in Annex A.14 | TBD - ΔOTAREFSENS |

In RAN4#114-bis it was agreed (among other proposals):

Sub-topic 3-4: Dynamic range in TS 36.108

- Issue 3-4-1: A15-1 & A15-2 configurations for dynamic range

Agreement:

- Option 1:

  - Reference channel A15-1 (with a 15kHz configuration) can be reused for NTN TDD IoT NTN dynamic range computation.

  - Reference channel A15-2 (with a 3.75kHz configuration) can be removed for NTN TDD IoT NTN dynamic range computation.

Table A.15-1 FRC parameters for NB-IoT dynamic range

| Reference channel | A15-1 for both FDD & TDD | A15-2 for FDD only |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Modulation | π/4 QPSK | π/4 QPSK |
| Diversity | No | No |
| Frequency offset | 0 | 0 |
| IMCS / ITBS | 7 / 7 | 7 / 7 |
| Payload size (bits) | 104 | 104 |
| Allocated resource units | 1 | 1 |
| Transport block CRC (bits) | 24 | 24 |
| Coding rate (target) | 2/3 | 2/3 |
| Coding Rate | 0.67 | 0.67 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks – C | 1 | 1 |
| Total symbols per resource unit | 96 | 96 |
| Total number of bits per resource unit | 192 | 192 |
| Tx time (ms) | 8 | 32 |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |

- Option 2:

  - Reference channel A15-1 (with a 15kHz configuration) can be reused for NTN TDD IoT NTN dynamic range computation.

  - If companies are providing a 3.75kHz configuration for Tx time below 8 ms, reference channel A15-2 can be kept for NTN TDD IoT NTN dynamic range computation.

At RAN4#115 we propose:

Proposal 7: RAN4 shall consider for NTN TDD IoT NTN dynamic range:

  - Reference channel A15-1 (with a 15kHz configuration) can be reused for NTN TDD IoT NTN dynamic range computation.

  - Reference channel A15-2 (with a 3.75kHz configuration) can be removed for NTN TDD IoT NTN dynamic range computation.

Table A.15-1 FRC parameters for NB-IoT dynamic range

| Reference channel | A15-1 for both FDD & TDD | A15-2 for FDD only |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Modulation | π/4 QPSK | π/4 QPSK |
| Diversity | No | No |
| Frequency offset | 0 | 0 |
| IMCS / ITBS | 7 / 7 | 7 / 7 |
| Payload size (bits) | 104 | 104 |
| Allocated resource units | 1 | 1 |
| Transport block CRC (bits) | 24 | 24 |
| Coding rate (target) | 2/3 | 2/3 |
| Coding Rate | 0.67 | 0.67 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks – C | 1 | 1 |
| Total symbols per resource unit | 96 | 96 |
| Total number of bits per resource unit | 192 | 192 |
| Tx time (ms) | 8 | 32 |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |

In RAN4#114-bis it was agreed (among other proposals):

- Issue 3-4-2: Equations and parameter values to compute dynamic range

Agreement: RAN4 to consider the following equations and parameter values to compute dynamic range for TDD NTN NB-IoT SAN:

- P_int = -174 dBm/Hz + 10 x log10 (BW) + NF + IoT;

- P_wan = -174 dBm/Hz + 10 x log10 (BW) + NF +IoT + SNR +IM;

with the following parameters:

  - SNR dynamic range NB-IoT = 4.8 dB (as per TR 36.802 and not -2,1 dB as for REFSENS);

  - IM=2.7 dB;

  - BW_Channel = 200 kHz for interferer;

  - BW_Channel =15 kHz for wanted signal;

  - IoT LEO=15 dB (IoT GEO =N/A);

  - NF LEO=4.3 dB (NF GEO=7.4 dB).

In RAN4#114-bis it was agreed (among other proposals):

- Issue 3-4-3: Dynamic range double-check

Agreement:

- Re-evaluate dynamic range for NB-IoT LEO case. RAN4 to double-check the dynamic range values for TDD NTN NB-IoT SAN since computed values are P_int=-101,7 dBm; P_want=-105,4 dBm for LEO (it seems there is an error also for FDD NTN NB-IoT SAN values for LEO, and GEO should not have been considered):

Table 7.3.2-3: Dynamic range of SAN supporting standalone NB-IoT operation (LEO class payload)

| SAN channel bandwidth (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWChannel | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | FRC A15-1 in Annex A.15 | -89.4 | -85.7 | AWGN |
| 200 | FRC A15-2 in Annex A.15 | -95.3 | -85.7 | AWGN |

- Further discuss if dynamic range for NB-IoT GEO case should be de-scoped or not, in Rel-17 (and should have been the case also for NB-IoT in Rel-18) it was decided not to define any dynamic range requirement for SAN GEO class.

Table 7.3.2-2: Dynamic range of SAN supporting standalone NB-IoT operation  (GEO class payload)

| SAN channel bandwidth (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWChannel | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | FRC A15-1 in Annex A.15 | -97.3 | -93.6 | AWGN |
| 200 | FRC A15-2 in Annex A.15 | -103.2 | -93.6 | AWGN |

At RAN4#115 we propose:

Proposal 8: RAN4 shall consider adding a new table for NTN TDD IoT NTN dynamic range for LEO:

Table 7.3.2-x: Dynamic range of SAN supporting standalone NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWChannel | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | FRC A15-1 in Annex A.15 | -105.4 | -101.7 | AWGN |
| 200 | FRC A15-2 in Annex A.15 | -95.3 | -85.7 | AWGN |

Proposal 9: RAN4 shall consider correcting previous NTN FDD IoT NTN dynamic range for LEO:

Table 7.3.2-3: Dynamic range of SAN supporting standalone NB-IoT operation in FDD (LEO class payload)

| SAN channel bandwidth (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWChannel | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | FRC A15-1 in Annex A.15 | [-89.4] | [-85.7] | AWGN |
| 200 | FRC A15-2 in Annex A.15 | [-95.3] | [-85.7] | AWGN |

Proposal 10: RAN4 shall not define any NTN TDD IoT NTN dynamic range for GEO.

Proposal 11: RAN4 shall consider removing NTN FDD IoT NTN dynamic range for GEO.

Table 7.3.2-2: Dynamic range of SAN supporting standalone NB-IoT operation  (GEO class payload)

| SAN channel bandwidth (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWChannel | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | FRC A15-1 in Annex A.15 | -97.3 | -93.6 | AWGN |
| 200 | FRC A15-2 in Annex A.15 | -103.2 | -93.6 | AWGN |

Proposal 12: RAN4 shall consider adding a new table for OTA NTN TDD IoT NTN dynamic range for LEO:

Table 10.4.2-x: Dynamic range of SAN supporting standalone NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWChannel | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | FRC A15-1 in Annex A.15 | -105.4 - ΔOTAREFSENS | -101.7 - ΔOTAREFSENS | AWGN |
| 200 | FRC A15-2 in Annex A.15 | -95.3 - ΔOTAREFSENS | -85.7 - ΔOTAREFSENS | AWGN |

Proposal 13: RAN4 shall not define any OTA NTN TDD IoT NTN dynamic range for GEO.

Proposal 14: Further discuss in next RAN4#116 meeting the NTN NB-IoT TDD test configuration for SAN ACS requirement (interfering signal mean power, wanted signal power) and if any update needs to be done.

In RAN4#114-bis it was agreed (among other proposals):

Sub-topic 3-5: General SAN requirements for NTN TDD NB-IoT

- Issue 3-5-1: Number of tones

Agreement: RAN4 may consider number of tones =1 for deriving the requirements, as long as transmission time less than 8ms.

- Issue 3-5-2: Repetition number

Agreement: RAN4 can consider Repetition number=1 to be used for TDD NB-IoT SAN Rx requirements compatible with D/U=8 (8 ms transmission/reception time) and transmission periodicity N=9 (90 ms) for the new TDD frame structure.

In RAN4#114-bis it was agreed (among other proposals):

Sub-topic 3-6: Work Split for SAN specification TS 36.108

- Issue 3-6-1: Work Split for NB-IoT TDD NTN in TS 36.108

Agreement:

| Clause/Requirement | Company |
| --- | --- |
| Definitions/Abbreviations | Iridium |
| Band definition (Clause 5.2) | THALES |
| Conducted REFSENS update | THALES |
| Radiated REFSENS update |  |
| Dynamic range update | Iridium |
| Conducted ON/OFF requirements | Iridium |
| EVM Annex | ZTE |
| Other, miscellaneous |  |

Proposal 15: RAN4 shall consider updating Work Split for TS 36.108.

| Clause/Requirement | Company |
| --- | --- |
| Definitions/Abbreviations | Iridium |
| Band definition (Clause 5.2) | THALES |
| Conducted REFSENS update | THALES |
| Radiated REFSENS update | TBD |
| Dynamic range update | Iridium |
| Conducted ON/OFF requirements | Iridium |
| EVM Annex | ZTE |
| Other, miscellaneous | TBD |

Proposal 16: Agree to update the regional requirement (clause 4.5) in TS 36.108 as follows

## 4.5	Regional requirements

Some requirements in the present document may only apply in certain regions either as optional requirements, or as mandatory requirements set by local and regional regulation. It is normally not stated in the 3GPP specifications under what exact circumstances the regional requirements apply, since this is defined by local or regional regulation.

Table 4.5-1 lists all requirements in the present specification that may be applied differently in different regions.

Table 4.5-1: List of regional requirements

| Clause number | Requirement | Comments |
| --- | --- | --- |
| 5.2 | Operating bands | Satellite operating bands may be applied regionally. |
| 6.6.4, 9.7.4 | Out-of-band emissions OTA out-of-band emissions | For band 249 and band 255 operation in US, Limits in FCC Title 47 apply. |
| 6.6.5 9.7.5 | Tx spurious emissions, OTA Tx spurious emissions | For band 249 and band 255 operation in US, Limits in FCC Title 47 apply. |

Proposal 17: Since TS 36.108 decisions look stable now, RAN4 shall start to discuss/update TS 36.181 starting from next meeting RAN4#116.

Note: There are many corrections to be made, for instance ETC6 is defined in Clause 4.2.7 and then never used throughout all Technical Specification.

Proposal 18: Discuss Work Split for TS 36.181 (if any) during the meeting:

- Definitions/Abbreviations

- REFSENS update

- Dynamic range update

- ON/OFF requirements

- EVM Annex

- Other, miscellaneous

Note: Other identified Clauses are not precluded

# Conclusion

Proposal 1: RAN4 to agree with keeping single tone 15 kHz (with different variants) and remove 3.75 kHz single-tone configuration for SAN reference sensitivity for NTN TDD NB-IoT in TS 36.108 (Option 1/REFSENS in TS 36.108).

Proposal 2: RAN4 to consider (at least) new FRC configuration A14-1 Option 1 below for NTN TDD NB-IoT:

Table A.14-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A14-1 for FDD | A14-2 for FDD | A14-1 new Option1 for TDD | A14-1 new Option2 for TDD | A14-1 new Option3 for TDD |
| --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 kHz | 15 kHz | 15 kHz |
| Number of tone | 1 | 1 | 1 | 1 | 1 |
| Diversity | No | No | No | No | No |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK | [π/4] QPSK | [π/4] QPSK |
| Frequency offset | 0 | 0 | 0 | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 | 4 ms | 4 ms | 4 ms |
| Number of NPUSCH repetition | 1 | 1 | 1 | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 | 0 / 0 | 3 / 3 | 2 / 1 |
| Payload size (bits) | 32 | 32 | 16 bits | 40 bits | 24 bits |
| Allocated resource unit | 2 | 2 | 1 | 1 | 1 |
| Code rate (target) | 1/3 | 1/3 | ⅓ | ⅓ | ⅓ |
| Code rate (effective) | 0.29 | 0.29 | 0.42 | 0.33 | 0.25 |
| Transport block CRC (bits) | 24 | 24 | 24 bits | 24 bits | 24 bits |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 96 | 192 | 192 |
| Total symbols per resource unit | 96 | 96 | 96 | 96 | 96 |
| Tx time (ms) | 16 | 64 | 8 ms | 8 ms | 8 ms |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |

Proposal 3: RAN4 to consider (at least) new SNR target value corresponding to the new configuration NTN TDD NB-IoT A14-1 Option 1 below for NTN TDD NB-IoT:

Table A.14-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A14-1 for FDD | A14-2 for FDD | A14-1 new Option1 for TDD | A14-1 new Option2 for TDD | A14-1 new Option3 for TDD |
| --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 kHz | 15 kHz | 15 kHz |
| Number of tone | 1 | 1 | 1 | 1 | 1 |
| Diversity | No | No | No | No | No |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK | [π/4]QPSK | [π/4]QPSK |
| Frequency offset | 0 | 0 | 0 | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 | 4 ms | 4 ms | 4 ms |
| Number of NPUSCH repetition | 1 | 1 | 1 | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 | 0 / 0 | 3 / 3 | 2 / 1 |
| Payload size (bits) | 32 | 32 | 16 bits | 40 bits | 24 bits |
| Allocated resource unit | 2 | 2 | 1 | 1 | 1 |
| Code rate (target) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 | 0.42 | 0.33 | 0.25 |
| Transport block CRC (bits) | 24 | 24 | 24 bits | 24 bits | 24 bits |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 96 | 192 | 192 |
| Total symbols per resource unit | 96 | 96 | 96 | 96 | 96 |
| Tx time (ms) | 16 | 64 | 8 ms | 8 ms | 8 ms |
| SNR target value (dB) | -2.1 dB as per TR 36.802 | as per TR 36.802 | -1,17 dB | 0,54 dB | -0,61 dB |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |

Proposal 4: The following equations and parameter values to compute REFSENS for TDD NTN NB-IoT SAN can be considered:

REFSENS [dBm] = -174 dBm/Hz noise density +10*LOG10(15*1000 Hz) bandwidth + SAN NF + IM + SNR_target,

where:

- SAN NF (Noise Figure):

  - NF LEO = 4.3 dB

  - NF GEO = 7.4 dB

- IM (Implementation Margin) = 2 dB

- (new TDD only) SNR_target values:

  - A14-1 new Option1 for TDD: -1.17dB;

  - A14-1 new Option2 for TDD: 0.54 dB;

  - A14-1 new Option3 for TDD: -0.61 dB,

Proposal 5: In accordance with new A14-1 configurations, RAN4 to select between following REFSENS values for TDD NB-IoT NTN band 249 SAN specification:

- Option 1:

  - GEO: -124.0 dBm;

  - LEO: -127.1 dBm;

- Option 2:

  - GEO: -122.3 dBm;

  - LEO: -125.4 dBm;

- Option 3:

  - GEO: -123.4 dBm;

  - LEO: -126.5 dBm;

- Other options not precluded.

| Reference channel | A14-1 for FDD | A14-2 for FDD | A14-1 new Option1 for TDD | A14-1 new Option2 for TDD | A14-1 new Option3 for TDD |
| --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 kHz | 15 kHz | 15 kHz |
| Number of tone | 1 | 1 | 1 | 1 | 1 |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK | QPSK | QPSK |
| Tx time (ms) | 16 | 64 | 8 ms | 8 ms | 8 ms |
| SNR target value (dB) | -2.1 dB as per TR 36.802 | .. as per TR 36.802 | -1,17 dB | 0,54 dB | -0,61 dB |
| REFSENS (dBm) | GEO: -124.9 LEO: -128.0 (current) |  | GEO: -124.0 LEO: -127.1 (new proposed) | GEO: -122.3 LEO: -125.4 (new proposed) | GEO: -123.4 LEO: -126.5 (new proposed) |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |

Proposal 6: For SAN TS 36.108, adapt FRC configuration with new TDD frame structure (maximum 8 ms transmission time), for example:

  - For conducted specifications:

For SAN supporting NTN NB-IoT operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.1 with parameters specified in Table 7.2.2-3 and 7.2.2-4 for SAN type 1-H in all FDD operating bands and specified in Table 7.2.2-5 and 7.2.2-6 for SAN type 1-H in all TDD operating bands.

Table 7.2.2-3: Reference sensitivity levels of SAN supporting NB-IoT operation in FDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -124.9 |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -130.9 |

Table 7.2.2-4: Reference sensitivity levels of SAN supporting NB-IoT operation in FDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -128.0 |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -134.0 |

Table 7.2.2-5: Reference sensitivity levels of SAN supporting NB-IoT operation in TDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 new Option 1 in Annex A.14 | -124.0 |
| 200 | 3.75 | TBD in Annex A.14 | TBD |

Table 7.2.2-6: Reference sensitivity levels of SAN supporting NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 new Option 1 in Annex A.14 | -127.1 |
| 200 | 3.75 | TBD in Annex A.14 | TBD |

  - For OTA specifications:

For SAN supporting standalone NB-IoT operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameter specified in Table 10.3.2-3 and Table 10.3.2-4 for FDD bands and Table 10.3.2-5 and Table 10.3.2-6 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

Table 10.3.2-3: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in FDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -124.9 - ΔOTAREFSENS |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -130.9 - ΔOTAREFSENS |

Table10.3.2-4: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in FDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -128.0 - ΔOTAREFSENS |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -134.0 - ΔOTAREFSENS |

Table 10.3.2-5: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in TDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 new Option 1 in Annex A.14 | -124.0 - ΔOTAREFSENS |
| 200 | 3.75 | TBD in Annex A.14 | TBD - ΔOTAREFSENS |

Table10.3.2-6: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 new Option 1 in Annex A.14 | -127.1 - ΔOTAREFSENS |
| 200 | 3.75 | TBD in Annex A.14 | TBD - ΔOTAREFSENS |

Proposal 7: RAN4 shall consider for NTN TDD IoT NTN dynamic range:

  - Reference channel A15-1 (with a 15kHz configuration) can be reused for NTN TDD IoT NTN dynamic range computation.

  - Reference channel A15-2 (with a 3.75kHz configuration) can be removed for NTN TDD IoT NTN dynamic range computation.

Table A.15-1 FRC parameters for NB-IoT dynamic range

| Reference channel | A15-1 for both FDD & TDD | A15-2 for FDD only |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Modulation | π/4 QPSK | π/4 QPSK |
| Diversity | No | No |
| Frequency offset | 0 | 0 |
| IMCS / ITBS | 7 / 7 | 7 / 7 |
| Payload size (bits) | 104 | 104 |
| Allocated resource units | 1 | 1 |
| Transport block CRC (bits) | 24 | 24 |
| Coding rate (target) | 2/3 | 2/3 |
| Coding Rate | 0.67 | 0.67 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks – C | 1 | 1 |
| Total symbols per resource unit | 96 | 96 |
| Total number of bits per resource unit | 192 | 192 |
| Tx time (ms) | 8 | 32 |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |

Proposal 8: RAN4 shall consider adding a new table for NTN TDD IoT NTN dynamic range for LEO:

Table 7.3.2-x: Dynamic range of SAN supporting standalone NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWChannel | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | FRC A15-1 in Annex A.15 | -105.4 | -101.7 | AWGN |
| 200 | FRC A15-2 in Annex A.15 | -95.3 | -85.7 | AWGN |

Proposal 9: RAN4 shall consider correcting previous NTN FDD IoT NTN dynamic range for LEO:

Table 7.3.2-3: Dynamic range of SAN supporting standalone NB-IoT operation in FDD (LEO class payload)

| SAN channel bandwidth (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWChannel | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | FRC A15-1 in Annex A.15 | [-89.4] | [-85.7] | AWGN |
| 200 | FRC A15-2 in Annex A.15 | [-95.3] | [-85.7] | AWGN |

Proposal 10: RAN4 shall not define any NTN TDD IoT NTN dynamic range for GEO.

Proposal 11: RAN4 shall consider removing NTN FDD IoT NTN dynamic range for GEO.

Table 7.3.2-2: Dynamic range of SAN supporting standalone NB-IoT operation  (GEO class payload)

| SAN channel bandwidth (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWChannel | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | FRC A15-1 in Annex A.15 | -97.3 | -93.6 | AWGN |
| 200 | FRC A15-2 in Annex A.15 | -103.2 | -93.6 | AWGN |

Proposal 12: RAN4 shall consider adding a new table for OTA NTN TDD IoT NTN dynamic range for LEO:

Table 10.4.2-x: Dynamic range of SAN supporting standalone NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWChannel | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | FRC A15-1 in Annex A.15 | -105.4 - ΔOTAREFSENS | -101.7 - ΔOTAREFSENS | AWGN |
| 200 | FRC A15-2 in Annex A.15 | -95.3 - ΔOTAREFSENS | -85.7 - ΔOTAREFSENS | AWGN |

Proposal 13: RAN4 shall not define any OTA NTN TDD IoT NTN dynamic range for GEO.

Proposal 14: Further discuss in next RAN4#116 meeting the NTN NB-IoT TDD test configuration for SAN ACS requirement (interfering signal mean power, wanted signal power) and if any update needs to be done.

Proposal 15: RAN4 shall consider updating Work Split for TS 36.108.

| Clause/Requirement | Company |
| --- | --- |
| Definitions/Abbreviations | Iridium |
| Band definition (Clause 5.2) | THALES |
| Conducted REFSENS update | THALES |
| Radiated REFSENS update | TBD |
| Dynamic range update | Iridium |
| Conducted ON/OFF requirements | Iridium |
| EVM Annex | ZTE |
| Other, miscellaneous | TBD |

Proposal 16: Agree to update the regional requirement (clause 4.5) in TS 36.108 as follows

## 4.5	Regional requirements

Some requirements in the present document may only apply in certain regions either as optional requirements, or as mandatory requirements set by local and regional regulation. It is normally not stated in the 3GPP specifications under what exact circumstances the regional requirements apply, since this is defined by local or regional regulation.

Table 4.5-1 lists all requirements in the present specification that may be applied differently in different regions.

Table 4.5-1: List of regional requirements

| Clause number | Requirement | Comments |
| --- | --- | --- |
| 5.2 | Operating bands | Satellite operating bands may be applied regionally. |
| 6.6.4, 9.7.4 | Out-of-band emissions OTA out-of-band emissions | For band 249 and band 255 operation in US, Limits in FCC Title 47 apply. |
| 6.6.5 9.7.5 | Tx spurious emissions, OTA Tx spurious emissions | For band 249 and band 255 operation in US, Limits in FCC Title 47 apply. |

Proposal 17: Since TS 36.108 decisions look stable now, RAN4 shall start to discuss/update TS 36.181 starting from next meeting RAN4#116.

Note: There are many corrections to be made, for instance ETC6 is defined in Clause 4.2.7 and then never used throughout all Technical Specification.

Proposal 18: Discuss Work Split for TS 36.181 (if any) during the meeting:

- Definitions/Abbreviations

- REFSENS update

- Dynamic range update

- ON/OFF requirements

- EVM Annex

- Other, miscellaneous

Note: Other identified Clauses are not precluded

# Annex – previous meeting agreements

4.1 Agreements from RAN4#113

The WF covers the contributions submitted under the following AI:

- 7.28.1 General aspects and work plan	[IoT_NTN_TDD]

- 7.28.2 Study of impact due to periodic pattern	[IoT_NTN_TDD-Core]

### Issue 0: Parameters of the baseline TDD pattern for analysis on the impact of requirements

Agreement:

- RAN4 to consider the following values for analyzing the impact of requirements, as necessary:

  - N = 9, D = {8, 20, 30}, U = {8, 20, 30}

  - N = 8, D = 20, U = 20

- Among the above sets of values, the set with N = 9 and D = 8 is considered the baseline TDD pattern for analysis, as it represents the most challenging case in terms of requirement analysis.

Note 1: The above values, based on RAN1’s discussion in RAN1#118bis meeting, are recommended examples for assessing the impact of requirements on RF, RRM, and Demodulation aspects. The exact values are subject to RAN1’s decision.

Note 2: In terms of SNR, the above analysis focuses on the requirements for normal coverage.

### Issue 1-2: Impact on SAN RF requirements

Agreement:

- The assessment of the impact on SAN RF requirements (TS 36.108) shown in the table below will serve as a baseline for further discussion on the exact requirement definition. For the requirements to be impacted for [249] band, if any, NTN IoT 254 L-band Tx/Rx requirements in TS 36.108 will serve as a baseline. If identified, new requirements can be defined for SAN RF, e.g. transient period.

| 6.2 Satellite Access Node output power | Not band specific --> No specification impact. |
| --- | --- |
| 6.3.2 RE power control dynamic range | Not band specific --> No specification impact. |
| 6.3.3 Total power dynamic range | Not band specific --> No specification impact. |
| 6.4 Transmit ON/OFF power | Add this specification to SAN starting from Rel-19 |
| 6.5 Transmitted signal quality | Not band specific --> No specification impact. |
| 6.6.2 Occupied bandwidth | Not band specific --> No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Not band specific --> No specification impact. |
| 6.6.4 Out-of-band emissions | Band specific --> FFS: Specification impact. |
| 6.6.5 Transmitter spurious emissions | No specification impact. |
| 6.7 Transmitter intermodulation | Not applicable to SAN. |
| 7.2 Reference sensitivity level | Not band specific --> No specification impact. |
| 7.3 Dynamic range | Not band specific --> No specification impact. |
| 7.4.1 Adjacent Channel Selectivity (ACS) | Not band specific --> No specification impact. |
| 7.4.2 In-band blocking | Not applicable for SAN. |
| 7.5 Out-of-band blocking | Not band specific --> No specification impact. |
| 7.6 Receiver spurious emissions | Not applicable for SAN. |
| 7.7 Receiver intermodulation | Not applicable for SAN --> No specification impact. |
| 7.8 In-channel selectivity | Not band specific --> No specification impact. |

### Issue 1-3: Impact on UE RF requirements

Agreement:

- The assessment of the impact on UE RF requirements (TS 36.102) shown in the table below will serve as a baseline for further discussion on the exact requirement definition.

| 36.102 Clause: Requirements | Band Agnostic | Re-use for TDD band [249] | Note |
| --- | --- | --- | --- |
| 6.2B.1: UE maximum output power | Yes | Yes | Support UE power class 3 (23 dBm) Support UE power class 5 (20 dBm) |
| 6.2B.1: UE maximum output power tolerance | No |  |  |
| 6.2B.2: UE maximum output power reduction | Yes | Yes |  |
| 6.2B.3: UE additional maximum output power reduction | No | - | Note: Reuse which band A-MPR, B254 A-MPR is not defined yet and the band [249] UL frequency range is not fully aligned with B254. |
| 6.2.B.4: Configured transmit power | Yes | Yes |  |
| 6.3: Output power dynamics | Yes | Yes FFS | Note: For 36.102 6.3B.4, 36.102 refers to 36.101 6.3.5F, the testing period for aggerate power control may depend on different UL pattern. |
| 6.4: Transmit signal quality | Yes | Yes |  |
| 6.5B.3.2: Spectrum emission mask | Yes | Yes |  |
| 6.5B.3.4: Adjacent Channel Leakage Ratio | Yes | Yes |  |
| 6.5B.4.2: General spurious emissions | Yes | Yes |  |
| 6.5B.4.3: Spurious emissions for UE co-existence | No |  | Band [249] needs to be added in Table 6.5A.4.3-1 Spurious emission band UE co-existence. |
| 6.5B.4.4: Additional spurious emissions | No |  | Additional spurious emissions requirement for B254 can be reused for the new IoT-NTN TDD L-band. UL frequency range of B254 is superset of [B249]. |
| 6.6: Transmit intermodulation for category NB1 and NB2 | Yes | Yes |  |
| 7.2: Diversity characteristics | Yes | Yes |  |
| 7.3: Reference sensitivity power level | No | No | To be studied for IoT-NTN TDD mode. |
| 7.4: Maximum input level | Yes | Yes |  |
| 7.5: Adjacent Channel Selectivity | Yes | Yes |  |
| 7.6B.2: In-band blocking | No | Yes | Different band group may have different In-band blocking tables for Cat-M1 and NB1/NB2 separately. Currently, bands 255,254, and 253 apply the same IBB requirements. Only for B256 lower frequency range is modified to accommodate specific implementation. [B249] can re-use IBB requirements from B255,254 and 253 |
| 7.6B.3: Out-of-band blocking | No |  | Different Out-of-band requirements for IoT NTN bands. Currently, bands 255,254, and 253 apply the same OBB requirements. Only for B256 lower frequency range is modified to accommodate specific implementation. [B249] can re-use OBB requirements from B255, 254 and 253 |
| 7.6B.4: Narrow band blocking | No | Yes |  |
| 7.7: Spurious response | Yes | Yes |  |
| 7.8: Intermodulation characteristics | Yes | Yes |  |
| 7.9: RX spurious emission | No | Yes | Re-use spurious emission requirements from Table 7.9.1-1 of 36.101 |
| A.1 DL reference measurement channels |  |  | NPDSCH Reference Channel for TDD should be introduced. Fixed Reference Channel for TDD should be introduced. |
| A.2 OFDMA Channel Noise Generator (OCNG) |  |  | OCNG Patterns for TDD should be introduced. |
| A.3 Testing related to Satellite Access |  |  | No impact. |
| A.4 UL reference measurement channels |  |  | Reference measurement channels for TDD should be introduced. |

### Issue 2-1: Impact on RRM requirements

Agreement:

- From the RRM requirement point of view, no impact on the following requirements is expected due to the new periodic pattern(s):

  - UE uplink timing

  - PUR

Agreement:

- For the following RRM requirements, modifications are expected due to the new periodic pattern(s):

  - RLM (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.)

  - MSG3-based channel quality report (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.)

  - Cell re-selection (example modifications include extended cell detection/evaluation/measurement periods, side conditions, etc.)

  - RRC Re-establishment (example modifications include extended cell detection/measurement periods, side conditions, etc.)

  - RRC Connection Redirection to Non-anchor Carrier (example modifications include updates to the definition of the symbols, side conditions, etc.)

  - Cell measurements (example modifications include extended cell detection/measurement periods, side conditions, etc.)

Note 1: The exact requirement modifications are subject to further discussion in future RAN4 meetings based on RAN1 conclusion/progress on this feature.

Note 2: Different modifications can be made for different TDD patterns, if multiple, DRX configurations, SNR values, etc.

Note 3: Not all existing requirements for NB-IoT NTN may be applicable to the IoT_NTN_TDD feature, which is subject to further discussions in future RAN4 meetings.

Note 4: It is assumed that UE will be made aware of the periodic pattern of frequency layers to be measured.

Note 5: UE power consumption aspect may be considered when modifying the above requirements as needed.

Note 6: The requirement applicability rule defined in Rel-17/18 applies to the IoT_NTN_TDD feature as well. For instance, the above requirements may not be applicable for inter-satellite scenarios if the UE is not provided with the target inter-satellite’s information.

Demodulation aspect:

No observation in this meeting.

4.2 Agreements from RAN4#114

General requirements:

Issue 1-1-1: Work Plan presented under R4-2500136.

Agreement: Updated Work Plan to be agreed under R4-2502291 (see Annex for the updated work plan).

Issue 1-2-1: Band number

Agreement: Adopt [249] E-UTRA operating band number for the new TDD 1616-1626.5 MHz NTN NB-IoT frequency band introduction.

Issue 1-2-2: E-UTRA operating band

Agreement: Confirm the proposed band plan as follows:

E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 256 | 1980 MHz | - | 2010 MHz | 2170 MHz | - | 2200 MHz | FDD |
| 255 | 1626.5 MHz | - | 1660.5 MHz | 1525 MHz | - | 1559 MHz | FDD |
| 254 | 1610 MHz | - | 1626.5 MHz | 2483.5 MHz | - | 2500 MHz | FDD |
| 2532 | 1668 MHz | - | 1675 MHz | 1518 MHz | - | 1525 MHz | FDD |
| 252 | 2000 MHz | - | 2020 MHz | 2180 MHz | - | 2200 MHz | FDD |
| [249] | 1616 MHz | - | 1626.5 MHz | 1616 MHz | - | 1626.5 MHz | TDD |
| NOTE 1: Satellite bands are numbered in descending order from 256 NOTE 2: UE assigned to channels and allocated frequency resources in the lower portion of Band 253 may experience blocking or harmful interference from terrestrial networks in adjacent or nearby frequencies when operating in the proximity with terrestrial base stations. NOTE 3: Band 249 is defined only for NB-IoT operation. |  |  |  |  |  |  |  |

Issue 1-2-3: Gap between EARFCN FDD NTN and EARFCN TDD NTN

Agreement: Consider a gap between previous introduced EARFCN FDD NTN (starting from 2^18-1=262143 for NUL and 2^18-2^15-1=229375 for NDL) and new (first) introduced EARFCN TDD NTN.

Issue 1-2-4: EARFCN for TDD NTN

Agreement: For the NTN E-UTRA TDD EARFCN number, consider to start the NUL/NDL upper range from 2^18-2^16-1=196607 (below FDD NTN NOffs-DL).

Issue 1-2-5: Channel raster for TDD NTN

Agreement: Use a channel raster of 100 kHz and a Step size of <1>.

Issue 1-3-1: TS 36.108 Table 5.4A.2-1: E-UTRA channel numbers

Agreement: Update table as part of E-UTRA channel numbers in Section 5.4A.2, TS 36.108:

Table 5.4A.2-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 256 | 100 | 2170 | 229076 | 229076 -<1>- 229375 | 1980 | 261844 | 261844 -<1>- 262143 |
| 255 | 100 | 1525 | 228736 | 228736 -<1>- 229075 | 1626.5 | 261504 | 261504 -<1>- 261843 |
| 254 | 100 | 2483.5 | 228571 | 228571 -<1>- 228735 | 1610 | 261339 | 261339 -<1>- 261503 |
| 253 | 100 | 1518 | 228501 | 228501 -<1>- 228570 | 1668 | 261269 | 261269 -<1>- 261338 |
| [249] | 100 | 1616 | 196607 | 196607-<1>-196711 | 1616 | 196607 | 196607-<1>-196711 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

Issue 1-3-2: TS 36.108 5.7.3 Clause

Agreement: As an initial working assumption, with respect to the definition of IoT-NTN TDD L-band, add the following information in 5.7.3 for the carrier frequency and EARFCN for DL and UL respectively:

- “The carrier frequency of NB-IoT in the downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 – 262143 and the Offset of NB-IoT Channel Number to EARFCN in the range {-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,-0.5,0,1,2,3,4,5,6,7,8,9} for FDD and TDD band 249.”

- “The carrier frequency of NB-IoT in the uplink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 –262143, and the Offset of NB-IoT Channel Number to EARFCN in the range {-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,0,1,2,3,4,5,6,7,8,9} for FDD and TDD band 249.”

Moderator Note: Companies to further check.

Issue 1-4-1: TS 36.102 Table 5.4A.2-1: E-UTRA channel numbers

Agreement: Update TS 36.102 with:

Table 5.4A.2-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 256 | 100 | 2170 | 229076 | 229076 -<1>- 229375 | 1980 | 261844 | 261844 -<1>- 262143 |
| 255 | 100 | 1525 | 228736 | 228736 -<1>- 229075 | 1626.5 | 261504 | 261504 -<1>- 261843 |
| 254 | 100 | 2483.5 | 228571 | 228571 -<1>- 228735 | 1610 | 261339 | 261339 -<1>- 261503 |
| 253 | 100 | 1518 | 228501 | 228501 -<1>- 228570 | 1668 | 261269 | 261269 -<1>- 261338 |
| [249] | 100 | 1616 | 196607 | 196607-<1>-196711 | 1616 | 196607 | 196607-<1>-196711 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

Issue 1-4-2: TS 36.102 5.4B Clause

Agreement: Update TS 36.102 with:

== Start Change == ## 5.4B Channel arrangement for category NB1 and NB2 ### 5.4B.1 Channel spacing Nominal channel spacing for UE category NB1 and NB2 in stand-alone mode is 200 kHz. ### 5.4B.2 Channel raster, carrier frequency and EARFCN The channel raster of UE category NB1/NB2 shall be as defined in clause 5.4A.2, and the channel raster per-frequency band shall be as defined in table 5.4A.2-1. The carrier frequency of UE category NB1/NB2 in the downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) as defined in Table 5.4A.2-1, and the Offset of category NB1/NB2 Channel Number to EARFCN in the range of {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, -0.5, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9} for FDD and TDD band 249. The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL is the downlink carrier frequency of category NB1/NB2, FDL_low and NOffs-DL are given in table 5.4A.2-1, NDL is the downlink EARFCN, MDL is the Offset of category NB1/NB2 Channel Number to downlink EARFCN. FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL) The carrier frequency of UE category NB1/NB2 in the uplink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) as defined in Table 5.4A.2-1, and the Offset of category NB1/NB2 Channel Number to EARFCN in the range of {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9} for FDD and TDD band 249. The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the uplink is given by the following equation, where FUL is the uplink carrier frequency of category NB1/NB2, FUL_low and NOffs-UL are given in table 5.4A.2-1, NUL is the uplink EARFCN, MUL is the Offset of category NB1/NB2 Channel Number to uplink EARFCN. FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL) NOTE 1: Guard-band operation and in-band operation for NB-IoT are not supported in this version of the specification. NOTE 2: For the carrier including NPSS/NSSS for stand-alone operation, MDL = 0. == End Change ==

SAN requirements:

Issue 3-1-1: Requirement set applicability TS 36.108

Agreement: RAN4 to update the following Requirement set applicability from TS 36.108:

Table 4.6-1: Requirement set applicability

| Requirement | Requirement set |  |
| --- | --- | --- |
|  | SAN type 1-H | SAN type 1-O |
| SAN output power | 6.2 |  |
| … | … |  |
| Transmit ON/OFF power | NA 6.4 |  |
| … | … |  |
| Transmitter intermodulation | NA | NA |
| … | … |  |
| Performance requirements | 8 |  |
| Radiated transmit power | 9.2 | 9.2 |
| … |  | … |
| OTA transmit ON/OFF power |  | NA 9.5 |
| … |  | … |
| OTA ACLR | NA | 9.7.3 |
| … |  | … |

Issue 3-1-2: Transmit ON/OFF power in TS 36.108

Agreement: Update the following Clause 6.4 in TS 36.108 with “The requirements in clause 6.4 are only applied for NB-IoT TDD SAN.” Also, update relevant clause “transmit ON/OFF power” for NB-IoT TDD SAN in TS 36.181 with TS 36.108 agreement for Transmit ON/OFF power.

## 6.4	Transmit ON/OFF power

The requirement is not applicable in this version of the specification.

The requirements in clause 6.4 are only applied for NB-IoT TDD SAN.

### 6.4.1	Transmitter OFF power

Transmitter OFF power is defined as the mean power measured over 70 µs filtered with a square filter of bandwidth equal to the transmission bandwidth configuration of the SAN (BWConfig) centred on the assigned channel frequency during the transmitter OFF period.

#### 6.4.1.1	Minimum Requirement

The transmitter OFF power spectral density shall be less than -85dBm/MHz.

For SAN capable of multi-band operation, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

### 6.4.2	Transmitter transient period

The transmitter transient period is the time period during which the transmitter is changing from the OFF period to the ON period or vice versa. The transmitter transient period is illustrated in Figure 6.4.2-1.

Figure 6.4.2-1 Illustration of the relations of transmitter ON period, transmitter OFF period and transmitter transient period.

#### 6.4.2.1	Minimum requirements

The transmitter transient period shall be shorter than the values listed in Table 6.4.2.1-1.

Table 6.4.2.1-1 Minimum requirements for the transmitter transient period

| Transition | Transient period length [us] |
| --- | --- |
| OFF to ON | [17] |
| ON to OFF | [17] |

Issue 3-1-3: OTA Transmit ON/OFF power in TS 36.108

Agreement: Update the following Clause 9.5 in TS 36.108 with “The requirements in clause 9.5 are only applied for NB-IoT TDD SAN”. Also, update relevant clause “transmit ON/OFF power” for NB-IoT TDD SAN in TS 36.181 with TS 36.108 agreement for OTA Transmit ON/OFF power.

## 9.5	OTA transmit ON/OFF power

The requirement is not applicable in this version of the specification.

### The requirements in clause 9.5 are only applied for NB-IoT TDD SAN.

### 9.5.2	OTA transmitter OFF power

#### 9.5.2.1	General

OTA transmitter OFF power is defined as the mean power measured over 70 us filtered with a square filter of bandwidth equal to the transmission bandwidth configuration of the SAN (BWConfig) centred on the assigned channel frequency during the transmitter OFF period.

For multi-band RIBs and single band RIBs supporting transmission in multiple bands, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

#### 9.5.2.2	Minimum requirement for SAN type 1-O

The total power from all co-location reference antenna conducted output(s) shall be less than -106 dBm/MHz.

### 9.5.3	OTA transient period

#### 9.5.3.1	General

The OTA transmitter transient period is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1.

This requirement shall be applied at each RIB supporting transmission in the operating band.

#### 9.5.3.2	Minimum requirement for SAN type 1-O

For SAN type 1-O, the OTA transmitter transient period shall be shorter than the values listed in the minimum requirement table 9.5.3.2-1.

Table 9.5.3.2-1: Minimum requirement for the OTA transmitter transient period for SAN type 1-O

| Transition | Transient period length (µs) |
| --- | --- |
| OFF to ON | [17] |
| ON to OFF | [17] |

Issue 3-2-1: Remove redundant A.3-1 FRC

Agreement: RAN4 to clean up the spec TS 36.108 as part of maintenance CR (not specifically related to this WID) and remove redundant data e.g. A.3-1 FRC (redundant with A.14-1) not cited at all in all the text.

Table A.3-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A14-1 | A14-2 |
| --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Diversity | No | No |
| Modulation | π/2 BPSK | π/2 BPSK |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) Note 1 | 4 | 16 |
| Number of NPUSCH repetition | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 |
| Payload size (bits) | 32 | 32 |
| Allocated resource unit | 2 | 2 |
| Code rate (target) | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 |
| Total symbols per resource unit | 96 | 96 |
| Tx time (ms) | 16 | 64 |
| Note: Channel estimation lengths are included in the table for information only. |  |  |

Issue 3-2-2: Remove redundant A.4-1 FRC

Agreement: RAN4 to clean up the spec TS 36.108 as part of maintenance CR (not specifically related to this WID) and remove redundant data e.g. A.4-1 FRC (redundant with A.15-1) not cited at all in all the text.

Table A.4-1 FRC parameters for NB-IoT dynamic range

| Reference channel | A15-1 | A15-2 |
| --- | --- | --- |
| Sub carrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Modulation | π/4 QPSK | π/4 QPSK |
| Diversity | No | No |
| Frequency offset | 0 | 0 |
| IMCS / ITBS | 7 / 7 | 7 / 7 |
| Payload size (bits) | 104 | 104 |
| Allocated resource units | 1 | 1 |
| Transport block CRC (bits) | 24 | 24 |
| Coding rate (target) | 2/3 | 2/3 |
| Coding Rate | 0.67 | 0.67 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks – C | 1 | 1 |
| Total symbols per resource unit | 96 | 96 |
| Total number of bits per resource unit | 192 | 192 |
| Tx time (ms) | 8 | 32 |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 |
| Note: Channel estimation lengths are included in the table for information only. |  |  |

Issue 3-3-1: NTN NB-IoT SAN FRC for reference sensitivity and in-channel selectivity

Agreement: RAN4 to further discuss FRC for reference sensitivity and in-channel selectivity in SAN TS 36.108 with respect to band 249 for better adaptation to 8ms TDD Tx time (corresponding to D/U=8).

- Option 1: Further discuss if to reuse A14-1 reference channel and segment the signal into 2 different 8 ms Tx/Rx times (e.g. with 90 ms periodicity between 2 transmissions, if 1 transmission can be postponed) for TDD NTN NB-IoT SAN requirements. Further discuss if A14-2 could be descoped for TDD NTN NB-IoT SAN requirements.

- Option 2: Further discuss if to define new FRC(s) for NTN NB-IoT to cope with 8 ms Tx/Rx time, other options are not precluded.

Table A.14-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A14-1 | A14-2 |
| --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Diversity | No | No |
| Modulation | π/2 BPSK | π/2 BPSK |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 |
| Number of NPUSCH repetition | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 |
| Payload size (bits) | 32 | 32 |
| Allocated resource unit | 2 | 2 |
| Code rate (target) | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 |
| Total symbols per resource unit | 96 | 96 |
| Tx time (ms) | 16 | 64 |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |

Issue 3-3-2: NTN NB-IoT SAN FRC dynamic range

Agreement: RAN4 to further discuss FRC for dynamic range in SAN TS 36.108 with respect to band 249 for better adaptation to 8ms TDD Tx time (corresponding to D/U=8).

- Further discuss if reference channel A15-1 can be reused for NTN TDD IoT NTN.

- Further discuss if reference channel A15-2 can be descoped for NTN TDD IoT NTN.

- Other options are not precluded.

Table A.15-1 FRC parameters for NB-IoT dynamic range

| Reference channel | A15-1 | A15-2 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Modulation | π/4 QPSK | π/4 QPSK |
| Diversity | No | No |
| Frequency offset | 0 | 0 |
| IMCS / ITBS | 7 / 7 | 7 / 7 |
| Payload size (bits) | 104 | 104 |
| Allocated resource units | 1 | 1 |
| Transport block CRC (bits) | 24 | 24 |
| Coding rate (target) | 2/3 | 2/3 |
| Coding Rate | 0.67 | 0.67 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks – C | 1 | 1 |
| Total symbols per resource unit | 96 | 96 |
| Total number of bits per resource unit | 192 | 192 |
| Tx time (ms) | 8 | 32 |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |

Issue 3-4-1: General requirements

Agreement: RAN4 to further discuss FRCs, related SNR targets for specific configurations and equations for driving requirements (e.g. REFSENS, in-channel selectivity, dynamic range).

  - NOTE: TR 36.802 can be used as starting point for the equations, FRC (when applicable) and related SNR values.

Issue 3-4-2: Number of tones

Agreement: RAN4 to discuss the number of tones to be used for deriving the requirements. Companies may consider increasing the number of tones in order to decrease transmission time less than 8 ms (for compatibility with D/U=8 for the new TDD frame structure).

Issue 3-4-3: Repetition number

Agreement: RAN4 to discuss the repetition number to be used for TDD NB-IoT SAN Rx requirements compatible with D/U=8 (8 ms transmission/reception time) and transmission periodicity N=9 (90 ms) for the new TDD frame structure.

Issue 3-4-5: New REFSENS SAN requirements for NTN TDD in TS 36.108

Agreement: For SAN TS 36.108, adapt FRC configuration with new TDD frame structure (maximum 8 ms transmission time), for example:

  - Option 1: Adapt the existent SAN FRC.

    - For conducted specifications:

For SAN supporting NTN NB-IoT operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.1 with parameters specified in Table 7.2.2-3 and 7.2.2-4 for SAN type 1-H in all FDD operating bands and specified in Table 7.2.2-5 and 7.2.2-6 for SAN type 1-H in all TDD operating bands.

Table 7.2.2-3: Reference sensitivity levels of SAN supporting NB-IoT operation in FDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -124.9 |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -130.9 |

Table 7.2.2-4: Reference sensitivity levels of SAN supporting NB-IoT operation in FDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -128.0 |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -134.0 |

Table 7.2.2-5: Reference sensitivity levels of SAN supporting NB-IoT operation in TDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 | TBD |
| 200 | 3.75 | TBD in Annex A.14 | TBD |

Table 7.2.2-6: Reference sensitivity levels of SAN supporting NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 | TBD |
| 200 | 3.75 | TBD in Annex A.14 | TBD |

    - For OTA specifications:

For SAN supporting standalone NB-IoT operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameter specified in Table 10.3.2-3 and Table 10.3.2-4 for FDD bands and Table 10.3.2-5 and Table 10.3.2-6 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

Table 10.3.2-3: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in FDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -124.9 - ΔOTAREFSENS |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -130.9 - ΔOTAREFSENS |

Table10.3.2-4: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in FDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -128.0 - ΔOTAREFSENS |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -134.0 - ΔOTAREFSENS |

Table 10.3.2-5: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in TDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 | TBD- ΔOTAREFSENS |
| 200 | 3.75 | TBD in Annex A.14 | TBD - ΔOTAREFSENS |

Table10.3.2-6: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 | TBD - ΔOTAREFSENS |
| 200 | 3.75 | TBD in Annex A.14 | TBD - ΔOTAREFSENS |

  - Option 2: Reuse UE FRC (from TS 36.101) for SAN FRC.

Table A.3.2-2e Fixed Reference Channel for Receiver Requirements (TDD) – for CAT-NB1 and CAT-NB2

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 0.2 |
| Number of subcarriers |  | 12 |
| Uplink-Downlink Configuration (Note 7) |  | 1 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Number of HARQ Processes | Processes | 1 |
| Maximum number of HARQ transmissions |  | 1 |
| Transport block size | Bits | 88 |
| Number of Sub-Frames per transport block |  | 1 |
| Transport block CRC | Bits | 24 |
| Binary Channel Bits Per Sub-Frame | Bits | 320 |
| LTE CRS port |  | N/A |
| Number of NRS ports |  | 1 |
| Number of NPDSCH repetitions Number of NPUSCH repetitions |  | 1 |
| UE DL Category UE UL Category |  | NB1 or NB2 |
| Note 1: Category NB1 or NB2 in stand-alone mode has been considered here. Note 2: Reference signal, Synchronization signals and NPBCH allocated as per TS 36.211. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 4: Parameters related to NPDSCH scheduling are defined in Table A.3.2-1e to Table A.3.2-1g. Note 5: NPDCCH and information bit payload are not transmitted in the subframes used for transmission of SI messages. Note 6: SI messages transmission should be prioritized over NPDCCH transmission in case of collision. NPDCCH transmission is postponed until the next NB-IoT downlink subframe in case NPDCCH transmission occurs in a non NB-IoT downlink subframe, where an NB-IoT downlink subframe is a subframe that does not contain NPSS/NSSS/NPBCH/SIB1-NB transmission. Note 7: As per Table 4.2-2 in TS 36.211 [4] Note 8: Number of repetition NRep as defined in table 16.4.1.3-2 in TS 36.213 [6]. |  |  |

Issue 3-4-6: Add EVM phrase for NTN TDD in TS 36.108

Agreement: Add EVM TDD phrase in TS 36.108 “For TDD the averaging in the time domain can be calculated from subframes of different frames and should have a minimum of 10 subframes averaging length. TDD special fields (DwPTS and GP) are not included in the averaging.”

4.3 Agreements from RAN4#114-bis

Topic #1: General part and band definition for [IoT_NTN_TDD]

Sub-topic 1-2: Other general topics

Issue 1-2-1: Remaining issues from other bands

Agreement: Remaining issues from 254 band introduction will remain out of scope of the IoT_NTN_TDD WI.

Issue 1-2-2: Operating bands for category NB1 and NB2

Agreement: Update the following phrase:

## 5.2B Operating bands for category NB1 and NB2 Category NB1 and NB2 UE are designed to operate in the E-UTRA satellite access operating bands defined in Table 5.2-1. Category NB1 and NB2 UE operate in HD-FDD duplex mode. Category NB1 and NB2 UE operate in TDD duplex mode for 249 band.

Topic #2: UE Requirements for [IoT_NTN_TDD]

Sub-topic 2-1: UE general core requirements

Issue 2-1-1: Discussion on UE Power Class (PC) for NB-IoT TDD NTN in TS 36.102

Agreement: Use PC3 and PC5 as work assumptions. The NB-IoT TDD NTN WI shall not consider the definition of a new power class.

Note: the intention of NB-IoT TDD NTN WI was to re-use current legacy NB-IoT devices.

Issue 2-1-2: Discussion on UE AMPR for NB-IoT TDD NTN in TS 36.102

Agreement: RAN4 to further discuss what are the regulatory documents applicable for the emission requirements of 249.

Agreement: RAN4 to consider at least the following references: ETSI EN 301 441, ITU-R M.1343-1, other references not precluded.

Agreement: RAN4 should discuss if regulatory requirements are defined per band or per channel, and the simulation/measurement assumption for NB-IoT NTN channel positioning with respect to the band.

Agreement: As a starting point, RAN4 can consider for discussions:

  - Option 1: Since similar frequency range between 249 and 254, A-MPR requirement for 249 shall follow simulation results in TS 36.764 and A-MPR requirements (to be defined) for 254 in TS 36.102.

1a/ As starting point, for 1-tone allocation (priority 1) – see TR 36.764:

Table 6.2.3.2-3: 1-Tone A-MPR for NS_04N power class 3

| Modulation | π/4 QPSK |  |  |
| --- | --- | --- | --- |
| Tone positions for 15kHz 1-Tone allocation | 0/11 | 1/10 | 2/3/4/5/6/7/8/9 |
| A-MPR | ≤ 0.5 dB | 0 dB | 0 dB |

Table 6.2.3.2-4: 1-Tone A-MPR for NS_05N power class 3

| Modulation | π/4 QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 15kHz 1-Tone allocation | 0/11 | 1/10 | 2/9 | 3/4/5/6/7/8 |
| A-MPR | ≤ 1.5 dB | ≤ 1.0 dB | 0 dB | 0 dB |

1b/ As starting point, for multiple-tone allocation (priority 2) – see TR 36.764:

Table 6.2.3.2-1: A-MPR for NS_04N power class 3

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | ≤ 0.7 dB | 0.2 dB |  | ≤ 0.7 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| A-MPR | 0 dB |  | 0 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| A-MPR | 0 dB |  |  |  |

Table 6.2.3.2-2: A-MPR for NS_05N power class 3

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | ≤ 1.5 dB | 0.5 dB |  | ≤ 1.5 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| A-MPR | ≤ 0.7 dB |  | ≤ 0.7 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| A-MPR | 0 dB |  |  |  |

1c/ Other options not precluded.

  - Option 2: Re-evaluate A-MPR for 249 based on PC3/PC5 (priority for PC3) assumption using at least the following references: ETSI EN 301 441, ITU-R M.1343-1

Issue 2-1-3: Discussion on frequency error requirement for NB-IoT TDD NTN in TS 36.102

Agreement:

  - Do not change the current frequency error requirement from TS 36.102 for NTN TDD NB-IoT:

Table 6.4B.1-1: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

  - Add a note to the following text [Clause 6.4B.1of TS36.102] as follows:

“When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3]. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission.

Note: For 249 band, UE is allowed to perform pre-compensation between two UpLink bursts”.

Sub-topic 2-2: Work Split for UE specification TS 36.102

Issue 2-2-1: Work Split for NB-IoT TDD NTN in TS 36.102

| Clause/Requirement | Company |
| --- | --- |
| Band definition (Clause 5.2) | Iridium |
| REFSENS update (Clause 7.3B) | THALES |
| A-MPR |  |
| Frequency error requirement | THALES |
| Blocking update (Clause 7.6B) | ZTE |
| Other topics (if any) e.g. [EVM Annex] |  |

Topic #3: SAN Requirements for [IoT_NTN_TDD]

Sub-topic 3-1: TDD specific requirements for NTN NB-IoT in TS 36.108

Issue 3-1-1: Add ON/OFF/transient definitions in TS 36.108

Agreement:

- Add the following definitions (currently not available in TS 36.108):

  - Transmitter ON period: time period during which the SAN transmitter is allowed to transmit data and/or reference symbols.

  - Transmitter OFF period: time period during which the SAN transmitter is not allowed to transmit.

  - Transmitter transient period: time period during which the transmitter is changing from the OFF period to the ON period or vice versa.

- Update relevant clause “transmit ON/OFF power” for NB-IoT TDD SAN in TS 36.181 with TS 36.108 agreement for OTA Transmit ON/OFF power.

Issue 3-1-2: OTA Transmit ON/OFF power in TS 36.108

Agreement:

- Remove Clause 9.5 (OTA transmit ON/OFF power) previously agreed at RAN#114 (while keeping Clause 6.4):

## OTA transmit ON/OFF power

The requirement is not applicable in this version of the specification.

- Update relevant clause “transmit ON/OFF power” for NB-IoT TDD SAN in TS 36.181 with TS 36.108 agreement for OTA Transmit ON/OFF power.

Issue 3-1-3: Requirement set applicability TS 36.108

Agreement: RAN4 to update the following Requirement set applicability from TS 36.108:

Table 4.6-1: Requirement set applicability

| Requirement | Requirement set |  |
| --- | --- | --- |
|  | SAN type 1-H | SAN type 1-O |
| SAN output power | 6.2 |  |
| … | … |  |
| Transmit ON/OFF power | NA 6.4 |  |
| … | … |  |
| Transmitter intermodulation | NA | NA |
| … | … |  |
| Performance requirements | 8 |  |
| Radiated transmit power | 9.2 | 9.2 |
| … |  | … |
| OTA transmit ON/OFF power |  | NA 9.5 |
| … |  | … |
| OTA ACLR | NA | 9.7.3 |
| … |  | … |

Sub-topic 3-2: In-channel selectivity in TS 36.108

Issue 3-2-1: In-channel selectivity/OTA in-channel selectivity from specification TS 36.108

Agreement: Do not consider in-channel selectivity/OTA in-channel selectivity in the Rel-19 specification TS 36.108 (clauses 7.8 for conducted receiver characteristics and 10.9 for radiated receiver characteristics) as SAN requirement for TDD NB-IoT NTN band 249, since it was not considered in previous Rel-18 TS 36.108 specification for previous defined NB-IoT NTN bands.

Issue 3-2-2: Tables A.14-1 for TS 36.108

Agreement: Remove “in-channel selectivity” from e.g. tables A.14-1.

Sub-topic 3-3: REFSENS in TS 36.108

Issue 3-3-1: 15 kHz tone vs 3.75 kHz tone

Agreement:

- Option 1: Keep single tone 15 kHz (with different variants) and remove 3.75 kHz single-tone configuration for SAN reference sensitivity for NTN TDD NB-IoT in TS 36.108.

- Option 2: 3.75 kHz can be kept if companies are bringing contributions to support this configuration with a Tx time below 8 ms.

Issue 3-3-2: New A14-1 variants (FRC REFSENS)

- Option 1: Consider at least one of the new A14-1 configurations below for NTN TDD NB-IoT:

Table A.14-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A14-1 for FDD | A14-2 for FDD | A14-1 new Option1 for TDD | A14-1 new Option2 for TDD | A14-1 new Option3 for TDD |
| --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 kHz | 15 kHz | 15 kHz |
| Number of tone | 1 | 1 | 1 | 1 | 1 |
| Diversity | No | No | No | No | No |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK | QPSK | QPSK |
| Frequency offset | 0 | 0 | 0 | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 | 4 ms | 4 ms | 4 ms |
| Number of NPUSCH repetition | 1 | 1 | 1 | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 | 0 / 0 | 3 / 3 | 2 / 1 |
| Payload size (bits) | 32 | 32 | 16 bits | 40 bits | 24 bits |
| Allocated resource unit | 2 | 2 | 1 | 1 | 1 |
| Code rate (target) | 1/3 | 1/3 | ⅓ | ⅓ | ⅓ |
| Code rate (effective) | 0.29 | 0.29 | 0.42 | 0.33 | 0.25 |
| Transport block CRC (bits) | 24 | 24 | 24 bits | 24 bits | 24 bits |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 96 | 192 | 192 |
| Total symbols per resource unit | 96 | 96 | 96 | 96 | 96 |
| Tx time (ms) | 16 | 64 | 8 ms | 8 ms | 8 ms |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |

- Option 2: 3.75 kHz can be kept for NTN TDD NB-IoT if companies are bringing contributions to support this configuration with a Tx time below 8 ms.

Issue 3-3-3: SNR targets for new A14-1 variants (FRC REFSENS)

- Option 1: Consider the SNR target values corresponding to the new NTN TDD NB-IoT A14-1 configurations:

Table A.14-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A14-1 for FDD | A14-2 for FDD | A14-1 new Option1 for TDD | A14-1 new Option2 for TDD | A14-1 new Option3 for TDD |
| --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 kHz | 15 kHz | 15 kHz |
| Number of tone | 1 | 1 | 1 | 1 | 1 |
| Diversity | No | No | No | No | No |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK | QPSK | QPSK |
| Frequency offset | 0 | 0 | 0 | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 | 4 ms | 4 ms | 4 ms |
| Number of NPUSCH repetition | 1 | 1 | 1 | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 | 0 / 0 | 3 / 3 | 2 / 1 |
| Payload size (bits) | 32 | 32 | 16 bits | 40 bits | 24 bits |
| Allocated resource unit | 2 | 2 | 1 | 1 | 1 |
| Code rate (target) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 | 0.42 | 0.33 | 0.25 |
| Transport block CRC (bits) | 24 | 24 | 24 bits | 24 bits | 24 bits |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 96 | 192 | 192 |
| Total symbols per resource unit | 96 | 96 | 96 | 96 | 96 |
| Tx time (ms) | 16 | 64 | 8 ms | 8 ms | 8 ms |
| SNR target value (dB) | -2.1 dB as per TR 36.802 | as per TR 36.802 | -1,17 dB | 0,54 dB | -0,61 dB |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |

- Option 2: 3.75 kHz can be kept for NTN TDD NB-IoT if companies are bringing SNR values to support this configuration with a Tx time below 8 ms.

Issue 3-3-4: Equations and parameter values to compute REFSENS

Agreement: As starting point, the following equations and parameter values to compute REFSENS for TDD NTN NB-IoT SAN can be considered:

REFSENS [dBm] = -174 dBm/Hz noise density +10*LOG10(15*1000 Hz) bandwidth + SAN NF + IM + SNR_target,

where:

- SAN NF (Noise Figure):

  - NF LEO = 4.3 dB

  - NF GEO = 7.4 dB

- IM (Implementation Margin) = 2 dB

- (new TDD only) SNR_target values:

  - A14-1 new Option1 for TDD: -1.17dB;

  - A14-1 new Option2 for TDD: 0.54 dB;

  - A14-1 new Option3 for TDD: -0.61 dB,

Issue 3-3-5: Resulted REFSENS values (different options)

Agreement: In accordance with new A14-1 configurations, RAN4 to select between following (one or more) REFSENS values for TDD NB-IoT NTN band 249 SAN specification:

- Option 1:

  - GEO: -124.0 dBm;

  - LEO: -127.1 dBm;

- Option 2:

  - GEO: -122.3 dBm;

  - LEO: -125.4 dBm;

- Option 3:

  - GEO: -123.4 dBm;

  - LEO: -126.5 dBm;

- Other options not precluded.

| Reference channel | A14-1 for FDD | A14-2 for FDD | A14-1 new Option1 for TDD | A14-1 new Option2 for TDD | A14-1 new Option3 for TDD |
| --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 kHz | 15 kHz | 15 kHz |
| Number of tone | 1 | 1 | 1 | 1 | 1 |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK | QPSK | QPSK |
| Tx time (ms) | 16 | 64 | 8 ms | 8 ms | 8 ms |
| SNR target value (dB) | -2.1 dB as per TR 36.802 | .. as per TR 36.802 | -1,17 dB | 0,54 dB | -0,61 dB |
| REFSENS (dBm) | GEO: -124.9 LEO: -128.0 (current) |  | GEO: -124.0 LEO: -127.1 (new proposed) | GEO: -122.3 LEO: -125.4 (new proposed) | GEO: -123.4 LEO: -126.5 (new proposed) |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |

Issue 3-3-6: REFSENS (with updates from previous meeting RAN4#114)

Agreement:

- Option 1: For SAN TS 36.108, adapt FRC configuration with new TDD frame structure (maximum 8 ms transmission time), for example:

  - For conducted specifications:

For SAN supporting NTN NB-IoT operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.1 with parameters specified in Table 7.2.2-3 and 7.2.2-4 for SAN type 1-H in all FDD operating bands and specified in Table 7.2.2-5 and 7.2.2-6 for SAN type 1-H in all TDD operating bands.

Table 7.2.2-3: Reference sensitivity levels of SAN supporting NB-IoT operation in FDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -124.9 |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -130.9 |

Table 7.2.2-4: Reference sensitivity levels of SAN supporting NB-IoT operation in FDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -128.0 |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -134.0 |

Table 7.2.2-5: Reference sensitivity levels of SAN supporting NB-IoT operation in TDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 new Option | TBD |
| 200 | 3.75 | TBD in Annex A.14 | TBD |

Table 7.2.2-6: Reference sensitivity levels of SAN supporting NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 new Option | TBD |
| 200 | 3.75 | TBD in Annex A.14 | TBD |

  - For OTA specifications:

For SAN supporting standalone NB-IoT operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameter specified in Table 10.3.2-3 and Table 10.3.2-4 for FDD bands and Table 10.3.2-5 and Table 10.3.2-6 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

Table 10.3.2-3: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in FDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -124.9 - ΔOTAREFSENS |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -130.9 - ΔOTAREFSENS |

Table10.3.2-4: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in FDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -128.0 - ΔOTAREFSENS |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -134.0 - ΔOTAREFSENS |

Table 10.3.2-5: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in TDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 new Option | TBD- ΔOTAREFSENS |
| 200 | 3.75 | TBD in Annex A.14 | TBD - ΔOTAREFSENS |

Table10.3.2-6: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 new Option | TBD - ΔOTAREFSENS |
| 200 | 3.75 | TBD in Annex A.14 | TBD - ΔOTAREFSENS |

- Option 2: 3.75 kHz can be kept for NTN TDD NB-IoT REFSENS if companies are bringing contributions to support this configuration with a Tx time below 8 ms.

Sub-topic 3-4: Dynamic range in TS 36.108

- Issue 3-4-1: A15-1 & A15-2 configurations for dynamic range

Agreement:

- Option 1:

  - Reference channel A15-1 (with a 15kHz configuration) can be reused for NTN TDD IoT NTN dynamic range computation.

  - Reference channel A15-2 (with a 3.75kHz configuration) can be removed for NTN TDD IoT NTN dynamic range computation.

Table A.15-1 FRC parameters for NB-IoT dynamic range

| Reference channel | A15-1 for both FDD & TDD | A15-2 for FDD only |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Modulation | π/4 QPSK | π/4 QPSK |
| Diversity | No | No |
| Frequency offset | 0 | 0 |
| IMCS / ITBS | 7 / 7 | 7 / 7 |
| Payload size (bits) | 104 | 104 |
| Allocated resource units | 1 | 1 |
| Transport block CRC (bits) | 24 | 24 |
| Coding rate (target) | 2/3 | 2/3 |
| Coding Rate | 0.67 | 0.67 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks – C | 1 | 1 |
| Total symbols per resource unit | 96 | 96 |
| Total number of bits per resource unit | 192 | 192 |
| Tx time (ms) | 8 | 32 |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |

- Option 2:

  - Reference channel A15-1 (with a 15kHz configuration) can be reused for NTN TDD IoT NTN dynamic range computation.

  - If companies are providing a 3.75kHz configuration for Tx time below 8 ms, reference channel A15-2 can be kept for NTN TDD IoT NTN dynamic range computation.

- Issue 3-4-2: Equations and parameter values to compute dynamic range

Agreement: RAN4 to consider the following equations and parameter values to compute dynamic range for TDD NTN NB-IoT SAN:

- P_int = -174 dBm/Hz + 10 x log10 (BW) + NF + IoT;

- P_wan = -174 dBm/Hz + 10 x log10 (BW) + NF +IoT + SNR +IM;

with the following parameters:

  - SNR dynamic range NB-IoT = 4.8 dB (as per TR 36.802 and not -2,1 dB as for REFSENS);

  - IM=2.7 dB;

  - BW_Channel = 200 kHz for interferer;

  - BW_Channel =15 kHz for wanted signal;

  - IoT LEO=15 dB (IoT GEO =N/A);

  - NF LEO=4.3 dB (NF GEO=7.4 dB).

- Issue 3-4-3: Dynamic range double-check

Agreement:

- Re-evaluate dynamic range for NB-IoT LEO case. RAN4 to double-check the dynamic range values for TDD NTN NB-IoT SAN since computed values are P_int=-101,7 dBm; P_want=-105,4 dBm for LEO (it seems there is an error also for FDD NTN NB-IoT SAN values for LEO, and GEO should not have been considered):

Table 7.3.2-3: Dynamic range of SAN supporting standalone NB-IoT operation (LEO class payload)

| SAN channel bandwidth (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWChannel | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | FRC A15-1 in Annex A.15 | -89.4 | -85.7 | AWGN |
| 200 | FRC A15-2 in Annex A.15 | -95.3 | -85.7 | AWGN |

- Further discuss if dynamic range for NB-IoT GEO case should be de-scoped or not, in Rel-17 (and should have been the case also for NB-IoT in Rel-18) it was decided not to define any dynamic range requirement for SAN GEO class.

Table 7.3.2-2: Dynamic range of SAN supporting standalone NB-IoT operation  (GEO class payload)

| SAN channel bandwidth (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWChannel | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | FRC A15-1 in Annex A.15 | -97.3 | -93.6 | AWGN |
| 200 | FRC A15-2 in Annex A.15 | -103.2 | -93.6 | AWGN |

Sub-topic 3-5: General SAN requirements for NTN TDD NB-IoT

- Issue 3-5-1: Number of tones

Agreement: RAN4 may consider number of tones =1 for deriving the requirements, as long as transmission time less than 8ms.

- Issue 3-5-2: Repetition number

Agreement: RAN4 can consider Repetition number=1 to be used for TDD NB-IoT SAN Rx requirements compatible with D/U=8 (8 ms transmission/reception time) and transmission periodicity N=9 (90 ms) for the new TDD frame structure.

Sub-topic 3-6: Work Split for SAN specification TS 36.108

- Issue 3-6-1: Work Split for NB-IoT TDD NTN in TS 36.108

Agreement:

| Clause/Requirement | Company |
| --- | --- |
| Definitions/Abbreviations | Iridium |
| Band definition (Clause 5.2) | THALES |
| Conducted REFSENS update | THALES |
| Radiated REFSENS update |  |
| Dynamic range update | Iridium |
| Conducted ON/OFF requirements | Iridium |
| EVM Annex | ZTE |
| Other, miscellaneous |  |