3GPP TSG-RAN WG4 Meeting #115	R4-2507104

St. Julian’s, Malta, 19 May – 23 May 2025

Agenda Item:	7.10.5.3

Source:	Ericsson

Title:	UE demodulation requirements with time varying Doppler shift and propagation delay model

Document for:	Discussion

# 1	Introduction

RAN4#114bis agreed with the way forward on the TE-emulated channel modeling for NGSO NTN testing [1]. The way forward captures the initial discussion results on NTN UE demodulation requirements with the time-varying Doppler shift and propagation delay model.

This contribution focuses on the UE demodulation performance requirements with the time varying Doppler shift and propagation delay.

# 2	NGSO NTN test environment

Figure 1	Assumed test environment for NGSO NTN UE testing.

Figure 1 is an overview of the NGSO NTN test environment. During the test, DUT (UE) estimates the Doppler shift and propagation delay based on the satellite location/velocity (ephemeris) information and UE’s location information at a given time. In the DUT, the UE location information is pre-configured or provided via AT command, as specified in TS 38.508 (See C.1). The UE uses the estimated Doppler shift and propagation delay to perform the pre-compensation before the baseband processing.

Figure 2 shows an example of Doppler shift and propagation delay using LEO-600 maximum positive Doppler test parameters where the initial evaluation angle is 30 degrees.

Figure 2	Doppler shift and propagation delay for LEO-600 maximum positive Doppler scenario.

# 3	UE demodulation requirements

Impact on RAN4 demodulation requirements with TE-emulated time vary doppler shift and propagation delay models - Option 1: - Keep the same NR NTN PDSCH requirements (SNR) even if the time-varying Doppler shift and propagation delay model is applied. - Keep the same IoT NTN UE demodulation requirements (SNR) even if the time-varying Doppler shift and propagation delay model is applied. - Option 2: - Considering worst case for frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply. Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models - Option 3: - RAN4 shall start to agree parameters for using the NGSO channel model in order to understand whether the margin produced is acceptable to define requirements with. - RAN4 shall use the new NGSO channel model for all performance requirements, in it’s current state. - Option 4: - After concluding discussions on the time-varying Doppler shift and propagation delay, initiate simulation campaigns to analyse the impact on demodulation requirements.

## 3.1	Doppler shift and propagation delay estimation error

RAN4 assumes the TE provides the ephemeris information every 10.24 seconds for UE demodulation requirements. Although the TE sends the ephemeris information every 10.24 seconds, the TE updates and applies the Doppler shift and propagation delay once per 1ms. Therefore, UE need to continue to estimate the Doppler shift and propagation delay for pre-compensation every e.g., one slot.

We have investigated the Doppler shift and propagation delay estimation error between the TE and UE. Although RAN4#114 agreed that the UE uses the Keplerian model or Equation of motion to derive the satellite location/velocity information [1], we assumed that UE only uses the ephemeris information, that is, UE does NOT run the satellite propagator model, which can be considered the as the worst case.

Here is our assumption for the UE estimating the Doppler shift and propagation delay:

- UE receives the satellite ephemeris information every 10.24 seconds.

- UE receives the satellite ephemeris information at least 10.24 seconds ahead.

- UE does not run the satellite propagator model.

- UE estimates the Doppler shift and propagation delay from the ephemeris information.

- UE updates the Doppler shift and propagation delay every slot by interpolating until UE receives the new ephemeris information.

  - We investigated three interpolation methods: Linear (first order) interpolation, Quadratic (second order) interpolation, and Cubic spline interpolation.

Figure 3 and Figure 4 compare the interpolated Doppler shift and propagation delays at UE, respectively, with LEO-600 maximum positive Doppler scenario.

Figure 3	Doppler shift estimation in UE (LEO-600 maximum positive Doppler scenario).

Figure 4		Propagation delay estimation in UE (LEO-600 maximum positive Doppler scenario).

The lower figures in Figure 3 and Figure 4 are the Doppler shift and propagation delay estimation errors between TE and UE. Note we assume the TE updates the satellite position/velocity every 1.00 seconds by running the satellite propagation model and then calculates Doppler shift and propagation delay. Between the satellite position updates, the TE generates the Doppler shift and propagation delay with the linear interpolation every 1ms as discussed in WF [1]. The lower right figures highlight the difference among three interpolation methods.

The figures show that the estimation errors with linear interpolation are ~70 Hz for Doppler shift and ~3.5 us for propagation delay. We don’t think these errors are negligible considering the Tx frequency error of 0.1ppm (=200Hz@2GHz) and the cyclic prefix of 4.7us for SCS=15kHz.

On the other hand, if we assume UE uses the quadratic interpolation or cubic spline interpolation, the mismatch from TE is small enough, that is, ~2Hz/0.05us for the quadradic interpolation and ~1Hz/0.035us for the cubic spline interpolation methods.

Since RAN4 agreed that the UE uses the satellite propagator model, e.g., Keplerian model, to estimate the satellite position/velocity vectors, the estimation errors of Doppler shift and propagation delay are smaller than those observed from Figure 3 and Figure 4.

Observation 1: Estimation errors of Doppler shift and propagation delay between TE and UE could be less than 2Hz and 0.05us, respectively, if UE receives the ephemeris information every 10.24 seconds.

When RAN4 defined the NR PDSCH demodulation requirements and eMTC/NB-IoT (N)PDSCH demodulation requirements, RAN4 first averaged companies’ impairment results and then added the additional margin 0.5 to 1.0dB according to the modulation order. Since the estimation errors are small enough compared with the UE Tx frequency error requirements of 0.1ppm (=200Hz@2GHz) and cyclic prefix of 0.47us (SCS=15kHz), we can keep the same requirements without extra margin even if the time-varying Doppler shift and propagation delay model is applied.

Proposal 1: Keep the same NR NTN PDSCH requirements (SNR) even if we apply the time-varying Doppler shift and propagation delay model.

Proposal 2: Keep the same IoT-NTN (N)PDSCH requirements (SNR) even if we apply the time-varying Doppler shift and propagation delay model.

## 3.2	NR NTN UE demodulation requirements

Test cases for NR NTN with applying time varying Doppler Shift and propagation delay model Agreement - Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 38.101-5 Clause 8.2.1.2.2.1 Test 1-1 and Test 1-2. - FFS on test 1-3 and 1-4 - Note: test 1-3 is related with UE capability to support 32 HARQ process, and test 1-4 is related with UE capability with HARQ disable

In Rel-17, RAN4 defined four new PDSCH demodulation requirements for FR1-NTN as shown in Table 1.

Table 1	PDSCH test cases for FR1-NTN in TS 38.101-5 Table 8.2.1.2.2.1.1-3.

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | 0.3 |
| 1-2 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | 7.6 |
| 1-3 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | -0.4 |
| 1-4 | R.PDSCH.1-1.1 FDD(1) | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | 1.1 |
| Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. |  |  |  |  |  |  |  |

Regarding the channel model applicability, we think the new model should apply to at least Tests 1-1 and 1-2. We may not need to apply to Tests 1-3 and 1-4, because Tests 1-3 and 1-4 reuse the same test setup as Test 1-1, but Test 1-3 configures 32 HARQ processes and 1-4 applies HARQ retransmission disabling. We do not think the time-varying Doppler shift and propagation delay model affects to the UE demodulation performances with larger HARQ processes and HARQ retransmission disabling.

Proposal 3: Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 38.101-5 Clause 8.2.1.2.2.1 Test 1-1 and Test 1-2.

## 3.3	IoT NTN UE demodulation requirements

### 3.3.1	NB-IoT

Test case for NB-IoT NTN with applying the time-varying Doppler shift and propagation delay model Agreement - Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 36.102 Clause 8.3.1.1 Test 1 - FFS If the LEO-600 model is not enough to cover the test time of Test 1, RAN4 should consider using the LEO-1200 model - Test case for eMTC NTN with applying the time-varying Doppler shift and propagation delay model

One of the differences of IoT-NTN UE demodulation requirements from NR NTN is the IoT-NTN UE demodulation requirements uses repetitions considering the coverage enhancement. This feature makes the test time longer to measure the SNR to achieve 70% of the peak throughput.

The latest RAN5 conformance test specification has defined the minimum test time for NDDSCH for UE Category NB1 in TS 36.521-4 Table G.3.5-1. According to the requirements, NB-IoT Test 1 requires 243,360 subframes, which corresponds to 244 sec. As shown in 3.1, the maximum simulation time is around 250 seconds for LEO-600 30 degrees elevation angle condition. Since the minimum test time is determined by the time it took the throughput curve to settle within +/- 2% of the target throughput value across all seeds (See TS 36.521-4 G.3.5.1), we think RAN4 can apply LEO-600 model (instead of LEO-1200) for IoT-NTN NB-IoT UE demodulation requirements.

Proposal 4: Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 36.102 Clause 8.3.1.1 Test 1.

### 3.3.2	Cat-M1

Test case for eMTC NTN with applying the time-varying Doppler shift and propagation delay model Agreement - FFS to consider Cat-M1 demodulation requirement with applying time varying Doppler shift and propagation delay model - If considering Cat-M1 demodulation requirement with applying time varying Doppler shift and propagation delay model - Cover test 1 and test 2 according to TS 36.102 Clause 8.2.1.1 - FFS to cover test 3

Table 2	PDSCH test case for IoT NTN Cat-M1 (in TS 36.102 Table 8.2.1.1.1.1-2)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 1.4MHz 16QAM 1/2 | R.1 FDD | OP.1 FDD | NTN-TDLC5-30 | 1x1 | 70 | 10.4 | M1 |
| 2 | 1.4MHz QPSK 1/3 | R.2 FDD | OP.1 FDD | NTN-TDLA100-200 | 1x1 | 70 | -4.2 | M1 |
| 3 | 1.4MHz QPSK 1/10 | R.3 FDD | OP.1 FDD | NTN-TDLA100-10 | 1x1 | 70 | -11.5 | M1 |

Regarding Cat-M1, although RAN5 has not defined the conformance test specifications, we think RAN4 should specify PDSCH demodulation performance requirement for NGSO, according to WID objective [2].

Regarding the applicable test cases, we propose to apply the time-varying channel model to Tests 1 and 2. RAN4 does not need to apply the GNSO channel model to Test 3, because this test is applicable for CE Mode B deep coverage scenario, and it requires longer test times.

Regarding Tests 1 and 2, according to TS 36.102 A.1.2.1, TE schedules PDSCH in two subframes per 10ms for Test 1, and 8 subframes per 32ms for Test 2. If we compare these test times with NB-IoT Test 1, TE schedules one NB-IoT NPDSCH transport block once per 96ms (See TS 36.102 Table 8.3.1.1.1.1-1). This means the test times of eMTC Tests 1 and 2 are shorter than NB-IoT Test 1, and LEO-600 scenario is sufficient to cover eMTC tests 1 and 2.

Proposal 5: Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 36.102 Clause 8.2.1.1 Test 1 and Test 2.

# 4	Summary

Observations:

Observation 1: Estimation errors of Doppler shift and propagation delay between TE and UE could be less than 2Hz and 0.05us, respectively, if UE receives the ephemeris information every 10.24 seconds.

Proposals:

Proposal 1: Keep the same NR NTN PDSCH requirements (SNR) even if we apply the time-varying Doppler shift and propagation delay model.

Proposal 2: Keep the same IoT-NTN (N)PDSCH requirements (SNR) even if we apply the time-varying Doppler shift and propagation delay model.

Proposal 3: Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 38.101-5 Clause 8.2.1.2.2.1 Test 1-1 and Test 1-2.

Proposal 4: Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 36.102 Clause 8.3.1.1 Test 1.

Proposal 5: Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 36.102 Clause 8.2.1.1 Test 1 and Test 2.

# References

- R4-2504700, “Way Forward for [114bis][323] NTN_testing_NGSO_channel_model_and_demod”, Samsung.

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, RAN4 chair.

# Appendix

## A.1	Minimum Test time for PDSCH (TS 38.521-5)

Table G.1.5-1: Minimum Test time for PDSCH demodulation (TS38.521-5)

| Test number | Reference Channel | Propagation condition | Minimum number of active subframes (MNAS) | MNAS to MNS Scaling factor (Note 3) | Minimum Number of Subframes (MNS) after rounding up to nearest thousand MNS= |
| --- | --- | --- | --- | --- | --- |
| 1-1 | R.PDSCH.1-1.1 FDD | NTN-TDLA100-200 | 30000 (Note 1) | 1.0526 | 32000 |
| 1-2 | R.PDSCH.1-2.1 FDD | NTN-TDLC5-200 | 72000 (Note 1) | 1.0526 | 76000 |
| 1-3 | R.PDSCH.1-1.1 FDD | NTN-TDLC5-200 | 80000 (Note 1) | 1.0526 | 85000 |
| 1-4 | R.PDSCH.1-1.1 FDD | NTN-TDLA100-200 | 30000 (Note 1) | 1.0526 | 32000 |
| Note 1: MNAS determined by simulations. Note 2: For cases where MNAS is not determined by simulations, use same MNAS as the similar case simulated (same doppler speed) Note 3: MNS/MNAS ratio decided by scheduling pattern and is ratio of all slots to DL slots. |  |  |  |  |  |

## A.2	Minimum Test time for NPDSCH (TS 36.521-4)

Table G.3.5-1: Minimum Test time for PDSCH for UE Category NB1

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | Min Gap between consecutive NPDCCH taking into account RTT (ms) | Actual Gap between consecutive NPDCCH taking into account RTT (ms) | MNSF (Min No Sub frames) HD-FDD (Note2) |
| --- | --- | --- | --- | --- | --- |
| 8.3.1.1.1 Test 1 | R.NB.1 FDD (200kHz, QPSK,1/2) (1x1) NTN TDLC5-200 | 845 (Note 1) | 241 | 288 | 243360 |
| 8.3.1.1.1 Test 2 | R.NB.2 FDD (200kHz, QPSK,1/3) (1x1) NTN TDLA100-10 | 523 (Note 1) | 881 | 1152 | 602496 |
| 8.3.1.1.2 Test 1 | R.NB.6 FDD (200kHz, QPSK,1/2) (1x1) EPA5 | 683 (Note 1) | 489 | 576 | 393408 |
| 8.3.1.1.2 Test 2 | R.NB.6-1 FDD (200kHz, QPSK,1/2) (1x1) ETU1 | 440 (Note 1) | 1982 | 2304 | 1013760 |
| Note1: Minimum test time has been derived by simulating the test scenario taking into account the minimum scheduling gap needed between successive NPDCCH/NPDSCH Note2: For MNSF calculations, with the current assumption of SIB2/SIB31 scheduling every 640ms with max 8 subframes and max 2 repetition per SI window, 16 additional subframes are needed every 640ms to avoid collisions with NPDSCH. |  |  |  |  |  |