3GPP TSG-RAN WG4 Meeting #115	R4-2507106

St. Julian’s, Malta, 19 May – 23 May 2025

Agenda Item:	7.21.7

Source:	Ericsson

Title:	PMI reporting requirements with CSI prediction

Document for:	Discussion

# 1	Introduction

RAN4#114bis agreed with the way forward on AI/ML for NR air interface [1]. The way forward captured the agreements on CSI prediction as follows:

Issue 1-1: Codebook to be used in the tests Agreement: - In Rel-19 only PMI requirements based on AI/ML CSI prediction will be defined - use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions Issue 1-2: Requirement baseline Agreement: - Requirement to be introduced for Doppler domain basis N4=1 - N4>1 can be further discussed in the future if needed Issue 1-4: Test metric Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4) - γAI/ML=tue_AL/MLtrnd - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/ML using the precoders configured according to the UE reports, - trnd is the throughput measured at SNRue_AI/ML with random precoding. - trnd is obtained with Rel-15 type I single panel codebook - Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1,follow2 in section 6.3 of 38.101-4

This contribution discusses the PMI reporting requirements with CSI prediction.

# 2	PMI reporting requirements

## 2.1	SGCS of predicted channel estimates

RAN4 has already agreed that the test metric of AI/ML-based PMI reporting requirements is the throughput ratio of follow PMI with predicted PMI and random PMI. However, RAN4 agreed to try to align the SGCS of the predicted channel estimates before RAN4 evaluates the throughput ratio. The alignment of SGCS consists of two steps [2]:

- Step 1: SGCS between SVD of model output (before PMI generation) and SVD of the ground truth raw channel, average over sub-bands, based on ideal channel estimates.

- Step 2: SGCS between predicted PMI and SVD of the ground truth raw channel, average over sub-bands, based on ideal channel estimates.

After we confirm the alignment of SGCS, RAN4 will check the PDSCH throughput with follow PMI as Step 3.

- Step 3: Check PDSCH throughput with the predicted PMI.

Table 1 summarizes our simulation results of step 1 according to [2]. We have evaluated the average of SGCS based on two CSI prediction models. The first model is trained with the dataset that is same as the test. For example, when we run the simulation with TDLA30-20, we use the model trained with TDLA30-20. On the other hand, the second model is trained with the mixed dataset with TDLA30-20, -50, and -100. The simulation results do not show the difference of SGCS due to the trained dataset at least for TDLA30.

The simulation summary also shows the SGCS of ‘sampling and hold’ for each scenario. The summary shows the SGCS values of predicted CSI model are higher than those of ‘sample and hold’, for 20Hz and 50Hz scenarios, although the SGCS values of predicted CSI model are same as those of ‘sample and hold’ for 100Hz. This means the CSI prediction does not work well for TDLA30-100.

Table 1	Summary of average SGCS between the SVD of model output and SVD of ground-truth (Step 1).

| Number of CSI-RS ports | Channel model for test | Layer (Rank 2) | SGCS of Predicted CSI (dataset is same as the channel model for test) | SGCS of Predicted CSI (mixed dataset) | SGCS of ‘sample and hold’ |
| --- | --- | --- | --- | --- | --- |
| 32 | TDLA30-20 | 1 | 0.99 | 0.99 | 0.80 |
| (4x4x2) |  | 2 | 0.99 | 0.98 | 0.66 |
|  | TDLA30-50 | 1 | 0.94 | 0.93 | 0.40 |
|  |  | 2 | 0.87 | 0.87 | 0.24 |
|  | TDLA30-100 | 1 | 0.36 | 0.35 | 0.34 |
|  |  | 2 | 0.22 | 0.21 | 0.22 |
| 16 | TDLA30-20 | 1 | 0.99 | 0.99 | 0.79 |
| (4x2x2) |  | 2 | 0.99 | 0.99 | 0.65 |
|  | TDLA30-50 | 1 | 0.93 | 0.93 | 0.36 |
|  |  | 2 | 0.86 | 0.86 | 0.22 |
|  | TDLA30-100 | 1 | 0.29 | 0.28 | 0.27 |
|  |  | 2 | 0.20 | 0.20 | 0.18 |

Observation 1: The average SGCS does not change regardless of the trained dataset for TDLA30 with 20/50/100Hz.

Observation 2: AI/ML-based PMI prediction works for TDLA30-20 and TDLA30-50 regarding to the average of SGCS. The average SGCS from AI/ML-based CSI prediction model is close to that from ‘sample and hold’ for TDLA30-100.

From the observations, we propose to exclude TDLA30-100 from the requirements because we do not think the model can predict the PMI with TDLA30-100.

Proposal 1: RAN4 does not use TDLA30-100 for Rel-19 AI/ML-based CSI prediction performance requirements.

## 2.2	Scheduling for PMI reporting requirements

Figure 1 and Figure 2 are the scheduling of CSI-RS, PMI reporting, and PDSCH, for FDD and TDD scenarios, respectively [3], used for Rel-18 Type-II Doppler PMI reporting requirements. RAN4 agreed these configurations based on the CSI processing time specified in TS 38.214 5.4.

In our understanding, RAN1 is still discussing the CSI processing time for Rel-19 CSI prediction together with CSI processing unit. Since the CSI processing time requirements affect the PMI reporting delay, RAN4 should wait for RAN1 conclusion to discuss the detailed simulation assumption for Step 3.

Figure 1	Test setup for FR1 FDD case of TypeII-Doppler-r18 codebook.

Figure 2	Test setup for FR1 TDD case of TypeII-Doppler-r18 codebook.

Observation 3: RAN1 is still discussing the CSI processing time for Rel-19 CSI prediction.

Proposal 2: RAN4 waits for the RAN1 conclusion on CSI processing time for AI/ML-based CSI prediction to discuss the detailed simulation assumption for the follow PMI performance evaluation.

# 5	Summary

Observations:

Observation 1: The average SGCS does not change regardless of the trained dataset for TDLA30 with 20/50/100Hz.

Observation 2: AI/ML-based PMI prediction works for TDLA30-20 and TDLA30-50 regarding to the average of SGCS. The average SGCS from AI/ML-based CSI prediction model is close to that from ‘sample and hold’ for TDLA30-100.

Observation 3: RAN1 is still discussing the CSI processing time for Rel-19 CSI prediction.

Proposals:

Proposal 1: RAN4 does not use TDLA30-100 for Rel-19 AI/ML-based CSI prediction performance requirements.

Proposal 2: RAN4 waits for the RAN1 conclusion on CSI processing time for AI/ML-based CSI prediction to discuss the detailed simulation assumption for the follow PMI performance evaluation.

# References

- R4-2505105, “WF on requirements for AI/ML air interface normative work”, Qualcomm Incorporated.

- R4-2505229, “Updated simulation assumptions for AIML CSI prediction”, OPPO, et. al.

- R4-2406111, “WF on [110bis][328] NR_MIMO_evo_DL_UL_demod”, Samsung.