3GPP TSG-RAN WG4 Meeting #115                                          R4-2507119

# Malta, 19th – 23rd of May 2025

Title: 	Updates on RRM requirements for NTN NB-IoT TDD

Agenda Item:	7.33.6 – RRM core requirements

Release:	Rel-19

Source: 	THALES, Iridium Satellite LLC

Document for:	Discussion

# WID information

A new WID Introduction of IoT-NTN TDD mode (RP-242415, New WID on introduction of IoT-NTN TDD mode, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024) was approved in RAN#105. That WID has been revised in RAN#106, as the study phase had been completed. Then, a work plan for Revised WID on Introduction of IoT-NTN TDD mode (RP-243293, Revised WID on introduction of IoT-NTN TDD mode, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024) is proposed for RAN 1, 2, and 4.

## Core part WI

The work item aims to specify enhancements for NB-IoT NTN to enable NTN operation with a NB-IoT TDD mode leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures for the NB-IoT operation in the targeted unpaired MSS allocated band (TN deployment is not expected in this band). The feature is not intended to be applicable to existing 3GPP bands.

The work objectives assume the following:

- LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763)

- Target the 1616-1626.5 MHz MSS allocated band

- Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT)

- Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with N=9 for the target MSS allocated band.

This work item includes the following objectives:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including:

  - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2]

      - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period. RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band.

  - Other necessary impacts on higher layers [RAN2]

  - RRM and RF core requirements [RAN4]

- Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4].

  - Specify band numbering

  - Specify SAN and UE RF characteristics

  - Specify DL and UL channelization.

  - Specify channel bandwidth as 200 kHz

  - Note1: No NTN-NTN coexistence study needed.

  - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands

Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements)

## Performance part WI

Specify the following:

- RRM performance requirements, as needed [RAN4]

- Necessary SAN and UE demodulation requirements [RAN4]

- SAN conformance testing requirements [RAN4]

## Current Work Plan

Current Work Plan from R4-2502291 (from RAN4#114):

| RAN#107 (Mar. 2025) | RAN1#121 | 0.5 | - Conclude on the solutions for the TDD support in NB IoT NTN - Further discuss (if needed) related RRC parameters and send LS to RAN2 - Finalize RAN1 CRs |
| --- | --- | --- | --- |
|  | RAN2#130 | 0.5 | - Complete the definition and RRC configuration of signaling of the periodic pattern, and associated UE procedures (if any) - Conclude support of TDD mode in NB IoT NTN - Discuss stage-2 design of TDD support in NB IoT NTN - Discuss RAN2 CRs |
|  | RAN4#115 | 0.5+0.25 | - Continue discussion on the UE AMPR, UE frequency error requirement, - Continue discussion on repetition number to be considered (associated with D/U=8 for 8 ms Rx/Tx time and N=9 for 90 ms periodicity) for both UE and SAN, - Continue discussion (at least) FRC for REFSENS, in-channel selectivity, dynamic range, and related parameters such as number of tones, Tx/Rx time, related SNR target for specific FRC configuration, - Derive related values for the at least for previous mentioned requirements: REFSENS, in-channel selectivity, dynamic range, etc., - Start discussion on UE and SAN CRs for RF part and SAN conformance testing, - Start CR split to prepare Draft CRs for the Core requirements |

Previous SAN & UE agreements from RAN4#114 were described in WF R4-2502292 (“Way Forward for [114][318] IoT_NTN_TDD_UE_SAN_RF”, THALES) and from RAN4#114-bis in WF R4-2504719 (“Way Forward for [114bis][316] IoT_NTN_TDD_UE_SAN_RF”).

Previous RRM WF from RAN4#114-bis was captured in R4-2504915 “WF on RRM requirements for IoT_NTN_TDD”.

# RRM TDD NB-IoT specific requirements in TS 36.133

## RRM agreements from RAN4#114

Agreement:

- For the following RRM requirements, modifications are expected due to the new periodic pattern(s):

  - RLM (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.)

  - MSG3-based channel quality report (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.)

  - Cell re-selection (example modifications include extended cell detection/evaluation/measurement periods, side conditions, etc.)

  - RRC Re-establishment (example modifications include extended cell detection/measurement periods, side conditions, etc.)

  - RRC Connection Redirection to Non-anchor Carrier (example modifications include updates to the definition of the symbols, side conditions, etc.)

  - Cell measurements (example modifications include extended cell detection/measurement periods, side conditions, etc.)

Note 1: The exact requirement modifications are subject to further discussion in future RAN4 meetings based on RAN1 conclusion/progress on this feature.

Note 2: Different modifications can be made for different TDD patterns, if multiple, DRX configurations, SNR values, etc.

Note 3: Not all existing requirements for NB-IoT NTN may be applicable to the IoT_NTN_TDD feature, which is subject to further discussions in future RAN4 meetings.

Note 4: It is assumed that UE will be made aware of the periodic pattern of frequency layers to be measured.

Note 5: UE power consumption aspect may be considered when modifying the above requirements as needed.

Note 6: The requirement applicability rule defined in Rel-17/18 applies to the IoT_NTN_TDD feature as well. For instance, the above requirements may not be applicable for inter-satellite scenarios if the UE is not provided with the target inter-satellite’s information.

At the previous RAN4#114 meeting the following WF has been approved:

R4-2502607  WF on RRM requirements for IoT_NTN_TDD

Type: other                   For: Approval
                                                           Source: Qualcomm

Decision:                   Approved.

With the following agreements during RAN4#114 RRM:

Issue 0: General aspects

Agreement:

For TDD NB-IoT in NTN, the following aspects will be further considered in future RAN4 meetings:

- The target SNR

- Applicable coverage (whether the EC needs to be considered)

- Applicable Rmax

- Applicable DRX cycle

  - Note: The DRX cycle might not be non-uniformly periodic due to the H-SFN not matching with the new TDD pattern. This issue will be addressed by other working groups. In the meantime, RAN4 assumes the DRX cycle is uniformly periodic.

Issue 1: Cell reselection (4.6A.2)

Agreement:

For the cell reselection for TDD NB-IoT in NTN, discuss the following details in the next meeting:

- Tdetect, Tevaluation and Tmeasure

Issue 3: RRC re-establishment (6.5A)

Agreement:

In the core requirement, For the delay requirement (TUL_grant + TUE_re-establish_delay_NB-IoT) of RRC re-establishment for TDD NB-IoT in NTN,

- No update to TUL_grant

- For the following parameters in TUE_re-establish_delay_NB-IoT

  - No update:

    - TPRACH_NB-IoT

    - TSI_NB1-NC [and TSI_NB1-EC]

  - Consider update, and the details are FFS:

    - Tsearch_NB1-NC,i [and Tsearch_NB1-EC,i] for unknown cell

Issue 4: RRC Connection redirection to Non-Anchor Carrier (6.9A)

Agreement:

For the latency requirement (TRRC_procedure_delay + Tperiod_DL_bitmap + TUL_grant + TDL-UL switch + TNPUSCH) of RRC Connection redirection to Non-Anchor Carrier for TDD NB-IoT in NTN,

- No update to TRRC_procedure_delay, TUL_grant, and TNPUSCH

- Update the following parameters

  - Tperiod_DL_bitmap: to be updated based on RAN1 design

  - TDL-UL switch: to be updated based on RAN1 design

Issue 5: RLM (7.23A)

Agreement:

Further discuss the following aspects for RLM for TDD NB-IoT in NTN:

- FFS update the hypothetical NPDCCH format (including Rmax) specified in Table 7.23A.2-1 upon RAN1 design in future meetings.

- Evaluation period when DRX is not in use:

  - Option 1: Reuse the existing requirement defined in 7.23A.2.1, unless RAN1 changes cause impact to this requirement

  - Option 2: The evaluation period defined in 7.23A.2.1 is scaled

    - Option 2a: scaled by N=9

    - Option 2b: scaled by FFS

    - Option 2c: no scaling, and define requirement based on 8ms DL subframes

- Evaluation period when DRX is in use:

  - FFS whether the evaluation period defined in 7.23A.2.1 needs to be scaled.

Issue 6: Channel quality report (8.14A.4)

Agreement:

Channel report requirement for TDD NB-IoT in NTN is defined as follows:

- FFS Reuse the existing measurement period

- In future meetings, may need to revisit the following aspects based on RAN1 design:

  - Hypothetical NPDCCH format specified in Table 8.14A.4-1

  - Report mapping tables defined in [9.1.22.15]

Issue 7: PUR (4.6A.3)

Agreement:

The existing PUR requirement defined for NB-NTN applies to the TDD NB-IoT NTN, if PUR requirements is in the WI scope.

Issue 8: Uplink transmit timing (7.20A)

Agreement:

The existing UE transmit timing requirement defined for NB-NTN applies to the TDD NB-IoT NTN.

- If any updates are identified upon RAN1 agreements related to 'uplink transmission gap' and 'segmented uplink transmission,' the corresponding updates will be made accordingly.

## RRM agreements from RAN4#114-bis

## Issue 0: General aspects

Agreement (online):

For TDD NB-IoT in NTN, discuss whether to define the RRM requirements for:

- Earth moving and fixed cell, normal coverage with SNR ≥ -1dB

Agreement:

Further discuss the following questions in the next meeting:

- Q1: Is the measurement/evaluation solely based on NPSS, NSSS or NRS?

- Q2: How many samples of the reference signal(s) were needed in a normal coverage scenario with SNR ≥ [-6, -1] dB?

- Q3: Is the UE required to monitor the reference signals during DL NB-IoT subframes that are in the middle of ongoing UL transmissions?

|  | Question Q1 | Question Q2 | Question Q3 |
| --- | --- | --- | --- |
| Issue 1: Cell reselection (4.6A.2) |  |  |  |
| Issue 2: Cell measurement in connected mode (8.14A) |  |  |  |
| Issue 3: RRC re-establishment (6.5A) |  |  |  |
| Issue 4: RRC Connection redirection to Non-Anchor Carrier (6.9A) |  |  |  |
| Issue 5: RLM (7.23A) |  |  |  |

## Issue 1: Cell reselection (4.6A.2)

Agreement (online):

For the cell reselection for TDD NB-IoT in NTN,

- The requirements are applicable only if UE is provided with the active periods of the neighbour cells configured for the measurements.

- For earth fixed cell and earth moving cell:

  - The existing values for Tdetect, Tmeasure, and Tevaluate in Table 4.6A.2.2-1 and Table 4.6A.2.2-2 can be applicable if it is confirmed that the current requirement is achievable without requiring more than one NPSS/NSSS sample for half the DRX cycle length [due to the TDD pattern]

    - Further discuss the values if -1dB will be used.

Ksatellite = 1 for the cells measured in DL opportunities of a neighbour satellite that are in non-overlapping with DL opportunities to be measured in the serving satellite

Further discuss the following:

- For earth moving cell:

  - Option 2-A: The existing values for Tdetect, Tmeasure, and Tevaluate in Table 4.6A.2.2-1 and Table 4.6A.2.2-2 can be applicable if it is confirmed that the current requirement is achievable without requiring more than one NPSS/NSSS sample for half the DRX cycle length

  - Option 2-B: Re-evaluate the maximum latency in cell detection, measurement, and evaluation required for this scenario, as well as the minimum number of NPSS, NSSS, and NRS samples needed for cell detection and measurement at an SNR of [-1] dB, and enhance Tdetect, Tmeasure, and Tevaluate in Table 4.6A.2.2-1 and Table 4.6A.2.2-2.

## Issue 2: Cell measurement in connected mode (8.14A)

Further discuss the following:

For the cell measurement period in connected mode for TDD NB-IoT in NTN:

- Requirements are applicable if the UE is provided with assistance information on the DL periodic patterns of the cells to be measured.

- For intra-frequency measurement for both cases where DRX is not configured and is configured,

  - Option 1-A: Max {existing values, the time period for the UE to obtain the same number of NPSS and NSSS samples as those assumed for the existing requirements}

  - Option 1-B: Delay requirements are scaled by a factor of [2 or 9]

    - FFS on side condition, e.g. only when the UE measures cells across 3 or more different patterns of DL opportunities

  - FFS for earth moving cell

- For inter-frequency measurement for both cases where DRX is not configured and is configured,

  - Option 2-A: Max {existing value, periodicity of corresponding RS}

  - Option 2-B: Reused the existing Tmeasure and Tdetect

  - The minimum length of measurement occasions is [82] ms

  - FFS for earth moving cell

## Issue 3: RRC re-establishment (6.5A)

Further discuss the following:

For the delay requirement (TUL_grant  + TUE_re-establish_delay_NB-IoT) of RRC re-establishment for TDD NB-IoT in NTN,

- Tsearch_NB1-NC,i in TUE_re-establish_delay_NB-IoT

  - Option 1: Reuse the existing value (1400ms per frequency layer)

  - Option 2: Scale it by a factor of  9

  - FFS for earth moving cell

## Issue 4: RRC Connection redirection to Non-Anchor Carrier (6.9A)

Further discuss the following:

For the latency requirement (TRRC_procedure_delay + Tperiod_DL_bitmap + TUL_grant + TDL-UL switch + TNPUSCH) of RRC Connection redirection to Non-Anchor Carrier for TDD NB-IoT in NTN,

- Update Tperiod_DL_bitmap and TDL-UL switch to be compliant with the new IoT-NTN TDD frame structure and RAN1/2 specifications.

- The exact updates can wait for further RAN1/2 progress

## Issue 5: RLM (7.23A)

Further discuss the following:

For OOS/INS evaluation period requirements for TDD NB-IoT in NTN:

- When DRX is not in use:

  - Option 1: Reuse the existing requirement defined in Table 7.23A.2.1-1

  - Option 2: Update it to 1200/600 ms from 400/200 ms when configured NPDCCH Rmax ≤ 64

  - Option 3: FFS for earth moving cell

- When DRX is in use:

  - Option 1: Scale it by a factor of [3 or 9 or multiple of 9]

  - Option 2: FFS for earth moving cell

## Issue 6: Channel quality report (8.14A.4)

Further discuss the following:

For channel report requirement for TDD NB-IoT in NTN:

- No modification to the measurement period.

# Discussion

Proposal 1: RAN4 to consider the following replies to the questions from RRM WF R4-2504915:

|  | Question Q1 General answer: Generally speaking, the UE should not do something different in TDD that is already existing/used in FDD. | Question Q2 General answer: Generally speaking, this input will be very useful if UE vendors could share their view. | Question Q3 General answer: N/A during IDLE mode. |
| --- | --- | --- | --- |
| Issue 1: Cell reselection (4.6A.2) | NRS by default (legacy). Optionally, NSSS can be used. (NW has no idea what measurement UE is using; NPSS is currently used only for synchronization) | To be further discussed/proposed by UE vendors. | N/A (IDLE mode only, not applicable) |
| Issue 2: Cell measurement in connected mode (8.14A) | NRS by default (legacy). Optionally, NSSS can be used depending on NW configuration and UE implementation. (NW has no idea what measurement UE is using; NPSS is currently used only for synchronization) | To be further discussed/proposed by UE vendors. | Up to UE implementation, not mandatory Observation: Only one DL opportunity with at least 3 or 4 NRS subframes and max 1 NSSS and/or NPSS. |
| Issue 3: RRC re-establishment (6.5A) | NRS by default (legacy). Optionally, NSSS can be used depending on NW configuration and UE implementation. (NW has no idea what measurement UE is using; NPSS is currently used only for synchronization) | To be further discussed/proposed by UE vendors. | N/A Can be in connected mode, but no consecutive UL transmissions expected during RRC re-establishment. |
| Issue 4: RRC Connection redirection to Non-Anchor Carrier (6.9A) | Optionally, NSSS can be used depending on NW configuration and UE implementation. (NW has no idea what measurement UE is using; NPSS is currently used only for synchronization) | To be further discussed/proposed by UE vendors. | N/A Can be in connected mode, but no consecutive UL transmissions expected during RRC re-establishment. |
| Issue 5: RLM (7.23A) | Optionally, NSSS can be used depending on NW configuration and UE implementation. (NW has no idea what measurement UE is using; NPSS is currently used only for synchronization) | To be further discussed/proposed by UE vendors. | Up to UE implementation, not mandatory Observation: Only one DL opportunity with at least 3 or 4 NRS subframes and max 1 NSSS and/or NPSS. |

In TR 38.821, the following information can be found:

Table 7.1-1: NTN scenarios versus delay constraints, Source [2]

| NTN scenarios | A | B | C1 | C2 | D1 | D2 |
| --- | --- | --- | --- | --- | --- | --- |
|  | GEO transparent payload | GEO regenerative payload | LEO transparent payload |  | LEO regenerative payload |  |
| Satellite altitude | 35786 km |  | 600 km |  |  |  |
| Relative speed of Satellite with respect to earth | negligible |  | 7.56 km per second |  |  |  |
| Min elevation for both feeder and service links | 10° for service link and 10° for feeder link |  |  |  |  |  |
| Typical Min / Max NTN beam foot print diameter (note 1) | 100 km / 3500 km |  | 50 km / 1000 km |  |  |  |
| Maximum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE | 541.46 ms (Worst case) | 270.73 ms | 25.77 ms |  | 12.89 ms |  |
| Minimum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE | 477.48 ms | 238.74 ms | 8 ms |  | 4 ms |  |
| Maximum Round Trip Delay variation as seen by the UE (note 2) | Negligible |  | Up to +/- 93.0 µs/sec (Worst case) |  | Up to +/- 47.6 µs/sec |  |
| NOTE 1: The beam foot print diameter are indicative. The diameter depends on the orbit, earth latitude, antenna design, and radio resource management strategy in a given system. NOTE 2: The delay variation measures how fast the round trip delay (function of UE-satellite-NTN gateway distance) varies over time. The worst case is when the satellite moves towards/away from the UE at 10⁰ elevation angle, assuming the UE speed is 1200 km/h. It is expressed in µs/s and is negligible for GEO scenario. NOTE 3: Void NOTE 4: Speed of light used for delay calculation is 299792458 m/s. |  |  |  |  |  |  |

And also the following information:

“Neglecting UE movement, a UE served by an NTN LEO cell of diameter 50 km and 1000 km may remained connected for a maximum of 6.61 seconds and 132.38 seconds respectively due to satellite movement. Considering UE movement, this will vary by approximately +/- 4%. By neglecting satellite speed and setting UE speed to 500 km/hr as per table 7.1-1, this is equivalent to a terrestrial UE being served by a cell diameter ranging from approximately 0.918 km to 18.39 km.

Table 7.3.2.1.4-1: Time to HO for min/max cell diameter and varying UE speed

| Cell Diameter Size (km) | UE Speed (km/hr) | Satellite Speed (km/s) | Time to HO (s) |
| --- | --- | --- | --- |
| 50 (lower bound) | +500 | 7.56 | 6.49 |
|  | -500 |  | 6.74 |
|  | +1200 |  | 6.33 |
|  | - 1200 |  | 6.92 |
|  | Neglected |  | 6.61 |
| 1000 (upper bound) | +500 |  | 129.89 |
|  | -500 |  | 134.75 |
|  | +1200 |  | 126.69 |
|  | - 1200 |  | 138.38 |
|  | Neglected |  | 132.28 |

”

Using same equations we can derive worst case for NB-IoT TDD NTN use case:

| Cell Diameter Size (km) | NB-IoT UE Speed (km/hr) | Satellite Speed (km/s) | Time to HO (s) |
| --- | --- | --- | --- |
| 200 (lower bound) | +500 | 7.56 | 25.97 |
|  | -500 |  | 26.95 |
|  | +1200 |  | 25.33 |
|  | - 1200 |  | 27.67 |
|  | Neglected |  | 26.45 |
| 600 (upper bound) | +500 |  | 77.93 |
|  | -500 |  | 80.85 |
|  | +1200 |  | 76.01 |
|  | - 1200 |  | 83.02 |
|  | Neglected |  | 79.36 |

Proposal 2: Given the work assumption with Earth Moving Beams for NTN TDD NB-IoT WI, RAN4 RRM should consider a maximum visibility time of a cell for 83.02 seconds and a minimum visibility time of a cell for 25.33 seconds.

Observation 1: RAN4 could add to TS 36.133 timing error requirement the following phrase: “For band 249, NTN NB-IoT UE in TDD mode adjusts its time pre-compensation before/at the beginning of each set of consecutive uplink sub-frames.” However, it may not be sufficient and it could also restrict implementation.

Proposal 3 (too restrictive): Add to TS 36.133 timing error requirement the following phrase: “For band 249, NTN NB-IoT UE in TDD mode adjusts its time pre-compensation before/at the beginning of each set of consecutive uplink sub-frames.”

We could therefore better propose:

Proposal 3bis (still too restrictive): Add to TS 36.133 timing error requirement the following phrase: “For band 249, NTN NB-IoT UE in TDD mode shall not adjust its time pre-compensation during the 8 consecutive uplink transmitted sub-frames.”

Or even:

Proposal 3ter (better, but in this case the requirement not clear anymore): Add to TS 36.133 timing error requirement the following phrase: “For band 249, NTN NB-IoT UE in TDD mode adjusts its time pre-compensation before/at the beginning of/during each set of consecutive uplink sub-frames.”

Or better:

Proposal 3quater: Do not define any specific TS 36.133 timing error pre-compensation requirement for band 249, NTN NB-IoT UE in TDD mode. For band 249, NTN NB-IoT UE in TDD mode can reuse the current Te timing error limit, and UE may adjust its time pre-compensation before/at the beginning of/during each set of consecutive uplink sub-frames.

FSS: whether pre-compensation gap is needed and the specific duration of the gap (which could be limited to a few symbols) depending on the UE capabilities.

Moreover, with an assumption of up to +/-47.6µs/sec delay variation (for regenerative payload) or up to +/-93.0µs/sec delay variation (for a transparent payload) as per TS 38.821 with 1200km/h UE and 10° for service link + 10° for feeder link assumption (worst case), the perceived timing error would be minimal during 8ms transmission time with respect to 97*TS (97*32.552ns=3.1575 µs), see Table 7.20A.2-1 for Te Timing Error Limit (TS 36.133):

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | 97*TS |
| Note 1: TS is the basic timing unit defined in TS 36.211 |  |

As a matter of fact, one can find below a computation of timing error per different configurations:

| Time considered for timing variation computation | Maximum timing shift | Observations |
| --- | --- | --- |
| 1 Tx burst for regenerative payload (8ms of consecutive 8 UL sub-frames) | 0.381 µsec | < 3.1575 µs (97*TS) Maximum timing shift = approx. 12% of Te timing error limit |
| 1 Tx burst for transparent payload (8ms of consecutive 8 UL sub-frames) | 0.744 µsec | < 3.1575 µs (97*TS) Maximum timing shift = approx. 23.5% of Te timing error limit |

If we update the SNR threshold for normal coverage to -1dB (instead of existing - 6dB), cell detection time can be scaled up by the factor of 9.

Proposal 4: The cell detection/cell reselection (4.6A.2) time from TS 36.133 does not need to be scaled up by the factor of 9. Existing value for Nserve_NB_NC should be sufficient for TDD as per table 4.6A.2.1-1 and 4.6A.2.1-2 as scaling up by 9 can result in UE going out of coverage before even starting neighbour cell measurements.

With SNR>-1dB assumption, there is no need to change the values for Tdetect, Tmeasure and Tevaluate from the existing values.

Observation 2: NPSS is only needed for cell detection and no requirement for half DRX. Only for measurements, two measurement samples should be apart by minimum half DRX cycle where measurements could be based on NSSS or NRS. If more NRS or NSSS is required for measurement accuracy, UE implementation can combine measurements across multiple downlink opportunities at the expense of UE power consumption.

Proposal 5: TS 36.133 - cell reselection (4.6A.2) - For earth moving cell, the existing values for Tdetect, Tmeasure, and Tevaluate in Table 4.6A.2.2-1 and Table 4.6A.2.2-2 can be reused.

Proposal 6: TS 36.133 - cell measurement in connected mode (8.14A) - the existing value of Tmeasure_intra (s) = 5 DRX cycle should be sufficient. No change is required for channel quality reporting as it all depends on serving cell measurement. (8.14A.4)

Proposal 7: TS 36.133 - RRC re-establishment (6.5A) - Cell search time Tsearch_NB1-NC should be scaled up by 9 for normal coverage.

Proposal 8: TS 36.133 - Clause 6.9A, RRC Connection redirection to Non-Anchor Carrier - RAN4 may consider to change TDL-UL switch from 8 to 50ms (based on RAN1 design).

Proposal 9: TS 36.133 - RLM (7.23A) When DRX is not in use (connected mode DRX is not configured):

  - No change is required for Rmax > 64;

  - For Rmax <=64 existing value of 400ms/200ms may not provide adequate number of NRS sub-frames for accurate measurement so it shall be scaled up as per option 2: “Update it to 1200/600ms from 400/200ms when configured NPDCCH Rmax ≤ 64”

Observation 3: RAN1 is currently discussing how to restrict minimum allowed NPDCCH value to 64ms.

Proposal 10: TS 36.133 - RLM (7.23A) When DRX is used,

  - When NPDCCH period is configured as >=64ms (NOTE: RAN1 is currently discussing how to restrict minimum allowed NPDCCH value to 64ms), existing values should be sufficient for Qin and Qout detection.

  - When NPDCCH period is configured as <64ms:

    - NPDCCH period value lower than 64ms does not make sense for NTN TDD mode when there is only one 8ms DL opportunity every 90ms.

    - If lower values are allowed (subject to RAN1 decision), then RAN4 will need to adjust values for RLM (when DRX is configured).

# Conclusion

Proposal 1: RAN4 to consider the following replies to the questions from RRM WF R4-2504915:

|  | Question Q1 General answer: Generally speaking, the UE should not do something different in TDD that is already existing/used in FDD. | Question Q2 General answer: Generally speaking, this input will be very useful if UE vendors could share their view. | Question Q3 General answer: N/A during IDLE mode. |
| --- | --- | --- | --- |
| Issue 1: Cell reselection (4.6A.2) | NRS by default (legacy). Optionally, NSSS can be used. (NW has no idea what measurement UE is using; NPSS is currently used only for synchronization) | To be further discussed/proposed by UE vendors. | N/A (IDLE mode only, not applicable) |
| Issue 2: Cell measurement in connected mode (8.14A) | NRS by default (legacy). Optionally, NSSS can be used depending on NW configuration and UE implementation. (NW has no idea what measurement UE is using; NPSS is currently used only for synchronization) | To be further discussed/proposed by UE vendors. | Up to UE implementation, not mandatory Observation: Only one DL opportunity with at least 3 or 4 NRS subframes and max 1 NSSS and/or NPSS. |
| Issue 3: RRC re-establishment (6.5A) | NRS by default (legacy). Optionally, NSSS can be used depending on NW configuration and UE implementation. (NW has no idea what measurement UE is using; NPSS is currently used only for synchronization) | To be further discussed/proposed by UE vendors. | N/A Can be in connected mode, but no consecutive UL transmissions expected during RRC re-establishment. |
| Issue 4: RRC Connection redirection to Non-Anchor Carrier (6.9A) | Optionally, NSSS can be used depending on NW configuration and UE implementation. (NW has no idea what measurement UE is using; NPSS is currently used only for synchronization) | To be further discussed/proposed by UE vendors. | N/A Can be in connected mode, but no consecutive UL transmissions expected during RRC re-establishment. |
| Issue 5: RLM (7.23A) | Optionally, NSSS can be used depending on NW configuration and UE implementation. (NW has no idea what measurement UE is using; NPSS is currently used only for synchronization) | To be further discussed/proposed by UE vendors. | Up to UE implementation, not mandatory Observation: Only one DL opportunity with at least 3 or 4 NRS subframes and max 1 NSSS and/or NPSS. |

Proposal 2: Given the work assumption with Earth Moving Beams for NTN TDD NB-IoT WI, RAN4 RRM should consider a maximum visibility time of a cell for 83.02 seconds and a minimum visibility time of a cell for 25.33 seconds.

Observation 1: RAN4 could add to TS 36.133 timing error requirement the following phrase: “For band 249, NTN NB-IoT UE in TDD mode adjusts its time pre-compensation before/at the beginning of each set of consecutive uplink sub-frames.” However, it may not be sufficient and it could also restrict implementation.

Proposal 3quater: Do not define any specific TS 36.133 timing error pre-compensation requirement for band 249, NTN NB-IoT UE in TDD mode. For band 249, NTN NB-IoT UE in TDD mode can reuse the current Te timing error limit, and UE may adjust its time pre-compensation before/at the beginning of/during each set of consecutive uplink sub-frames.

FSS: whether pre-compensation gap is needed and the specific duration of the gap (which could be limited to a few symbols) depending on the UE capabilities.

Proposal 4: The cell detection/cell reselection (4.6A.2) time from TS 36.133 does not need to be scaled up by the factor of 9. Existing value for Nserve_NB_NC should be sufficient for TDD as per table 4.6A.2.1-1 and 4.6A.2.1-2 as scaling up by 9 can result in UE going out of coverage before even starting neighbour cell measurements.

Observation 2: NPSS is only needed for cell detection and no requirement for half DRX. Only for measurements, two measurement samples should be apart by minimum half DRX cycle where measurements could be based on NSSS or NRS. If more NRS or NSSS is required for measurement accuracy, UE implementation can combine measurements across multiple downlink opportunities at the expense of UE power consumption.

Proposal 5: TS 36.133 - cell reselection (4.6A.2) - For earth moving cell, the existing values for Tdetect, Tmeasure, and Tevaluate in Table 4.6A.2.2-1 and Table 4.6A.2.2-2 can be reused.

Proposal 6: TS 36.133 - cell measurement in connected mode (8.14A) - the existing value of Tmeasure_intra (s) = 5 DRX cycle should be sufficient. No change is required for channel quality reporting as it all depends on serving cell measurement. (8.14A.4)

Proposal 7: TS 36.133 - RRC re-establishment (6.5A) - Cell search time Tsearch_NB1-NC should be scaled up by 9 for normal coverage.

Proposal 8: TS 36.133 - Clause 6.9A, RRC Connection redirection to Non-Anchor Carrier - RAN4 may consider to change TDL-UL switch from 8 to 50ms (based on RAN1 design).

Proposal 9: TS 36.133 - RLM (7.23A) When DRX is not in use (connected mode DRX is not configured):

  - No change is required for Rmax > 64;

  - For Rmax <=64 existing value of 400ms/200ms may not provide adequate number of NRS sub-frames for accurate measurement so it shall be scaled up as per option 2: “Update it to 1200/600ms from 400/200ms when configured NPDCCH Rmax ≤ 64”

Observation 3: RAN1 is currently discussing how to restrict minimum allowed NPDCCH value to 64ms.

Proposal 10: TS 36.133 - RLM (7.23A) When DRX is used,

  - When NPDCCH period is configured as >=64ms (NOTE: RAN1 is currently discussing how to restrict minimum allowed NPDCCH value to 64ms), existing values should be sufficient for Qin and Qout detection.

  - When NPDCCH period is configured as <64ms:

    - NPDCCH period value lower than 64ms does not make sense for NTN TDD mode when there is only one 8ms DL opportunity every 90ms.

    - If lower values are allowed (subject to RAN1 decision), then RAN4 will need to adjust values for RLM (when DRX is configured).