3GPP TSG-RAN WG4 Meeting #115		               R4-2507131

Malta, MT, 19th – 23rd May, 2025

Source:	Sony

Title:	Discussion on the phase continuity and GNSS issue for NR NTN

Agenda item:	7.31.3.1

Document for:	Approval

# Background

In response to RAN4's original LS in R4-2419898, RAN2 has replied with further questions on asking the periodicity and duration of IDC free time to help the UE read the GNSS in RAN4#114bis [1]. However, no conclusion from the RAN4 side has been reached, and further discussion is needed.

In addition, another LS was sent by RAN1 in R4-2505305 [2], which consulted RAN4 about whether the same phase continuity requirements as for DMRS bundling can be applied for OCC length 2 and/or OCC 4 length.

In this contribution, we provide our views on the issues mentioned above.

# Phase continuity for OCC grouping

In the coming LS from RAN1, it states:

RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH.

•	FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling.

action

RAN1 respectfully ask RAN4 whether the same phase continuity requirements as for DMRS bundling can be applied for OCC length 2 and/or OCC length 4 under the same conditions as for DMRS bundling, or if new requirements are needed.

RAN4 requirements on DMRS bundling for slot < 8 is defined as the phase difference between slot p-1 and slot p is below 25 degrees as shown below:

Therefore, RAN4 requirements can guarantee the phase variation of each UE individually within a certain range. However, with OCC grouping, to ensure the paired UE can actually maintain orthogonality with each other, we understand the phase difference between the paired UE is a more critical issue. Taking OCC-4 as an example, presumable the network could group with 2 UEs with the same initial phase, e.g., phase difference is 0 degree on the first slot. In the worst case, the phase difference on the last slot can be up to 150 degrees, and whether such a phase difference is acceptable requires further study.

Observation 1: Unlike DMRS bundling, the phase difference between paired UE is more critical for OCC grouping than the phase variation of each UE. The phase difference between the grouped UE can be up to 150 degrees in the worse under current DMRS bundling requirements.

A preliminary simulation result with a simplified assumption is presented in Fig. 1 to verify if the existing DMRS bundling requirement would be sufficient for OCC-4. In this simulation, the phase variation between slot p-1 to slot p is assumed to be uniformly distributed from -φ to – φ, but there is no phase variation within each slot. It is also assumed that the initial phase of both UEs is 0, and the receiver knows the channel.

Figure 1 . The performance with respect to different phase variations between slot p and slot p+1 with OCC 4.

This very preliminary simulation result shows that the SNR degradation for BER = 1% is below 1 dB for phase variation below 40°. From this result, the existing DMRS bundling requirement should be sufficient. Therefore, RAN4 can take existing phase continuity requirements as a starting point and define the requirement for UE support OCC. Meanwhile, the same applicability of phase continuity requirements as for DMRS bundling shall still be applied.

Observation 2: the existing DMRS bundling requirement on the phase difference between slot p-1 and slot p with 25 degrees is sufficient to ensure the OCC performance with grouped UE.

Meanwhile, the same applicability of phase continuity requirements as for DMRS bundling shall still be applied.

-RB allocation in terms of length and frequency position does not change, and intra-slot and inter-slot frequency hopping is not activated.

-Modulation order does not change.

-No network commanded TA takes effect.

-The TPMI precoder does not change.

-There is no change in UE transmission power level, and no change in the level of P-MPR applied by the UE.

-UE is not scheduled with uplink transmission of other physical channel/signal in-between the PUSCH or PUCCH transmissions.

-Doppler conditions are set to zero and delay conditions are set to constant.

Proposal 1: RAN4 takes the existing phase continuity requirement as a starting point to define the phase continuity requirement for UE support OCC. Meanwhile, the same applicability of phase continuity requirements as in 38.101-5 for DMRS bundling shall still be applied.

# GNSS IDC

In the LS replied from RAN2, it states [1]:

RAN2 respectfully asks RAN4 to take the aforementioned RAN2 decisions into account and provide the corresponding requirements, e.g. the periodicity and duration of IDC free time to help the UE read the GNSS.

Meanwhile, the current IDC-TDM solution is specified as below in 38.331:

Based on the information above, it is our understanding that RAN2 requires information on how often the GNSS measurement event would happen and how long the measurement gap would require.

GNSS validity duration was specified for IoT NTN since Rel-17, an UE autonomously reported value indicating the remaining GNSS validity duration in the UE.

In our view, the large range of validity duration specified for IoT NTN can also be valid for NR NTN as well, as it can accommodate the scenario under different speeds, etc. Therefore, it is proposed that those numbers of NR NTN be reused.

Proposal 2: the GNSS validity duration values of IoT NTN in TS 36.331 can be reused for NR NTN.

Measurement gap on GNSS has been introduced for IoT NTN since Rel-17, which ranges up to 31s (see table below). Per our understanding, such a long measurement gap was defined to accommodate a cold start of the GNSS receiver with IoT NTN devices, as the GNSS receiver will be totally shut off during the NTN transmission for the sake of power consumption. However, it is unclear if such a limitation would also be valid for NR NTN. For a normal measurement gap without a cold start, a measurement gap of a few seconds would be sufficient.

Proposal 3: A measurement gap of a few seconds would be sufficient if the GNSS receiver is not cold-started.

# Conclusion

In this paper, we presented our view on the phase continuity for OCC grouping and GNSS IDC issue, where the following observations and proposals are made:

Observation 1: Unlike DMRS bundling, the phase difference between paired UE is more critical for OCC grouping than the phase variation of each UE. The phase difference between the grouped UE can be up to 150 degrees in the worse under current DMRS bundling requirements.

Observation 2: the existing DMRS bundling requirement on the phase difference between slot p-1 and slot p with 25 degrees is sufficient to ensure the OCC performance with grouped UE.

Proposal 1: RAN4 takes the existing phase continuity requirement as a starting point to define the phase continuity requirement for UE support OCC. Meanwhile, the same applicability of phase continuity requirements as in 38.101-5 for DMRS bundling shall still be applied.

Proposal 2: the GNSS validity duration values of IoT NTN in TS 36.331 can be reused for NR NTN.

Proposal 3: A measurement gap of a few seconds would be sufficient if the GNSS receiver is not cold-started.

# References

- R2-2501568	Response LS on simultaneous operation between GNSS and NR NTN

- R4-2505305 	LS on requirements for the phase continuity and power consistency for OCC with PUSCH in NR NTN Ph3