3GPP TSG-RAN WG4 Meeting #115	R4-2507135

Malta, MT, May 19th – May 23rd, 2025

Source:	Nokia

Title:	Discussion on General Aspects of AI/ML for NR Air Interface Phase 2

Agenda item:	7.22.2

Document for:	Discussion

Introduction

In RAN#107, the revised SID of the Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2 was agreed [1]. In this revised SID, the following scope was agreed for RAN4:

- Testability and interoperability [RAN4]: - Further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis

In this contribution, we share our views on the following topics:

- Relation to legacy requirements

- LCM-related aspects for two-sided model case, including relation to LCM of UE-sided models and performance monitoring

Discussion

Relation to legacy requirements

The following agreement was made in RAN4#112bis regarding the relation to legacy requirements for CSI prediction and beam management use cases.

Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

Along the lines of the first FFS in the above agreement, the performance requirements for CSI prediction were agreed to be defined based on the gamma ratio, defined in the current PMI reporting requirements in 38.101-4. The related agreement was made in last meeting RAN4#114bis as follows:

Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4) - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/ML [a given SNR] using the precoders configured according to the UE reports, - trnd is the throughput measured at SNRue_AI/ML [the same SNR] with random precoding. - trnd is obtained with Rel-15 type I single panel codebook - Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1,follow2 in section 6.3 of 38.101-4

For CSI compression, whether this gamma ratio is relevant and how it can be used for defining CSI compression performance requirements are examples of points that need further clarification.

RAN4 to discuss the relevance of the minimum performance requirements of PMI reporting defined in 38.101-4 for the CSI compression use case.

Life-Cycle Management related aspects for two-sided models

Relation to LCM of UE-sided models

UE-sided model LCM-related aspects are being addressed in RAN4 under both the General Aspects and the use case-specific discussions. For instance, delay requirements for AI/ML functionality/configuration activation/deactivation/fallback are discussed as part of the LCM requirements. These discussions rely heavily on the progress made by RAN1/RAN2 on LCM definitions and procedures.

RAN4 should consider reusing UE-sided model LCM framework for the LCM of the UE-part of 2-sided models.

Performance monitoring

According to TR 38.843, performance monitoring in CSI compression using two-sided model use case can be NW-side or UE-side.

- NW-side performance monitoring: NW monitors the performance and make decisions of model/functionality activation/ deactivation/updating/switching. Impact to enable performance monitoring using an existing CSI feedback scheme as the reference, including the association between AI/ML scheme and existing CSI feedback scheme for monitoring, are considered. Note: The metric for monitoring and comparison includes intermediate KPI and eventual KPI. - UE-side performance monitoring: UE monitors the performance and reports to Network, NW makes decisions of model/functionality activation/deactivation/updating/switching. Impact on triggering and means for reporting the monitoring metrics, including periodic/semi-persistent and aperiodic reporting, and other reporting initiated from UE, are not precluded.

We quote below the agreements related to performance monitoring in CSI compression from several RAN1 meetings.

RAN1#117 Agreement Further study following monitoring options in Rel-19, including the necessity and feasibility, - NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1) - SRS-based monitoring - UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the output of the CSI reconstruction model at the UE (Case 2-1) - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side. - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2) - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model - Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook Note: for UE-side monitoring, the final reported monitoring output, if specified, may be different, e.g., be further derived based on the output of the above approaches. RAN1#120bis Conclusion For NW-side monitoring with target CSI reporting - Target CSI reporting via legacy CSI codebooks can be used for NW-side monitoring - Target CSI reporting with CSI codebook enhancement via higher-resolution parameter combination may be beneficial for improving NW-side monitoring with additional cost of complexity and overhead at UE side. RAN1#120bis Conclusion UE-side monitoring is feasible. RAN1#120bis Observation - Some companies think that, at least in some UE-side monitoring options, NW-side monitoring with target CSI reporting is needed to check the reliability of UE-side monitoring reports.

- In case of NW-side performance monitoring, UE will report Target CSI via legacy eTypeII codebook or eTypeII-like high-resolution codebook. This means that UE is supposed to report the codebook-based CSI report as in the legacy scenario. Hence, the delay requirements of such reports should not differ from legacy requirements.

In case of NW-side performance monitoring in the two-sided CSI compression use case, UE should follow legacy delay requirements for the performance monitoring reporting.

In case of UE-side performance monitoring, UE is expected to report the performance monitoring metric (e.g., intermediate KPI). Such reports are different from legacy PMI reports, hence, RAN4 should consider defining delay requirements for performance monitoring reporting from the UE in this case.

In case of UE-side performance monitoring in the two-sided CSI compression use case, RAN4 should study the impact on delay requirements for performance monitoring reporting from the UE.

Conclusion

The following proposals were made in this contribution:

Proposal 1: RAN4 to discuss the relevance of the minimum performance requirements of PMI reporting defined in 38.101-4 for the CSI compression use case.

Proposal 2: RAN4 should consider reusing UE-sided model LCM framework for the LCM of the UE-part of 2-sided models.

Proposal 3: In case of NW-side performance monitoring in the two-sided CSI compression use case, UE should follow legacy delay requirements for the performance monitoring reporting.

Proposal 4: In case of UE-side performance monitoring in the two-sided CSI compression use case, RAN4 should study the impact on delay requirements for performance monitoring reporting from the UE.

- References

- RP-250308, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2”, Qualcomm, RAN#107, March 12-14, 2025, Incheon, South Korea.