TSG-RAN Working Group 4 (Radio) meeting #115	R4-2507155

Malta, EU, 19th to 23th April 2025

Source:	Ericsson

Title:	Draft CR to TS 38.141-2: improvements of Annex with TRP measurement methods

Agenda item:	7.13.5.1

Document for:	Approval

# Introduction

One of the work areas of the work item on NR base station (BS) RF requirement evolution for FR1/FR2 and testing simplification, targets the BS TRP estimation methods in Annex I in TS 38.141-2. The goal is to improve applicability and description. As this annex is normative, it is imperative to clearly describe how the manufacturer performs the TRP measurement, the applicability conditions for each method and what are the criteria for deciding compliance to the TRP limits.

The current Annex I of TS 38.141-2 contains alternatives of measurement grids that are supposed to produce similar outcomes in terms of pass/fail (at least according to the MU analysis in the TR 37.941). Some of the methods aim for an accurate estimation of the TRP, while others aim to an overestimation that would provide a safe margin to the limit.  There are probably even more alternatives which are not included at this moment. Due the lack of precise instructions on how each method can be applied, regulators have difficulties assessing the risk of passing a failing device. For this reason, there is a need for establishing a “reference method” which would be the one providing an accurate estimation of TRP. However, in some situations, such reference method may lead to unpractical test time, so a “preferred method” can be considered instead, given that the outcome will be the same as for “reference method”. Note that in-band and out-of-band measurements might be associated a different “preferred method”.

In this contribution we discuss a few proposals for improving the Annex I in TS 38.104 by adding applicability conditions, reference TRP method and preferred methods.

At the end of this contribution a text proposal is attached for approval.

# Discussion

The last WF approved in RAN4 #114-bis in [1] mentions:

Agreement for test method/ sampling grid simplification:

  - To follow the previous RAN4 agreement in RAN4#114 meeting: to keep the current TRP testing method, TRP sampling grid and testing chamber in the specification

Agreement for applicability for different requirement:

  - For OTA base station output power, ACLR and operating band unwanted emissions, use the following option 1 as starting point.

    - Option 1:

      - Step 1: [Orthogonal cut grid] (two-cuts and optional three cuts) as described in annex I.5 or Beam-based directions as described in annex I.10 may be performed if chosen for the test, go to step 2 if this step is not chosen or the BS under test fails the test in this step.

      - Step 2: One of the TRP measurement procedure as described in annex I.2 to I.9 except annex I.5 shall be performed for the test.

  - For OTA base station spurious emission, use the option 1 as starting point

    - Option 1:

      - Step 1: Pre-scan as described in annex I.13 if chosen for the test, go to step 2 if this step is not chosen.

      - Step 2: [Orthogonal cut grid] (two-cuts and optional three cuts) as described in annex I.5 or Peak method as described in annex I.11 or Equal sector with peak average as described in annex I.12 may be performed if chosen for the test, go to step 3 if this step is not chosen or the BS under test fails the test in this step.

      - Step 3: One of the TRP measurement procedure as described in annex I.2 to I.9 except annex I.5 shall be performed for the test.

Agreement:

  - Encourage companies to provide the draft CR focusing on how to improve the test procedures for Pre-scan approach next meeting and further discuss the improvement based on draft CR directly.

Starting from these agreements in the WF we have the following proposals for going forward with the revision of Annex I in TS 38.141-2 (and similar annex in other specifications).

Proposal 1: Adopt a “reference method” for measuring the TRP of any signal regardless power level or frequency.

We propose that there should be one reference method, preferably the one called the “Spherical equal angle grid” with angular steps not exceeding the “reference steps”. The equi-angular spherical grid is well understood and widely studied in literature. The analysis is based on Spherical wave expansions wherein the dimensions of the device in terms of minimum sphere and cylinder determines the needed angular sampling. The reference steps correspond to λ/2 sampling on the minimum sphere and cylinder, respectively. Using reference steps on a spherical grid gives accurate TRP results.

In Annex I we find a few other grids, e.g. Fibonacci, that can be demonstrated to generate an accurate value of TRP if the reference angular steps are employed. In general, all methods based on using reference angular steps can be considered as accurate as the one based on equal angle grid.

Proposal 2: Adopt Spherical Equal Angle Grid with reference angular steps (I.2.2) as the “reference method” for TRP measurements.

Proposal 3: For each requirement, adopt a “preferred method” for TRP measurements. This method can be chosen if the amount of measurement points on the spherical grid (i.e. number of samples) is exceeding 700. This would correspond to 10 deg angular step in Table 2-1.

Proposal 4: Agree on a minimum number of pointts on the spherical grid that shall be considered (when number f points is too large to be practical) – this “pain threshold” is intended to show that manufacturers and testing laboratories are committed to a minimum effort that is acceptable from a practical perspective.

Table 2-1 Spherical equal angle grid: number of measurement points (samples) on the sphere as a function of angular step.

Observation 1:  Annex I is called “TRP measurement methods” but we observe that we have both methods and procedures for estimating a TRP value, but also methods and procedures to avoid estimation of a TRP value (e.g. I.11 Peak method, I.13 Pre-scan). The latter provides pass/fail a criteria based on an overestimation of TRP, with a significant margin

Proposal 5: Merge the Peak Method I.11 with the Pre-scan I.13. In this way we can have a more comprehensive pre-scan that can provide an “early exit” for spurious emissions which are well below the limit.

Particularities of the pre-scan are:

- measurement distance can be short, given that measurement antenna directivity is low (i.e. wide lobe)

- measurements can be fast, without averaging for many frames

- beam scanning can be used together with max hold readings

- should allow a continuous smooth movement of the positioner

Proposal 6: Remove the Equal sector with peak average I.12, for the reason that it was defined related to I.11 and it is very unclear how the number of sectors K is chosen. It also does not bring any advantage compared to full sphere with sparse sampling.

Let us analyse an example of test procedure, here is the one for OBUE in TS 38.141-2

##### 6.7.4.4.2	Procedure

The following procedure for measuring TRP is based on the directional power measurements as described in annex I. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber. If so, follow steps 1, 3, 4, 6 and 9. When calibrated and operated within the guidance of 3GPP TR 37.941 [29] the measurement methods are applicable and selected depending on availability at the test facility.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	The measurement devices characteristics shall be:

-	measurement filter bandwidth: defined in clause 6.7.4.5.

-	detection mode: true RMS voltage or true power averaging.

As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity, efficiency and avoiding e.g. carrier leakage, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in Table 4.1.2.2-1 for FR1 and Table 4.1.2.2-2 for FR2.

4)	For single carrier operation, set the BS to transmit according to the applicable test configuration in clause 4.8 using the corresponding test model(s) in clause 4.9.2 at manufacturers declared rated carrier output power (Prated,c,TRP).

For a BS declared to be capable of multi-carrier and/or CA operation, use the applicable test signal configuration and corresponding power setting specified in clause 4.7.2 and 4.8 using the corresponding test model(s) in clause 4.9.2 on all carriers configured.

5) Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex I).

6)	Sweep the centre frequency of the measurement filter in contiguous steps and measure emission power within the specified frequency ranges with the specified measurement bandwidth.

7)	Repeat step 5-6 for all directions in the appropriated TRP measurement grid needed for TRPEstimate (see annex I).

8)	Calculate TRPEstimate using the measurements made in step 6.

9) For BS type 1-O and multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

Observation 2: In the test procedure for different requirements there is a reference to “appropriate TRP grid”. Use of word “appropriate” allows for too much flexibility in the choices of the measurement grid, therefore we need more clarification what “appropriate” means. Also, not all methods are based on a “grid”.

Observation 3: The test procedures always a mention TRP measurement grid and TRPEstimate as the value to compare to the limit. Some methods are not based on a spherical grid and neither on estimation of TRP.

# Summary

Observation 1:  Annex I is called “TRP measurement methods” but we observe that we have both methods and procedures for estimating a TRP value, but also methods and procedures to avoid estimation of a TRP value (e.g. I.11 Peak method, I.13 Pre-scan). The latter provides pass/fail a criteria based on an overestimation of TRP, with a significant margin

Observation 2: In the test procedure for different requirements there is a reference to “appropriate TRP grid”. Use of word “appropriate” allows for too much flexibility in the choices of the measurement grid, therefore we need more clarification what “appropriate” means. Also, not all methods are based on a “grid”.

Observation 3: The test procedures always a mention TRP measurement grid and TRPEstimate as the value to compare to the limit. Some methods are not based on a spherical grid and neither on estimation of TRP.

Proposal 1: Adopt a “reference method” for measuring the TRP of any signal regardless power level or frequency.

Proposal 2: Adopt Spherical Equal Angle Grid with reference angular steps (I.2.2) as the “reference method” for TRP measurements.

Proposal 3: For each requirement, adopt a “preferred method” for TRP measurements. This method can be chosen if the amount of measurement points on the spherical grid (i.e. number of samples) is exceeding 700. This would correspond to 10 deg angular step in Table 2-1.

Proposal 4: Agree on a minimum number of pointts on the spherical grid that shall be considered (when number f points is too large to be practical) – this “pain threshold” is intended to show that manufacturers and testing laboratories are committed to a minimum effort that is acceptable from a practical perspective.

Proposal 5: Merge the Peak Method I.11 with the Pre-scan I.13. In this way we can have a more comprehensive pre-scan that can provide an “early exit” for spurious emissions which are well below the limit.

Proposal 6: Remove the Equal sector with peak average I.12, for the reason that it was defined related to I.11 and it is very unclear how the number of sectors K is chosen. It also does not bring any advantage compared to full sphere with sparse sampling.

# References

[1]	R4-2504740 Way Forward for [114bis][304] NR_BS_RF_Part3_OTA_TRP

TEXT PROPOSAL:

Annex I (normative):
TRP measurement procedures

# I.1	General

The annex describes various procedures for BS OTA TRP measurments. These procedures can provide either an accurate or an over-estimate of TRP values.

The procedures for an accurate estimate can be applied to all TRP requirements. However, if a TRP requirement does not need accurate TRP estimate then the procedures for over-estimate of TRP may be used in order to have a reasonable OTA test time.

Pre-scan is an optiona procedure and does not provide an accurate TRP estimate or over-estimate of TRP. Pre-scan is a fast but coarse method that is used to identify the spurious emission frequencies with emission power as described in clause I.13. A sequential measurement is then made at the emission frequencies, to assess the TRP as described in clause I.2 to clause I.9.

When making TRP measurements the alignment between EUT and measurement antenna is important to achieve expected measurement uncertainty:

1.	The measurement antenna needs to be aligned tangential to the measurement surface forming a sphere around the EUT, in order to correctly measure the TRP.

2.	Test methods described in clauses I.5.1, I.5.2, I.10require angular alignment between the selected measurement grid and EUT radiation pattern in order to measure peak values in the main beams. Angular misalignment can lead to differences in the actual and measured angular positions of the intended maximum EIRP.

3.	Test methods described in clause I.5.3, I.6 and I.9 are designed to be independent of rotations of the angular grid, and hence angular alignment between the measurement grid and EUT is not needed.

The reference method for measuring TRP for both in-cand and out-of-band emissions is the “Spherical equal angle grid” described in I.2, with angular steps not exceeding the reference steps that correspond to λ/2 sampling on the minimum sphere and cylinder, respectively.

An EUT that fails compliance to the TRP limit when the reference method is employed cannot be declared compliant when employing any other methods described in this Annex.

The methods in I.3, I.4, I.6 are also considered accurate if the reference angular step criteria in I.2.2 is met.

When the number of samples on the spherical grid exceeds [700] (i.e. that corresponds to a an angular step of 10 deg in I.2) it is acceptable to apply sparse sampling together with a correction factor, only in the case of spurious emissions.

For in-band measurements the methods in I.5 (2-3 orthogonal cuts) or I.10 (beam based directions) can be considered accurate and utilized as a preferred method, given that all the applicability conditions for these methods are demonstrated.

Table I.1-1: Applicability of TRP measurement methods and chambers to the type of emissions to be measured

|  |  | BS output power | Unwanted emissions: ACLR (Note 1) | Unwanted emissions: SEM, OBUE | Spurious emissions (Note 7) |
| --- | --- | --- | --- | --- | --- |
|  | Full sphere using reference steps (accurate) (I.2, I.3, I.4) | X | X | X | X (Note 2) |
| Methods applicable to | Full sphere using sparse sampling (overestimate) |  |  |  | X (Note 2, 3) |
| anechoic chambers | Two cuts + Pattern multiplication (accurate) (I.5) (Note 4) | X | X | X | (Note 3) |
|  | Two/three cuts (overestimate) |  | X | X | X (Note 2, 3) |
|  | Beam-based directions | X | X (Note 5) | X (Note 5) |  |
|  | Peak method |  |  | X | X (Note 2) |
|  | Equal sector with peak average |  |  | X | X (Note 2) |
|  | Pre-scan (Note 2) |  | X | X | X |
| Reverberation chamber |  | X | X | X | X (Note 2) |
| Note 1: Two TRP measurements are needed, i.e. one for wanted channel, and the other one for adjacent channel. Note 2: Pre-scan is needed to identify the frequencies of interest for more accurate TRP measurements. Note 3: At harmonic frequencies the use of this method is not applicable. Note 4: Pattern multiplication is conditional, as detailed in clause 6.3.4.5. Note 5: Applicable if the directivity of corresponding requirement at the reference direction is equivalent to the directivity at the reference direction when BS emits Prated,c,TRP and Prated,c,EIRP. Note 6: If box is blank the method is not excluded but the methodology has not been described in clause 6.3.3 on angular alignment in TRP measurements; if a suitable analysis is shown the method may be applied. Note 7: Applicability of any chamber for the spurious emissions measurements is subject to the supported frequency range of the chamber, and specifically its upper frequency limit. |  |  |  |  |  |

For each requirement, the TRP measurement method shall be included in the test report.

# I.2	Spherical equal angle grid

## I.2.1	General

TRPEstimate is defined as

TRPEstimate=π2NMn=0N-1m=0M-1EIRP(θn,ϕm)sinθn

when EIRP measurements is used or as

TRPEstimate=4πd2π2NMn=1N-1m=0M-1PD(θn,ϕm)sinθn

when power density measurements are used, and d is the test distance. N and M are the number of samples in the θ and ϕ angles. Each (θn,ϕm) is a sampling point. The sampling angular intervals for θ and ϕ angles are Δθ=πN and Δϕ=2πM. The sampling intervals Δθ and Δϕ are described in I.2.2.

## I.2.2	Reference angular step criteria

The reference angular steps Δϕref and Δθref in degrees are defined as:

Δϕref= min⁡(180∘πλDcyl,15∘)

Δθref=⁡min(180∘πλD,15∘)

The upper limit for these reference angular steps of 15∘ ensures a low Summation Error (SE) when λ is large compared to the EUT dimensions.

Dcyl and D are calculated as:

`	Dcyl=d2+w2

D=d2+w2+h2

The definition of d, w and h is shown in figure I.2.2-1. The radiation source can be EUT antenna array or the whole of EUT.

Figure I.2.2-1: Dimensions of a radiation source: depth (d), width (w) and height (h)

Optionally, in the case of Uniform Linear Array (ULA), when d is negligible (d ≈0) and the EUT is mounted along the yz plane as shown in figure I2.2-2, the reference angular steps, in degrees, can be determined by

∆θref=min(180°πarcsin⁡(λDz),15°)

Δϕref=min(180°πarcsin⁡(λDy),15°)

Where Dy is the length of radiating parts of EUT along y-axis, Dz is the length of radiating parts of EUT along the z-axis and  is wavelength for the measured frequency.

Figure I2.2-2: Spherical coordinate for OTA conformance testing of EUT

SF=maxΔθgridΔθref,ΔϕgridΔϕref

Δθgrid Δϕgrid

Alternatively, when EUT radiating dimensions are not known, for each frequency within the downlink operating band including ΔfOBUE, the reference angular steps can be specified in terms of the beamwidth of the wanted signal as

∆ϕref=λλo BeWϕ

Δθref=λλo BeWθ

where λo is the wavelength of the wanted signal, and BeWφ and BeWθ are the beamwidth of the wanted signal in the φ-axis and θ-axis, respectively.

BeWφ and BeWθ may be set to beamwidth declared for the OTA BS radiated transmit power requirement provided the same beam is applied to test in-band TRP requirements.

NOTE:	Beamwidth is approximately equal to half the first-null beam width.

# I.3	Spherical equal area grid

TRPEstimate is defined as

TRPEstimate=1Nn=1NEIRP(θn,ϕn)

N is the total number of samples and specified as

The sampling intervals Δθref and Δϕref are described in annex I.2.2. Each (θn,ϕn) is a sampling point.

# I.4	Spherical Fibonacci grid

TRPEstimate is defined as

TRPEstimate=1Nn=0N-1EIRP(θn,ϕn)

N is the total number of samples and specified as

The sampling intervals Δθref and Δϕref are described in annex I.2.2. Each (θn,ϕn) is a sampling point, where θn and ϕn, in degrees, are defined as:

θn=cos-11-2n+1N

ϕn=360°nΨ,  Ψ=1+52

# I.5	Orthogonal cut grid

## I.5.1	General

Here, at least two cuts (default) shall be used, an optional third cut can be used. The alignment of the cuts must be along the symmetry planes of the antenna array. No alignment is required for spurious emissions.

When alignment is required:

1)	The first mandatory cut is a horizontal cut passing through the peak direction of the main beam.

2)	The second mandatory is a vertical cut passing through the peak direction of the main beam. Using the data from these two mandatory cuts, a conditional pattern multiplication can be used.

3)	The third optional cut is a vertical cut orthogonal to the first and the second cut.

When alignment is not required, the cuts can be aligned arbitrarily.

Once the number and the orientation of the cuts are decided, the total EIRP is measured on the orthogonal cuts and the TRP is then calculated as follows: First the contributions from each cut is calculated as

EIRPav,cut-n=1Pj=1PEIRP(j)

where P is the number of sampling points in the cut. The final contribution for all cuts is calculated as

TRPEstimate=1Nn=1NEIRPav,cut-n

where N is the number of cuts. Note that when orthogonal cuts are measured, the interclause points are measured multiple times and the repeated values can be removed from the samples before averaging.

When two cuts measurements are used, a conditional pattern multiplication can be applied. The following are the conditions for applying pattern multiplication:

i.	The vertical cut (and the main beam) is in the xz -plane

ii.	The frequency of the emission is within the downlink operating band.

iii.	The bandwidth of the emission is the same as the bandwidth of the in-band modulated signal

iv.	The emission appears/disappears when the Tx power is turned on/off.

v.	The antenna arrays of the EUT

1)	Have rectangular grids of antenna element positions

2)	Have symmetry planes that are vertical and horizontal.

3)	Have parallel antenna planes

The antenna array is here assumed to be placed in the yz-plane. The pattern multiplication is performed in uv-coordinates and the data in the two cuts are denoted EIRPcut1ϕ at θ=θH and a vertical cut with data EIRPcut2θ at ϕ=0. The data is split in two parts corresponding to the forward and backward hemispheres. The uv-coordinates are the projections of the angular directions onto the antenna plane, here the yz-plane. Using the spherical coordinates as depicted in figure I.2.2-1 the u and v coordinates are defined as:

u=sinθsinϕv=cosθ

Note that only the data on the cuts are measured.

Calculate power density/EIRP values outside the two cardinal cuts as

EIRPu,v=EIRPcut1uEIRPcut2vEIRP0,vH

The pattern multiplication is applied separately for the forward (fwd) and backward (bwd) hemisphere. The TRP is then calculated as

TRP=14πfwdEIRPfwdu,vdudv1-u2-v2+bwdEIRPbwdu,vdudv1-u2-v2

NOTE:	The numerical singularity at u2+v2=1 must be treated with care, e.g. by change of variables.

## I.5.2	Operating band unwanted emissions

The procedure is as follows:

1)	Follow steps described in annex I.5.1 for the first two mandatory cuts and calculate the TRPEstimate.

2)	Compare the TRPEstimate to the limit.

3)	If the TRPEstimate is above the limit, perform the measurement on an additional third cut and repeat steps 1 to 2.

## I.5.3	Spurious unwanted emissions

The procedure is as follows:

1)	Follow steps described in annex I.5.1 for two cuts and calculate the preliminary TRPEstimate.

2)	Add the appropriate correction factor ΔTRP according to table I.5.3-1 to ensure overestimation with 95% confidence.

3)	Compare the corrected TRPEstimate (including ΔTRP) to the limit.

4)	If the corrected TRPEstimate is above the limit, perform the measurement on an additional third cut and repeat steps 1 to 3.

Table I.5.3-1: The correction factor for two or three cuts dense sampling

|  | Three cuts | Two cuts |
| --- | --- | --- |
| Correction factor ΔTRP (dB) | 2.0 | 2.5 |

# I.6	Wave vector space grid

If EUT is mounted along the yz plane as shown in figure I.2.2-1, the reference step in wave vector space can be determined by

Δuref=λDy

Δvref=λDz

where Dy is the length of radiating parts of EUT along y-axis, Dz is the length of radiating parts of EUT along the z-axis.

According to the relationship between the normalized wave vector and spherical coordinate, the wave vector can be represented as following:

u=sinθsinϕ, v=cos⁡(θ)

The total radiated power (TRP) in the wave vector space is determined by:

TRPestimate=ΔurefΔvref4πu2+v2<1cosϕ>0EIRPθn,ϕm,nsinθncosϕm,n +u2+v2<1cosϕ<0EIRPθn,ϕm,nsinθncosϕm,n

For spurious Tx or Rx emissions and where due to practical reasons such as time constraints or turn-table precision, measurement with the reference steps is not practical, sparser grids can be used. Use of sparse grids can lead to errors in TRP assessment. In order to characterize these errors, the SF (sparsity factor) of the grid is defined as

SF=max∆ugrid∆uref,∆vgrid∆vref

Where ∆ugridand ∆vgrid are the actual steps used in the wave vector space in the measurement and the upper bound of their value is 1/12, corresponding to 15 degree steps close to boresight.

# I.7	Void

# I.8	Void

# I.9	Full sphere with sparse sampling

The procedure is as follows:

1)	Set the angular grid:

a.	Non-harmonic frequencies: choose the angular steps Δϕ and Δθ smaller than or equal to 15 degrees. Calculate the sparsity factor (SF) as

SF=maxΔϕΔϕref,ΔθΔθref

and the correction factor as:

ΔTRP=SF-1SFmax-1⋅1.0 dB,

where SFmax corresponds to 15 degrees angular step. If the sparsity factor is smaller than 1, the correction factor ΔTRP is 0 dB.

Harmonic frequencies with fixed beam test signal: choose the angular steps smaller than or equal to the reference angular steps Δϕref and Δθref. Correction factor ΔTRP is 0 dB.

Harmonic frequencies with beam sweeping test signal: set the angular steps to 15 degrees. Correction factor is ΔTRP 0 dB.

2)	Apply a suitable numerical integration to calculate the preliminary TRPEstimate.

3)	Add the appropriate correction factor ΔTRP according to step 1 to ensure an overestimation with 95 % confidence.

4)	Compare the corrected TRPEstimate (including ΔTRP) with the limit. If the corrected TRPEstimate is above the limit, choose a smaller angular step and repeat steps 2-4. If the sparsity factor is less than one, no significant improvement of accuracy is expected.

# I.10	Beam-based directions

Beam-based direction can be used in the base station operating band only if the directivity of the radiation pattern of the emssions being measured is known. TRPEstimate is defined as

TRPEstimate=EIRPpeakDEUT, where EIRPpeak is the maximum EIRP in the beam peak direction within a particular beam direction pair and DEUT is the directivity of the EUT.

NOTE: For this method to be applicable, in case of ACLR and OBUE measurements the assumption on same directivity of the radiation pattern as with main beam needs to be validated.

# I.11

# void

# I.12	Equal sector with peak average

ϕ

TRPEstimate=1Kk=1KEIRPkEIRPk

# I.13	Pre-scan

Pre-scan is an optional first step in identifying the frequencies with spurious emission power levels above a certain threshold. The pre-scan does not provide an estimate of TRP, but offers a criteria to declare compliance to the limit based on coarse measurements around the object and employing a large margin towards the TRP limit. An emission frequency identified by a pre-scan shall be further investigated by any of the TRP measurement methods in this annex.

Measurement antenna shall be able to illuminate the whole EUT and the measurement distance shall be larger than the Fraunhofer far-field distance of the measurement antenna. Noise floor in the test set-up shall be at least 30 dB under the specified limit.

If possible, use a sweeping test signal, where a number of beam directions is quickly scanned during the measurement period.

The procedure for pre-scan is as follows:

1)	Scan the entire surface around EUT

2)	Rotate test antenna to cover all possible polarizations of emissions and identify the maximum levels readings

3)	Record the list of frequencies and corresponding unwanted emission power levels, EUT spatial positions, and test antenna polarization for which the maximum emission levels occur.

4)	Emissions  presenting maximum levels reading that are 20 dB or more below the specified TRP limit shall not require further measurements. This criteria is sufficient to prove compliance to the TRP limit.

5)	Emissions presenting maximum levels reading that exceed (TRP limit - 20 dB) shall be subject to further investigations. Steps 6 to 8 are optional.

6)	Move EUT and test antenna to the same position where the peak emission is recorded during the pre-scan.

7)	Place the test antenna at a measurement distance that meets the far-field of the EUT antenna, if possible. The actual test distance shall be considered against the requirement to maintain the system noise floor being at least 10 dB below the TRP limit. Move the EUT around the position and test antenna orientation to find the final peak EIRP or power density.

8)	In order to prove compliance to the TRP limit, the measured peak power density or EIRP of the emissions shall not exceed the TRP limit.