3GPP TSG-RAN WG4 Meeting #115	R4-2507157

St. Julian’s, Malta, 19 May – 23 May 2025

Source:			Ericsson

Title:			RRM impact of 7 MHz channel bandwidth

Agenda item:		7.8.5

Document for:		Discussions

# Introduction

In RAN#106, a new WI ([1]) has been approved to specify 7 MHz channel bandwidth for NR bands n5 and n26.

In this contribution we discuss and identify potential RRM impact.

# Discussion

The WI to specify new channel BW of 7 MHz targets NR bands n5 and n26 which are both low frequency FR1 bands supporting 15 kHz SCS.

Reference measurement channels (RMCs) are defined for PDSCH channels and CORESET RMSI/RMCs in A.3.1.1 and A.3.1.3/A.3.1.3 in [2]. It is noted that both are defined in a generic way which makes them applicable also for operation with 7 MHz channel BW. Moreover, RF has agreed that the maximum transmission bandwidth for 7MHz CBW is 35 RBs, see the running CR in [3] (see below). Hence, there is no impact to the PDCCH/CORESET due to 7MHz CBW.

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1 [R4-2505138]

| SCS (kHz) | 3 MHz | 5 MHz | 7 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 35 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | N/A | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

Observation 1: No impact to PDCCH/CORESET reception due to 7MHz CBW.

Observation 2: RMCs defined in current specification support flexible channel BW.

In addition, OCNG patterns are used in different RRM test cases and the patterns defined in NR are generic covering variable number PRBs, see section A.3.2.1 in [2]. Therefore, we don’t foresee any impact on OCNG pattern due to 7MHz channel bandwidth operation.

Observation 3: OCNG patterns used in current RRM specification supports flexible channel BW.

RAN4 needs to introduce the new band supporting 7MHz channel bandwidth to the frequency band groups defined in section 3.5 of TS 38.133. This is, however, normal procedure when introducing a new band and thus we do not foresee any new impact. It is further noted that RF may define new REFSENS for this band which is based on existing 5Mhz REFSENS scaled to the new bandwidth. Hence, no RRM impact is expected to the frequency band grouping section.

Observation 4: No impact to the side conditions tables due to the new band supporting 7MHz channel bandwidth.

It shall be noted the NR bands n5 and n26 supporting 7MHz CBW also support 10MHz channel bandwidth, see Table 5.3.5-1 in running CR [3]. This means a UE supporting 7MHz CBW is also required to support 10 MHz CBW.

Observation 5: NR bands n5 and n29 support both 7MHz CBW and 10MHz CBW.

Since there exists already FR1 RRM test cases with test configuration 10MHz CBW with 15kHz SCS and considering the above observations, our view is that there is no need to introduce any new test cases to cover 7MHz CBW, instead the UEs can apply the existing test cases designed for 10MHz CBW with 15KHz SCS.

Proposal 1: No need to introduce any new RRM test cases or test configurations to support 7MHz CBW, instead UE shall apply existing FR1 test cases designed for 10MHz CBW with 15kHz SCS.

# Conclusion

In this contribution, we analysed the RRM specification impact when introducing 7 MHz channel bandwidth and made following observations and proposals:

Observation 1: No impact to PDCCH/CORESET reception due to 7MHz CBW.

Observation 2: RMCs defined in current specification support flexible channel BW.

Observation 3: OCNG patterns used in current RRM specification supports flexible channel BW.

Observation 4: No impact to the side conditions tables due to the new band supporting 7MHz channel bandwidth.

Observation 5: NR bands n5 and n29 support both 7MHz CBW and 10MHz CBW.

Proposal 1: No need to introduce any new RRM test cases or test configurations to support 7MHz CBW, instead UE shall apply existing FR1 test cases designed for 10MHz CBW with 15kHz SCS.

# References

- RP-243323, New WID on 7 MHz Channel Bandwidth for n26 and n5, China Telecom, T-Mobile USA

- TS 38.133 V.19.0.0

- R4-2505138, Draft running CR to TS 38.104 – Introduction of 7 MHz channel BW, Ericsson