3GPP TSG-RAN WG4 Meeting #115	R4-2507158

St. Julian’s, Malta, 19 May – 23 May 2025

Agenda item:			7.1.3

Source:	Ericsson

Title:	Discussions on RRM performance requirements of Rel-19 WI on UE RF enhancements

Document for:	Discussions

# Introduction

The performance requirements of introducing 6Rx UE was discussed at previous RAN4#114-bis meeting and following agreements were reached [1]:

Issue 2-1: Antenna connection configuration for 6 Rx capable UE Agreement: New antenna connection configurations are to be defined for 6 Rx capable UE, taking the antenna connection configuration of existing 8RX capable UE as reference. Issue 2-2: Whether a new clause of antenna connection for 6 Rx capable UEs is to be introduced Agreement: A new clause of antenna connection for 6 Rx capable UEs is to be introduced Issue 2-3: SNR level for RLM and BFD/CBD testing for 6Rx capable UE Agreement: RLM and BFD testing for 6Rx capable UEs reuse SNR levels specified for 4Rx capable UE.

In this contribution we provide our view on the new antenna connection based on the agreement above.

# Discussions

At last meeting it was agreed to introduce a new clause for antenna connection configuration for 6Rx UE based on existing 8Rx antenna configuration which is defined in clause A.3.6.1.2.2.1 of TS 38.133, see below:

##### A.3.6.1.2.2 Principle of testing A.3.6.1.2.2.1 Single carrier tests For 8RX capable UEs supporting at least one band where 2RX is supported and either 4RX or 8RX is not supported, all single carrier tests specified in clauses A.4 and A.6 except those in clauses A.4.7 and A.6.7 shall be tested on any band where 2RX is supported and 8RX is not supported with the antenna connection specified in clause A.3.6.1.2.2.4. For single carrier tests specified in clauses A.4.7 or A.6.7, all tests shall be tested with the antenna connection specified in clause A.3.6.1.2.2.4 for bands where 2RX is supported and 8RX is not supported, and the antenna connection specified in clause A.3.6.1.2.2.6 for bands where 8RX is supported. For 8RX capable UEs supporting at least one band where 4RX is supported and 8RX is not supported but without supporting any band where only 2RX is supported, all single carrier tests specified in clauses A.4 and A.6 except those in clauses A.4.7 and A.6.7 shall be tested on any band where 4RX is supported and 8RX is not supported with the antenna connection specified in clause A.3.6.1.2.2.5. For single carrier tests specified in clauses A.4.7 or A.6.7, all tests shall be tested with the antenna connection specified in clause A.3.6.1.2.2.5 for bands where 4RX is supported and 8RX is not supported, and the antenna connection specified in clause A.3.6.1.2.2.6 for bands where 8RX is supported. For radio link monitoring tests, the SNR levels are modified according to table A.3.6.1.1.2.1-1 and table A.3.6.1.1.2.1-2. For beam failure detection and link recovery tests, the SNR levels are modified according to table A.3.6.1.1.2.1-3. For 8RX capable UEs which do not support any band where 8RX is not supported, all tests specified in clauses A.4 and A.6 shall be tested using the antenna connection specified in clause A.3.6.1.2.2.6. For radio link monitoring tests, the SNR levels are modified according to table A.3.6.1.1.2.1-1 and table A.3.6.1.1.2.1-2. For beam failure detection and link recovery tests, the SNR levels are modified according to table A.3.6.1.1.2.1-3.

Following the same principle, we take the existing principle of testing and adapt it to 6Rx as follows:

#### A.3.6.1.x Antenna connection for 6 Rx capable UEs ##### A.3.6.1.x.1 Introduction All tests in clause A.4 and A.6 are specified for UEs supporting 2RX. In this clause, the antenna connection method for applying 2RX tests or 4RX tests to UEs supporting 6RX antenna ports is specified. No tests are currently specified in clause A.4 or A.6 which are applicable only to 6RX antenna ports, so 6RX capable UEs are always tested by reusing tests which were originally specified for 2RX UEs or 4Rx UEs. ##### A.3.6.1.X.2 Principle of testing A.3.6.1.X.2.1 Single carrier tests For 6RX capable UEs supporting at least one band where 2RX is supported and either 4RX or 6RX is not supported, all single carrier tests specified in clauses A.4 and A.6 except those in clauses A.4.7 and A.6.7 shall be tested on any band where 2RX is supported and 6RX is not supported with the antenna connection specified in clause A.3.6.1.2.2.4. For single carrier tests specified in clauses A.4.7 or A.6.7, all tests shall be tested with the antenna connection specified in clause A.3.6.1.2.2.4 for bands where 2RX is supported and 6RX is not supported, and the antenna connection specified in clause A.3.6.1.2.2.5A tfor bands where 6RX is supported. For 6RX capable UEs supporting at least one band where 4RX is supported and 6RX is not supported but without supporting any band where only 2RX is supported, all single carrier tests specified in clauses A.4 and A.6 except those in clauses A.4.7 and A.6.7 shall be tested on any band where 4RX is supported and 6RX is not supported with the antenna connection specified in clause A.3.6.1.2.2.5. For single carrier tests specified in clauses A.4.7 or A.6.7, all tests shall be tested with the antenna connection specified in clause A.3.6.1.2.2.5 for bands where 4RX is supported and 8RX is not supported, and the antenna connection specified in clause A.3.6.1.2.2.5A for bands where 6RX is supported. For radio link monitoring tests, the SNR levels are modified according to table A.3.6.1.1.2.1-1 and table A.3.6.1.1.2.1-2. For beam failure detection and link recovery tests, the SNR levels are modified according to table A.3.6.1.1.2.1-3. For 6RX capable UEs which do not support any band where 6RX is not supported, all tests specified in clauses A.4 and A.6 shall be tested using the antenna connection specified in clause A.3.6.1.2.2.5A . For radio link monitoring tests, the SNR levels are modified according to table A.3.6.1.1.2.1-1 and table A.3.6.1.1.2.1-2. For beam failure detection and link recovery tests, the SNR levels are modified according to table A.3.6.1.1.2.1-3.

In addition, antenna configuration for bands where 6Rx is supported can be defined following similar principles as 8Rx as follows:

A.3.6.1.2.2.6 Antenna connection for bands where 8RX is supported For bands where 6RX is supported, all 6 RX antennas are connected with data source from system simulator. The system simulator shall provide independent noise and fading (low correlation) for each antenna port. Except for the modifications to radio link monitoring thresholds described in clause A.3.6.1.1.2.1, no test parameters or requirements are modified.

In addition, RAN4 may need to modify the existing antenna connection for bands for 2Rx and 4 Rx as follows:

A.3.6.1.2.2.4 Antenna connection for bands where 2RX is supported For bands where 2RX is supported and either 4RX, 6Rx or 8RX is not supported, it is left to the UE declaration and antenna port configuration to decide which 2 of the 6 Rx or 8 Rx ports are connected with data source from system simulator. The remaining RX ports shall be connected with zero input. No test parameters or requirements are modified. A.3.6.1.2.2.5 Antenna connection for bands where 4RX is supported For bands where 4RX is supported and 6Rx or 8RX is not supported, it is left to the UE declaration and antenna port configuration to decide which 4 of the 6 Rx or 8 Rx ports are connected with data source from system simulator. The remaining RX ports shall be connected with zero input. Except for the modifications to radio link monitoring thresholds and beam failure detection thresholds described in clause A.3.6.1.1.2.1, no test parameters or requirements are modified.

- New antenna configurations as shown above are to be defined for 6 Rx capable UEs.

- RAN4 to modify existing antenna connection for bands for 2Rx and 4Rx as shown above.

In addition, RAN4 may need to define separate antenna connection for bands where 6Rx is supported as follows:

A.3.6.1.2.2.5A Antenna connection for bands where 6RX is supported For bands where 6RX is supported, all 6 RX antennas are connected with data source from system simulator. The system simulator shall provide independent noise and fading (low correlation) for each antenna port. Except for the modifications to radio link monitoring thresholds described in clause A.3.6.1.1.2.1, no test parameters or requirements are modified.

- RAN4 to define new antenna connection for bands where 6Rx is supported.

# Conclusion

In the previous sections, we made the following observations:

- New antenna configurations as shown above are to be defined for 6 Rx capable UEs.

- RAN4 to modify existing antenna connection for bands for 2Rx and 4Rx as shown above.

- RAN4 to define new antenna connection for bands where 6Rx is supported.

# References

- R4-2504892, WF on RRM requirements for NR_ENDC_RF_Ph4, Huawei, HiSilicon