3GPP TSG-RAN WG4 Meeting # 115	R4-2507167

# Malta, MT, 19th – 23rd May, 2025

Title:	Discussion on NR-NTN HPUE Tx requirements

Source:	Samsung

Agenda item:	7.10.3.2.1

Document for:	Approval

1	Introduction

In last RAN4 #114bis meeting, the WF[1] had further progressed in RF requirements for NR-NTN PC2/1.5/1. And at the same time, the work split was agreed in [1].

In this paper, we will focus on the remaining open issues for Tx requirements.

2	Tx requirements

MPR for PC1:

In [1], the MPR for PC2 and PC1.5 were agreed together with the ACLR of these power classes. And the PC1 ACLR also reached a tentative agreement as [34] dBc after a long debate in #114bis meeting.

Issue 1-7-2: PC1 ACLR Tentative agreement: - PC1 ACLR is [34] dBc - Companies to further evaluate this, and to be confirmed in next meeting.

Also, a general principle was agreed, which is to reuse existing Tx requirements as much as possible.

Issue 1-4: General Agreement: - As a baseline approach, existing Tx requirements from the TN specifications are re-used for the high-power NTN UE.

In our last contribution [2], we proposed to re-use the existing PC1 MPR from TN specification TS 38.101-1, which was under the assumption of re-use TN PC1 ACLR as 37dBc. Given the ACLR was then agreed tentatively to be relaxed to 34dBc, the MPR potentially could be relaxed.

We targeted to carry out a MPR analysis for PC1 with updated ACLR as 34dBc, but at the time of submission, we don’t have the solid results at hand. From our previous observations in [2], we believe the MPR table would be slightly relaxed, but not much from the TN PC1 MPR table. And from such observation, we would suggest to start with TN PC1 MPR table and further check with our own and other companies simulation results if update is needed.

Observation 1: For NR-NTN PC1 MPR, to use the TS 38.101-1 Table 6.2.2-4b as starting point, and to further check if update is needed with agreed [34]dBc ACLR.

Table 6.2.2-4b: Maximum power reduction (MPR) for power class 1 for bands other than Band n14 Modulation; MPR (dB). Edge RB allocations; Outer RB allocations; Inner RB allocations. DFT-s-OFDM; Pi/2 BPSK w/ Rel-15 DMRS; (NOTE 1); ≤ 0.5; 0. Pi/2 BPSK w Pi/2 BPSK DMRS; (NOTE 1); 0; 0. QPSK; (NOTE 1); ≤ 1; 0. 16 QAM; (NOTE 1); ≤ 2; ≤ 1. 64 QAM; (NOTE 1); ≤ 2.5. 256 QAM; (NOTE 1); ≤ 4.5. CP-OFDM; QPSK; (NOTE 1); ≤ 3; ≤ 1.5. 16 QAM; (NOTE 1); ≤ 3; ≤ 2. 64 QAM; (NOTE 1); ≤ 3.5. 256 QAM; (NOTE 1); ≤ 6.5. NOTE 1: MPR for all modulations for Edge RB allocation is defined as following for two distinguished channel bandwidths groups as: Within the <50MHz channel bandwidth group: MPR= CEIL(7.2 dB- 6 dB∙ CBW100 MHz, 0.5 dB) Within the ≥50MHz channel bandwidth group: MPR= CEIL5.35 dB+3.15 dB∙ CBW100 MHz, 0.5 dB where CEIL(x,0.5 dB) means rounding x upwards to the closest multiple of 0.5 dB..

ACLR for PC1:

In our last contribution [2], we had expressed our analysis over the ACLR for PC1 considering both the co-ex study outcomes and RF feasibilities. In the end, the meeting had agreed in [1] with a tentative agreement of [34]dBc as PC1 ACLR.

As the co-ex study was agreed to focus in Scenario 4, which actually investigated the impacts from NTN UL to TN UL, i.e. the interference in adjacent channel from NTN HPUE to TN BS in this case. Considering the ACS of TN BS assumed as 46dBc, the resulting ACIR is 33.7 dBc.

| NTN HPUE ACLR | BS ACS | Resulting ACIR |
| --- | --- | --- |
| 34 | 46 | 33.7 |

At least from our study results, even with isolation distance as 0km, for PC1, our results indicate a required ACIR as 30.8dBc should be sufficient to protect the TN BS uplink.

Observation 2: From our study results, even without isolation distance, the 34dBc NTN HPUE ACLR provided a ACIR as 33.7 dBc, which is sufficient for adjacent channel interference protection purpose.

Proposal 1: To confirm the PC1 ACLR as 34dBc as a single value.

Configured transmitted power:

In last meeting WF[1] and [3], the power boosting, duty-cycle based power class back-off were discussed, but not reached consensus so far. And in the current TS 38.101-5, the configured transmitted power of UE simply refers to the TS 38.101-1, which includes many components that not yet supported by NTN.

The wording of TS 38.101-5 in this section is shown below:

6.2.4 Configured transmitted power The requirements for configured transmitted power defined in subclause 6.2.4 of 3GPP TS 38.101-1 [5] clause 6.2.4 shall apply to NTN satellite UE.

And the equations in section 6.2.4 of TS 38.101-1 is shown below:

PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c with PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) } PCMAX_H,f,c = MIN {PEMAX,c, PPowerClass – ΔPPowerClass + ΔPPowerBoost}

To simply re-use the existing wording can be a simple way, however it will create ambiguities that the NTN UE already supports power boost, power class fall-back, etc. Moreover, some components like P_powerclass, MPR and A-MPR should also mapped to the 38.101-5 other than 38.101-1.

Observation 3: The configured tx power in TS 38.101-5 simply refer to TS 38.101-1 would create ambiguities and confusion that NTN UE has agreed or been introduced with same features as TN UE, especially considering at least the power boosting, CA, power class back off, etc., were actually not yet supported by NTN UE.

Observation 4: The components, like MPR, AMPR, P_powerclass, etc., should be refer to the sub-clauses in TS 38.101-5 other than TS 38.101-1. This was not clear in the existing specification text.

To be crystal clear of what was expected from the NTN UE, including HPUE, in this release of the specifications, we think it’s better to at least remove the un-relevant parts of the equation as in TS 38.101-1 for now as a starting point, and if some similar or new component can be agreed, it can be added to the equation later.

Proposal 2: To capture the similar equations from TS 38.101-1 to TS 38.101-5, other than simply referring to TS 38.101-1. And,

- To remove the ∆TC,c, ΔPPowerClass, ΔPPowerBoost, ∆TIB,c from the equation;

- To clarify the PPowerClass, MPRc and A-MPRc refers to the corresponding section in TS 38.101-5.

3	Conclusion

In this contribution, we provide views on NR-NTN HPUE Tx requirements:

Observation 1: For NR-NTN PC1 MPR, to use the TS 38.101-1 Table 6.2.2-4b as starting point, and to further check if update is needed with agreed [34]dBc ACLR.

Observation 2: From our study results, even without isolation distance, the 34dBc NTN HPUE ACLR provided a ACIR as 33.7 dBc, which is sufficient for adjacent channel interference protection purpose.

Proposal 1: To confirm the PC1 ACLR as 34dBc as a single value.

Observation 3: The configured tx power in TS 38.101-5 simply refer to TS 38.101-1 would create ambiguities and confusion that NTN UE has agreed or been introduced with same features as TN UE, especially considering at least the power boosting, CA, power class back off, etc., were actually not yet supported by NTN UE.

Observation 4: The components, like MPR, AMPR, P_powerclass, etc., should be refer to the sub-clauses in TS 38.101-5 other than TS 38.101-1. This was not clear in the existing specification text.

Proposal 2: To capture the similar equations from TS 38.101-1 to TS 38.101-5, other than simply referring to TS 38.101-1. And,

- To remove the ∆TC,c, ΔPPowerClass, ΔPPowerBoost, ∆TIB,c from the equation;

- To clarify the PPowerClass, MPRc and A-MPRc refers to the corresponding section in TS 38.101-5.

4	Reference

[1] R4-2505113, WF on HPUE RF requirements for NR-NTN, Samsung

[2] R4-2504333, Discussion on NR-NTN HPUE Tx requirements, Samsung

[3] R4-2505114, WF on HPUE RF requirement for IoT-NTN, vivo