3GPP TSG-RAN WG4 Meeting # 115	R4-2507169

# Malta, MT, 19th – 23rd May, 2025

Title:	Discussion on Ku-band coex study

Source:	Samsung

Agenda item:	7.11.3

Document for:	Approval

1	Introduction

In last RAN4 #114bis meeting, the WF[1] had further achieved progress in co-existence scenarios and assumptions.

In this paper, we will provide analysis for phased-array antenna with agreed parameters in [1], and propose the way forward for further progress the co-ex work.

2	Discussion

Phased array parameters:

The phased array antenna parameters agreed in [1] were listed below:

1; VSAT Antenna Characteristics. 1.1; Antenna pattern; TR 38.921, TR 38.922; TR 38.921, TR 38.922; Circular Aperture TR 38.863. 1.2; Element gain (dBi) (Note 1); 3.5; 3.5; N/A. 1.3; Y axis / Z axis element 3 dB beam width of single element (degree); 90° for Y axis 90° for Z axis; 90° for Y axis 90° for Z axis; N/A. 1.4; Y axis / Z axis element front-to-back ratio (dB); 30 dB; 30 dB; N/A. 1.5; Antenna polarization (Note 5); Circular (RHCP or LHCP); Circular (RHCP or LHCP); Circular (RHCP or LHCP). 1.6; Antenna array configuration (Y axis × Z axis)(Note 3); 45x45 elements; 78x78 elements; 60 cm diameter aperture. 1.7; Number of supported polarizations, P; 1; 1; 1. 1.8; Y axis / Z axis radiating element spacing; 0.5 lambda; 0.5 lambda; N/A. 1.9; Array Ohmic loss (dB) (Note 1); 2; 2; N/A. 1.10; Conducted power (before Ohmic loss) per antenna element (dBm) (Note 2); 6.9; 9.1; N/A. 1.11; Tx Transmit power; 10 Watts for GEO and for LEO; 50 Watt for GEO; 10 Watts for GEO and for LEO. 1.12; Maximum coverage angle in the horizontal plane (degrees); 0~360 degrees; 0~360 degrees; 0~360 degrees. 1.13; Vertical coverage range (degrees) between beam direction and normal direction; 0~60 degrees; 0~60 degrees; 0~60 degrees. 1.14; Normal direction; Toward X+ axis; Toward X+ axis; Toward X+ axis. 1.15; Max EIRP Value; 76.6 dBm; 88.2 dBm; 76.6 dBm. 1.16; Noise Figure; 6 dB; 6.0 dB for GEO; 2.5 dB for both LEO and GEO. Note 1: The element gain in row 1.2 includes the loss given in row 1.9. Note 2: The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element). Note 3: All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane. Note 4: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration (Y axis × Z axis). Note 5: Antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization evaluations are conducted by converting to linear polarization.

45x45 phased array antenna

45x45 Phased array antenna pattern:

Based on the AAS antenna modelling and the parameters above, and by assuming the NTN VSAT panel is placed with its normal direction pointing to 90-degree in elevation. The antenna gain of 45x45 phased array parameters are depicted in the figures below.

Figure 1. The antenna pattern of a 45x45 phased array with its normal direction to 90-degree in elevation

(Left) Electrical steering = 0-degree; (Right) Electrical steering = 55-degree.

The maximum gain comes from the 3.5 dBi per element gain, with total 45x45 elements, which sums up to the maximum gain to 3.5+10*log10(45*45) = 36.6 dBi.

45x45 Phased array e.i.r.p.:

Then by considering the conducted power 6.9dBm per element, with the total 45x45x1 elements and 2dB ohmic loss, the total input power sums up to 6.9+10*log10(45x45x1)-2 = 38.0 dBm.

Thus, the maximum e.i.r.p. is around 36.6+38.0 = 74.6 dBm.

Observation 1: The above calculation has slightly difference to the ‘Max EIRP value’ agreed in [1], my calculation shows max eirp is around 74.6 dBm, while the agreed Max EIRP value in [1] is 76.6 dBm, there’s 2dB difference which equals to the ohmic loss.

Proposal 1: Discuss and if agreed, to update the ‘Max EIRP value’ for 45x45 phased array antenna in [1] to 74.6 dBm by applying ohmic loss, which seems was not applied.

By adding the power to the antenna gain in all elevation and azimuth, we get the figure below as the eirp distribution for 45x45 phased array antenna, when it’s normal direction to the panel is pointing 90-degree in elevation.

Figure 2. The e.i.r.p. distribution of a 45x45 phased array with its normal direction to 90-degree in elevation

(Left) Electrical steering = 0-degree; (Right) Electrical steering = 55-degree.

Observation 2: The max e.i.r.p. will drop around 5dB when purely utilizing electrical beam steering with a 55-degree in elevation domain.

45x45 Phased array off-axis e.i.r.p.:

Theoretically, electrical beam steering would worsen the off-axis e.i.r.p. as the side lobe will increase. This issue was raised in last meeting #114bis discussion, but not with any numerical analysis. Hence, we provided the direct comparison below.

In the figures below, we assumed the NTN VSAT is transmitting with the minimum channel bandwidth to be specified in Ku-band NTN, i.e. 5MHz from [2].

Sub-topic 2-3: UE channel bandwidth per operating band Table 5.3.5-1: Channel bandwidths for each NTN satellite band in FR1-NTN NTN satellite band; SCS kHz; UE Channel bandwidth (MHz). 5; 10; 15; 20; 25; 30 (NOTE); 35; 50; 70; 100. 15; 5; 10; 15; 20. n248; 15; 10; 15; 25; 35; 50. 30; 10; 20; 25; 35; 50; 70; 100. n247; 15; 10; 15; 25; 35; 50. 30; 10; 20; 25; 35; 50; 70; 100. NOTE: Deployment of 30 MHz channel bandwidth for NTN SAN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements.. Table 5.3.5-2: Channel bandwidths for each NTN satellite band in FR2-NTN SAN Operating Band; SCS (kHz); SAN channel bandwidth (MHz). 50; 100; 200; 400. n509; 120; 50; 100; 200; 400. n508; 120; 50; 100; 200; 400.

And for comparison purpose, we choose the CEPT off-axis EIRP requirements agreed in [3] and shown as the ‘off-axis limit’ in the figures below.

Issue 1-2-3: Off-axis EIRP requirements for the CEPT Ku band Agreement: - Adopt the following off-axis EIRP requirements for the CEPT Ku band n248 and n509 - For co-polarized transmissions θ value; Maximum Off-axis EIRP (dBm); Measurement bandwidth (kHz). 2.5° ≤ θ ≤ 7°; 63 – 25log10(θ) – K; 40. 7° ≤ θ ≤ 9.2°; 42 – K; 40. 9.2° ≤ θ ≤ 48°; 66 – 25log10(θ) – K; 40. 48° < θ; 24 – K; 40. NOTE 1: K=10log(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one mobile VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.3.2 in EN 301 428. The manufacturer shall declare the value of N. NOTE 2: For θ> 70° the values given above may be increased to 4 - K dBW over the range of angles for which the particular feed system may give rise to relatively high levels of spillover.. - For cross-polarized transmissions θ value; Maximum Off-axis EIRP (dBm); Measurement bandwidth (kHz). 2.5° ≤ θ ≤ 7°; 53 – 25log10(θ) – K; 40. 7° ≤ θ ≤ 9.2°; 32 – K; 40. NOTE: K=10log(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one mobile VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.3.2 in EN 301 428. The manufacturer shall declare the value of N..

The 5MHz transmitted e.i.r.p. were down-scaled to 40kHz, same as the measurement bandwidth of CEPT off-axis e.i.r.p. limit, for direct comparison in the figure below.

Figure 3. 45x45 phased array: compliance to CEPT off-axis e.i.r.p limits in meas. BW 40kHz

(Left) Electrical steering = 0-degree; (Right) Electrical steering = 55-degree.

It should be noted that the above figure only shows one 2-D slice in Azimuth = 0-degree, and only show the electrical steering in elevation domain with 55 degrees. The compliance to the off-axis e.i.r.p., to our understanding, should be examined in 3D domain of the antenna emissions.

Observation 3: The 45x45 phased array, when operating in 5MHz as minimum supported chbw in this release, when electrically steered to 35 degrees elevation angle probably don’t need any power reduction for compliance to off-axis eirp limits to CEPT.

Observation 4: The compliance of this 45x45 phased array antenna to FCC and other regional/country-wide off-axis eirp limits need further study.

78x78 phased array antenna

In similar approaches as we did for 45x45 phased array antenna above, the results for 78x78 phased array antenna are shown below.

78x78 Phased array antenna pattern:

Figure 4. The antenna pattern of a 78x78 phased array with its normal direction to 90-degree in elevation

(Left) Electrical steering = 0-degree; (Right) Electrical steering = 55-degree.

The maximum gain comes from the 3.5 dBi per element gain, with total 78x78 elements, which sums up to the maximum gain to 3.5+10*log10(78*78) = 41.3 dBi.

78x78 Phased array e.i.r.p.:

Then by considering the conducted power 9.1dBm per element, with the total 78x78x1 elements and 2dB ohmic loss, the total input power sums up to 9.1+10*log10(78x78x1)-2 = 44.9 dBm.

Thus, the maximum e.i.r.p. is around 41.3+44.9 = 86.2 dBm.

Observation 5: The above calculation has slightly difference to the ‘Max EIRP value’ agreed in [1], my calculation shows max eirp is around 86.2 dBm, while the agreed Max EIRP value in [1] is 88.2 dBm, there’s 2dB difference which equals to the ohmic loss.

Proposal 2: Discuss and if agreed, to update the ‘Max EIRP value’ for 78x78 phased array antenna in [1] to 86.2 dBm by applying ohmic loss, which seems was not applied.

By adding the power to the antenna gain in all elevation and azimuth, we get the figure below as the eirp distribution for 78x78 phased array antenna, when it’s normal direction to the panel is pointing 90-degree in elevation.

Figure 5. The e.i.r.p. distribution of a 78x78 phased array with its normal direction to 90-degree in elevation

(Left) Electrical steering = 0-degree; (Right) Electrical steering = 55-degree.

Observation 6: The max e.i.r.p. will drop around 5dB when purely utilizing electrical beam steering with a 55-degree in elevation domain, similar as the results from 45x45 antenna.

78x78 Phased array off-axis e.i.r.p.:

Figure 6. 78x78 phased array: compliance to CEPT off-axis e.i.r.p limits in meas. BW 40kHz

(Left) Electrical steering = 0-degree; (Right) Electrical steering = 55-degree.

It can be observed from the above figures, that the off-axis e.i.r.p. from a 78x78x1 phased array with parameters agreed in [1] will exceed the CEPT off-axis e.i.r.p. limits as given in [3]. And such exceedance even occurs when there’s no electrical steering, as depicted in the left of Figure 6 above.

Observation 7: The NTN VSAT with 78x78x1 phased array parameters agreed in [1] would result in exceedance to the CEPT off-axis eirp limit given in [3], even without any electrical beam steering, when transmitting in 5MHz channel bandwidth.

Even though the co-existence studies would focus on 100MHz channel bandwidth, where the larger bandwidth would dramatically reduce the e.i.r.p. in 40kHz measurement bandwidth, we believe it’s worthy to bring this observation to the meeting.

Proposal 3: To have the proponent of this 78x78x1 phased array antenna to double check if the parameters needs any update and/or if they target on larger channel bandwidth than 5MHz, considering our Observation 7.

For co-existence study

Based on our observations above, the issues regarding off-axis eirp compliance majorly happens in small transmission bandwidth. The wider transmission bandwidth would result in lower power spectral density, and it will actually help the measurement results in each of the measurement bandwidth, theoretically.

The bandwidth documented in [4] reflects that the co-existence study would focus on 100MHz channel bandwidth, which we agreed as the worst case from adjacent channel interference perspective. And with 100MHz channel bandwidth, even it was not covered by our figures and observations above, it probably does not have any issue regarding the off-axis eirp limits from either CEPT or FCC or any other.

Proposal 4: No need to consider the power adjustment in co-existence study assuming 100MHz, and the UE RF requirement should reflect the reduction may be required for certain cases at least similar to Ka-band VSAT.

In offline discussion, one company also shared their figures of antenna patterns with and without beam steering, and the differences between us is around 2dB at peak gain direction when antenna steered. Given the phased array is not really considered in previous NTN studies, and these parameters are newly proposed. We believe a short calibration phase before collecting co-ex results can be helpful.

Observation 8: Given the phased array is not really considered in previous NTN studies, and these parameters are newly proposed. We believe a short calibration phase before collecting co-ex results can be helpful.

3	Conclusion

In this contribution, we provide views on co-ex study for Ku bands NTN:

Observation 1: The above calculation has slightly difference to the ‘Max EIRP value’ agreed in [1], my calculation shows max eirp is around 74.6 dBm, while the agreed Max EIRP value in [1] is 76.6 dBm, there’s 2dB difference which equals to the ohmic loss.

Proposal 1: Discuss and if agreed, to update the ‘Max EIRP value’ for 45x45 phased array antenna in [1] to 74.6 dBm by applying ohmic loss, which seems was not applied.

Observation 2: The max e.i.r.p. will drop around 5dB when purely utilizing electrical beam steering with a 55-degree in elevation domain.

Observation 3: The 45x45 phased array, when operating in 5MHz as minimum supported chbw in this release, when electrically steered to 35 degrees elevation angle probably don’t need any power reduction for compliance to off-axis eirp limits to CEPT.

Observation 4: The compliance of this 45x45 phased array antenna to FCC and other regional/country-wide off-axis eirp limits need further study.

Observation 5: The above calculation has slightly difference to the ‘Max EIRP value’ agreed in [1], my calculation shows max eirp is around 86.2 dBm, while the agreed Max EIRP value in [1] is 88.2 dBm, there’s 2dB difference which equals to the ohmic loss.

Proposal 2: Discuss and if agreed, to update the ‘Max EIRP value’ for 78x78 phased array antenna in [1] to 86.2 dBm by applying ohmic loss, which seems was not applied.

Observation 6: The max e.i.r.p. will drop around 5dB when purely utilizing electrical beam steering with a 55-degree in elevation domain, similar as the results from 45x45 antenna.

Observation 7: The NTN VSAT with 78x78x1 phased array parameters agreed in [1] would result in exceedance to the CEPT off-axis eirp limit given in [3], even without any electrical beam steering, when transmitting in 5MHz channel bandwidth.

Proposal 3: To have the proponent of this 78x78x1 phased array antenna to double check if the parameters needs any update and/or if they target on larger channel bandwidth than 5MHz, considering our Observation 7.

Proposal 4: No need to consider the power adjustment in co-existence study assuming 100MHz, and the UE RF requirement should reflect the reduction may be required for certain cases at least similar to Ka-band VSAT.

Observation 8: Given the phased array is not really considered in previous NTN studies, and these parameters are newly proposed. We believe a short calibration phase before collecting co-ex results can be helpful.

4	Reference

[1] R4-2504723, Way Forward for  [114bis][313] NR_NTN_Ku_Band_Coexistence, Moderator: Intelsat

[2] R4-2504761, Way Forward for [114bis][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group)

[3] R4-2504720, Way Forward for [114bis][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL)

[4] R4-2502315, Co-existence simulation assumptions for NR NTN Ku band (Running document), Ericsson